THE DOS+ RELOCATOR SYSTEM - INSTRUCTION MANUAL

==============================================

Copyright (c) 1980, 1986 by C.B. Falconer.

Released to the public domain for non-profit use only.

INTRODUCTION

============

This  system  allows  the user to construct his own "MOVDOS+"  or  "MOVCPM"

program,  containing  his own BIOS and BOOTSTRAP (instead of that  for  the

Intel  MDS  on  the original MOVCPM program or Kaypro 83  on  the  original

MOVDOS+ program).  After construction the final system will perform identi-

cally  to the original Digital Research supplied MOVCPM program,  with  the

following additions:

1:     As stated, the users BIOS and BOOTSTRAP are present.

2:     The  system can relocate DOS+ or CPM on 256  byte  bound-

aries. This is specified by adding "/n", where n is 1, 2,

or  3,  to the normal size specifier.  This will relocate

the  system  that number of 256 byte  pages  higher  than

normal.

3:     Should  the user make his own patches to the DOS+ or  CPM

systems,  they can be incorporated in the DOSRELOC  prog-

ram,  and  that modified system can then be relocated  as

desired.

NOTE:

The  CP/M system proper is not supplied with this system.

It  is  up  to the user to ensure that he  uses  a  valid

system,   with   the  appropriate  serial  numbers   etc.

embedded.  CP/M is the trademark of DIGITAL RESEARCH, who

can  supply the system.   DOSRELOC will copy  all  serial

numbers etc correctly,  if installed as listed below,  so

that all Digital Research subsystems (e.g.  PIP,  SYSGEN,

etc.) will function properly.   DOS+ and CCP+ is supplied

with the system, and executes on Z80 cpus only.

NOTE (for MOVCPM generation):

If you already have a suitable MOVCPM for your system, it

is not necessary to create all  the modules  below.   The

existing bios and boot portions are probably satisfactory

images.  Such images are probably to be found 080h offset

from the locations needed for this system.  Check it.

OPERATION

=========

1:     IF  YOUR SYSTEM IS NOT A Kaypro 2/4 (83) edit the  source

for   DOSRELOC   (the   initial  set   of   customization

equates) and re-assemble to create DOSRELOC.HEX.

2:     Generate  the  appropriate  CBIOS00.HEX  and  SBOOT00.HEX

files,  as outlined in the appendix below.  Make sure you

have   backup   copies  of  everything  in  case   of   a

malfunction.  Make  sure you can boot the system up  from

the  one  disc  you are using.  This disc  will  need  to

contain  copies  of  the  system,  DDTZ.COM,  MOVCPM.COM,

CPMRELOC.HEX, SBOOT00.HEX and CBIOS00.HEX.

3:     THIS  STEP only needed if you wish to generate a  MOVCPM.

DOS+ systems are supplied.

With DDTZ, load the existing "MOVCPM" program by

A>DDTZ MOVCPM.COM

and  move  the existing CCP/BDOS (as supplied by  Digital

Research) image down by 080h bytes by:

-MA00,1FFF,980

For clarity, clean out the junk by:

-F100,97f,0

Now install the assembled DOSRELOC by:

-IDOSRELOC.HEX

-R

3a:    Insert your own BIOS, prepared as below, by

-ICBIOS00.HEX

-R980

and your own bootstrap, again prepared as below, by

-ISBOOT00.HEX

-Rvalue        (see notes for value)

(Usually 900 for boot at 0,)

exit with control-c (^C) and save the result by

A>SAVE 34 DOSRELOC.COM

(or ^ other number for your system)

4:     Run the relocator generator by

A>DOSRELOC

the response will be

Now "SAVE 34 MOVDOS+.COM"        DO IT

A>SAVE 34 MOVDOS+.COM

CAUTION

DOSRELOC  cannot be run under DDT etc., since it uses the

actual  CCP code to generate relocation  information.  In

addition  the  system disc,  when DOSRELOC is  run,  must

contain  the proper booter for the running system AND the

identicaal system must be running.

5:     Read  Digital Research's manual on the use of MOVCPM  and

treat MOVCPMY in exactly the same manner,  EXCEPT that to

generate  an immediately executing system the  2nd  para-

meter must be "G" (rather than defaulting to execution) .

Unless you modify your bootstrap or your bios you can now

generate any size DOS+ or CPM system in about 2  seconds,

including  immediately executing systems.  The output  of

MOVDOS+ is suitable for inserting new BIOS's, bootstraps,

etc.,  just  as  was the original MOVCPM output.  If  you

generate a system by

A>MOVDOS+ 32             {for example}

you may follow this immediately by a

A>SYSGEN

and  input the CR to skip system loading.   I.E  when  in

doubt, follow Digital Research's instructions.

CAUTION

If an immediate execution system is generated, it will be

unable to reboot itself properly, because the disk proper

contains  the original system.   However,  until a reboot

occurs everything will be normal.   Remember     that any

program that does a "JMP 0", or a control-c, will usually

reboot.  DDT and DDTZ always exits with a reboot.

APPENDIX

========

To  use this system the user MUST have an operational DOS+ or CP/M  system,

of  sufficient size to load MOVCPM with DDT.  For CP/M 2.2 this requires  a

17K minimum system,  although when completed the system will run in a 16/1K

system.  In addition the user should have the source code for his BIOS  and

BOOTER.  The  BIOS  must be assembled for a DOS+ or CPM that originates  at

zero,  i.e. as if CCP begins at address zero, and, for CP/M 2.2, as if BDOS

begins at address 0800H.  In this case the BIOS would be assembled as if it

began at address 01600H.  The booter image will vary,  depending on how  it

operates.  The point is to place a booter,  assembled as if it would load a

DOS+  or CPM that began at zero,  at addresses 900H through 97FH in step  3

above.  Remember that the booter will normally operate in a fixed  location

for  all size systems,  but will load the system into different  addresses,

and transfer control accordingly.

CAUTION

Remember that the CBIOS and SBOOT loaded in step 3 above  MUST

be  identical  to those in the executing  system,  except  for

execution  addresses.   If you can't change just one value  in

the source code to generate either,  this system will probably

not work.  If you don't have the source code for these modules

complain viciously to your supplier.

If  patches or changes are made to the DOS+ or CPM  systems,  corresponding

patches   should  be  made  in  the  zero  based  image  contained  in  the

DOSRELOC.COM module generated in step 3.  The result should relocate satis-

factorily, provided that no unclean practices (such as self-modifying code)

have  been used.  Again,  if the CBIOS uses self-modifying code the  system

probably  will  not function,  and you should complain to  the  knucklehead

(possibly yourself) who wrote such code.