Re:  IMP4MDS.LBR                  Ilbert Butler                 04/04/88

This library contains in crunched form:

IMP4MDS.DOC     You are reading it.

I2MDA-55.ASM    Current Morrow MD computer overlay for IMP245

IMPMD3R1.COM    IMP for Rev.1 Morrow MD2s & MD3s, ROM Rev. 1.x

IMPMD3R2.COM    IMP for Rev.2 Morrow MD2s & MD3s, ROM Rev. 2.x-3.x

IMPMDHDA.COM    IMP for Morrow hard disk CP/M 3.0 computers,

using Auxiliary (JPC) Serial Port (switchable)

NO ONE (!) SHOULD UNCRUNCH AND ATTEMPT TO RUN ANY (!) IMP PROGRAM

IN THIS LIBRARY WITHOUT READING AND FOLLOWING THESE INSTRUCTIONS!

=================================================================

There are THREE VERSIONS of IMP245 in this library. All are set for 1200

baud.  Your attempt to  run THE WRONG  VERSION FOR YOUR SYSTEM could re-

sult, not only in failure of the program to work, due to failure  to in-

stall it properly for your system, but also:

RUNNING AN IMP PROGRAM WITHOUT INSTRUCTIONS

COULD SERIOUSLY DAMAGE SOME SYSTEMS

Morrow hard  disks may  become  trashed  by software  intended  only for

floppy systems. While this is virtually impossible, use a little caution

in determining you selected the appropriate version from the above list.

Morrow hard disks and floppy  computers use different serial controllers

and different commands to address them.

========================================================================

The three following programs are ALMOST READY TO RUN, BUT NOT QUITE! You

must first run SETUP.COM (SETBAUD on  Rev.1 systems) and verify which of

three common null modem cables/adapters you have, and then  INSTALL with

IMPATCH.COM (or  reassemble)  for  1) modem type, 2) maximum modem speed

and 3) cable/null adapter  type.  There  are THREE ESSENTIAL SETTINGS TO

MAKE BEFORE YOU SHOULD RUN THEM!

IMPMDHDA.COM   IMP for Morrow hard disk using AUXiliary port.

You can reset within the program to use the

Printer/Modem port.

IMPMD3R1.COM   IMP for Morrow floppy disk computers with Rev.

1 boards & ROM 1.x, incl. Rev. 1 MD2s & MD3s.

IMPMD3R2.COM   IMP for Morrow floppy disk computers with Rev.

2 boards & ROM 2.x or 3.x (mainly MD3s).

CHANGES TO DEFAULT SETTINGS

Defaults are:       8 data bits, 1 stop bit, no parity

(to change these you MUST reassemble)

IF ANY OF THESE     1) Hayes-compatible  modem  (not ProModem or Racal)

3 SETTINGS DO       2) 1200 baud highest speed

NOT MATCH, YOU      3) NODTR equate is off--meaning you must have

MUST CHANGE WITH       a correct null adapter permitting you to

IMPATCH OR  REAS-      send a DTR signal from computer pin 6 to

SEMBLE. YOU  CAN'T     modem pin 20, which signal IMP turns off to

CHANGE WHILE  RUN-     hang up the phone--change NODTR to on, f

NING THE PROGRAM       if you don't have the correct cable/adapter.

(optional to        intercharacter delay 4 ms

change these)       end of line delay 400 ms

ABOUT SETUP.COM (or SETBAUD.COM)

For IMP to work, you must run  SETUP or SETBAUD, and set the serial port

you will use to connect the modem to the same baud rate as  your highest

modem speed.  You  must also set that port to `Hardware Handshake on Pin

20'.  Please remember you must not only set these settings for immediate

use, BUT ALSO save them to any  disk  from which you will be running IMP

and reboot from that disk before running IMP.

ABOUT THE LIMITATIONS OF THE IMP `SET' COMMAND & NEEDING IMPATCH

What's this about running IMPATCH?  You thought you could set your modem

speed from within IMP?  The answer is,  "Yes in some respects, but a re-

sounding No in others."  The SET command within  IMP  actually  sets the

speed of communication without a modem, for hard cable transfers between

computers in the same location. It can also be used to lower speed below

the modem's  highest  portential.  However, the highest speed your modem

can run at is a separate  setting,  because it is a combination of make,

model, and initialization instructions, all in one setting.  So, if your

version is set for a Hayes 1200, you cannot  run  an  Avatex,  Hayes, or

other 2400  baud  modem, because the program-to-modem initialization in-

structions are different.

0103h  05  MSPEED;  DB   5     ; 5 for 1200 baud 6 for 2400 baud

0104   00  HS2400:  DB   NO    ; No=00, Yes=FF for highest speed

0105   FF  HS1200:  DB   YES   ; No=00, Yes=FF for highest speed

0106   00  RACAL:   DB   NO    ; Yes for Racal-Vadic result code

0107   00  PROMODM: DB   NO    ; Yes for Prometheus 1200b modems

You can change any of those in  a few seconds with EDFILE, PATCH18, SID,

etc.  Or you can edit the overlay and then reassemble.  The other method

is to run  IMPATCH, a program not by Irv Hoff, the author of IMP and not

approved, tested or monitored by him.  Please don't report a bug in IMP,

based on failure of IMPATCH to work the same as reassembly.

TEST BEFORE REPORTING BUGS:

ONLY IMPMDHDA has been  thoroughly  tested & debugged (using ZSID).  Re-

liable test reports from those using IMPMD3R1 &  IMPMD3R2  will  be wel-

comed.  However, this  is  not  a  tutorial for computer users, who have

never tried out a public domain modem program on their own before.  This

is an invitation to `veteran' computer users to report anomalies, diffi-

culties, etc.  Please do not report  a  bug, without first observing the

following MINIMUM TEST PROCEDURES:

1.   Load IMP with caplocks on, the modem on ready for use,

with ALL dipswitches forcing lines turned OFF, and the

correct null adapter.  DTR indicator light on the mo-

dem should be ON both before and after IMP loads.  Re-

member that, if you change dipswitches with the modem

turned on, you may have to turn it off and back on to

get it to read its own settings.  Or enter terminal

mode with capslock on and type ATZ<cr>.  When the mo-

dem resonds `Ok', it has also read its settings.

2.   DSC command in COMMAND mode should briefly turn DTR

off and SD send light on for a fraction of a second,

then reverse the process AND send a message that line

has been disconnected.

3.   ESC-N in TERMINAL mode should do the same thing AND

exit from TERMINAL to COMMAND mode.  Back in TERMINAL

mode, ESC-Q should turn send light on briefly, WITHOUT

turning off DTR, without disconnecting or exiting

TERMINAL mode, AND(!) ESC-N should work as before, es-

pecially AFTER ESC-Q has been run.

4.   Back in COMMAND mode, BYE should act like DSC, PLUS

exiting the program to the system prompt, and DTR

light, although briefly off, upon exit should be back

ON (return to normal with no program control).

THE THREE CORRECT MORROW NULL MODEM CABLES/ADAPTERS:

Of course for this test to work at all, you need either a port correctly

reconfigured for straight-through connection  to  a  modem, or a correct

null adapter.  In this regard, please be advised:

A.   If you rejumper, the jumper diagrams in the manual may be a

mirror image (or the view from the bottom of the board,

looking through it with X-ray vision), and NOT A VISUAL

REPRESENTATION of the jumpering.  To check this, check both

pin numbers and Row A & Row B pin locations of your board

vs. the jumpering diagrams.

B.   There are only three correct null adapters for Morrow

serial ports with factory default settings.  All others are

a hazard, especially the one with only pins 2 & 3 reversed

and all other 23 pins straight through.  The ones sold in

stores as null adapters are usually for PCs and similarly

ported computers (Atari, Amiga, Tandy laptops, etc.)  Even

if they work, they may be a test of your MD serial port's

ability to withstand short circuits (connecting 2 outputs).

The ONLY three correct & completely safe adapters are:

(Pins not shown are not connected.  In both diagrams, com-

puter pins are on left, modem pins on right.)

MITE manual             Morrow Q&A              Minimum Type

===========             ===========            ==============

1 ----- 1 <-optional->  1 ----- 1 <-optional-> 1 ------ 1

2 ----- 3               2 ----- 3              2 ------ 3

3 ----- 2               3 ----- 2              3 ------ 2

4 ----- 5               4 ----- 5              7 ------ 7

5 ----- 4               5 ----- 4

6 ---- 20               6  ---- 20         (This  type does not

20 ----- 8              20 ----- 6          connect DTR,  so you

7 ----- 7               7 ----- 7          need to set NODTR: to

YES at 011Eh, by using

set NODTR: NO           set NODTR: NO        IMPATCH.COM, SID or by

just reassembling.

ok all programs          all but MITE

may need  input signal

(Both of these work with NODTR set to `No'.)    on MD pin 4 = CTS in.)

- end -

#######################