PRIMER

on

FILE TRANSFER METHODS

by

Terry Smythe

Sysop, Z-Node 40

INTRODUCTION

From time to time, many of us have had a need to transfer data from our

own micro computer to another micro computer.  The 2 computers might be

close together, or they may be across town or across the nation.  It's

when they are not close together that problems occur because the

telephone system becomes the link between the two computers, rather

than a short piece of shielded wire.

This "voice" telephone network is not "clean", and is a major source of

aggravation and heartburn as it far too often adds extraneous

characters commonly referred to as "line noise", to legitimate data as

it moves through the wire.  The net effect is a document, file,

program, etc., that is often made virtually unusable by the involuntary

insertion of these additional characters.

The problem has its origin with the historical development, around the

turn of the century, of our nationwide telephone network.  Basically,

solid copper connected telephones across town, across the nation,

across the continent, and it had many mechanically actuated switches

scattered along the way.  These switches and many line connections

eventually wear out, provoking a progressive deterioration of network

efficiency.

Many of these switches and much of the wire passes near other

electrical equipment throwing off magnetic fields which adds to

unreliability because of the principle of induced current.  A magnetic

field need only be brought near copper wire to "induce" a foreign

current in it, in addition to legitimate data also passing through it

at the same time.

Until the late '40's, early 50's, there was no real concern because the

network carried only "voice" signals.  The combination of the human

ear, coupled to an discerning brain, had the wonderful natural ability

to filter intelligent conversation.  What remained was simply thought

of as "static", and largely ignored.

CURRENT SITUATION

These "voice" telephone circuits are still a major highway over which a

huge amount of computerized data now flows.  Unfortunately, computers

still lack the smarts inherent within a typical human brain, and have

great difficulty sorting out the good from the bad in what is passing

through the wire.  The ability to ignore the junk along the way is

still an elusive goal.

True, networks have been made much more reliable in recent years, with

the development and implementation of microwave networks, fibre optics,

electronic switches, etc.  Each in their own way contributing in some

significant way to reducing the amount of "noise" passing through the

network.

Today, the network is no longer solid copper wire.  It is more likely a

mix of solid copper wire, microwave transmission, and fibre optics.

However, even in a contemporary system there are still elements of the

old copper network with its increasingly unreliable mechanical

switches, deteriorating connections, etc.

The bottom line is that network quality is really a function of the

weakest link between 2 points.  If your computer is connected into a

computerized electronic switch you will enjoy relatively clean data

transmission if you are connected to another computer which itself is

connected into a computer switch, possibly the same one in the

telephone company's local switching centre.

However, if one of the computers is still hooked into an older

mechanically switched centre, significant line noise is inevitable.

Curiously, it can make itself known at one end, and not the other;

where one user can be looking at an incredibly scrambled screen

display, while the other user is looking at a spotless screen display,

wondering what's going on.

The net result here is a high probability that when data is sent

through the wire in an uncontrolled single trip fashion, it will pick

up line noise somewhere along the way.  In the case of conventional

ASCII data (plain, readable text), the presence of a few extraneous

characters is of no great consequence, relatively easy to detect, and

fix with your favorite word processor or full screen editor.

However, in the case of computer programs, the presence of a single

extraneous character can make it totally unuseable.  Detection is

almost impossible, certainly for the neophyte, and patching the fix is

not a trivial task.  A considerable array of computer smarts is needed

here, something that most users neither have nor want.

ERROR DETECTION

In the eyes (brain?) of the computer, characters introduced by line

noise are every bit as legitimate as live data passing through

simultaneously.  It would take extraordinary programming talent to

develop a communications system capable of sorting out the good from

the bad as it streams by.  The task is mind boggling, such that it

simply is not done.

However, error detection methods have been developed, are in place now,

and are quite effective and reliable.  Fundamentally, they are all

based upon the perception that if errors have been introduced enroute,

then the file as received will be different from the file as

transmitted.  They do not really care what the specific error is, only

that an error of some kind has been detected.

Most of these error detection methods are based upon some simple

arithmetic formulae applied to the same file at both ends, and the

results compared.  If the file as received has the same result as the

file when sent, then it's reasonable to assume that the file has been

transferred correctly.  If different, then the file must be sent

through again, and again, until it does come through the wire clean and

correct.

Doing such a calculation on an entire file is very inefficient. You

really should not have to find out after an hour's transmission that

errors have crept in.  At this rate, it could take days to send a large

file through the wire accurately.

The simple fix is to break up the file into small blocks, typically 128

bytes long.  This way, only those blocks where an error has been

detected, need be re-transmitted again.  So, a file of say 1500 blocks

might take about an hour to transmit cleanly.  Even on a noisy line, a

maximum number of bad blocks likely would not exceed 30-50.  In this

way, that portion of the file needing to be re-transmitted is reduced

to a manageable level.

ERROR DETECTION METHODS

In August 1977, Ward Christensen, a pioneer in data communications,

developed a method of file transmission with simultaneous error

detection.  He simply called it MODEM2 (Release 2.0), but very quickly

it became affectionately known as the "Christensen" protocol.

In its simplest form, this original, somewhat primitive error detection

scheme added up the values of all characters in the 128 byte data

stream, and sent this value through the wire.  The receiver meanwhile

was adding up the values of the characters as they arrived, and

compared the result with the "CHECKSUM" value sent through by the

sender.

If these 2 numbers did not agree, the receiver sent through a code

telling the sender to repeat the transmission of that bad block.  This

process was repeated, if necessary, up to 10 times for a particular bad

block.  Only when the 2 numbers were identical, did the receiver send

through a code acknowledging correct block received.  The sender would

then move on to the next block of 128 characters, repeating the process

all over again.

This early method of error detection was deliberately made super-

simple, so that it could apply to a whole host of different machines,

under an almost infinite array of data transmission conditions.

However, because of its simplicity, it did let a few technically

obscure errors sneak through.  Consequently, Ward Christensen and Chuck

Forsberg collaborated in the development, and release in 1982, of the

CRC (Cylic Redundancy Checking) error detection scheme which has

remained in widespread use to this day.

Because it guarantees a minimum level of error detection confidence of

not less than 99.9969%, CRC is accepted as a reliable method of

ensuring clean and accurate file transfer.  Most systems of file

transfer now employ CRC, or a derivative of it, as their principal

method of error detection.  Please note this is error detection, not

error correction, a function still best left to human intelligence.

Uncertain how or when, but this protocol became universally known as

XMODEM.  The original CheckSum method was never abandoned, and to

distinguish between them, they are universally known as:

XMODEM     - CheckSum protocol

XMODEM CRC - CRC protocol

Where the CheckSum method simply added up the values of the characters

in a 128 character block, the CRC method does sequential division on

each character in the block, resulting in a significant improvement in

error detection.  Looks something like this:

Discard Quotient

_____________________________________

Constant )    Character

Constant x Quotient

--------------------------------

Remainder + next character

Constant x Quotient

---------------------------------

Remainder + next character

etc

Constant x Quotient

----------------------------------

Remainder       <-- CRC Value

Note: Constant = (X+1)(X15+X+1)

When there are no more characters for sequential division, the final

remainder is the CRC value sent through by the sender.  The receiver

applies the same calculation to the incoming characters, and compares

the results with the incoming CRC value.   If equal, the block is

acknowledged and the next block is allowed to come through.  Inequality

would require re-transmission of the block, to a maximum of 10 times.

If still unequal after 10 tries, the transmission will be automatically

terminated.

ENHANCED FILE TRANSFER METHODS

With normal equipment upgrades, such as microwave and fibre optics,

telephone companies around the world have progressively improved their

abilities to transfer data more reliably over voice grade lines.  As

line quality improves, line "noise" decreases, and data files may be

successfully transferred with fewer "hits".   In fact, it is

commonplace today to experience file transfers with no "hits" at all.

This improvement in data transmission capability provoked a realization

that the 128 character block size had become inefficient because of its

associated overhead.  Furthermore, new methods of data transmission,

such as DATAPAC, resulted in dramatically inefficient use of the

telephone network. (e.g. a DATAPAC "packet" capable of carrying 1024

characters was carrying only 128 characters!)

To overcome this inefficiency, Chuck Forsberg developed the YMODEM

protocol, where the block size was increased to 1024 characters.  In

it, he inserted a rather nifty feature where the protocol would

automatically step down to 128 character block size if line noise got

so bad as to degrade elapsed file transmission time.  This auto step-

down has been universally adopted at 3 consecutive "hits" (bad blocks).

The YMODEM protocol has only a modest improvement in elapsed file

transmission time over the conventional voice network.  However, it

provided a dramatic improvement on the DATAPAC network by simply using

the packet size more efficiently.

Not satisfied with this improvement, Chuck Forsberg continued with his

development activities and came up with YMODEM BATCH.  This allowed

rapid transmission of a group of files sequentially, to reduce the

overhead associated with keyboard entries to set up the communications

programs at both ends with the transfer of each file.

While YMODEM is referred to as a protocol, it really is a "method" of

file transfer.  The CRC protocol is still in use at its heart, no

matter if in 128 or 1024 character block size.

Ever vigilant to technological developments, Chuck continued to

perceive opportunities for further improvements and has recently

developed and released to public domain a new file transfer protocol

which he calls ZMODEM.  It is a new, sophisticated protocol aimed at

efficient file transfer with time sharing systems, satellite relays,

and wide area packet switching networks.

ZMODEM will work only if both ends support this new protocol, but it

has built into it a fall-back routine whereby it will automatically

fall-back to YMODEM protocol, if ZMODEM is not supported at the other

end.   It uses a "streaming" technique whereby data is flowing

continuously, with simultaneous error detection in a moving window of

up to 256 characters, depending on line quality, using the capabilities

of the full duplex network.

This is an oversimplified description of ZMODEM.  It is quite

sophisticated, complex to learn and use, and not yet in widespread use.

No attempt will be made here to describe this in anything other than

this crude overview.  Those interested otherwise are encouraged to read

Chuck Forsberg's paper on his ZMODEM protocol (ZMODEM.DOC).

There are other protocols, some somewhat obscure, some very complex,

and some proprietory.  For example, KERMIT, MNP, BLAST, BISYNC, SDLC,

HDLC, X.25, X.PC, etc., which, with perhaps the exception of Kermit,

are not in widespread use, tend to be tightly bound to the fortunes of

their suppliers, and which the average users will not likely encounter.

Suffice to note their presence so those interested may do additional

research.

BAUD RATES

While not normally a function of file transfer methods, it does seem

appropriate to briefly consider the speed at which data flows through

the telephone wire.  BAUD is simply an international unit of

measurement that has become synonymous with BPS (Bits Per Second).  The

latter has come into popular useage, and tends to be a much more

meaningful term.

Most users will encounter modem/computer/communications system

configurations using baud rates of 300 BPS, 1200 BPS, and 2400 BPS.

Lower or higher baud rates are still extremely rare.  By a huge margin,

the most popular is 1200 Bits Per Second, and is the one most

frequently recommended, and at modest cost.  300 baud configurations

should be avoided for they deliver data through the wire at painfully

slow speeds.  In fact, 300 baud becomes cost prohibitive if employed

over long distances.

By way of comparison of how long it may take to transfer a file over

the wire at various baud rates, consider the following example of a

typical file taking 24 minutes to pass through the wire at 300 baud:

File transferred at 300 baud  - 24 minutes

Same file at 1200 baud        -  6 minutes

Same file at 2400 baud        -  3 minutes

Modems capable of transferring files at baud rates higher than 2400 are

available, but they are complex, expensive, and typically require the

identical modem at both ends, because of the absence to date of

consistent universal standards of methods of file transfer at 4800 and

9600 baud.  These standards will ultimately emerge, but for the

present, most users will likely choose to stay with proven techniques

at baud rates of 2400 or 1200.

COMMUNICATIONS TOOLS

There is no shortage of software out there to achieve reliable data

communications, using these protocols.  It ranges from costly dedicated

utilities, such as for AES equipment, to low cost generic systems

placed into the world of "ShareWare" software.  A few of the more

prominent of these ShareWare products are:

PROCOMM v. 2.42 - Excellent,  supports all protocols

but ZMODEM

QMODEM v 2.4    - Very good, supports most protocols.

ZCOMM v 2.0     - Excellent,  but complex.  Supports

all protocols discussed here.

These are good, and they are cheap.  As with most ShareWare software

products, prices in the $40 - $60 (U.S.) range are commonplace.  There

are others, too many to discuss here.  See them out, do your homework,

choose that which suits you best.

SUMMARY

Most users will be presented with the following optional methods of

transfering files from one micro-computer to another:

1. ASCII

Straight one way trip of data without any form of error detection in

place.  Highly vulnerable to data corruction by normal line noise

adding extraneous characters to the file.

2. XMODEM

Very early method of file transfer, using primitive CheckSum

protocol, at a fixed 128 character block size.  Risk of a few

obscure errors slipping through.

3. XMODEM CRC

Reliable method of file transfer using the CRC protocol at a fixed

128 character block size.  Not very efficient, but highly compatible

with most communications systems.

4. YMODEM

Reliable method of file transfer using the CRC protocol at both 128

and 1024 character size blocks.  Reasonably efficient, and

reasonably compatible with many communications systems.

5. YMODEM BATCH

Reliable method of file transfer using the CRC protocol at both 128

and 1024 character size blocks, with an added option of sending a

number of files in Batch mode.  Quite efficient, and marginally

compatible with a few communications systems.

6. ZMODEM

Sophisticated, reliable, and efficient method of file transfer,

using a modified CRC protocol of up to 256 character block size with

auto step-down in accordance with line quality.  Marginally

compatibility with very few communications systems.  Currently

rarely found.

TYPICAL APPLICATIONS

1. Generic public domain software may be reliably transferred between 2

computers having incompatible disk formats.  If the 2 computers are

together in the same room, they may simply be connected together

through their serial ports, then 2 compatible communications systems

may then facilitate file transfer between the 2 at very high speeds

(baud rates).  Over distance, the same can be achieved with modems

at both ends, and "talking" over the voice telephone network at much

lower baud rates.  The CRC protocol virtually guarantees accurate

transfer.

2. Text files, saved in ASCII form, may be transferred over the

telephone wires to most any location.  Typical application  might be

the content of a magazine article, or a book, where the author

finalizes content as to language and spelling, etc., and transmits

it to a printer.  While faithfully preserving content, the printer

sets it up as to publication format, type style, etc., and can go

directly to press.  A "proof" copy is not sent back to the author

for proofreading.  The CRC  protocol virtually guarantees error free

transfer, where ASCII would be a disaster.

3. Business data, such as accounting, inventory, sales, etc.,  may be

reliably transferred, using the CRC protocol from a remote site, to

a central computer for consolidated processing.  Also possible to

set up this kind of file transfer as an automatic interrogation

during the middle of the night when rates are lowest.

CONCLUSIONS

There was a time (not so long ago) where it was considered quite

inapproriate to use a computer to send files through the voice

telephone network.  Between an absence of standards on file transfer

protocols and line noise, received files were rendered almost totally

useless.

However, this is no longer true.  Reliable file transfer protocols are

now in place, and files may now be transferred between micro-computers

with a high degree of reliability.

While this may mean reduced revenues for some industries, in particular

the publishing industry where transcriptions (and related revenues) can

now be a thing of the past, business and industry can look forward to

substantial improvements in staff productivity and significant

reductions in publishing costs, by the application of the "type-it-

once" principle.

In these tight money times, now is the time for business and industry

to be creative in their use of micro-computers and data communications

capability.

Were it not for the vision and foresight of Ward Christensen and Chuck

Forsberg, and others like them, these wonderful tools would be denied

to us, and companion benefits unattainable.   What they have done is

indeed very much appreciated today.  They are to be commended for their

achievements.

Terry Smythe

Sysop, Z-Node 40

55 Rowand Ave

Winnipeg, Manitoba

Canada   R3J 2N6

(204) 832-3982 (Voice-res)

(204) 832-4593 (Z-Node)

(204) 945-6713

15 Apr 87

####################################################################################