Application Notes

for

Mercury 1200

(a.k.a. FHR 1200)

Intelligent Modem

and

NUBYE or IMP

by

Roger Warren

(San Diego, CA.)

Introduction

The  MERCURY 1200 is a cheapie HAYES clone which is supplied  with

minimal documentation.  The information supplied here is the results

of  some experimentation in the application of the MERCURY,  and  is

intended  to help those who are trying to use this modem with  NUBYE

or IMP.

Hardware Notes

1)	VERY  IMPORTANT!!  The nomenclature on the case of some of the

MERCURY modems is INCORRECT,  and NOT ALL are supplied with an

errata sheet!   From the rear of the device,  the  connections

and switches ARE (from left to right): POWER, LINE, PHONE, RS-

TTL  switch,  DCE-DTE  switch.   The incorrect  case  markings

transpose the RS-TTL and DCE-DTE switch!

2)	SW4- Internal DIP switch:

Position 2 is called HCD and is documented as follows:

In  the  OFF  position  (default)  Carrier  Detect  operates

independent  of the DTR input.   In the ON position  Carrier

detect is held off until DTR is presented.

Experimentation has yielded the following information:

With  position 2 in the OFF (default) position.   DTR has NO

effect  on MODEM operation.   The modem escape sequence  (if

online) followed by the ATH command MUST be used to hang  up

the  phone!  With position 2 in the ON position.  DTR can be

used  to hang up the phone and hold  off  answer.   However,

with  this  setting  the  modem  will  dial,  but  will  not

originate a carrier under IMP.

The author has made the following mod to his modem:

SW4  position 2 is left open and a separate switch has  been

run  external  to the case and attached in parallel  to  DIP

switch.   I've labelled this ANS/ORIG.   In the OPEN  (ORIG)

position  the modem will ORIGINATE under IMP,  but will  not

use  the DTR signal.  In the CLOSED (ANS) position the modem

can be used under NUBYE using the DTR signal to hang up  the

phone.

Of  course,  you can always leave the case off and change  the

switch MANUALLY!

REMEMBER  to  turn modem power OFF when  changing  any  switch

setting!!

IMP installation

To originate under IMP,  the switch discussed above should  be

OPEN  (in  the ORIG mode).   DTR will have no effect on  modem

operation. Under IMP244 , use the NODTR feature to inform IMP.

In previous versions,  you'll have to code in your own  online

ESCAPE and HANGUP command in the IMP overlay.

NUBYE installation

Use under NUBYE is not so straightforward.

USE the ANS mode (SW4 position 2 CLOSED) if possible!!!

USE the following NUBYE EQUATE settings:

IMODEM	equ	YES	; intelligent modem

IMAT	equ	YES	; MODEM uses protocol

ECHO	equ	NO	; MERCURY does not echo ESC chars,if used

ANCHOR	equ	YES	; MERCURY ACCEPTS the extended commands,

; but IGNORES them.  THE ANCHOR subset is

; sufficient.  ALSO, when answering at

; 300 BAUD it returns the resutl code at

; at 300 (as does the ANCHOR)

HS1200	equ	YES	;

NODTR	equ	NO	; use the DTR line to hang up

NOATA	equ	NO	; MERCURY USES ATA

SHORTB	equ	xx	; doesn't matter, use NO.

NOTE:	If  you  MUST use NODTR equ YES then you'll have to  edit  the

NUBYE  source code.   The combination of EQUATES ANCHOR =  YES

and  NODTR = YES causes NUBYE to initialize the modem with  an

escape character of 128d (80h).  While the MERCURY ACCEPTS the

setting, sending three bytes of 80h (preceeded and followed by

a 1 second pause) does not cause an online escape.  Change the

code  to eliminate the S2=128 initialization  command!   NUBYE

will use the +++ escape sequence. SET SW4 position 2 to OPEN!

Special NOTE:

Setting  ANCHOR=YES  and  NODTR=YES  will  currently  cause  a

combination  where  the  modem  (not  just  the  MERCURY)   is

programmed as having the escape character as 80h,  while NUBYE

issues an ascii +++.

########################################################################################################################