12/28/86

by Steven Hirsch

Files:

NEW2E80.DVR

PCPISCRD.DRV

Purpose:

This library contains files and information necessary to

implement the 'advanced' features of BGii on the PCPI AppliCard

CP/M system.

Bridger Mitchell's BackGrounder ii is a minor miracle! Hopefully

many folks have already tried the demo version (available on fine

BBS systems everywhere), and maybe even purchased the full-blown

version. Like many others, I was dismayed to find that the only

screen drivers supplied were for Heath terminals and certain

Kaypro machines.

After a weekend of headbanging, I have generated a new version of

the PCPI FAST2E80.DVR file. This updated driver supports the

'report cursor position', and the 'transmit screen' function. For

lack of imagination, I used the Heath/Zenith conventions:

ESC, 'n' ===> Report cursor position. A string is sent (via

console input) which consists of :

ESC, '=', Y+32, X+32

ESC, '#' ===> Transmit screen. The entire screen (1920 chars) is

sent via conin. A <cr> is appended at the end. Inverse characters

have their hi-bit set.

Unfortunately, the source code for the driver is proprietary, and

though it is floating around on BBS's, I will not assist in it's

propogation.

It was also necessary to hack up Bridger's H19SCRDV file, to

produce PCPISCRD.DRV. Again, the source is copyrighted. I will

forward my modified source to Plu-Perfect, and they will likely

include it in future accessory disk releases.

A minor patch to the PCPI Z80 bios is necessary, in order to

permit the return of 8-bit data from the screen. Just do this:

ddt PCPICPM

(blah, blah)

-s1A36

E6  00

7F  00

xx  .

-g0

A>save 54 NEWCPM (or whatever..)

Now, generate a boot disk using NEW2E80.DVR instead of

FAST2E80.DVR (yes, Martha, you must have a IIe!!), and give

INSTALL the name of your patched system file. Test this disk to

make sure that it is OK, and see if programs using direct cursor

addressing (WordStar, etc.) perform correctly.

If all goes well, install the PCPISCRD.DRV file into LOADBG.COM

using the SETBG.COM utility. The directions are fairly clear on

this. Fire up BGii and enjoy cut, paste, flip, swap, and all of

the other marvelous tricks!

You will note a spurious cursor appearing in the upper left of

the screen on every other 'suspend' operation. This is being

worked on as you are reading this, and should be solved in a

future release. It is **not** in the file, and does not appear on

screen dumps!

Please leave messages for me on Lilliput (#2), Z-CENTRAL, and/or

Downspout if any problems are encountered.#