Addenda to PUBLIC Article in DDJ

11/18/84

Since the publication of the article, a minor bug in the PUBLIC.ASM

file has been drawn to our attention.  This causes it to refuse

to make files larger than 512K PUBlic.  The correction is noted

in the source code contained on this disk.

To use the relocater contained on this disk, simply run the following:-

RELPUBLC<CR>

and you will be prompted to give the address of the start of your BIOS.

You can find this address simply by running DDT and inspecting

location zero.

e.g.

DDT<CR>

d0,F<CR>

the address formed by location 0002 and 0001 points to the warrm

boot jump which is at BIOS+3.  The number you should enter to the relocator

is location 0002 followed by two zeros. (i.e. if you get the

following display from DDT

0000 C3 03 EA .. .. .. .. etc

then you would enter

EA00

to the relocator.)

The relocator will produce a file called PUBPATCH.HEX which is a standard

Intel format hex file which can be merged with your copy of CP/M

in the manner described in the article.  Note that numbers larger

than 0FFFFH wrap,  so for the above system with the BIOS at

0EA00H the offset would be 3580H.

CCPBASE_MEMORY	= 0EA00H - 01600H = 0D400H

CCPBASE_IMAGE	= 00980H

0D400H+03580H	= 0980H	(after wrapping)