ZCPR-2 on the KAYPRO 10

=======================

----------

REVISION NOTE:

This file has been revised since its original release.  The major

changes  are to accomodate the three (!) different  operating  systems

that  have been released for the Kayrpo 10 and provide two options  of

CPR systems for general use and SYSOP's of RCPM systems.

----------

The  accompanying files allow installation of Rich Conn's     replace#

ment  console  command  processor on the Kaypro  10.   This  processor

replacement, and its rich collection of utilities, greatly enhance the

utility  and  flexibility of this hard disk machine.   It is  in  this

environment that ZCPR-2 really shines!

The  Kaypro 10 has limited graphics capability by defining each  char#

acter  block into 8 segments.   These 8 segments are turned on by  the

appropriate characters in the 128 - 255 character range,  i.e.,  those

with the eighth bit set.   Since some programs (and some ZCPR-2 utili#

ties)  depend  on this eighth bit being set,  a method was  needed  to

prevent  this graphics character being displayed when  standard  ascii

text should be displayed.

This has been accomplished by the simple expedient of adding a mask in

the  CONOUT routine of the BIOS which tests a specific byte in  memory

and,  if it is set,  allows graphic character display; if it is clear,

the  eighth  bit is masked off and the standard character set is  dis#

played.  This byte, located at 04FH, is set by a command that has been

added to the ZCPR2 CPR.   This command, "GRAF", is used like any other

resident command, such as, REN or ERA.  It toggles the graphic byte at

04FH.   This  is indicated on the display by the CPR prompt  ">".   In

ZCPR-2 installations, this prompt character has the eighth bit set so,

when the graphics mode is enabled, a block character is displayed next

to  the drive/user number.   Merely type "GRAF <cr>" and the  standard

cpr prompt ">" appears.

The  Kaypro 10 has an open block of memory between 0F100 and 0F600 and

this space is used for the ZCPR-2 buffers.   This allows  installation

without  further shrinking the available TPA.   A full featured ZCPR-2

installation  is provided with the sole exception of  IO  redirection.

(My assembly language programming isn't good enough for this yet.)

There  are  three versions of the CPM operating system that  has  been

released  by Kaypro for the 10.   These may be identified by the  cold

boot sign on message.  In the remainder of this document, I will refer

to  them  as  versions 0,  D and F.   The sign  on  messages  and  the

corresponding versions are as follows:

.pa

Sign on Message                    Version

---------------                    -------

KAYPRO 10 CP/M Version 2.2            0

KAYPRO 10 CP/M Version 2.2D           D

KAYPRO 10 CP/M Version 2.2F           F

The  following  description  of the two  "systems"  describe  the

options that may be generated using the appropriate BIOSPAT.ASM  along

with the Z2STD.HEX/Z2SEC.HEX files.   The source code modifications to

ZCPR2.ASM  and ZCPRHDR.LIB are described in the accompanying Z2PAT.DOC

file.  The only files you need to supply are your distribution copy of

PUTSYS.COM and DDT.COM.

PUTSTD.COM

----------

This  file is used in place of PUTSYS.COM,  as supplied with  the

Kaypro.  This version of ZCPR2 is a more or less standard version with

resident DIR, TYPE, LIST commands.  Because it is a full featured CCP,

external  buffers are used for the CCP stack as well as the  CCP  file

control   block.    This  version  is  intended  as  a  "general  use"

implementation.

PUTSEC.COM

----------

This version is also used in place of PUTSYS.COM but is  intended

for  use in situations requiring security such as RCPM  systems.   The

DIR,  LIST  and  TYPE commands have been deleted to allow  replacement

with  limited  transient programs to accomplish  the  same  functiion.

Additionally,  a  patch has been added to ZCPR2 CCP to allow  limiting

the maximum disk and user when the WHEEL byte is reset.  All remaining

ZCPR2 resident commands,  except the DU: command, have been made wheel

oriented commands.   Thus, a SYSOP can have a reasonably secure system

and  be  able  to  open it up for his personal use without  a  lot  of

passworded files.   In fact,  using the limited user area approach, it

is  possible  to  put  up a RCPM system with  NO  .COM  files  in  the

accessible  user  areas  and  yet through a  double  password  system,

totally open up the machine.   Much of this implementation depends  on

the  use of the powerful ZCPR2 utilities such as CD,  MKDIR,  STARTUP,

PATH,  LD  along  with customized versions of RBBS,  XMODEM  and  BYE.

(Note  that  CD MUST be modified to run under BYE with BYE LOW  as  is

done with the Kaypro 10.)

.pa

Buffer Locations

----------------

Buffer Name              PUTSTD.COM          PUTSEC.COM

-----------              ----------          ----------

WHEEL BYTE               003E                003E

External Path            0040-004E           0040-004E

GRAPHICS BYTE            004F                004F

CPR FCB                  F2D0-F2F0           Internal to CPR

Named Directory          F300-F4FF           F300-F4FF

Multicommand line        F500-F5CF           F500-F5CF

CPR Stack                F5D0-F600           Internal to CPR

Files Included

--------------

BIOSPAT0.ASM   Patch file for version 0

BIOSPATD.ASM   Patch file for version D

BIOSPATF.ASM   Patch file for version F

Z2STD.HEX      Overlay file for PUTSTD.COM

Z2SEC.HEX      Overlay file for PUTSEC.COM

Z2PAT.DOC      Description of ZCPR2 CPR Modifications.

NZ2KP10.DOC    This file

.pa

Installation Procedure

----------------------

The  only  files  needed beyond those supplied here  are  DDT.COM  and

PUTSYS.COM.  The latter writes CPM on the hard disk. Before beginning,

I  suggest you edit the PATH:  section of the appropriate  BIOSPAT.ASM

file to suit your needs.   As this one is coded,  the PATH is  current

user, drive A, then Drive A, user 0.  The idea is to overlay CCP and a

CCP  patch  added at the end of the BIOS with the ZCPR-2 CPR  and  the

BIOSPATCH routine.   This is accomplished as follows:  (Lines you type

indicated by <--- )

A0>ASM BIOSPATx.AAZ <cr>        <--- (Insert 0,D or F for x as

CP/M ASSEMBLER VERSION 2.0            appropriate)

nnnn                            <--- (Depends on version: 0 - EC4B

000H USE FACTOR                                           D - ECFE

END OF ASSEMBLY                                           F - ED11)

Warm Boot

A1>DDT PUTSYS.COM <cr>		<---

DDT VERS 2.2

NEXT  PC

2300 0100

IZ2STD.HEX			<--- (or, for PUTSEC, IZ2SEC.HEX)

R3580				<---

NEXT  PC

2300 D400

IBIOSPATx.HEX			<--- (Insert 0, B or D for x as

R3580				<---  appropriate.)

NEXT  PC

2300 CC68                       <--- PC depends on version

G0				<---

Warm Boot

A0>SAVE nn PUTSTD.COM		<--- (or PUTSEC.COM. Note that nn

the  decimal  equivalent  of

the hex number given under

NEXT when DDT is invoked.  Values

for nn are: 0 - 35

D - 34

F - 40)

You  can now use PUTSTD.COM or PUTSEC.COM to rewrite the modified CP/M

on  the system track of the hard disk.   Run it then do a  cold  boot.

You  should get a new sign-on message with the line  "STARTUP?".   The

Multicommand line buffer is initialized to look for and execute a file

called "STARTUP.COM" so you can rename any desired starting program to

get an automatic execution on cold boot.  (If anything goes wrong, put

a floppy disk with CP/M on it, into the drive, do a cold boot, and run

PUTSYS.COM  (not PUTSTD.COM or PUTSEC.COM) and that will get you  back

to go.)

Assuming all is ok to here, you will see a block character next to A0.

Type "GRAF <cr>" and you should see the normal CPR prompt.

The following hex values associated with this ZCPR-2 installation:

Some  excellent  manuals on all the ZCPR2 documentation are  available

from:

New York Amateur Computer Club

P. O. Box 106, Church St. Station

New York, NY  10008

Ask  for Book 6:  ZCPR2 User & Reference Manual and Book  7:  ZCPR2  &

SYSLIB These are $10.00 each and well worth it.

Thanks  to  Rich Conn for his excellent contribution to public  domain

software.   I  hope  I  have  done his effort  justice  by  the  above

installation.   Any  "bug" reports can be left on the Syracuse  Kaypro

Network at 315-468-2887.

John C. Smith

Manlius, NY

December 4, 1983

########################################################################################################################