Sydex

22DISK

A CP/M-to-DOS Diskette Interchange Utility

Sydex

153 North Murphy Ave.

Sunnyvale, CA  94086

(408) 739-4866

22DISK

A CP/M-to-DOS Diskette Interchange Utility

Version 1.30, February, 1989

Copyright 1988, 1989, Sydex

Sydex

153 North Murphy Ave.

Sunnyvale, CA  94086

(408) 739-4866

All Rights Reserved

NOTICE

THIS IS NOT FREE  SOFTWARE!   If you  paid  a "public domain"   vendor

for  this  program,   you paid for the service of copying the program,

and not for the program itself.   Rest assured that nothing ever  gets

to the originators of this product from such a sale.  You may evaluate

this product, but if you make use of it, you must register your copy.

We  offer  several inducements to you for registering.   First of all,

you receive the most up-to-date copy of the program that  we  have  --

and  we  do  update the product on a regular basis.   You also receive

support for 22DISK -- which can be  quite  valuable  at  times.   And

finally,  we  have  a  few  utilities  not  included in the evaluation

package.   Make no mistake,  however - this is a fully functional ver-

sion of 22DISK and not "crippled" in any way.

REGISTRATION INFORMATION

The  non-commercial  single-user registration fee for 22DISK is $25.00

US.  The commercial and multisystem site fee for 22DISK is $100.00.

Send a check or company P.O. for the appropriate amount to:

Sydex

153 North Murphy Ave.

Sunnyvale, CA 94086

Please indicate what product you are ordering and if you  have  a  re-

quirement for 3.5" media.   We normally ship 5.25" 360K diskettes, but

will furnish 3.5"  720K  media  upon  request.   Customers outside  of

the United States and Canada should include an extra $5.00 for airmail.

If you are a registered user of this product  and  desire  an  update,

please  send  $5.00  for handling  and we will ship you an up-to-date

copy.  Please add $10.00 for printed documentation to cover duplication

and shipping costs.

DISTRIBUTION NOTICE

This is "user-supported" software.   You are hereby granted a  license

by Sydex to distribute this evaluation copy of 22DISK and its documen-

tation, subject to the following conditions:

1.   22DISK  may  be  distributed  freely without charge in evaluation

form only.

2.   22DISK may not be sold,  licensed,  or a fee charged for its use.

If a fee is charged in connection with 22DISK,  it must cover the

cost of copying or dissemination only.   Such  charges  must  be

clearly  identified  as such by the originating party.   Under no

circumstances may the purchaser be given the impression  that  he

is buying 22DISK itself.

3.   22DISK  must  be  presented  as  a complete unit,  including this

documentation.   Neither 22DISK  nor  its  documentation  may  be

amended or altered in any way.

4.   By  granting  you  the right to distribute the evaluation form of

22DISK, you do not become the owner of 22DISK in any form.

Any other use,  distribution or representation of 22DISK is  expressly

forbidden without the written consent of Sydex.

_______________________________________________________________

CP/M, CP/M-80 and CP/M 2.2 are trademarks of Digital Research.  MS-DOS

is a trademark of Microsoft.   IBM PC,  PC-XT and PC-AT are trademarks

of International Business Machines.  V20, V30, uPD70108,  uPD70116 are

trademarks  of  NEC  Electronics.   CompatiCard  is  the trademark of

MicroSolutions, Inc.  Some products of Lear-Siegler,  Kaypro,  Osborne

and Televideo are also trademarked.

Table of Contents

22DISK, Briefly..............................................1

Installing 22DISK............................................2

Getting Started..............................................2

Selecting A Disk Format......................................4

22DISK CP/M File Naming Conventions..........................5

22DISK Command Syntax Conventions............................6

A Quick Reference Guide to 22DISK Commands...................7

CTOD -- Convert A CP/M Diskette To DOS.......................9

DTOC -- Convert DOS Files TO CP/M Files.....................10

CTYPE -- Display A CP/M File................................10

CDIR -- Display A CP/M Directory............................11

CERA -- Erase CP/M Files....................................12

CFMT -- Format A CP/M Diskette..............................13

Building Custom Diskette Definitions........................14

Some Hints on Developing a Definition.......................19

Indexing A Large Definition File with GENINDEX..............21

Diskette Densities and Recording Modes......................21

Modifying A PC-XT "Clone" Controller For Single-Density Re..23

In Closing..................................................25

Other Sydex Products........................................26

22DISK, Briefly

22DISK  is  a  product  which enables one to convert files on diskette

from various CP/M formats to MS-DOS,  and vice versa.   It is  a  com-

panion product to 22NICE,  a Z-80/8080 CP/M 2.2 emulator.   The opera-

tion of one part is not dependent on the other--if  you  want  to  use

22DISK  to  convert data from other disk formats to DOS or vice-versa,

you need not install the 22NICE CP/M emulator.  Similarly,  if you al-

ready  have CP/M programs available as DOS files,  you may not want to

use the 22DISK disk conversion utilities.

Our packaging for 22NICE and 22DISK  offers  the  22DISK  program  set

separately  for those who may be using a different emulator or have no

need of a CP/M emulator.   The 22NICE emulation package,  however,  is

bundled with 22DISK because of the need to transport CP/M files to the

PC before emulation can be performed.

The 22DISK disk conversion package offers the following features:

*    "Roll  your  own"  disk  formats  -  we  provide support for

several common CP/M 2.2 diskette formats,  but if you have a

format  that  isn't  in our list,  you can simply define the

characteristics in an  ASCII  parameter  file.   Registered

users get an even larger file of diskette types.

*    Support   for  single-  double-  and  quad-density  formats,

hardware permitting.

*    Support of the the following operations:

CP/M-to-DOS

DOS-to-CP/M

Directory display

Display CP/M files

Erase CP/M files

Format CP/M diskette

*    Full user number support.

*    Support of MicroSolutions' CompatiCard.

22DISK currently will not handle "hard sectored" diskettes,  nor  will

it work with diskettes written on an Apple ][ or Commodore 64 or 128.

Page 1

Installing 22DISK

22DISK will not run without first being installed.   To do this, enter

the following command at the DOS prompt:

A:22DINST            and press the ENTER key.

We're assuming that the file 22DINST.EXE is located on the diskette in

drive A:; if you are installing 22DISK from some other drive or direc-

tory, substitute it for the A: in this command.

Generally, the installation process is self-explanatory.  A few of the

more important points to note are:

1.   22DISK will support up to 8 diskette drives--4 per  control-

ler card. Support for two controller cards is provided.

2.   If  you're using 22DISK on an XT-class computer with a high-

density controller,  you'll need to specify that  your  com-

puter  is an AT-type computer.   This is necessary to ensure

that your controller is correctly programmed by 22DISK.

3.   22DINST installs all files in the same directory.   However,

the  file  CPMDISKS.DEF  and  the files with an extension of

"LBL" may be placed in any directory specified  by  the  DOS

PATH environment variable.  For information on PATH, consult

the DOS reference manual.

4.   The files comprising 22DISK can be installed or re-installed

"in place".   That is, the source drive and directory may be

the same as the destination.

You'll need about 512K of base memory to install 22DISK.  If you don't

have that much available,  you can still install 22DISK by creating an

installation   diskette   having   all  of  the  22DISK  files  except

CPMDISKS.DEF and all of the files having a extension of "LBL".   These

files  can be copied over to your destination drive or directory after

you've run 22DINST.

Getting Started

The following sections deal with various specific features of  22DISK.

If  the  diskette format desired is one of those included in the stan-

dard definition file, entering the command

CMENU

will cause a menu-driven interface to 22DISK to be executed.   The ap-

propriate function may then be selected from the menu.  CMENU displays

the following choices:

Page 2

+------------------------------------------------+

|                                                |

|       22DISK Version 1.30 (12-Mar-89)          |

|                                                |

|  Please select one of the following:           |

|                                                |

|       0.  Exit to DOS                          |

|       1.  Set CP/M diskette type               |

|       2.  Set CP/M diskette drive              |

|       3.  Copy CP/M file(s) to DOS             |

|       4.  Copy DOS file(s) to CP/M             |

|       5.  Format a CP/M diskette               |

|       6.  Display a CP/M directory             |

|       7.  Display (type) CP/M file(s)          |

|       8.  Erase CP/M file(s)                   |

|                                                |

+------------------------------------------------+

No diskette type selected yet.

Please enter function number (0...8):

When CMENU is used for the first time,  choice 1 should be selected to

determine what CP/M diskette format will be used.   Until a format  is

selected,  no  CP/M diskette operations can be performed;  attempts to

perform any operation before selecting a type will  produce  an  error

message.

When choice 1 is selected,  a list of CP/M diskette types,  in display

"pages" of 23 types will be displayed.  The cursor keys may be used to

select a format type.  When the desired type is located,  pressing the

ENTER  keys will select that format.   A screen of general information

about systems requirements will be displayed,  after which the opening

menu is again shown.

After  the  diskette  type is selected,  CMENU will "remember" it from

session to session.   The next step is to select choice 2  to  specify

what diskette drive will be used to access CP/M diskettes.

The  desired function may then be selected from the menu.   Each func-

tion may then request additional input for items such as CP/M diskette

type and names of source and destination files.

Page 3

CMENU prompts for parameters and formats them into a command line  for

one of 22DISK's utility programs.  If a menu interface is not desired,

each program may also be invoked as a normal DOS command.  The follow-

ing  sections deal with each specific function in terms of the program

that performs that function.

Selecting A Disk Format

All of the 22DISK utilities make use of a common interface to  specify

the format of CP/M diskette to be used. Over 200 different formats are

provided with 22DISK and "custom" definitions may also be written.

Each  different  CP/M  format is given a unique one- to four-character

tag.  For example, the Osborne 1, single-sided,  double-density format

is  given  a  tag  of "OSB2",  while the single-sided,  double-density

Kaypro 2is assigned a tag of "KAY1".  So, to specify a format, its tag

is used.

The format tag may be indicated to the  22DISK  utilities  in  any  of

three ways:

(1)  The tag may be specified interactively.  This is the default

situation if the tag has not been indicated by either of the

other  two  ways.   The following prompting message is dis-

played -

Enter 1-4 character disk format type, ? for list -

If a question mark is entered, followed by the ENTER key,  a

list  of  diskette  formats and their tags is displayed,  23

lines at a time.  The cursor keys (up, down,  PgUp and PgDn)

may  be  used  to select a type.   Pressing ENTER causes the

current type to be used.

(2)  The tag may be specified directly on the DOS  command  line.

In this case,  the tag is the first field of the 22DISK com-

mand and is preceded with a slash (/) character.   For  ex-

ample,

CFMT /KAY1 A:

would  result in the diskette in drive A: being formatted as

a Kaypro 2 single-sided, double-density diskette.

(3)  The DOS environment variable "CPMDISK" may be set to the tag

corresponding to the desired format.  So,

SET CPMDISK=OSB2

Page 4

would result in all 22DISK  operations  being  performed  in

terms of Osborne 1 double-density format,  unless overridden

by an explicit DOS command line  specification  as  in  (2),

above.

The  file  named  CPMDISKS.DEF  contains  definitions for all diskette

types and must be present somewhere in the DOS  command  search  path.

That  is,  one  of the directories indicated by the DOS PATH statement

must contain the CPMDISKS.DEF file.

In addition to the predefined formats given in  CPMDISKS.DEF  as  sup-

plied  with 22DISK,  custom definitions may also be prepared using any

ASCII text editor.   For details,  consult the section titled Building

Custom Diskette Definitions.

22DISK CP/M File Naming Conventions

All  of  the  22DISK utilities use the same convention for naming CP/M

files,  which corresponds roughly to that used by CP/M.   However,  to

supply a means of indicating CP/M user numbers and to afford an alter-

nate way to refer to diskette drives, the convention has been extended

somewhat.

A  CP/M  file  designator  as used by 22DISK consists of the following

items:

*    A disk drive specifier.  This is a letter referring to those

drives configured by the 22DINST program.

*    An optional user number between 0 and 31.  If omitted,  user

number 0 is assumed.   A wildcard,  or asterisk (*),  may be

specified to signify all user numbers.

*    A colon (:).

*    The file name consisting of up to 8 letters, numbers or spe-

cial characters.   Upper- and lower-case  letters  are  con-

sidered to be equivalent.  A special character may be any of

the following -

! @ # $ % ^ & ( ) - _ = + { } [ ] ' " , /

*    The   characters "?" and "*" serve as "wildcard" specifiers;

"?" matches any single character.  "*" matches any remaining

characters in the file name.

*    A period (.),  which may be omitted if no file extension  is

present (see next item).

Page 5

*    The  file extension which consists of up to 3 letters,  num-

bers or special characters.   Its  rules  of  formation  are

identical to those associated with the file name,  including

use of wildcard characters.

Here are some examples of CP/M file names accepted by the 22DISK  con-

version utilities:

A:*.COM                     All  files  with  an extension of

"COM" from drive A:,  user number

0.

D*:*.*                      All files from any user number on

diskette drive D.

B2:X*.DAT                   All  files having an extension of

"DAT" under user number  2,  from

drive B:,  starting with the let-

ter "X".

In all of the 22DISK utilities, a drive designator followed by a colon

without any filename or extension assumes all  files  on  that  drive.

Thus,

B:                   is identical in meaning to

B:*.*

Sometimes,  as  with the CFMT utility,  a drive designator is required

without a file name.  In these cases, no user number, file name or ex-

tension is used, but physical unit designations are still allowed.

22DISK Command Syntax Conventions

In order to discuss command syntax in 22DISK,  a number of conventions

have been adopted in this document:

(1)  All commands are assumed to be followed by depression of the

ENTER  key.   Sometimes this key is referred to as RETURN or

CARRIAGE RETURN.

(2)  All upper-case (capital)  letters  are  entered  exactly  as

shown in the command description.  For instance,

CFMT

is entered as depressions of the "C", "F", "M" and "T" keys.

Actual  case  of the entered characters is irrelevant;  that

is, either upper- or lower-case letters may be entered.

Page 6

(3)  Any underlined item enclosed in italics refers to a name  of

a  particular  type  of  item  and  is  not  to  be  entered

literally.  For example:

CP/M file

refers to a general CP/M file designator as described in the

preceding section, and NOT the characters ""CP/M file"".

(4)  Any item enclosed in parentheses ("(" and ")") refers to  an

item  which  may be omitted from the command line.   For ex-

ample:

(disk type)

refers to a disk type item which may be left out  completely

from the command statement.

(5)  Any  blank  character may be replaced by more than one blank

character or a tab character.

These conventions will be used throughout  this  document  with  addi-

tional explanations where required.

A Quick Reference Guide to 22DISK Commands

The  following  is  a  short description of the command syntax of each

conversion program:

CONVERT CP/M FILES TO DOS

CTOD  (/disk type)  CP/M file name  DOS disk unit

Example:

CTOD  /KAY1  A:*.DAT  C:

CONVERT DOS FILES TO CP/M

DTOC  (/disk type)  DOS file name  CP/M disk unit

Example:

DTOC  C:*.BAS  A:

DISPLAY A CP/M DISKETTE DIRECTORY

CDIR  (/disk type)  CP/M file name

Page 7

Example:

CDIR A:*.OVR

ERASE A CP/M DISKETTE FILE

CERA  (/disk type)  CP/M file name

Example:

CERA  /IBM1  WS?.COM

DISPLAY (TYPE) A CP/M DISKETTE FILE

CTYPE  (/disk type)  CP/M file name

Example:

CTYPE  2:*.DOC

FORMAT A CP/M DISKETTE

CFMT  (/disk type)  CP/M disk unit

Example:

CFMT  /OSB1  B:

GENERATE AN INDEXED DEFINITION FILE

GENINDEX  source file  destination file

Example:

GENINDEX  CPMDISKS.NOI  CPMDISKS.DEF

STRIP AN INDEX FROM A DEFINITION FILE

STRIPIDX  source file  destination file

Example:

STRIPIDX  CPMDISKS.DEF  CPMDISKS.NOI

Page 8

CTOD -- Convert A CP/M Diskette To DOS

The program CTOD is used to read a CP/M diskette and copy files to any

selected DOS directory.

The general syntax of the CTOD command is -

CTOD  (/disk type)  CP/M file  (DOS path)

Disk type is the optional diskette tag as  discussed  earlier.   CP/M

file  refers  to  the CP/M file designator for the diskette drive con-

taining the CP/M diskette and the file(s) to be copied.   The wildcard

characters * and ? are allowed to specify a number of CP/M files.

DOS  path  refers  to  the name of a DOS disk drive or subdirectory to

which the CP/M files will be copied by CTOD.  If this term is omitted,

the current DOS default drive and path are used.

Some transformations of CP/M file names are made by CTOD when  copying

to  DOS.   CP/M file naming conventions allow just about any character

in a file name;  DOS naming conventions  are  much  more  restrictive.

Characters which are not accepted by DOS are translated by CTOD to the

character #, a legal DOS file name character.

In  addition,  the file extension COM is changed to CPM by CTOD.   DOS

considers any file with the extension COM to be executable;  if execu-

tion  of  a  CP/M  file is attempted by DOS,  the result could be dis-

astrous.

The 22NICE CP/M emulator also performs the same name  translation  in-

ternally  for  CP/M programs,  so programs will operate correctly with

the transformed file extensions.

Here are a few examples of CTOD commands:

CTOD  A:*.*

Copy all files from user 0, on drive A: to the current directory.

Use whatever format that was defined by the  CPMDISK  environment

variable, or prompt for one if CPMDISK is not defined.

CTOD  /A1  B*:*.COM  C:\CPMFILES

Copy  all  files  with  an extension of COM from all users,  from

drive B: to the directory named C:\CPMFILES.   Use  the  8"  SSSD

format.   These files, when copied, will be given an extension of

CPM.

CTOD

Produces no action, but gives a synopsis of the CTOD command syn-

tax.

Page 9

DTOC -- Convert DOS Files TO CP/M Files

The program DTOC is the complement of CTOD;  that is,  it  writes  DOS

files to a CP/M diskette.  The general syntax of the DTOC command is:

DTOC  (/disk type)  DOS file name  CP/M drive and user

The  optional  term disk type is the name of a CP/M diskette type,  as

discussed above.   DOS file name refers to the name of a DOS  file  or

group  of  files  and  may  include drive and subdirectory specifiers.

CP/M drive and user specifies the diskette drive to  receive  the  DOS

files and the user number under which they are to be created.

In  addition to copying DOS files to CP/M diskettes,  DTOC carries out

the CPM to COM name translation described in the CTOD command.   That

is,  all  DOS  files  with  an extension of CPM will be created as COM

files on the CP/M target diskette.

Some examples of DTOC commands are:

DTOC  C:*.*  A:

Copy all files from DOS drive C: to CP/M  diskette  A:,  user  0.

Use  whatever  format that was defined by the CPMDISK environment

variable, or prompt for one if CPMDISK is undefined.

DTOC  /A1  \UTILS\*.ASM  B3:

Copy all files from the DOS default  drive  in  the  subdirectory

named  UTILS  with an extension of ASM to user 3 on CP/M diskette

drive B:.  Use the 8" SSSD format.

DTOC

Produces no action, but gives a synopsis of the DTOC command syn-

tax.

CTYPE -- Display A CP/M File

The program CTYPE copies a CP/M diskette file or files to DOS standard

output.  Before each file is copied,  its name is also copied to stan-

dard  output.   CTYPE  is  the  analogue  of  the  DOS  and CP/M TYPE

utilities.

The general syntax of the CTYPE command is:

CTYPE  (/disk type)  CP/M file

Page 10

Disk type is the optional diskette  tag  code  as  discussed  earlier.

CP/M  file  refers  to the CP/M file designator for the diskette drive

containing the CP/M diskette and the file(s) to  be  displayed.   The

wildcard characters * and ?  are allowed in order that a group of CP/M

files may be specified.

An operation similar to CTYPE can be obtained  with  CTOD,  where  the

destination  DOS  drive  is given as CON.   Some examples of the CTYPE

command are:

CTYPE  A:*.*

Copy all files from user 0, on drive A: to standard output.

CTYPE  /A1  B*:*.DOC

Copy all files with an extension of DOC from  all  user  numbers,

from drive B: to standard output.  The source diskette is assumed

to be in 8" SSSD format.

CTYPE

Produces  no  action,  but  gives a synopsis of the CTYPE command

syntax.

CDIR -- Display A CP/M Directory

The program CDIR displays a list of CP/M diskette files  in  much  the

same way that the CP/M command DIR does.  However, the user number for

each  file  is also displayed,  along with the available CP/M diskette

space remaining.

The general syntax of the CDIR command is:

CDIR  (/disk type)  CP/M file

Disk type is the optional two character diskette  type  code  as  dis-

cussed earlier.   CP/M file refers to the CP/M file designator for the

diskette drive containing the CP/M diskette and the file(s) to be dis-

played.  The wildcard characters "*" and "?" are allowed in order that

a group of CP/M files may be specified.   Some examples  of  the  CDIR

command are:

CDIR  A:*.*

Displays  a directory of all files from all user areas,  on drive

A.

CDIR  /A1  B*:*.DOC

Page 11

Displays a directory of all files with an extension of  DOC  from

all user numbers, on drive B:.  The source diskette is assumed to

be in 8" SSSD format.

CDIR

Produces no action, but gives a synopsis of the CDIR command syn-

tax.

CERA -- Erase CP/M Files

The program CERA erases a list of CP/M diskette files in much the same

way  that  the  CP/M  command  ERA does.   In addition,  the available

remaining CP/M diskette space is reported.

The general syntax of the CERA command is:

CERA  (/disk type)  CP/M file

disk type is the optional diskette tag as  discussed  earlier.   CP/M

file  refers  to  the CP/M file designator for the diskette drive con-

taining the CP/M diskette and  the  file(s)  to  be  displayed.   The

wildcard  characters  "*" and "?" are allowed in order that a group of

CP/M files may be specified.

Each file specified is erased by writing the hexadecimal character  E5

as  the first byte (user number) of the CP/M diskette directory entry.

The rest of the affected directory entry is left alone.   Examples  of

the CERA command are:

CERA  A:*.*

Erase all files belonging to user 0, on drive A:.

CERA  /A1  B*:*.DOC

Erase  all  files with an extension of DOC from all user numbers,

on drive B:.   The source diskette is assumed to be  in  8"  SSSD

format.

CERA

Produces no action, but gives a synopsis of the CERA command syn-

tax.

Page 12

CFMT -- Format A CP/M Diskette

The  CFMT  utility  formats  blank  CP/M diskettes.   CFMT corresponds

directly to the CP/M FORMAT or NEWDISK command.

The general syntax of the CFMT command is:

CFMT  (/disk type)  CP/M unit name

Disk type is the optional diskette tag as  discussed  earlier.   CP/M

unit  name  refers to the designator for the diskette drive containing

the diskette to be formatted.

Before a diskette is formatted by CFMT, a prompt is issued to the con-

sole in order to confirm that a diskette is to be reformatted.  If the

letter "G" followed by the ENTER key is typed,  a blank format pattern

is created on the designated diskette.

Examples of the CFMT command are:

CFMT  /MOR1  A:

Format  the  diskette  in drive A: with a format pattern suitable

for a Morrow MD1 computer system.

CFMT  F:

Format the diskette in diskette unit F:  with  a  format  pattern

corresponding to whatever is specified by the CPMDISK environment

variable.  If CPMDISK is not defined, CFMT will prompt for a dis-

kette type.

Page 13

Building Custom Diskette Definitions

As  mentioned earlier,  the file CPMDISKS.DEF contains definitions for

various common CP/M diskette types.   This sections describes how  you

can add your own definitions to this file for diskette formats not al-

ready  present.   This  section  requires some knowledge of CP/M disk

structure; it is not intended for the casual user.

The following is a typical diskette format definition:

BEGIN

SUP3  Superbrain QD -- DSDD 48 tpi 5.25"

DENSITY

MFM,LOW

COMPLEMENT

CYLINDERS

35

SIDES

2

SECTORS

10,512

SIDE1 0

1,3,5,7,9,2,4,6,8,10

SIDE2 1

1,3,5,7,9,2,4,6,8,10

ORDER

CYLINDERS

BSH 4 BLM 15  EXM 1  DSM 169  DRM 63

AL0 080H  AL1 00H  OFS 2

END

Diskette definitions are keyword-oriented.   That is,  there are a few

words  which  have  special meaning to the routines with interpret the

diskette definitions.   Each keyword identifies  the  data  which  im-

mediately follows it.

The following rules of construction apply to a diskette definition:

(1)  Definitions are not sensitive to alphabetic case;  terms and

keywords may be in upper- or  lower-case  characters,  or  a

mixture of both.

(2)  Spaces,   tabs and line boundaries are treated as separators

between terms.  In some cases, a comma is required as an ad-

ditional separator.

(3)  A term must be completely contained on a single line; other-

wise,  line boundaries are generally ignored.   The only ex-

ceptions to this are the NOTE keyword, which causes the rest

Page 14

of  the  current line to be ignored,  and the BEGIN keyword,

which uses the rest of the line following the diskette  type

code or "tag" as a descriptive comment.

(4)  Numbers may be expressed in decimal,  hexadecimal or binary.

Decimal quantities are the default and are made up of digits

between 0 and 9 inclusive.   Hexadecimal quantities may also

include the letters A through F,  but must begin with a num-

ber.   Hexadecimal quantities are denoted by  a  final  "H".

Binary quantities are made up of the numbers 0 and 1 and are

denoted  by a final "B".   These are the same conventions as

those used in CP/M assembler coding.

(5)  All-blank or empty lines are ignored and may be used  freely

to enhance readability.

BEGIN  signals  the  start of a definition.   The first term following

this keyword is a one- to four-character diskette tag  code  by  which

this  definition  may  be accessed.   The first character of this type

code must be alphabetic;  the second character may be numeric  or  al-

phabetic.   The  remainder of the the line following the type code is

saved as commentary describing the diskette type.

DENSITY takes two terms,  separated by a comma.   The  first  term  is

either  MFM  for  a  double-density diskette or FM for single density.

Your disk controller must be capable of supporting FM operation if the

FM density option is given.   The second term is used to  program  the

diskette  clock  rate on AT-class machines.   LOW signifies that a 250

Kbps data rate should be used; HIGH signifies that a 500 Kbps (HD den-

sity) data rate is to be used.   When the PC-AT  1.2M  5.25"  diskette

drive is being used,  a data rate of LOW is taken to mean 300 Kbps in-

stead of 250 Kbps.

CYLINDERS specifies the number of cylinders contained in the  diskette

format.   If  this  number  is less than 50,  quad-density (80 track)

drives will be double stepped to go from cylinder to cylinder.

SKEW specifies the physical interleaving of sectors.   This specifica-

tion is optional; if omitted a 1-to-1 physical interleave is assumed.

SIDES specifies the number of tracks or surfaces per cylinder.   SIDES

may be either 1 or 2 for single- or double-sided media, respectively.

SECTORS specifies both the number of sectors per track (cylinder  sur-

face)  and the size of a sector.   The sector count and the size terms

must be separated by a comma.  SECTORS must precede SIDE1 and SIDE2 in

a definition.

Page 15

COMPLEMENT is used to denote certain  formats  which  record  data  in

complemented or NOT-ed form.  The primary offender in this area is the

Superbrain  line.   COMPLEMENT  instructs  22DISK  to complement data

before writing and after reading a CP/M diskette.   If  COMPLEMENT  is

not used within a definition, normal true format recording is assumed.

ORDER  instructs 22DISK as to the ordering of tracks on a double-sided

diskette.   Most  formats  record first one side,  and then the other

before moving to a new cylinder.   However,  some  diskette  formats,

notably CP/M-86 on the IBM PC, record all of the tracks on one side of

a diskette, then record the tracks on the other side in descending or-

der.  To specify that the cylinder is incremented before the side, use

CYLINDERS after the ORDER keyword.  To specify that the side is incre-

mented before the cylinder number,  use SIDES after the ORDER keyword.

The default when ORDER is not specified is SIDES;  that is,  first one

side is recorded,  then the other before the cylinder number is incre-

mented.  ORDER has no effect on single-sided media.

SIDE1, which must follow the SECTORS specification, specifies the sec-

tor ordering on the  first  cylinder  surface.   CP/M  2.2  allows  a

software  interleave  of  sectors on a diskette;  the terms given here

reflect that interleave.   The first term after the SIDE1  keyword  is

the  logical  side  ID  found  in  each ID address mark for that disk.

Usually,  this number is 0 for the first side of a diskette and 1  for

the second side.   Certain computers,  however,  deviate from this, as

shown by the Kaypro example above,  where the second side also  has  a

side ID of 0.   After the side ID, there follows a list of sector num-

bers in their CP/M ordering.   Each number specifies the next  logical

sector to be addressed.  Sector numbers in this list must be separated

by  commas;  the  number   given must correspond to that stated in the

SECTORS specification.

SIDE2 is specified only if the diskette has two surfaces per cylinder.

The terms following SIDE2 are the same as those for SIDE1, except that

they define the sector addressing and ordering for the second  surface

of the diskette.

LABEL  is specified for diskette formatting only.   The term following

the word LABEL is used as a file name with an implied suffix of  .LBL.

When a diskette is formatted, the contents of this file are written to

the diskette,  starting with the first sector.   If the data length of

the file does not work out to be an integral multiple of the  diskette

sector length,  the last sector is filled to its end with the value of

the last byte read from the file.   The LABEL feature may also be used

to enable one to create CP/M "system" diskettes.

The  remainder  of  the  keywords in the definition are taken from the

CP/M Disk Parameter Block for this diskette type.  The CP/M System Al-

teration Guide presents details on the meaning of each field.  A brief

description of each is presented here:

Page 16

BSH refers to the block shift or the number of left shifts needed

to translate an allocation block number to a relative sector

address.

BLM refers to the block mask or a bit mask of ones  corresponding

to the number of bits specified by BSH.  For example, if BSH

is 3, BLM is 7 (3 bits).

EXM  refers  to  the  extent  mask  or a bit mask of ones used in

delimiting extents, or groups of 128 128-byte records.

DSM refers to the total number of sectors present on a  diskette.

This  quantity,  like all of the CP/M system variables,  as-

sumes a 128-byte sector.

DRM refers to the total number of directory entries available  on

a diskette.  DRM is also related to the AL0 and AL1 fields.

AL0 and AL1 form a bit mask,  beginning with the most significant

bit in byte AL0 and ending with the least significant bit in

AL1.   These two bytes map the first 16 allocation blocks of

the  disk.   A 1 bit in a position indicates that an alloca-

tion block is reserved,  usually for the directory (see DRM,

above).

OFS  indicates the number of tracks that are used for storing the

CP/M system and BIOS at the beginning of a diskette.

SOFS  indicates the number of sectors that are  used  for  storing

the  CP/M  system  and  BIOS at the beginning of a diskette.

SOFS is used when the number of sectors used by the CP/M  is

not  an  integral  number of tracks,  such as for the Coleco

Adam.

Every diskette definition must end with the END keyword, which follows

the OFS specification.

If the following Microsoft BASIC program is run on the  host  CP/M  (8

bit)  system,  the values of the relevant DPB fields are displayed for

the currently logged-in drive:

100 REM PROGRAM TO DISPLAY A DEFAULT DPB

110 REM

120 REM FOR CP/M 2.2 ONLY

130 REM

140 DEF FNP(X) = PEEK(X) + 256*PEEK(X+1)

150 IF PEEK(0) <> 195 THEN PRINT "Requires CP/M" : STOP

160 PRINT "DPB VALUES FOR THE CURRENT DISK -- " : PRINT

170 REM

180 REM STUFF S$ WITH THE FOLLOWING PROGRAM -

Page 17

190 REM

200 REM PUSH H            ; Save address of FAC-3

210 REM LHLD 1            ; Get BIOS Cold boot entry

220 REM MVI L,27          ; Change to SELDISK vector

230 REM LDA 4        ; Get current disk

240 REM MOV C,A

250 REM MVI A,PCHL   ; for indirect call

260 REM STA 38H           ; set in RST 7 vector

270 REM RST 7        ; call SELDISK

280 REM POP D        ; FAC-3

290 REM XCHG

300 REM MOV M,E           ; Store pointer to DPH

310 REM INX H

320 REM MOV M,D

330 REM RET          ; exit...

340 REM

350 S$ = ""

360 READ SD : IF SD < 0 THEN 380  ' Read the USR code

370 S$ = S$ + CHR$(SD) : GOTO 360

380 SX = VARPTR(S$) +1

390 SR = FNP(SX)

400 DEF USR = SR               ' User program set up

410 DPH = USR(0)               ' Get DPH Pointer

420 IF DPH = 0 THEN PRINT "Select error" : STOP

430 SPT = FNP( FNP( DPH +10)) ' Get Sectors/Track

440 PRINT "Sectors per Track";TAB(30);SPT

450 IF FNP(DPH) = 0 THEN 510  ' if no interleave

460 PRINT " Sectors interleaved as follows:"

470 FOR I = 1 TO SPT

480  PRINT TAB(10); PEEK( FNP(DPH) + I-1)

490 NEXT I

500 PRINT : GOTO 520

510 PRINT " Sectors are not interleaved"

520 DPB = FNP(DPH+10)

530 PRINT "Block Shift (BSH)";TAB(30);PEEK(DPB+2)

540 PRINT "Block Mask (BLM)";TAB(30);PEEK(DPB+3)

550 PRINT "Extent Mask (EXM)";TAB(30);PEEK(DPB+4)

560 PRINT "Total Sectors (DSM)";TAB(30);FNP(DPB+5)

570 PRINT "Directory Entries (DRM)";TAB(30);FNP(DPB+7)

580 PRINT "Allocation 0 (AL0)";TAB(31);HEX$(PEEK(DPB+9));"H"

590 PRINT "Allocation 1 (AL1)";TAB(31);HEX$(PEEK(DPB+10));"H"

600 PRINT "Cylinder Offset (OFS)";TAB(30);PEEK(DPB+13)

610 PRINT

620 STOP

630 DATA  229,42,1,0,46,27,58,4,0,79,62,233,50,56,0

640 DATA  255,209,235,115,35,114,201,-1

650 END

Page 18

Some Hints on Developing a Definition

Quite frequently,  the only information available  when  developing  a

definition for 22DISK is a diskette with a few files and nothing more.

Here's a guide on what to do if you'd like to try working up a defini-

tion yourself:

1.   Get  a copy of Sydex's ANADISK.   This utility will fur-

nish far more information than will any other program.

2.   Run ANADISK on the diskette in SECTOR EDIT mode.   Note

that  the interleave (or SKEW) is given,  as well as the

number,  size and addresses of sectors.   You then  have

the figures for DENSITY,  CYLINDERS,  SIDES,  SKEW SIDE1

and SIDE2.

3.   Locate the  directory  on  the  diskette.   Almost  in-

variably, it starts on the first sector of a track.  The

number of tracks up the point where the directory begins

will give the figure for OFS.   The directory stands out

because it shows file names every  32  bytes,  or  every

other line on the ANADISK display.

4.   Examine the first directory entry.   The second 16 bytes

(the second line of the entry) gives a list  of  alloca-

tion unit numbers belonging to the file.   The first one

of these will tell how many allocation units  have  been

reserved to hold the directory.   Thus, if the first al-

location byte is 04, four allocation units have been al-

located to the directory.   Another  thing  to  note  is

whether  the  allocation  unit number appears to be an 8

bit or 16 bit quantity.   For example,  if an entry  has

the following form:

00 50 49 50 20 20 20 20 20 43 4F 4D 00 00 00 10  .PIP     COM....

04 00 05 00 06 00 00 00 00 00 00 00 00 00 00 00  ................

the  allocation unit designators are 4,  5 and 6 and are 16-

bit quantities.  On the other hand, the following item:

00 50 49 50 20 20 20 20 20 43 4F 4D 00 00 00 10  .PIP     COM....

04 05 06 00 00 00 00 00 00 00 00 00 00 00 00 00  ................

shows the same allocation units,  but as  8-bit  quantities.

If 16-bit quantities are involved,  more than 255 allocation

units are defined for the diskette,  if an  8-bit  quantity,

less than 256 units.   This serves as a check for later com-

putations.  In any case,  the first allocation number of the

first  directory entry gives the number of bits to be set in

AL0 and AL1.  Thus, for an allocation number of 04:

AL0 11110000B  Al1 0

Page 19

5.   Starting with the first directory sector, advance the sector

(and track,  if necessary) until you find the data belonging

to the first file.   Unused directory entries are almost al-

ways filled out with a pattern of E5's.   The number of sec-

tors  dedicated  to the directory will give an indication of

both the number of directory entries and the allocation unit

size.   Suppose that a our hypothetical  diskette  shows  16

sectors  of 512 bytes allocated to the directory.   Since a

directory entry is 32 bytes long,  the number  of  directory

entries possible for this format is:

16 entries/sector X 16 sectors = 256 entries total

Thus,  DRM is one less than this or 255.  Since we know that

these 16 sectors  represent  4  allocation  units  or  8,192

bytes, the allocation unit size is:

8,192 / 4  = 2,048 bytes

Given  this,  we  can use the following table to compute the

values for BSH and BLM:

+------------+---------+----------+

| Unit Size  |   BSH   |   BLM    |

+------------+---------+----------+

|    1,024   |    3    |    7     |        +----------+

|    2,048   |    4    |   15     |  ------+ This one |

|    4,096   |    5    |   31     |        +----------+

|    8,192   |    6    |   63     |

|   16,384   |    7    |  127     |

+------------+---------+----------+

7.   We still need a value for EXM and DSM.   DSM is  simply  the

total number of allocation units on the diskette, not count-

ing the system OFS area.  Let's assume that our diskette has

an  offset of 2 tracks and is single-sided with 40 cylinders

with 8 sectors per track.  The value of DSM is then:

((40 cylinders X 1 side) - 2 tracks) X 8 sectors X 512  bytes)  =

155,648 bytes / 2,048 = 76 Allocation units, total

To get EXM, we then use the following table:

+------------+-----------+-----------+

| Unit Size  | DSM < 256 | DSM > 255 |

+------------+-----------+-----------+

|    1,024   |      0    |   na      |

|    2,048   |      1    |    0      |

|    4,096   |      3    |    1      |

|    8,192   |      7    |    3      |

|   16,384   |     15    |    7      |

+------------+-----------+-----------+

Page 20

8.   That's  almost  the complete picture,  but for the SIDE1 and

SIDE2 sector ordering.   This is the hardest to determine of

any  of  the  parameters.   Find a readable text file on the

diskette and piece the text back together by  observing  how

words break across sector boundaries.   After a bit,  a pat-

tern in the ordering will become apparent, such as:

1, 3, 5, 7, 2, 4, 6, 8

9.   The ORDER value can be determined on double-sided  diskettes

by  observing  whether  file  data  appears to move from one

track to the next on the same side (CYLINDERS),  or  appears

to  go  from  one  side to the other and then advance to the

next cylinder (SIDES).

10.   COMPLEMENT is rare.  Generally,  if a diskette appears to be

formatted  with values of 1A instead of E5,  the data on the

diskette is complemented.

Indexing A Large Definition File with GENINDEX

CPMDISKS.DEF as supplied with 22DISK has been indexed so that the  en-

tire  file  does  not  need  to be searched for the specified diskette

definition.  CPMDISKS.DEF is also supplied in a  non-indexed  form  as

CPMDISKS.NOI.

To  generate an indexed definition file,  the GENINDEX command is used

as follows:

GENINDEX  source file  destination file

where source file is the non-indexed input definition  file  and  des-

tination  file  names  the file which will contain the indexed defini-

tions.

An editable form of CPMDISKS.DEF may  be  had  by  using  the  utility

STRIPIDX to remove the index from a definition file:

STRIPIDX  source file destination file

Diskette Densities and Recording Modes

The  diskette  has undergone quite a number of changes in its develop-

ment and 22DISK reflects some of these changes.   This  section  dis-

cusses some of the terms used in connection with 22DISK definitions:

1.   The physical size of the diskette.   By and large, most dis-

kettes come in one of three sizes: 8, 5.25 or 3.5".   The 8"

diskette is rare to find in use nowadays, but 22DISK is able

Page 21

to  handle  this  format,  given  the  appropriate hardware.

5.25" diskettes are the reigning media choice now,  but  may

soon be displaced by the 3.5" "micro" diskettes.  There have

been other sizes,  such as 3.25", but these have not enjoyed

much popularity.

2.   Recording performed on both surfaces of the diskette.   The

formats  using  only  one  surface  are called single-sided,

those  using  both  surfaces,  double-sided.   Double-sided

drives  were  rare until about 1978;  now they are the rule,

rather than the exception.

3.   The track density.   An analogue to this would be to ask how

close  the  grooves  on  a phonograph record are.  On 8" and

5.25" drives,  a track packing density of 48 tracks per inch

is  most  commonly  used,  although some high-capacity 5.25"

drives double this for 96 tracks per inch (the PC  AT's  1.2

megabyte  drive is a 96 tpi drive).   There was an alternate

5.25" standard in use between 1977 and 1980,  100 tracks per

inch,  but  it  was  always in the minority of applications.

3.5" diskettes use a track density of 135.5 tracks per inch.

4.   The spindle speed, or how fast the diskette spins inside its

jacket.   All 8"  diskettes  spin  at  360  revolutions  per

minute; almost all 3.5" diskettes spin at 300 rpm; the first

Sony drives spun at 600 rpm.  Most 5.25" drives also spin at

300 rpm,  with the exception of the PC AT 1.2 megabyte high-

capacity drive, which spins at 360 rpm.

5.   The recording method used.   Early diskette  drives  used  a

method  called  FM,  for frequency modulation,  in which one

pulse per time period signified a zero bit  and  two  pulses

signified  a  one  bit.   This is the method referred to as

single density.  Later,  a recording method called MFM,  for

Modified FM came into popular use.  This scheme uses a some-

what  more complex encoding scheme dependent on the position

of a pulse within a bit time window and  the  value  of  the

last bit written.   What MFM does is get twice the data rate

for the same bit rate,  hence the term double density.   Al-

most all modern diskette drives use MFM recording.

6.   The  bit  rate  at which data is read or written.   The data

rate for low-density 5.25" diskettes are recorded is 250,000

bits per second.   The PS/2 and PC AT  high-density  formats

make use of a data rate of 500,000 bits per second.  8" for-

mats generally make use of a 500,000  bit  per  second  data

rate.   The  PC  AT 1.2 megabyte high-capacity drive uses a

data rate of 300,000 bits per second  to  record  360K  dis-

kettes because of its higher spindle speed:

Page 22

360 rpm

---------- X 250,000 bps = 300,000 bps

300 rpm

7.    The  sector  demarcation method used.   All modern diskettes

use unique data patterns to show where a sector  begins  and

ends on a track.  On some older systems, however, additional

index  holes  were  used to mark sector boundaries--known as

"hard" sectoring.   22DISK does not  support  hard  sectored

formats.

The  common  PC  XT  diskette controller is able to handle single- and

double-sided,  double-density diskettes.   The common PC  AT  diskette

controller  possesses a variable-rate bit clock generator for 250,000,

300,000 and 500,000 bits per second to handle double density diskettes

in both 8", 5.25" and 3.5" sizes, as well as high-density diskettes in

the 5.25" and 3.5" profiles.

If there is a need to handle single-density diskettes in a 5.25"  form

factor,  a PC XT clone controller can easily be modified to do so,  at

no loss of normal functioning.  The next section shows how.

Modifying A PC-XT "Clone" Controller For Single-Density Reading

Got a bunch of Xerox 820 diskettes that you'd love to be able to read?

Sold your Osborne I at a swap meet and need your correspondence  files

from 1981?  Too bad, your PC can't handle single-density diskettes...

Or can it?

Well, it can be done with a little patience, a few bits of wire, and a

small  soldering  iron.    The result is a controller that remains PC-

compatible as far as any DOS software can tell,  but has an  extra  FM

mode in addition.

Do  not  attempt  this modification unless you really know what you're

doing and have worked with this kind of equipment before.   This is no

place for a novice.

First, you need to determine the type of controller you're using.  The

original  IBM PC and PC/XT controllers are NOT readily adaptable,  but

most inexpensive clones are.

The important thing to locate on  the  controller  card  is  the  data

separator chip; it's an 8-pin DIP.  It looks something like this:

Page 23

+--U--+

Pin 1   |     | Pin 8

Pin 2   |     | Pin 7

Pin 3   |     | Pin 6

Pin 4   |     | Pin 5

+-----+

(Top View)

This DIP (Dual Inline Package) will have pins 4,  5 and 6 grounded and

pin 8 connected to the +5 supply trace.    Numbering  of  this  device

varies  --  Western Digital (the prime source) has it as a WD 9216;  a

NEC part number is UM 8326.   But it's all the same as  far  as  we're

concerned.

Identify  the  diskette  controller  chip on your board -- this is the

large 40 pin DIP  labeled  uPD765A  (NEC),  Z765A  (Zilog),  or  8272A

(Intel).   Note that pin 26 is a no-connection on your card -- this is

the MFM/FM mode output pin.

The last thing you need to find is an unused inverter section.    This

is the most difficult part of this whole modification.  Most cards use

a  SN7404 or 'S04 as an oscillator (look near the crystal).   Usually,

not all sections are used.   Or look for a section of an 'LS00 gate or

some  such  thing that can be wired as an inverter (Hint: tie both in-

puts together).

You need to lift pin 5  of  the  8-pin  data  separator  (above)  from

ground.  Depending on how your card is laid out, this can be quick and

simple (cut the ground trace) or very messy (unsolder the whole chip).

Connect  a  wire  (wire-wrap wire is great) between this pin 5 and the

output of the inverter which you located.   Connect a second wire  be-

tween  pin 26 of the floppy disk controller (uPD 765) and the input of

the inverter.

That's it -- put the card back into your PC and  make  sure  it  still

handles regular DOS diskettes okay.

If  you've got an AT-style controller or a Western Digital 'Fox' dual-

density XT controller,  you can even drive SD and DD 8" diskettes with

this modification.

Note  that  this  simple  modification will enable you to read single-

density diskettes,  but not to write them.   If you're  really  adven-

turous,  an  XT-style controller can be made to write diskettes if the

write-precompensation circuitry is bypassed.   We do it  here  with  a

section of a 74LS157 driven from the MFM pin of the '765.  A schematic

of your controller is a must for this kind of work.

Page 24

In Closing

Thank you for taking the time to consider Sydex.   We hope that 22DISK

does its job well and that it meets your needs.   This product  repre-

sents several months of work and research; if you have any suggestions

on how it might be improved, please let us know.

Page 25

Other Sydex Products

SHAREWARE

CON>FORMAT  --  Concurrent "background" diskette formatter.  Fea-

tures "pop-up" operation and "hot key" activation.  You've got to

see it to believe.   Supports all  current  DOS  formats.  $15.00

($50.00 site) registration fee.

22DISK  --  Transfer  files,  format,  examine and erase files on

"foreign" CP/M diskettes on your PC.  Includes tips on supporting

8" and 5.25" single-density diskettes.   Contains definitions for

over  200  different formats.   $25.00 ($100.00 site) registration

fee.

22NICE -- A  CP/M  2.2  emulation  package.    Supports  the  NEC

V-series  chips  or  performs  emulation by software for both the

8080 and Z80 processors.   Includes terminal emulation  and  dis-

kette handling for common CP/M systems.  Includes 22DISK.  $40.00

($150.00 site) registration fee.

ANADISK  --  The  compleat  diskette  utility.    Nothing like it

anywhere else; scan, edit, repair and copy just about any kind of

diskette.  $25.00 ($150.00 site)  registration fee.

TELEDISK -- Turn any diskette into a  compressed  data  file  and

vice-versa.   Allows you to send and receive entire diskettes via

modem.  Even works with some "copy-protected" diskettes.   $20.00

($65.00 site) registration fee.

COPYQM -- Mass diskette duplicator.  Format, copy and verify mul-

tiple  diskettes from a single master.   Implements "no keyboard"

interaction mode and drive "round robin" servicing.  Supports all

standard DOS formats.  $15.00 ($50.00 site) registration fee.

FORMATQM -- Mass diskette formatter - format a box  of  diskettes

at  a single sitting.   Implements "no keyboard" interaction mode

and drive "round robin" servicing.   Supports  all  standard  DOS

formats.  $10.00 ($40.00 site) registration fee.

RETAIL SOFTWARE

SIM-CGA  4.2  --  Color Graphics Adapter simulation on monochrome

(Hercules-compatible) graphics-equipped machines.  Available from

better software  retailers  and  distributed  by  CSS,  ABCO  and

American Software Distributors.

SIM-EMS  -- Simulate Lotus/Intel/Microsoft Expanded Memory on PC-

AT class machines.   Conforms  to  EMS  4.0  specification.  Dis-

tributed by CSS and ABCO.

Page 26