RSXDEMO.DOC							7-15-83

Copyright (c) 1983 by George F Peace.  All rights reserved.

1.  Introduction

This document and the accompanying program are provided to assist the CP/M

Plus user in understanding the operation of the Resident System Extension

(RSX) feature.  Some of the information presented is also available in the

"CP/M Plus (CP/M Version 3) Operating System Programmer's Guide" by Digital

Research.

An RSX is a Page Relocatable (PRL) file with the file type RSX.  RSXs are

combined with a CP/M Plus program by the GENCOM utility.  At execution time,

the program loader dynamically relocates the RSXs to the top of the TPA

and updates the RSX headers and page zero address six to reflect the

presence of those routines.  Control is then passed to the COM file to

perform any operation required to initialize the RSXs and/or the CP/M Plus

environment.

CP/M-Plus is released with several RSXs or programs with attached RSXs.

These include GET, PUT, SAVE and SUBMIT. GET and PUT intercept console input

and output requests respectively and convert those requests into CP/M file

I/O requests.  SAVE intercepts the warm boot BDOS function following

execution of a program and allows the user to save memory to disk.  SUBMIT

functions similarly to GET by replacing console input requests with file I/O.

The CP/M Plus user can create an RSX to perform functions that in the past

typically required a detailed understanding of CP/M memory organization and

operation.  Those functions might include

- Restricting certain BDOS function calls according to

specific security requirements

- data manipulation (i.e. encryption)

- Implementation of RAM disks

- Implementation of hard disk interfaces

The RSX accompanying this document receives all BDOS function calls and

intercepts those that are console-related following its relocation and

initialization.  The program displays the function number for each one

intercepted.  If BDOS was CALLed via address 0005h, the address of that

CALL will also be displayed.  Its operation thus generates a run-time

trace of BDOS calls.

2.  Resident System Extension Structure

Each RSX must contain a standard 27 byte prefix.  The following is an

example of that structure.

0	serial:	db	0,0,0,0,0,0	;room to insert serial number

1	start:	jmp	ftest		;beginning of program

2	next:	db	0C3H		;jump instruction op-code

3		dw	0		;next module in line (or BDOS)

4	prev:	dw	0		;previous module

5	remov:	db	00h		;remove flag clear

6	nonbnk:	db	0		;>0 to load only in non-banked CP/M

7	rsxnam:	db	'RSXDEMO '	;the name of this RSX

8	loader:	db	0		;loader flag

9		db	0,0		;reserved

Updated				Legend

by

loader 	0 - This area is reserved for the program loader to insert the

CP/M Plus serial number.  This allows existing programs that

use that value (always expected to be the first six bytes of

the first page of BDOS) to operate without error in an RSX

environment.

user	1 - This is the jump to the actual start of the RSX code

loader	2 - This field points to the next higher RSX or BDOS if this

is the last RSX

loader	4 - This is the address of the previous RSX or 0005 if this is

the first RSX

user	5 - This flag is set to zero if the RSX is to remain in memory

indefinitely.  It is set to 0FFh to force its removal from

memory on the next warm boot

user   6 - Set this to 0FFh if this RSX is to be loaded only when being

executed on a non-banked CP/M Plus.  DIRLBL.RSX is an example

of the use of this flag.  The RSX handles BDOS function 100

in a non-banked environment

user   7 - This field is any eight byte literal to uniquely identify

the RSX

loader	8 - This flag is set to 0FFh if the RSX is the CP/M Plus LOADER

and zero if it is a user RSX

3.  Resident System Extension Logic Flow

The first thing a programmer needs to do in any RSX is save the caller's

environment to allow normal processing of the BDOS request in subsequent

RSXs.

Next, the RSX should retrieve the BDOS function number (from register C)

and test for one of those being intercepted.

If the BDOS function is not one of those being intercepted, restore the

caller's environment and jump to tag NEXT: (see line 2 in the RSX prefix

above).  Note that any time an RSX passes control to BDOS, tag NEXT: must

be used.  Remember that the RSX is essentially a part of BDOS and a call

to address 0005 will produce unpredictable results.

If the BDOS function is being intercepted, perform any processing

required and restore the caller's environment (except for any flags or

indicators being returned).

Now, determine whether this RSX preprocesses or replaces the BDOS function

and pass control either to label NEXT: to continue with the next RSX or an

RET instruction to return immediately to the caller.

4.  BDOS Function 60 (call Resident System Extension)

This is the special BDOS function to be used to communicate with an active

RSX.  The function is called with 60 (03Ch) in C and the RSX Parameter Block

address in DE.  The parameter block has a structure as follows:

RSXPB:	db	FUNCTION	;the RSX function number

db	NUMPARMS	;the number of parameter words

dw	PARAM1		;parameter word 1

dw	PARAM2		;parameter word 2

.

.

dw	PARAMn		;parameter word n

The FUNCTION byte must a number between 0 and 127.  By use of this BDOS

function, data and commands can be passed to or from an active RSX.  If

no RSX traps this BDOS function, the BDOS returns 0FFh in registers A and L.

It might be useful to designate parameter one of the Parameter Block as

a target RSX indicator (i.e. the RSX name).  In that way, multiple RSXs

obtained from different sources can each use the full range of subfunctions

and yet intercept only their own subfunctions.

5.  Preparing a Resident System Extension for Execution

An RSX is stored and loaded as part of a specially constructed COM file.

The RSX is typically attached to a COM file which performs specific

operations using the extensions provided by that RSX.  The RSX can,

however, be attached to a dummy COM file so that the only operation

performed is the RSX load.  In that case, the RSX is normally locked into

memory until triggered for removal.  The GENCOM utility constructs this

COM file according to user-defined parameter input.

5.1  Assembling the RSX

The RSX must be a page relocatable file because its execution address

is determined dynamically by the loader.  The RMAC assembler and LINK-80

must therefore be used to assemble and link the RSX.

A>RMAC RSXDEMO

A>LINK RSXDEMO [op]

5.2  Assembling the Main Program

As was indicated above, the RSX is typically attached to some COM file

that uses the extensions provided by that RSX.  In this example, an

assembler program will be used to demonstrate both RSX construction and

use of BDOS function 60 to control that RSX.  Use your word processor

to split the following program into RSXINIT.ASM.

; RSXINIT.ASM	copyright (c) 1983 by George F Peace

; usage: RSXINIT			loads the RSX

; 	 RSXINIT <any parameter>	unloads the RSX

dma	equ	80h

bdos	equ	5

wboot	equ	0

callrsx	equ	60

rsx$init equ	13

rsx$term equ	14

org	100h

lda	dma		;get command tail length

cpi	0		;anything there?

jnz	termrsx		;yes - assume terminate request

mvi	a,rsx$init	;get initialization function

loadrsx:

sta	rsxpb		;store subfunction in parameter block

mvi	c,callrsx	;get RSX command index

lxi	d,rsxpb		;get RSX parameter block address

call	bdos

mvi	c,wboot		;terminate the program but not the RSX

jmp	bdos

termrsx:

mvi	a,rsx$term	;get RSX termination function

jmp	loadrsx		;now go set up the RSX

rsxname: db	'RSXTEST '	;name of RSX we're calling

rsxpb:	db	rsx$init	;RSX initialization function

db	1		;1 parameter word follows

dw	rsxname		;parameter 1 address

end

The commands to assemble the control program are

A>MAC RSXINIT

A>HEXCOM RSXINIT

5.3  Attaching the RSX to the Main Program

The final step in RSX construction is the generation of the special COM

file containing both the main program and the attached RSX.  The GENCOM

command is used for this purpose.  GENCOM requires that the RSX have a

file type of RSX so it can be properly identified and included in the COM

file.

A>RENAME RSXDEMO.RSX=RSXDEMO.PRL

A>GENCOM RSXINIT RSXDEMO

5.4  Using the Resident System Extension

Now that the RSXINIT COM file has been updated by GENCOM, it's time to

try it out.  To load the RSX, type

RSXINIT

then execute a program or issue a CCP command.  When you are finished

testing the RSX, type

RSXINIT <any parameter>

to remove the RSX from memory.  You can return the RSXINIT.COM file to

its original unmodified state by typing

GENCOM RSXINIT

6.  Excuses and Closing Comments

There are certainly more ways to implement an RSX than the one presented

here.  My intent has been to provide a general introduction to the

Resident System Function capability rather than a detailed text.  I believe

the Digital Research documentation is an adequate reference text.  Why did

I even provide this text if the DRI documents are "adequate"?  My CP/M

Plus machine was provided with only the barest (non-DRI) documentation.  I

had to gather the material piece-by-piece until I purchased the DRI manual

set.  Maybe there are a few other frustrated Plus users out there too.

I can be contacted at:	George F Peace

1703 Headwaters Road

Midlothian, Virginia  23113

or the CP-MIG on CompuServe (id 71555,1501)

-----	-----	-----	-----	-----	-----	-----	-----	-----

CP/M is a registered trademark of Digital Research

CP/M Plus, LINK-80, MAC, and RMAC are trademarks of Digital Research

HEXCOM is anybody's guess

#rademark of Digital Research

CP/M Plus, LINK-80, MAC, and RMAC are trademarks of Digita