Z-80 Macro Library Documentation

--------------------------------

I.

The purpose of this library is to enable the assembly of the Z-80

instruction set on a CP/M sytem using the CP/M MAC macro assembler.

This library is invoked with the pseudo-op:

"  MACLIB  Z80 "

II.

The following symbols and notations are used in the individual macro

descriptions;

R - Any of the 8 bit registers: A, B, C, D, E, H, L, or M

RR - Any of the 16 bit register pairs: BC, DE, HL, or SP

NN - 8 bit immediate data (0 through 255)

D  - 8 bit signed displacment (-128 through +127)

NNNN - 16 bit address or immediate data (0 through 65535)

B  - bit number (0-7, 7 is most significant, 0 is least)

ADDR - 16 bit address within PC+127 through PC-128

M(ZZZ) - Memory at address "ZZZ"

III.

MACLIB ver.    Zilog ver     TDL ver

--------------  -------------  -------------

LDX R,D  LD R,(IX+D) MOV R,D(IX)

Load register from indexed memory (with IX)

LDY R,D  LD R,(IY+D) MOV R,D(IY)

Load register from indexed memory (with IY)

STX R,D  LD (IX+D),R MOV D(IX),R

Store register to indexed memory (with IX)

STY R,D  LD (IY+D),R MOV D(IY),R

Store register to indexed memory (with IY)

MVIX NN,D  LD (IX+D),NN MVI D(IX)

Move immediate to indexed memory (with IX)

MVIY NN,D  LD (IY+D),NN MVI D(IY)

Move immediate to indexed memory (with IY)

LDAI   LD A,I  LDAI

Move I to A

LDAR   LD A,R  LDAR

Move R to A

STAI   LD I,A  STAI

Move A to I

STAR   LD R,A  STAR

Move A to R

LXIX NNNN  LD IX,NNNN  LXI IX,NNNN

Load IX immediate (16 bits)

LXIY NNNN  LD IY,NNNN  LXI IY,NNNN

Load IY immediate (16 bits)

LBCD NNNN  LD BC,(NNNN) LBCD NNNN

Load BC direct (from memory at NNNN)

LDED NNNN  LD DE,(NNNN) LDED NNNN

Load DE direct

LSPD NNNN  LD SP,(NNNN) LSPD NNNN

Load SP direct

LIXD NNNN  LD IX,(NNNN) LIXD NNNN

Load IX direct

LIYD NNNN  LD IY,(NNNN) LIYD NNNN

Load IY direct

SBCD NNNN  LD (NNNN),BC SBCD NNNN

Store BC direct (to memory at NNNN)

SDED NNNN  LD (NNNN),DE SDED NNNN

Store DE direct

SSPD NNNN  LD (NNNN),SP SSPD NNNN

Store SP direct

SIXD NNNN  LD (NNNN),IX SIXD NNNN

Store IX direct

SIYD NNNN  LD (NNNN),IY SIYD NNNN

Store IY direct

SPIX   LD SP,IX  SPIX

Copy IX to the SP

SPIY   LD SP,IY  SPIY

Copy IY to the SP

PUSHIX   PUSH IX  PUSH IX

Push IX into the stack

PUSHIY   PUSH IY  PUSH IY

Push IY into the stack

POPIX   POP IX  POP IX

Pop IX from the stack

POPIY   POP IY  POP IY

Pop IY from the stack

EXAF   EX AF,AF'  EXAF

Exchange AF and the alternate, AF'

EXX   EXX   EXX

Exchange BC DE HL with BC' DE' HL'

XTIX   EX (SP),IX  XTIX

Exchange IX with the top of the stack

XTIY   EX (SP),IY  XTIY

Exchange IY with the top of the stack

LDI   LDI   LDI

Move m(HL) to m(DE), increment DE and HL, decrement BC

LDIR   LDIR   LDIR

Repeat LDI until BC = 0

LDD   LDD   LDD

Move m(HL) to m(DE), decrement HL, DE, and BC

LDDR   LDDR   LDDR

Repeat LDD until BC = 0

CCI   CPI   CCI

Compare A with m(HL), increment HL, decrement BC

CCIR   CPIR   CCIR

Repeat CCI until BC = 0  or  A = m(HL)

CCD   CPD   CCD

Compare A with m(HL), decrement HL and BC

CCDR   CPDR   CCDR

Repeat CCD until BC = 0 or A = m(HL)

ADDX D  ADD (IX+D)  ADD D(IX)

Indexed add to A

ADDY D  ADD (IY+D)  ADD D(IY)

Indexed add to A

ADCX D  ADC (IX+D)  ADC D(IX)

Indexed add with carry

ADCY D  ADC (IY+D)  ADC D(IY)

Indexed add with carry

SUBX D  SUB (IX+D)  SUB D(IX)

Indexed subtract

SUBY D  SUB (IY+D)  SUB D(IY)

Indexed Subtract

SBCX D  SBC (IX+D)  SBB D(IX)

Indexed subtract with "borrow"

SBCY D  SBC (IY+D)  SBB D(IY)

Indexed subtract with borrow

ANDX D  AND (IX+D)  ANA D(IX)

Indexed logical and

ANDY D  AND (IY+D)  ANA D(IY)

Indexed logical and

XORX D  XOR (IX+D)  XRA D(IX)

Indexed logical exclusive or

XORY D  XOR (IY+D)  XRA D(IY)

Indexed logical exclusive or

ORX D  OR (IX+D)  ORA D(IX)

Indexed logical or

ORY D  OR (IY+D)  ORA D(IY)

Indexed logical exclusive or

CMPX D  CP (IX+D)  CMP D(IX)

Indexed compare

CMPY D  CP (IY+D)  CMP D(IY)

Index compare

INRX D  INC (IX+D)  INR D(IX)

Increment memory at M(IX+D)

INRY D  INC (IY+D)  INR D(IY)

Increment memory at M(IY+D)

DCRX D  INC (IX+D)  INR D(IX)

Decrement memory at M(IX+D)

DCRY D  DEC (IY+D)  DCR D(IY)

Decrement memory at M(IX+D)

NEG   NEG   NEG

Negate A (two's complement)

IM0   IM0   IM0

Set interrupt mode 0

IM1   IM1   IM1

Set interrupt mode 1

IM2   IM2   IM2

Set interrupt mode 2

DADC RR  ADC HL,RR  DADC RR

Add with carry rr to HL

DSBC RR  SBC HL,RR  DSBC RR

Subtract with "borrow" RR from HL

DADX RR  ADD IX,RR  DADX RR

Add rr to IX (RR may be BC, DE, SP, IX)

DADY RR  ADD IY,RR  DADY RR

Add RR to IY (RR may be BC, DE, SP, IY)

INXIX   INC IX  INX IX

Increment IX

INXIY   INC IY  INX IY

Increment IY

DCXIX   DEC IX  DCX IX

Decrement IX

DCXIY   DEC IY  DCX IY

Decrement IY

BIT B,R  BIT B,R  BIT B,R

Test bit B in register R

SETB B,R  SET B,R  SET B,R

Set bit b in register r

RES B,R  RES B,R  RES B,R

Reset bit B in register R

BITX B,D  BIT B,(IX+D) BIT B,D(IX)

Test bit B in memory at M(IX+D)

BITY B,D  BIT B,(IY+D) BIT B,D(IY)

Test bit b in memory at M(IY+D)

SETX B,D  SET B,(IX+D) SET B,D(IX)

Set bit B in memory at M(IX+D)

SETY B,D  SET B,(IY+D) SET B,D(IY)

Set bit B in memory at M(IY+D)

RESX B,D  RES B,(IX+D) RES B,D(IX)

Reset bit b in memory at m(IX+D)

RESY B,D  RES B,(IY+D) RES B,D(IY)

Reset bit b in memory at m(IY+D)

JR ADDR  JR ADDR-$  JMPR ADDR

Jump relative unconditional

JRC ADDR  JR C,ADDR-$ JRC ADDR

Jump relative if Carry indicator true

JRNC ADDR  JR NC,ADDR-$ JRNC ADDR

Jump relative if carry indicator false

JRZ ADDR  JR Z,ADDR-$ JRC ADDR

Jump relative if Zero indicator true

JRNZ ADDR  JR NZ,ADDR-$ JRNZ ADDR

Jump relative if Zero indicator false

DJNZ ADDR  DJNZ ADDR-$  DJNZ ADDR

Decrement B, jump relative if non-zero

PCIX   JMP (IX)  PCIX

Jump to address in IX i.e., Load PC from IX

PCIY   JMP (IY)  PCIY

Jump to address in IY

RETI   RETI   RETI

Return from interrupt

RETN   RETN   RETN

Return from non-maskable interrupt

INP R  IN R,(C)  INP R

Input from port C to register R

OUTP R  OUT (C),R  OUTP R

Output from register R to port (C)

INI   INI   INI

Input from port (C) to M(HL), increment HL, decrement B

INIR   INIR   INIR

Input from port (C) to M(HL), increment HL, decrement B, repeat if B <> 0

OUTI   OTI   OUTI

Output from M(HL) to port (C), increment HL, decrement B

OUTIR   OTIR   OUTIR

Repeat OUTI until B = 0

IND   IND   IND

Input from port (C) to M(HL), decrement HL snd B

INDR   INDR   INDR

Repeat IND until B = 0

OUTD   OTD   OUTD

Output from M(HL) to port (C), decrement HL and B

OUTDR   OTDR   OUTDR

Repeat OUTD until B = 0

RLCR R  RLC R  RLCR R

Rotate left circular register

RLCX D  RLC (IX+D)  RLCR D(IX)

Rotate left circular indexed memory

RLCY D  RLC (IY+D)  RLCR D(IY)

Rotate left circular indexed memory

RALR R  RL R  RALR R

Rotate left arithmetic register

RALX D  RL (IX+D)  RALR D(IX)

Rotate left arithmetic indexed memory

RALY D  RL (IY+D)  RALR D(IY)

Rotate left arithmetic indexed memory

RRCR R  RRC R  RRCR R

Rotate right circular register

RRCX D  RRC (IX+D)  RRCR D(IX)

Rotate right circular indexed

RRCY D        RRC     (IY+D)        RRCR    D(IY)

Rotate right circular indexed

RARR R  RR R  RARR R

Rotate right arithmetic register

RARX D  RR (IX+D)  RARR D(IX)

Rotate right arithmetic indexed memory

RARY D  RR (IY+D)  RARR D(IY)

Rotate right arithmetic indexed memory

SLAR R  SLA R  SLAR R

Shift left register

SLAX D  SLA (IX+D)  SLAR D(IX)

Shift left indexed memory

SLAY D  SLA (IY+D)  SLAR D(IY)

Shift left indexed memory

SRAR R  SRA R  SRAR R

Shift right arithmetic register

SRAX D  SRA (IX+D)  SRAR D(IX)

Shift right arithmetic indexed memory

SRAY D  SRA (IY+D)  SRAR D(IY)

Shift right arithmetic indexed memory

SRLR R  SRL R  SRLR R

Shift right logical register

SRLX D  SRL (IX+D)  SRLR D(IX)

Shift right logical indexed memory

SRLY D  SRL (IY+D)  SRLR D(IY)

Shift right logical indexed memory

RLD   RLD   RLD

Rotate left digit

RRD   RRD   RRD

Rotate right digit

end

########################################################################