----------------------

| ZZZZZ  BBBB   K  K | \

|    Z   B   B  K K  | |

|   Z    BBBB   KK   | |  Z Blaster Kit 1.0

|  Z     B   B  K K  | |  (C) 1992 Steve Goldsmith

| ZZZZZ  BBBB   K  K | |  All Rights Reserved

---------------------- |

\_____________________\

Z Blaster Kit may be copied and distributed freely providing that

all files are kept intact, no fee is charged and it is not part of

a package for which a charge is made.  ZBF files can be used in

your own software royalty free as long as you display a message

stating that you are using Z Blaster files.

* OVERVIEW

Z Blaster Kit allows a C128 running CP/M 3.0 to record, play and

import digitized sound files!  These files are called Z Blaster

Files or ZBF files and have the .ZBF extension.  Included in the

kit are two ZBF file players.  Z Blaster Player is command line

driven and Z Blaster Mega Player is for 80 column VDC mode only.

If you were disappointed with the quality of the Covox Voice Master

then dust it off and check out Z Blaster Recorder.  It creates

better quality sound and has greater sampling range then the

original software that came with the Voice Master!  If you have an

IBM PC or other system give Z Blaster Import a try.  It will let

you create ZBF files from other systems!  Programmers and Hackers

should take a look at the ZBF file format.  It gives information on

decoding and playing ZBF files.

* REQUIREMENTS

+ A C128 or C128D equipped with CP/M 3.0

+ At least one 1541, 1541 compatible or better disk drive

+ 40 or 80 column monitor with sound output

+ Covox Voice Master in plastic enclosure (required for recording

only)

* Z BLASTER PLAYER

Z Blaster Player ZBPLAY.COM is a command line driven ZBF file

player.  To play a file:

ZBPLAY FILENAME

FILENAME = standard 1 to 8 character CP/M file name without .ZBF

extension.

ZBPLAY will load a ZBF file from the currently logged drive into

memory and allow you to play it until ESCAPE is pressed.  ZBPLAY

can handle up to 32640 bytes of digitized data.  ZBF files larger

than 32640 bytes will play, but only the first 32640 bytes are

used.

* Z BLASTER MEGA PLAYER

Z Blaster Mega Player ZBMEGA.COM allows you to play ZBF files from

a file name pick list until ESCAPE is pressed.  ZBMEGA supports 80

column VDC mode only.  To run:

ZBMEGA

You must be logged into the drive with the ZBF files you want to

play.  Also, no command line parameters are required.

* Z BLASTER RECORDER

Z Blaster Recorder ZBREC.COM creates ZBF files on the currently

logged drive without leaving CP/M!  To record a ZBF file:

ZBREC FILENAME SIZE HZ

FILENAME = standard 1 to 8 character CP/M file name without .ZBF

extension.

SIZE     = 128 to 32640 bytes.  SIZE will be rounded to the nearest

128 byte block. i.e. SIZE = (SIZE DIV 128)*128.

HZ       = 4000 to 17000 Hz sample rate.  HZ will be adjusted to

match the closest timer latch value.

You need to have a Covox Voice Master with a plastic enclosure.

The one in the aluminum enclosure will not work.  I'm not sure

about the Jr..  If you get the Jr. to work drop me a line, so I can

support it in future documentation releases.  ZBREC will not lock

up if you don't have a Voice Master.  It will just create a silent

sound file.

You may notice that when you plug the Voice Master into control

port 2 that it affects the keyboard polling.  This does not occur

in native 64 or 128 modes.  The only way around it is to plug the

Voice Master in when ZBREC tells you to and unplug it when

finished.  This can be a pain, but I was able to leave it plugged

in and just delete the extra characters that pop out.  You can use

the numeric key pad and ENTER since the Voice Master has no affect

on the extended keys.  This should help save some retyping!

Once ZBREC is up and running you can press SPACE to record, p to

play or y to save until ESCAPE is pressed.

* Z BLASTER IMPORT

Z Blaster Import ZBIMPORT.COM is for those of you that don't have

a Covox Voice Master or other means to create ZBF files.  ZBIMPORT

allows you to import one bit uncompressed sound files from the

currently logged drive.  To import a file:

ZBIMPORT SOURCE.EXT DEST HZ

SOURCE.EXT = standard 1 to 8 character CP/M file name with

extension.  Source file to read.

DEST       = standard 1 to 8 character CP/M file name without .ZBF

extension.  ZBF file to create.

HZ         = 4000 to 17000 Hz sample rate.  HZ will be adjusted to

match the closest timer latch value.

You may have to import a couple of times to get the Hz right.

ZBIMPORT allows you to import ANY file, so if you import a DOC file

expect strange sounds.  If you are sure it is a digitized sound

file make sure it is not compressed or higher resolution (i.e. 4 or

8 bit PCM).

* WORKING WITH Z BLASTER KIT

I have included ZBF files for you to experiment with.  The ZBF

files that start with HZ show the differences in quality.  I

recorded my voice from 4000 Hz to 16200 Hz while using the same

sample size.  Also included are various sounds recorded with a PC

Sound Blaster card and 128 CP/M Z Blaster Recorder.

ZBF errors may pop up while using any of the command line driven

programs.  If it is a positive value then it is a Turbo Pascal 3.1

I/O error.  If it is a negative value then it is an internal error.

If you can't figure what's causing it then drop me a line.  Most of

the time you will not get any errors.

* ZBF FILE FORMAT

The ZBF file format is straight forward and flexible.  If you are

a Programmer or Hacker you might want to play ZBF files in your own

programs or on other computers.  The first 128 bytes of a ZBF file

make up the header in the following format:

Field    Type         Description

Version  word         ZBF version

Compress byte         Type of compression

SamBits  byte         Bits per sample

Delay    word         Hardware timer value

Size     long integer Sample Size

Hz       word         Sample frequency

Filler   array        116 byte filler

Remember that words are arranged in Intel lo/hi order.  The current

ZBF version is 0100h which stands for 01.00.  Compress is set to 0

which means no compression.  Future releases may include silence

encoding and other encoding schemes to reduce file size.  SamBits

is set to 1 for one bit sample data.  Strings of 0s and 1s from

left to right make up the sound's shape.  You can play each bit

through the SID's volume control to produce the sound on the 128.

Set volume to 0 for 0 bits and 1 thru 15 for 1 bits.  Delay is

hardware dependent.  Files created on the C128 will have CIA timer

latch values and IBM files will have 8253 timer latch values.  It

is best to calculate the latch value from Hz instead of Delay.

Size is the sample size in bytes.  On the IBM I use the whole 32

bit long integer.  With the C128 I'm only using the low word which

gives a maximum length of 64K.  I support up to 32K right now due

to the limited memory on the 128 in CP/M mode.  Hz is the samples

per second frequency.  Higher Hz values create better sound quality

at a cost of more memory.  Filler is for future header expansion.

I set the filler bytes to 0 currently.

Immediately following the header is the digitized sound data.  Your

program can figure how much to read from the lo word of the Size

field.

* SG TOOLS

If you like the look and feel of Z Blaster Mega Player (ZBMEGA.COM)

then check out SG Tools Programmer's tool box for Turbo Pascal 2.0

or higher.  To get a copy contact:

Parsec, Inc.

PO Box 111

Salem, MA 01970-0111

(508) 745-9125

On-line on GEnie as C128.JBEE

* PAYMENT

I would like a payment of $10.00 US currency to the address below

if you use Z Blaster Kit or ZBF files.

* HOW TO CONTACT ME

Please report any problems, suggestions, etc. to me via GEnie as

S.GOLDSMITH2, voice phone (813) 925-1064 or mail to:

Steve Goldsmith

2805 Jamaica Street

Sarasota, FL 34231

###########################################################