84KP256A.LBR  - the new (2/88) update to the original 9/86 library. This

version is stand-alone, all you need is M80 and L80.  The mod costs about

40 dollars and takes about three hours.  It's well worth it.

* * * * *

The following is an update to Dr. Liddle's '83 Kaypro 256K RAMDRIVE

modification, which allows the fine mod to be made to the '84 machines.

Unfortunately, the mod will NOT work on the 10/83 series machines (those

with graphics, but without PIO chips,) but will work on any other Kaypro

system with graphics, and with Handyman too.  I have done a 10/83, and

will make that mod public, although it is very involved.  Those with

'83 non-graphics machines (you know - the FAST ones!) should use Dr.

Liddle's original KP256RAM library for conversion, available on GEnie.

Configure RAMDRIVE.MAC for your machine. I use the Advent Turbo-Rom, and

operate on a 61.75K system. My address for DIRBUF is 0FEE5H. I set RAMDRIVE

to load at 0F363H, which is above my ZCPRII stack and EXTFCB. Set the PIO

DATA and CONTROL equates to the appropriate port - see the PIO mods below to

determine port addresses.  Compile using M80 and link with L80.  Correct

syntax for my L80 is shown in RAMDRIVE.MAC.  For those without Turbo-Rom,

use PROB12 and PATCH18A to find those large chunks of unused memory above

CP/M and below the disk buffers. You need two "pages" of memory.

If your RAM is unsocketed, remove the 64K chips (U32, U33, U38, U39, U41, U42,

U47, U48). carefully - either by cutting the leads at the body and removing

them one by one, or by careful desoldering of all leads. Install sockets, and

wire all pin #1's together on RAM sockets. Re-install the 64K chips (or use

the 256K chips and tie the common pin #1 to +5v.)  Test the machine thoroughly

with the enclosed MT256.COM - which will crash if your CP/M is bigger than

62.75K - but it tests each location up to the point of crash for every possible

read/write value. An error will appear if any cell is bad.

Build the overlay in Figure 1 below. Mount the overlay behind the Monitor ROM,

so that all wires are as short as possible. Don't forget to bend out pin 24

from U-29 so that it doesn't go into the socket!

If your machine is a 4/84 OR 10/84 WITH internal modem or real time clock,

skip to the piggyback mod. If no RTC or modem, then install a PIO in position

U-35 and tie PIO-1,2,3 from the overlay below to 27,28,29 on U-35. If U-27

is missing, install it also. If your machine was previously modified to enable

interrupts (a jumper to the right of the PIO,) remove that jumper.  Your ports

for RAMDISK.MAC are 021H and 023H for data and control, respectively. Set

these equates near the end of RAMDRIVE.MAC.

FOR 4/84's and 10/84's WITH internal modem and/or RTC, add another PIO now.

Piggy-back a new Z80-PIO chip carefully on top of the old one as follows:

Solder Pins 1-3, 5-6, 11, 19-20, 22-26, 34-40 to the existing

PIO chip (carefully!).

Attach NEW PIO pin 4 to U-27 pin 13. This designates the NEW PIO

as Ports 029H and 02BH for data and control.

Tie PIO 1,2,3 from the overlay to 27, 28, 29 of NEW PIO. You also

end up with a spare side for another parallel port if you like.

Now install new 256K chips in the RAM sockets or remove common pin #1 from

the +5v bus. Wire the overlay, (table 1) keeping all wires, ESPECIALLY 16 MHZ

as short as possible, and keep them separated if possible.

Turn on your machine. It should boot and perform properly. Check memory

again with MT256, and check ALL hardware - if ANYTHING appears abnormal,

fix it first! Run RAMDRIVE and you should be able to log into the RAMDRIVE

(C or D as selected in RAMDRIVE.MAC)  Test thoroughly by NSWPing many files

to RAMdrive, or by running BD.COM (FINDBAD won't work here.)

The theory is simple: We need the MUX and RFSH signals that the '84's

lack. U6 buffers the 16 mhz clock signal, and inverts 1/RFSH from the

Z80. U5 generates a pulse from 1/MREQ and RFSH that allows U4 to

generate the missing MUX signal.  The rest of the circuit is a duplicate

of Dr. Liddle's KP256RAM circuit.

2/88 - I have re-written these docs to reflect the slight improvements made

since the 9/86 writing.  Mainly, RAMBUF must be internal to RAMDRIVE, or in

an unused area (Perfect Writer couldn't handle the buffer sharing) and the

74XXX04 can be anything fast and hi-z (HCU, HCT, probably F.)  I am waiting

for the 1 Meg chips to hit $10 before trying to expand the RAMdisk.

This mod has been made on about 15 machines, including those operating three

of the four ports on Jacksonville's Multi-Buss Network.  These machines have

been running without problem for 18 months now, and have logged over 130,000

callers.

I'd appreciate any comments/suggestions be left to me on GEnie (CWMCHAN)

or call AMY'S CRYSTAL PALACE I at 904-725-5435 (300/1200/2400) or  AMY'S

CRYSTAL PALACE II at 904-725-6122 (300/1200).

\_TABLE I\_

Signal on                  Chip on

Board                      Kaypro '84

1/RFSH                     U43 pin 28

1/MREQ                     U43 pin 19

A7                         U43 pin 37

A15                        U43 pin 5

RA7                        U32 pin 9

RA8                        U32 pin 1

PIO 1                      NEW PIO Pin 27

PIO 2                      NEW PIO Pin 28

PIO 3                      NEW PIO Pin 29

16 MHZ                     Jnct of C84/R40/Y5/U29-39

Also, lift Pin 24 of U-29 to disable the existing A7/15 line to the

RAM chips.

\_FIGURE 1\_

+-----------------------------+

|             U1              |

|         +--------+          |

|  +------|    7   |- 0v      |

|  |     -|    4   |----------+

|  |     -|    L   |-

|  |     -|    S   |-

|  |  0v -|    3   |-

+--|------|    9   |- 0v

| +5v -|    3  1|----------+

|      +--------+          |

|                          |

|          U2              |

33R    |      +--------+          |

RA7  <-- / /---|------|        |- 0v      |

+------|    7   |-         |

+---------|    4   |-         |

|        -|    L   |-         |

|        -|    S   |-         |

|        -|    1   |-         |

|     0v -|    5   |- 0v      |          (RFSH)

|    +5v -|    8  1|----------o--------------------------+

|         +--------+                                     |

|                                                        |

|             U3                                         |

|         +--------+                                     |

+---------|        |- 0v        33R                      |

A7   >------------o--|    7   |------------ / /--------> RA8        |

\_|    4   |-----------------o------< PIO 2      |

PIO 1 >---------------|    L   |-< +5v >-/\/\/\_/ 4.7k               |

0v -|    S   |-----------------o------< PIO 3      |

A15   >---------------|    1   |-< +5v >-/\/\/\_/ 4.7k               |

0v -|    5   |------------------------+            |

+5v -|    3  1|- 0v                    |            |

+--------+                        |  (MUX)     |

|            |

U4                            |            |

+--------+                        |            |

-|    7   |- 0v                    |            |

-|    4   |-                       |            |

+5v -|    L   |------------------------+            |

-|    S   |- +5v                                |

+5v -|    7   |-----------------------------+       |

-|    4   |- +5v                        |       |

+5v -|       1|------------------------+    |       |

+--------+                        |    |       |

|    |       |

U5                            |    |       |

+--------+                        |    |       |

0v -|    7   |- 0v                    |    |       |

0v -|    4   |- 0v                    |    |       |

-|    L   |- 0v                    |    |       |

0v -|    S   |-                       |    |       |

0v -|    0   |-------------------+    |    |       |

-|    2   |------> 1/MREQ     |    |    |       |

+5v -|       1|------------------------+    |       |

+--------+                   |         |       |

0v <--| |--> +5v                                  |         |       |

U6                       |         |       |

0.1 mfd         +--------+                   |         |       |

50 v.         -|    7   |- 0v               |         |       |

0v -|    4   |-----------------------------+       |

-|    H   |------> 16MHz      |                 |

0v -|    C   |-                  |                 |

-| T or U |- 0v               |                 |

0v -|    0   |-------------------o-----------------+

+5v -|    4  1|------> 1/RFSH

+--------+

##################################################################################################