SPEEDING UP DISK RESETS

WITH TURBOROM AND P*PS ENHANCEMENTS

5/28/86

Purpose:

The CP/M operating system disk reset function cause the allocation

table for drive A and the currently logged in drive to be rebuilt.

For a large hard disk with many blocks this can take a substantial

amount of time making the system appear very sluggish during warm boots.

The patch on this disk allows this shortcoming of CP/M to be removed for

those who have both the Advent TurboROM and Plu*Perfect Systems operating

system enhancements (CP/M 2.2e).

Operation:

The Advent TurboROM reserves the high bit of the first byte of the

extended DPH as a flag which specifies whether the allocation table of

this drive requires rebuilding.  The ROM initializes this so that all

drives require rebuilding.  Only those drives with a zero length checksum

vector are assumed to have non-removable media and only these are

candidates for avoiding the rebuilding of the allocation table.  Drive A

is a special case as it must still be scanned for the occurence of

'$$$.SUB'.  This process is still considerably faster than actually

rebuilding the vector.  Any program which plays tricks with the

allocation map should ALWAYS reset the high bit of the extended DPH so

that the allocation will be forced to rebuild.  Plu*Perfect Systems

utilities such as PUTDS (version 1.2 and later) and PUTBG (to be

released) always do this.  If you are unfamiliar with the extended

DPH, please refer to the notes available on the TurboROM developers

support disk.

The patches to the CP/M BDOS are internal, but require the space available

in the P*PS enhancements.  For other users of the TurboROM the source code

to the patch is included, but Plu*Perfect Systems cannot support and attempts

to modify this code.  Please do not ask us.

Installation:

A general purpose utility program MERGEPRL is included which will take any

CDL format REL file and merge it with a Digital Research format PRL file to

produce another PRL file.

Usage is

MERGEPRL   infile  outfile

and then you will be prompted for the patch file name.  The input and output

file names can be the same and so not need the PRL suffix.  If you use the

same input and output names, make sure that you have a backup.

The PRL file you will use should be TURBOSYS.PRL or TURBO-DS.PRL and

when prompted for the patch REL file you should type in SKIPMAP.

The output file name will require renaming to TURBOSYS.PRL or TURBO-DS.PRL

for relocation wiht MOVTURBO.

###################################################################################################################