MXSET  - The  Definitive configuration program for the  Epson  MX

printers.

This  program sets options on the MX80 or MX100.  With a compile-

time  setting the program can be customized to printers  with  or

without Graftrax.

The  program can be called from within WordStar to change printer

settings or even to type text directly to the printer.

The program must be compiled using Microsoft's M80 assembler.  It

uses  certain  routines  from  Richard  Conn's  SYSLIB   linkable

subroutines. The program contains Z80 instructions.

-----------------------------------------------------------------

Editor's Note:

An 8080 version, MXSET23.AQM, is included in this library file.

It has been compiled as MXSET23.COM as is. It has not been tested

completely because your editor has an Okidata printer, but it does

appear to work. We have also included the SYSLIB.REL file since

two routines are required should you recompile with M80.

-----------------------------------------------------------------

The main program is not specific to any operating system. It uses

the SYSLIB routines to talk to CP/M. By substituting for the 5 or

6 relatively simple SYSLIB routines used,  it could be adapted to

another DOS such as TRSDOS.

There are four COM versions in this library:

MX10.COM    - MX100 with Graftrax

MX8.COM     - MX80  with Graftrax

MX10NG.COM  - MX100 no  Graftrax

MX8NG.COM   - MX80  no Graftrax

All  versions  have the Clear Screen/Home cursor string  3  bytes

into  the  program.  It is set at 1AH,0 for Osborne  &  Televideo

compatibles. Patch the string in there for your system. There are

6  bytes  avalable plus the required terminating  zero.  If  your

system  does  not  support  clear  screen/cursor  home,patch   in

CR,LF,LF,LF... to leave blank lines between menus.

The  following functions are available.  The control sequence put

to  the printer is shown in parenthesis.  The notation GTRAX  and

non-GTRAX indicate those options available only on printers  with

or without Graftrax.

A: Select character font

1:  Set condensed ON     ( SI  )

2:  Set condensed OFF    ( DC2 )

3:  Set enlarged ON      ( SO  )

4:  Set enlarged OFF     ( DC4 )

5:  Set emphasized ON    ( ESC,'E' )

6:  Set emphasized OFF   ( ESC,'F' )

7:  Set italics ON       ( ESC,'4' )

8:  Set italics OFF      ( ESC,'5' )

9:  Set double strke ON  ( ESC,'G' )

10: Set double strke OFF ( ESC,'H' )

11: Set to subscript     ( ESC,'S',0 )

12: Set to superscript   ( ESC,'S',1 )

13: Set super/subscript OFF    ( ESC,'T' )

14: Set permanent enlarged ON  ( ESC,'W',1 )

15: Set permanent enlarged OFF ( ESC,'W',0 )

(7 - 15 are non-GTRAX)

B: Reset printer to defaults ( ESC,'@' ) - GTRAX

B: Select character set                  - non-GTRAX

1:  North American       ( ESC,'R',0 )

2:  British              ( ESC,'R',3 )

3:  French               ( ESC,'R',1 )

4:  German               ( ESC,'R',2 )

5:  Spanish              ( ESC,'R',7 )

6:  Italian              ( ESC,'R',6 )

7:  Danish               ( ESC,'R',4 )

8:  Swedish              ( ESC,'R',5 )

C: Set line spacing

1: Select 6 lines per inch    ( ESC,'2' )

2: Select 8 lines per inch    ( ESC,'0' )

3: Set line spacing in 1/72" increments  (ESC,'A',n )

4: Set line spacing in 1/216" increments (ESC,'3',n )

(4 is non-GTRAX only)

D: Set page size

1: Set form length in lines   ( ESC,'C',n )

2: Set form length in inches  ( ESC,'C',0,n )

3: Set width                  ( ESC,'Q',n )

E: Set uni-directional print    ( ESC,'U' )

- non-GTRAX

E: Set vertical tabs            ( ESC,'B',n,... )

- GTRAX

F: Set horizontal tabs          ( ESC,'D',n,... )

G: Set to top of form (FF)      ( FF )

H: Turn ON perforation skip     ( ESC,'N',n )

I: Turn OFF perforation skip    ( ESC,'O' )

J: Enable out of paper signal   ( ESC,'9' )

K: Disable out of paper signal  ( ESC,'8' )

L: Print test

ABCDEFGHIJKLMNOPQRSTUVWXYZ

abcdefghijklmnopqrstuvwxyz    CR LF

1234567890-=['\;,./

!@#$%^&*()_+]"|:<>?           CR LF LF

M: Set bit 8 handling      - GTRAX only

1:  Bit 8 as is          ( ESC,'#' )

2:  Bit 8 always ON      ( ESC,'>' )

3:  Bit 8 always OFF     ( ESC,'=' )

N: Home printhead          ( ESC,'<' )

- GTRAX only

O: Set medium density graphics ( ESC,'K',03H,30H ) MX100

- GTRAX only                ( ESC,'K',01H,E0H ) MX80

P: Set high density graphics   ( ESC,'L',06H,60H ) MX100

- GTRAX only                ( ESC,'L',03H,C0H ) MX80

Z: Keyboard to printer

All keys pressed are passed to the printer as pressed.

This includes all control keys, except ^Z.

Use ^Z to end. ^C is passed to printer in this mode.

Frans Van Duinen

Toronto, Ont

21 Apr 84#############################################################################################################################