DOCUMENTATION FOR BULLET80.CAL

This worksheet is the work of Chip Stratton, Dallas, Tx. I adapted it

to run under SuperCalc v1.05 from the Symphony IBM version. His original

documentation follows this note. Hope you enjoy Chip's work.

Chuck Cotham

41 L.O.G

Nederland,Tx.

DOCUMENTATION FOR BULLET

This worksheet will enable the rifleman to make trajectory predictions

given basic starting information. It can be particulary useful for

deciding the optimum range for sighting-in a given rifle and cartridge

combination. It will also allow one to make various changes in

parameters such as scope height, muzzle velocity, crosswind, ect. for

easy comparisons of different combinations. Following is an explanation

of the various data fields.

CARTRIDGE - This is for reference only, and is not used in calculations.

BULLET WEIGHT - This only affects the energy calculation. Enter the

weight of the bullet under consideration.

BULLET TYPE - For reference only, not used in calculations.

BALLISTIC COEFF. - This is a number that describes the ability of

the bullet to travel through air, and can be found in reloading

handbooks and bullet manufacturer literature. It is critical

to trajectory calculations. If you are unable to find out the

number for the particular bullet you are using, use the listed

bullets below for reference. Most factory bullets are either

spitzer or round nose types.

MUZZLE VELOCITY - Enter the published or hypothesized muzzle velocity

for your cartridge here. This is critical to all subsequent

calculations and can be found in cartridge manufacturer literature.

An increase or decrease will affect downrange energy and drop.

SIGHT IN - This is the range at which you want your rife zeroed in.

You may enter any range you wish, and can see where the bullet will be

both up and down range from this point (BUL PATH).

SCOPE HT. - This is the distance your scope's axis lies above the rifle

bore. Most are .9" to 1.5". This affects only the BUL PATH calculation.

ANGLE - This is the angle up or down from a level shot (e.g. up a

mountain). It will affect DROP and BUL PATH only.

CROSSWIND - This is a component of the wind that is at 90 degrees to

the bullet's path. It is used to calculate drift downrange.

DISTANCE - Yards downrange from rifle. Note that these can be changed

at will. If 0 yards is entered, you may note the velocity is not

exactly equal to the muzzle velocity - this reflects small errors in

the velocity formula, but the errors are small.

VELOCITY - Calculated from muzzle velocity, distance, and the bullet's

ballastic coefficient.

ENERGY - Calculated from velocity at given range and bullet weight.

FLT TIME - Time from leaving muzzle to arrival at given distance.

DROP - Drop from line of bore. This is independent of scope or sight

settings, though affected by ANGLE.

BUL PATH - This is the distance of the bullet below or above the line

of sight at a given distance. Dependent on muzzle velocity, ballistic

coefficient, sight height, and angle.

BUL DRIFT - The drift induced by crosswind component at given range.

The trajectories calculated are valid for muzzle velocities from

1500 to 5000 feet per second. Outside of this range, error will

increase. Formulas were taken from :

NOSLER RELOADING MANUAL NUMBER TWO, 1981 pp298-300 and from

"Practical Ballistics", The RIFLE magazine, pp40-42, May-June 1970

*********************************************************************

.22cal   50gr   spitzer  .244          60gr  spitzer   .290

.24cal   85gr   spitzer  .375          100gr spitzer   .381

.25cal   100gr  spitzer  .409          120gr spitzer   .471

.264cal  120gr  spitzer  .422          140gr spitzer   .491

.277cal  100gr  spitzer  .315          130gr spitzer   .400

150gr  spitzer  .482          160gr semi-spit .435

.284cal  120gr  spitzer  .434          150gr spitzer   .520

175gr  semi-spitz partition  .508

.308cal  150gr  spitz boat tail  .408   150gr spitz partition .370

180gr  spitz boat tail  .507   180gr spitz partition .355

180gr  semi-spitz part. .355   200gr spitz partition .584

.338cal  210gr  spitz partition  .386   250gr roundnose part. .364

Above examples are of NOSLER hunting bullets, and will vary somewhat

from bullets of other manufacturers.

**********************************************************************

Below are coefficients for some REMINGTON bullets, these numbers

were obtained by working backwards with this spreadsheet using

factory published ballistic tables.

.22cal   50gr  psp .20   55gr   psp .23

.243cal  80gr  psp .30   100gr  psp .40

.25cal   100gr psp .32   120gr  psp .40

.277cal  100gr psp .28   130gr  psp .37

.284cal  150gr psp .37   180gr  psp .52

.308cal  125gr psp .30   150gr  sp  .21   150gr psp .35

170gr sp  .27   180gr  psp .42   220gr sp  .32

.32cal   185gr psp .32   220gr  psp .40

SP= soft point (e.g. round nose)      PSP= pointed soft point

**********************************************************************

You may send futher inquiries via MCI Mail. Hope you enjoy

the spreadsheet.  Chip Stratton, Dallas,Tx  11/85

#**************

You may send futher inquiries via MCI Mail. Hope you enjoy

the spreadsheet.  Chip Str