DOCUMENTATION OF THE CP/M MODEM PROGRAM

by Ward Christensen

MODEM is a program for transferring files between CP/M systems.

Files are sent a sector at a time.  The program prints mes-

sages showing how things are going.  A hexadecimal sector

number is printed, in a message: "WAITING FOR SECTOR NN" on

one system, and "SENDING SECTOR NN" on the other system.

Users of the program should list it and study it if they

are interested in learning more about it.  At a minimum, it

must be edited and modified for the particular serial

interface used for the modem.  As it stands, it is for my

MITS dual serial I/O board, ports 4 and 5.

To use the program, one user types:

MODEM SEND filename.filetype

The other user types:

MODEM RECEIVE filename.filetype

(SEND and RECEIVE may be abbreviated S and R)

The program has an error counter.  Each block is tried up

to 10 times.  If 10 occur, it asks if you want to retry

or quit.  If you are near the beginning of a file, you

might want to quit, and re-dial the phone to get a better

connection.  If you are further along in the transmission,

you would likely want to retry.  Note that the program does

not close the file if you say quit.

There is one known design error in this program:  After the

file is sent, the sending program sends a single EOT character.

The receiving program, upon receiving an EOT, takes closes

the file, and returns to CP/M.  This is normally not a problem,

but means that if the receiving program gets a glitch when

it is looking for the start of a block, it will shut down

early.  Also if the valid EOT from the sender is glitched,

the receiving program will wait, eventually timing out.

I have nearly completed a version of the program which does

better handshaking at EOF, which closes the file if you

quit, and which allows a 'second logical channel' allowing

keyboard-to-keyboard communications in parallel with the

file transfer.  Thus, you could type "I'll send you spat.com

when this is done, OK" and this message will be sent after

the next data block is sent or received.  The other user

will see the message and can reply.

Ward Christensen

c/o CACHE

Box 52

South Holland, Il. 60473