GENERAL COMMENTS CONCERNING THE PROGRAM "N-ELT 1":

COL BOB MATTSON

HQ CAP

DEAR BOB:

ENCLOSED IS A COPY OF THE ELT DIRECTION FINDING ASSISTANCE PROGRAM WHICH

WE PLAN TO USE ON THE NELT SEARCHES. NO GREAT SHAKES, BUT IT IS ABLE TO PROVIDE

SOME SIMPLE STATISTICS AND ACCESSORY FUNCTIONS.

I AM REASONABLY CONFIDENT THAT THIS PROGRAM IS CORRECT IN WHAT IT DOES,

AS WE HAVE TESTED IT AND UNDER MOST CONDITIONS IT WILL PERFORM WELL.

A COUPLE OF COMMENTS ARE IN ORDER, THOUGH.

1) THE CROSSINGS OF BEARINGS (CUTS) ARE CALCULATED BY SIMULTANEOUS

SOLVING OF THE EQUATIONS FOR A LINE (SLOPE-INTERCEPT VERSION: Y=MX+B, WHERE 'M'

IS THE SLOPE, 'B' IS THE Y AXIS INTERCEPT).  SINCE THE SLOPE OF A LINE IS

INFINITE WHEN THE LINE OF BEARING IS 0 DEGREES(360) OR 180 DEGREES, THE PROGRAM

CALLS THESE 359.9 AND 179.9 DEGREES RESPECTIVELY.  THE ERRORS SO INTRODUCED

SHOULD BE NEGLIGIBLE.

2) AT TIMES RICK HAS SEEN ZERO DIVIDE IN THE PROGRAM, THOUGH HE HASN'T

GIVEN ME DETAILS (LINE NUMBER ETC).

3) THE PROGRAM FLAGS ANY CUTS WHOSE COMPONENT BEARINGS ARE LESS THAN

ABOUT 30 DEG (TAN<.5) SINCE LARGE ERRORS ARE POSSIBLE WITH THESE CLOSE ANGLES.

IT SHOULD BE EMPHASIZED THAT IT IS DANGEROUS TO DISCARD ANY DATA AT ALL,

SINCE ONE CANNOT BE SURE THE ACCURACY WOULD INCREASE JUST BECAUSE THE BEARINGS

AGREE BETTER.  THE WARNING IS TO (1) ALLOW THE USER TO KNOW HIS DATA IS OF THIS

NATURE AND (2) RECOMPUTE AND TOSS OUT CUTS JUST TO SEE HOW THESE CUTS AFFECT

THE RESULT.

4) THE PROGRAM IS FOR ASSISTANCE IN DIRECTION FINDING, AND IS ONLY ONE

OF SEVERAL TOOLS WHICH SHOULD BE USED TO PLAN THE SEARCH.  THERE MUST BE A MAP

(WITH X-Y OVERLAY GRID) AND MANUAL EVALUATION OF THE RESULTS BY THE USER

(COORDINATOR) TO FIT THIS DATA BACK INTO THE REAL WORLD (MOUNTAINS, HOUSES,

REFLECTIONS OF SIGNAL AND PROPAGATION CONDITIONS).

5) THE RESULTS ARE ONLY AS GOOD AS THE DATA ENTERED. IF THE TEAM IS

UNABLE TO OPERATE ITS DF GEAR, OR THE COORDINATOR DOESN'T TAKE REFLECTIONS AND

OTHER CONDITIONS INTO ACCOUNT, ALL IS FOR NAUGHT.  THE BEARINGS SHOULD BE

TAKEN WITH PROVEN ACCURATE GEAR, FAMILIAR USERS, AND FROM THE BEST AVAILABLE

SITES. THERE IS NO SUBSTITUTE FOR PRACTICE AND EXPERIENCE.

HOPES FOR THE FUTURE INCLUDE A MORE COMPLETE STATISTICAL ANALYSIS OF THE

DATA, MASSAGING IT FOR A BETTER ESTIMATE.  EXPERIENCE WILL TELL US HOW TO

DO THIS AS WE GO ALONG.

ENJOY - CHRIS

#

DESCRIPTION OF THE 'N-ELT 1' PROGRAM.

LINE #		DESCRIPTION

10-640		INITIALIZATION, COMMENTS, MENUE

650-710		DISPATCH TABLE FROM MENUE

720-880		DATA ENTRY, SLOPE & INTERCEPT CALCULATION

890-1080	CALCULATION OF CUTS

1090-1230	CALCULATION OF MEAN FIX X,Y

1240-1640	CALCULATION OF PROBABILITY ELLIPSE

1650-1920	FIND FIELD TEAM VIA BEARINGS TO KNOWN LANDMARKS

1930-1970	CHANGE PROBABILITY CONTOUR

1980-2260	EDIT BEARING TABLE

2270-2340	DECISION ON CLOSE BEARING CUTS

2350-2760	BASELINE AVERAGING CODE

2770-2790	CHANGE DECLINATION

#

USERS GUIDE TO 'N-ELT 1'

1) RUN THE PROGRAM

2) WHEN MENU APPEARS, CHOOSE AN OPTION OR ELSE CARRIAGE RETURN WILL ENTER YOU

TO DATA ENTRY PORTION OF PROGRAM.  WHEN DECLINATION IS ASKED FOR, ENTER

THE CORRECTION OF MAGNETIC FROM TRUE BEARINGS, IF USER WILL BE ENTERING

MAGNETIC BEARINGS.  ENTER A WEST MAGNETIC VARIATION AS A NEGATIVE NUMBER

AND EASTERLY VARIATION AS A POSITIVE NUMBER.

3) THE PROGRAM WILL ASK FOR DATA, AND THE USER SHOULD ENTER DATA WITH X FROM

-50 TO 50,PREFERABLY FROM 0 TO 20 BUT REAL NUMBERS ARE OK, SO A MAP COVERING

5 MILES SQUARE, DIVIDED INTO 20X20 INCREMENTS, EACH INCREMENT FURTHER

DEVIDED BY SUB-INCREMENTS OF 10 GIVES YOU 40,000 CELLS.  IF THE X INPUT IS

GREATER THAN 50, THE MENU IS REDISPLAYED AND COMMANDS MAY BE GIVEN.  IF THE

X OR Y DATA IS INCORRECTLY ENTERED, THE INPUT MAY BE CANCELLED BY ENTERING

A BEARING ANGLE OF 360 OR GREATER.

4) WHEN 2 OR MORE BEARINGS ARE ENTERED, THE PROGRAM WILL CALCULATE CUTS AND

DISPLAY THEM. WHEN 3 OR MORE CUTS ARE OBTAINED, THE PROGRAM WILL COMPUTE THE

MEAN OF THE CUTS AND THE PROBABILITY CONTOUR ELLIPSE SIZE.

THE ELLIPSE IS DESCRIBED AS FOLLOWS:

THETA=ANGLE(SK) OF SKEW FROM 0 DEGREES(DUE NORTH) OF THE 'A' AXIS

SKEW=THE TILT OF THE ELLIPSE

A=1/2 THE LENGTH OF THE ELLIPSE MAJOR AXIS ('A' AXIS)

B=1/2 THE WIDTH OF THE ELLIPSE (MINOR AXIS)('B' AXIS)

X,Y=MEAN LOCATION OF CUTS

5) THE INTERPRETATION OF THE ELLIPSE IS YET UNCLEAR TO ME, EXCEPT THAT THE

RELATIVE SIZE OF THE ELLIPSE IS A MEASURE OF AGREEMENT OF THE DATA.  SMALL

ELLIPSE GIVES GREATER CONFIDENCE IN THE FIX.

6) TO FIND A FIELD TEAM, ASK THEM FOR TWO BEARINGS TO KNOWN LANDMARKS,

FIND THE X,Y OF THESE LANDMARKS, ENTER DATA AND COMPUTER PRINTS THEIR

ESTIMATED X,Y.  THESE BEARINGS MAY BE EITHER OPTICAL OR RADIO BEARINGS.

7) AVERAGING BEARINGS OVER A BASELINE WILL IMPROVE ACCURACY IN MARGINAL

CONDITIONS.  START BY ENTERING ALL THE X AND Y LOCATIONS TO BE AVERAGED

AND THE BEARINGS FROM THOSE LOCATIONS.  IF ALL THE BEARINGS ARE TAKEN FROM

THE SAME GENERAL LOCATION, THEN ENTER THAT LOCATION FOR EACH BEARING.

AFTER YOU HAVE ENTERED ALL THE LOCATIONS/BEARINGS ENTERING A '51,0,0'

WILL CAUSE OF THIS THE PROGRAM TO BEGIN CALCULATIONS.  WE ENCOURAGE THE USE

OF THIS TECHNIQUE EITHER OVER SMALL DISTANCES (EG 300 FEET)OR LONG DISTANCES

(MILES) WHEN THE DATA DICTATES ITS USE.

8) UP TO THIRTY BEARINGS/LOCATIONS MAY BE ENTERED IN THIS PROGRAM.  MEMORY

REQUIREMENTS MAY BE REDUCED BY DECREASING THIS NUMBER (CHANGE THE DIMENSION

STATEMENTS) AND BY INTEGER ARITHMETIC.

- CHRIS