DFOCO DOCUMENTATION

COPYRIGHT 1979

BY S. J. SINGER

DFOCO IS A FAST FORMAT AND COPY UTILITY FOR CP/M AND THE DELTA

FLOPPY DISK CONTROLLER. IT PERMITS FORMATTING DISKS IN BOTH THE STANDARD

IBM FORMAT AND A VARIETY OF NON STANDARD FORMATS AS WELL AS COPYING

ENTIRE DISKS OR ONLY SPECIFIED TRACKS FROM ONE DISK TO ANOTHER.

IN ADDITION DFOCO PERMITS RELATIVELY EFFICIENT SINGLE DISK DRIVE COPIES

AND WILL COPY FORMATS AND DISPLAY THE FORMAT ON ANY SPECIFIED TRACK OF

ANY DISK. DFOCO SUPPORTS THE USE OF 4 DISK DRIVES.

DFOCO MAY BE USED BOTH SINGLE DENSITY (FM) AND DOUBLE DENSITY (MFM).

THE OPERATIONS PERFORMED ARE IN GENERAL THE SAME BUT MAY BE SLIGHTLY

MORE LIMITED IN THE CASE OF DOUBLE DENSITY.

DFOCO WILL RUN ON ANY 8080 OR Z-80 BASED SYSTEM USING THE DELTA

DISK CONTROLLER. THE PROGRAM USES CP/M FOR ALL CONSOLE INPUT-OUTPUT,

HOWEVER ALL DISK OPERATIONS ARE DIRECTLY EXECUTED BY DFOCO. THIS IS

NECESSARY BECAUSE DFOCO USES NON CP/M COMMANDS LIKE DISK TRACK READ,

AND DFOCO KEEPS CONTROL OF ALL DISK ERRORS TO FACILITATE ACCOUNTING

AND ERROR RECOVERY. IT SHOULD BE POSSIBLE HOWEVER, TO MODIFY DFOCO

TO RUN WITH ANY WESTERN DIGITAL 1771 OR 1791 BASED DISK CONTROLER.

THERE ARE TWO DIFFERENT VERSIONS OF DFOCO, ONE FOR STANDARD DISK DRIVES

AND THE OTHER FOR FAST SEEK PERSCI DRIVES. THE SINGLE DENSITY OPERATIONS

WILL BE DESCRIBED IN DETAIL FIRST.

* * * * * * *  SINGLE DENSITY OPERATIONS  * * * * * * *

VALIDATING DISKS

IT IS OFTEN USEFUL TO BE ABLE TO TELL WHETHER OR NOT THE RECORDING

SURFACE OF A DISK IS COMPLETELY INTACT BEFORE USING THE DISK. THIS IS

ESPECIALLY TRUE SINCE WHILE CP/M ALWAYS CHECKS FOR READ ERRORS THERE IS

NO CHECK DONE FOR WRITE ERRORS. YOU WILL NOT KNOW YOU HAVE A BAD COPY

OF A FILE UNTIL THE FIRST TIME YOU TRY TO READ IT AGAIN WHICH MAY BE DAYS

OR EVEN MONTHS LATER.

VALIDATING A DISK CONSISTS SIMPLY OF READING EACH SECTOR ON THE

DISK AND VERIFYING THAT THE CRC IS CORRECT. THIS SIMPLE PROCESS NOT ONLY

DETECTS PHYSICAL DAMAGE TO THE MEDIA SUCH AS SCRATCHES BUT ALSO FORMATTING

ERRORS SOMETIMES CAUSED BY NOISE TRANSIENTS, ERRORS WHICH WILL DISAPPEAR

UPON REFORMATTING THE DISK. THE FOLLOWING ARE EXAMPLES OF VALIDATION COMMANDS.

VALID			(DEFAULT IS DRIVE A:)

VALID C:

VALID D:  RETRY 20

THE PROGRAM WILL READ THE ENTIRE DISK AND REPORT ANY READ ERRORS.

IF A CRC ERROR OCCURS THE SECTOR WILL BE REREAD A MAXIMUM OF EITHER

THE NUMBER OF RETRYS SPECIFIED OR 10 TIMES IF NO RETRY COUNT IS ENTERED.

BOTH THE TOTAL NUMBER OF PERMANENT OR HARD ERROR AND RETRYS ARE REPORTED

AT THE END OF THE VALIDATION PROCESS.

SUCCESSFUL VALIDATION DRIVE B:

OR

PERMANENT READ ERROR TRACK 17 SECTOR 5 DRIVE B

PERMANENT READ ERROR TRACK 42 SECTOR 19 DRIVE B

THERE WERE 27 RETRYS AND 2 HARD ERRORS

COPYING DISKS

ALL OR PART OF A DISK MAY BE COPIED BETWEEN ANY SPECIFIED DRIVES.

THE FOLLOWING ARE EXAMPLES OF TYPICAL COPY COMMANDS:

COPY				(DEFAULT IS A: TO B:)

COPY A: TO C:

COPY D:  TO  B:  RETRY 20

COPY A: TO C: TRACK 0-1

COPY B: TO D:  TRACK  3 - 20

COPY A: TO A:			(SINGLE DISK TRANSFER)

DFOCO WILL RESPOND WITH AND ACKNOWLEDGMENT AND INSTRUCTIONS. FOR EXAMPLE:

COPYING DISK A TO DISK B

TYPE RETURN TO START

THE PROGRAM WILL HALT AND WAIT FOR DISKS TO BE CHANGED ETC. AND

WILL BEGIN THE ACTUAL COPY OPERATION AFTER RETURN IS TYPED. IF YOU HAVE

MADE AN ERROR AND WISH TO REENTER THE COMMAND TYPE CONTROL C. THE COPY

OPERATION IS OPTIMIZED FOR SPEED. THE PROGRAM WILL DETERMINE THE MAXIMUM

AMOUNT OF MEMORY AVAILABLE AND READ WHOLE TRACKS FROM THE SOURCE DISK

UNTIL THE MEMORY IS FILLED. THEN THE ENTIRE MEMORY BUFFER IS WRITTEN

ONTO THE DESTINATION DISK. ALL TRANSFERS ARE VERIFIED. AS EACH TRACK IS

WRITTEN ONTO THE DESTINATION DISK IT IS READ BACK AND COMPARED BYTE BY

BYTE WITH THE CONTENTS OF THE MEMORY BUFFER. ANY ERRORS ARE REPORTED.

IF THE SOURCE AND DESTINATION ARE THE SAME DFOCO WILL FIRST

RESPOND WITH A REQUEST THAT YOU VERIFY THAT YOU REALLY WANT TO PERFORM

A SINGLE DRIVE COPY. IF THE REQUEST IS ACKNOWLEDGED THE PROGRAM

WILL INDICATE WHICH DISK TO MOUNT IN THE SPECIFIED DRIVE.

AS WITH TWO DRIVE COPIES THE PROGRAM WILL FILL THE ENTIRE MEMORY BUFFER

WITH DATA. THE PROGRAM WILL THEN HALT AND NOTIFY YOU TO CHANGE DISKS.

THE NUMBER OF DISK CHANGES REQUIRED WILL OF COURSE DEPEND UPON THE

AMOUNT OF MEMORY AVAILABLE. WITH A 48K SYSTEM 8 SWAPS ARE REQUIRED, WITH

A 64K SYSTEM ONLY 6 ARE NEEDED.

COPYING DISKS WITH DIFFERENT SIZE SECTORS

DFOCO WILL COPY DISKS WITH SECTORS OF 128, 256 OR 512 BYTES. THE

PRIMARY LIMITATION OF THE PROGRAM IS THAT BOTH DISKS MUST HAVE THE SAME

SIZE SECTORS. THE SIZE SHOULD BE SPECIFIED IN THE COPY COMMAND.

COPY A: TO C:  SIZE 256

COPY B: TO A: SIZE 512

COPY C:  TO D:  SIZE 128

SPECIFYING A SECTOR SIZE OF OTHER THAN 128, 256 OR 512 BYTES WILL

GENERATE AN ERROR MESSAGE.

ERROR HANDLING DURING COPY OPERATIONS

ERROR HANDLING AND RECOVERY IS AN EXTREMELY INPORTANT PART OF ANY

DISK FILE MANAGEMENT SYSTEM. DFOCO IS DESIGNED TO PERMIT THE MAXIMUM RECOVERY

OF DATA FROM DAMAGED OR "CRASHED" DISKS. TWO TYPES OF ERRORS OCCUR DURING

COPY OPERATIONS, ERRORS IN READING FROM THE SOURCE DISK AND ERRORS WRITING

TO THE DESTINATION DISK. READ ERRORS ARE BY FAR THE MOST COMMON.

IF A READ ERROR OCCURS DFOCO WILL ATTEMPT TO REREAD THE SECTOR 10

(OR RETRY) TIMES. IF THE READ ERROR PERSISTS ONE OF TWO ACTIONS MAY BE CHOSEN.

THE DEFAULT IS TO FILL THE SECTOR WITH E5H AND WRITE IT ON THE DESTINATION

DISK. THE OTHER CHOICE IS SIMPLY TO ACCEPT THE DATA AS READ IN AND WRITE IT ON

THE DESTINATION DISK. THIS IS THE NOFILL OPTION. NOFILL PERMITS THE MAXIMUM

DATA RECOVERY BUT REQUIRES CAREFUL INSPECTION OF THE SECTOR CAUSING THE ERROR

SINCE THE ERRORS MAY NOT BE OBVIOUS.

WRITE ERRORS ARE HANDLED IN A SOMEWHAT DIFFERENT FASHION. SINCE THERE

IS USUALLY NO VALUABLE DATA ON THE DESTINATION DISK, THE NORMAL CHOICE IS TO

NOT PERMIT COPY OPERATIONS IF WRITE ERROR OCCUR. DFOCO WILL ATTEMPT TO WRITE

A SECTOR 10 (OR RETRY) TIMES. IF THE ERROR PERSISTS, DFOCO WILL STOP THE COPY

OPERATION AND VALIDATE THE DESTINATION DISK THUS REPORTING ALL BAD SECTORS.

IT IS POSSIBLE TO COPY IN THE PRESENCE OF WRITE ERRORS BY TURNING OFF THE

WRITE VERIFICATION, THE NOVERIFY OPTION. HOWEVER SINCE DISKS HAVE BECOME

RELATIVELY INEXPENSIVE THIS OPTION IS PROBABLY UNWISE EXCEPT IN VERY SPECIAL

CASES. SOME EXAMPLES ARE AS FOLLOWS:

COPY B: TO C: NOFILL   RETRY 25

COPY A: TO B: TRACK 0-5 NOVERIFY

COPY D: TO A: NOVERIFY NOFILL

DISK FORMATTING AND MAPPING

THE STANDARD IBM FORMAT FOR 8 INCH FLOPPY DISKS IS GIVEN IN THE

WESTERN DIGITAL DOCUMENTATION FOR THE 1791 DISK CONROLLER CHIP AND IN VARIOUS

IBM DOCUMENTS. THE USUAL FORMAT IS "SOFT SECTORED". THIS MEANS ESSENTIALLY

THAT THE TRACK AND SECTOR NUMBERS ARE ACTUALLY WRITTEN ON THE DISK AS DATA

RATHER THAN BEING DETERMINED BY THE PRESENCE OF PHYSICAL INDICATORS SUCH AS

HOLES IN THE DISK. FORMATTING A DISK CONSISTS OF WRITING BOTH THE TRACK

AND SECTOR NUMBERS AS WELL AS CLOCKING INFORMATION FOR THE CONTROLER ON THE

DISK. IT IS IMPORTANT TO REMEMBER THAT A "BLANK" DISK IS BY NO MEANS REALLY

BLANK. RATHER IT CONTAINS A GREAT DEAL OF FORMATTING INFORMATION WITHOUT

WHICH THE CONTROLER IS TOTALLY UNABLE TO READ IT.

BECAUSE OF THIS "SOFT SECTORING" IT IS SOMETIMES POSSIBLE AND OFTEN

USEFUL TO CHANGE THE FORMAT TO ALLOW INCREASED AMOUNTS OF DATA TO BE WRITTEN

ON THE DISK OR HIGHER SPEED OF OPERATION. THERE ARE TWO POSSIBLE CHANGES

THAT ARE USEFUL, CHANGING THE SIZE OF THE SECTORS OR CHANGING THEIR ORDERING.

THIS VERSION OF DFOCO SUPPORTS THREE DIFFERENT SECTOR SIZES USING

THE "IBM" SOFT SECTOR FORMAT. SECTORS MAY BE FORMATED WITH THE NORMAL

26 SECTORS OF 128 BYTES OR WITH 16 SECTORS OF 256 BYTES OR 8 SECTORS OF

512 BYTES EACH. USING 256 OR 512 BYTE SECTORS ALLOWS APPROXIMATELY

20% MORE DATA TO BE WRITTEN ON A SINGLE DISK, HOWEVER THERE ARE AT PRESENT

FEW PROGRAMS WHICH WILL SUPPORT THE USE OF LARGER SECTORS.

DFOCO ALSO SUPPORTS A WIDE VARIETY OF SECTOR ORDERINGS.

THE DESIGN OF CP/M DATES FROM A TIME WHEN DISK CONTROLLERS WERE

QUITE SLOW AND COMPUTER MEMORIES SMALL AND VERY COSTLY. SMALL MEMORIES

DICTATED SMALL SECTOR SIZES ON DISKS SINCE THE LARGER THE SECTORS THE LARGER

THE MEMORY BUFFERS REQUIRED. SLOW CONTROLERS MEANT THAT HAVING READ A SECTOR

FROM THE DISK IT WAS NECESSARY TO WAIT BEFORE ANOTHER SECTOR COULD BE READ.

DELAYS OF 5 SECTOR TIMES (ABOUT 25 MSEC) WERE COMMON. THUS CP/M IS SET UP

TO READ EVERY 6TH SECTOR AROUND THE DISK. THIS STRATEGY UNFORTUNATELY IS

FAR FROM OPTIMAL FOR PRESENT DAY CONTROLLERS WHICH CAN READ CONSECUTIVE

SECTORS FROM A DISK WITH EASE. NOTE THAT DFOCO WHICH READS AN ENTIRE TRACK

IN A SINGLE DISK REVOLUTION IS OVER 5 TIMES AS FAST AS PIP. UNFORTUNATELY

CP/M STANDARD SYSTEM PROGRAMS SUCH AS PIP AND THE ASSEMBLER CAN ONLY BE

SPEEDED UP A SMALL AMOUNT, ABOUT 20 PER CENT, BY CHANGING DISK FORMATS ALONE.

HOWEVER, NEW PROGRAMS WRITTEN TO TAKE ADVANTAGE OF FASTER CONTROLERS CAN

BE SPEEDED UP A GREAT DEAL MORE.

MAPPING DISK FORMATS

WHEN EXPERIMENTING WITH NONSTANDARD FORMATS IT IS OFTEN VERY USEFUL

TO BE ABLE TO READ AND DISPLAY THE ACTUAL DISK FORMAT. YOU CAN'T NECECESSARILY

TELL WHATS ON A DISK BY LOOKING AT THE LABEL. THE MAP COMMAND READS THE FORMAT

FROM A SINGLE TRACK ON A SPECIFIED DISK AND DISPLAYS IT. THERE ARE 26 SECTORS

PER TRACK IN THE STANDARD IBM FORMAT WHICH ARE NUMBERED IN SEQUENTIAL ORDER.

TO DISPLAY THE SECTOR ORDERING TYPE:

MAP			(DEFAULT IS TRACK 0 DRIVE A:)

MAP C: TRACK 76

THE PROGRAM WILL READ THE SPECIFIED TRACK AND DISPLAY THE PHYSICAL TO

LOGICAL SECTOR MAPPING. THE PHYSICAL SECTORS STARTING FROM THE SINGLE INDEX

HOLE IN THE DISK ARE SIMPLY NUMBERED 1 THRU 26. THE CORRESPONDING LOGICAL

SECTORS ACTUALLY WRITTEN ON THE DISK ARE DISPLAYED BESIDE THE PHYSICAL SECTOR

NUMBER.  FOR DISKS FORMATTED WITH LESS THAN 26 SECTORS, THE UNUSED LOGICAL

SECTOR NUMBERS DISPLAY AS A '-'.

OCCASIONALLY DFOCO WILL DISPLAY OBVIOUSLY INCORRECT MAPPING DATA, FOR

EXAMPLE TRACK 404 SECTOR NUMBER 201. THIS MEANS THE FORMAT ON THE DISK IS

INCORRECT. THE DISK CONTROLLER WILL OFTEN READ THESE DISKS CORRECTLY

BUT IT IS USUALLY A GOOD IDEA TO COPY THE DATA TO A CORRECTLY FORMATTED DISK.

FORMATTING PROBLEMS OF THIS TYPE OFTEN SHOW UP WHEN YOU ATTEMPT TO READ

DISKS PRODUCED ON ANOTHER COMPUTER SYSTEM. IF THE HEAD ALLIGNMENT IF ONLY

SLIGHTLY DIFFERENT FROM YOURS YOU MAY GET MAPPING ERRORS EVEN THOUGH YOU ARE

USUALLY ABLE TO READ THE DATA CORRECTLY.

FORMATTING DISKS

DFOCO PERMITS FORMATTING DISKS ON ANY DRIVE SUPPORTED BY CP/M. EITHER

THE ENTIRE DISK OR SPECIFIED TRACKS MAY BE FORMATTED. IT IS EVEN POSSIBLE

TO WRITE DIFFERENT FORMATS ON DIFFERENT TRACKS OF THE SAME DISK.

STANDARD FORMAT

TO WRITE THE STANDARD IBM FORMAT ON A DISK TYPE

FORMAT				(DEFAULT IS DRIVE A)

FORMAT B:

FORMAT C:  TRACK  20-40

THE PROGRAM WILL HALT AND THEN RESPOND WITH:

STANDARD IBM 3740 FORMAT

INSERT DISK TO BE FORMATTED IN DRIVE A

TYPE CARRIAGE RETURN

NON STANDARD FORMATS MAY HAVE ALMOST ANY FORM YOU SPECIFY. THE FIRST

VARIATION IS TO OFFSET THE SAME FORMAT FROM TRACK TO TRACK. THIS IS USEFUL

TO COMPENSATE FOR THE TIME IT TAKES TO STEP THE HEAD FROM ONE TRACK TO

ANOTHER AND IS ONE OF THE TECHNIQUES USED IN DFOCO TO INCREASE THE COPY SPEED.

THE FOLLOWING IS AN EXAMPLE OF TRACK OFFSETTING:

FORMAT B: OFFSET 5

THIS RESULTS IN THE FOLLOWING FORMAT ON THE DISK

TRACK 0		TRACK 1		TRACK 2		ETC.

SECTOR		  1		  6		  11

SECTOR		  2		  7		  12

SECTOR		  3		  8		  13

SECTOR		  4		  9 		  14

A SECOND FORMATTING VARIATION IS TO SKEW THE SECTORS BY A CONSTANT

AMOUNT. THIS CAN BE SPECIFIED AS FOLLOWS.

FORMAT B: SKEW 3

THIS WILL RESULT IN A DISPLAY OF THE PHYSICAL TO LOGICAL SECTOR MAPPING

AND PERMIT CHANGING THE SPECIFICATIONS BEFORE WRITING THE FORMAT ON THE DISK.

PHYSICAL	LOGICAL		PHYSICAL	LOGICAL

SECTOR		SECTOR		 SECTOR		SECTOR

1		  1		   14		  14

2		  4		   15		  17

3		  7		   16		  20

4		 10		   17		  23

5		 13		   18		  26

6		 16		   19		   3

7		 19		   20		   6

8		 22		   21		   9

9		 25		   22		  12

10		  2		   23		  15

11		  5	  	   24		  18

12		  8		   25		  21

13		 11		   26		  24

TYPE RETURN TO FORMAT, SECTOR NO TO CORRECT

TYPING A SECTOR NUMBER ALLOWS THE LOGICAL SECTOR NUMBER TO BE CHANGED.

BEFORE USING A SECTOR NUMBER IT MUST BE FIRST SET TO ZERO SINCE THE PROGRAM

CHECKS AND DOES NOT PERMIT TWO SECTORS WITH THE SAME NUMBER. THE SECTOR MAPPING

IS REDISPLAYED FOR VERIFICATION AFTER EACH CHANGE.

THE FINAL FORMATTING OPTION IS SIMPLY TO TYPE IN THE PHYSICAL TO LOGICAL

SECTOR MAPPING FOR EACH SECTOR. TO SELECT THIS OPTION TYPE:

SPECIAL FORMAT A:

THE PROGRAM WILL RESPOND BY DISPLAYING EACH PHYSICAL SECTOR NUMBER

AND WAITING FOR THE CORRESPONDING LOGICAL SECTOR NUMBER TO BE ENTERED. AGAIN

THE PROGRAM CHECKS THE SECTOR NUMBERS AS ENTERED AND WILL NOT ALLOW THE SAME

SECTOR NUMBER TO BE USED TWICE.

NOTE THAT THE VARIOUS OPTIONS MAY BE COMBINED IF DESIRED.

SPECIAL FORMAT B: TRACK 0-1

FORMAT C: OFFSET 6 SKEW 3 TRACK 10-76

IT IS EVEN POSSIBLE TO COPY THE FORMAT FROM ONE DISK TO ANOTHER

COPY FORMAT A: TO B:

COPY FORMAT B: TO D:  TRACK 10

FORMATTING WITH DIFFERENT SIZE SECTORS

THE DEFAULT SECTOR SIZE GENERATED BY DFOCO IS 128 BYTES, HOWEVER THE

PROGRAM WILL ALSO FORMAT TRACKS WITH 16 SECTORS OF 256 BYTES OR 8 SECTORS OF

512 BYTES. THE SECTOR SIZE IS SPECIFIED BY THE SIZE PARAMETER.

FORMAT B: SIZE 512

SPECIAL FORMAT A:  SIZE 256

FORMAT C: OFFSET 2 SIZE 512

CAUTION MUST BE USED WITH THE SPECIAL FORMAT OPTION SINCE DFOCO WILL

ALLOW SECTOR NUMBERS GREATER THAN THE NUMBER OF SECTORS ON A TRACK. THE

1791 WILL ACTUALLY READ SECTORS NUMBERED IN THIS FASHION. FOR EXAMPLE

A TRACK MAY BE FORMATTED WITH 8 512 BYTE SECTORS NUMBERED 11 THRU 18, HOWEVER

DISKS WRITTEN IN THIS FASHION MAY NOT VALIDATE CORRECTLY. IT IS ALSO POSSIBLE

TO FORMAT A DISK WITH DIFFERENT SIZE SECTORS ON DIFFERENT TRACKS. AGAIN, DISKS

WRITTEN IN THIS FASHION MAY NOT VALIDATE CORRECTLY.

REGARDLESS OF THE SECTOR SIZE AND MAPPING CHOSEN, TRACK 0 OF THE

DISK IS ALWAYS WRITTEN IN THE STANDARD IBM 3740 FORMAT. THIS IS DONE TO

FACILITATE IDENTIFICATION OF THE DISK FORMAT BY A PROGRAM. READ AND WRITE

OPERATIONS TO A DISK MAY ACTUALLY BE IMPOSSIBLE AND "HANG" THE CONTROLLER

IF THE PROGRAM EXPECTS A FORMAT THAT IS NOT PRESENT. THIS IS ESPECIALLY

TRUE IF THE INCORRECT DENSITY IS SELECTED.

PROGRAM TIMING

THE FOLLOWING TIMING FIGURES ARE TYPICAL OF A 64K SYSTEM AND WILL BE

SLIGHTLY HIGHER FOR SMALLER SYSTEMS. THE COPY TIMINGS VARY WITH DISK FORMATS.

IF THE FORMAT IS NON STANDARD BUT THE SAME ON BOTH DISKS THE TIMES ARE THE

SAME AS FOR STANDARD FORMATS BUT IF THE SECTOR FORMATS ARE DIFFERENT ON THE

TWO DISKS THE COPY TIMES WILL BE INCREASED. THE FOLLOWING TIMINGS ARE FOR

SINGLE DENSITY ONLY.

VALIDATING		17 SEC

FORMATTING		43 SEC (INCLUDES VALIDATION)

COPY SAME FORMAT	46 SEC

COPY DIFFERENT FORMAT	90 SEC AVERAGE

300 SEC WORST CASE

IF THE FORMAT IS DIFFERENT ON DIFFERENT TRACKS OF A DISK IT MAY BE

POSSIBLE TO INCREASE COPY SPEED WITH THE USING OPTION. SINCE A WRITE OPERATION

TAKES TWICE AS LONG AS A READ OPERATION THE PROGRAM CAN OPTIMIZE THE COPY

BY READING THE TRACK FORMAT FROM THE DESTINATION DISK AND USING IT TO

CONTROL READING AND WRITING. A SAMPLE COPY WITH THIS OPTION IS

COPY A: TO B: USING 3

THIS CAUSES TRACK 3 TO BE READ FROM THE DESTINATION DISK AND USED TO

CONTROL THE COPY OPERATION. THIS CAN OFTEN DOUBLE THE COPY SPEED IF THE FORMATS

ARE DIFFERENT ON THE TWO DISKS.

* * * * * * *  DOUBLE DENSITY OPERATIONS  * * * * * * *

WITH A FEW EXCEPTIONS THE SAME OPERATIONS ARE AVAILABLE DOUBLE

DENSITY AS ARE AVAILABLE SINGLE DENSITY. DOUBLE DENSITY MAY BE SELECTED EITHER

BY ADDING A D TO THE DESIRED COMMAND OR BY PLACING THE CODE DD ANYWHERE

ON THE COMMAND LINE. SOME EXAMPLES:

DVALID B:

VALID A: SIZE 512  DD

DCOPY A: TO B:  SIZE 256

DCOPY

COPY B: TO D: DD SIZE 512

DFORMAT

DFORMAT B: SIZE 512

FORMAT C: SIZE 256  DD

THE FOLLOWING EXCEPTIONS SHOULD BE NOTED. THE MAPPING FUNCTION

IS NOT AVAILABLE DOUBLE DENSITY. IT IS DIFFICULT TO DO AN ACCURATE

TRACK READ OF THE FORMAT ON A DOUBLE DENSITY DISK DUE TO THE FORMAT USED.

EVEN WHERE NOT SPECIFICALLY PROHIBITED, IT IS RECOMMENDED THAT OPERATIONS

REQUIRING TRACK READ SUCH AS COPY FORMAT OR COPY USING NOT BE ATTEMPED.

PROBLEMS IN READING THE FORMAT FROM DISKS MAY CAUSE UNPREDICTABLE RESULTS.

THE SPECIAL FORMAT FUNCTION IS NOT AVAILABLE DOUBLE DENSITY.

THE STANDARD FORMATS PROVIDED BY THE PROGRAM DOUBLE DENSITY ARE AS FOLLOWS:

128 BYTE SECTORS -

TRACK 0 STANDARD 3740 FORMAT SINGLE DENSITY. TRACKS 1-76 HAVE 51

SECTORS WRITTEN IN A 6 TO 1 INTERLACE PATTERN FOR USE WITH CP/M VERS 1.4.

THE SECTOR ORDER IS AS FILLOWS:

1,18,35,10,27,44,2,19,36,11,28,45,3,20,37,12,29,46,4,21,38,13,30,47,5,

22,39,14,31,48,6,23,40,15,32,49,7,24,41,16,33,50,8,25,42,17,34,51,9,26,43

ONE ADDITIONAL FEATURE IS PROVIDED FOR USE WITH CP/M. THE LAST BYTE

OF DATA ON TRACK ZERO SECTOR ONE IS WRITTEN WITH A SPECIAL FLAG BYTE TO

INDICATE THE FORMAT OF THE REST OF THE DISK. THE CODES ARE

E5	SINGLE DENSITY  (E5 IS THE STANDARD FILL CHAR)

DD	DOUBLE DENSITY 51 SECTORS PER TRACK

4D	"QUAD" DOUBLE DENSITY DOUBLE SIDED DISK

TO INSERT THE "QUAD" CODE INTO SECTOR ONE OF A 128 BYTE SECTOR DISK

THE FOLLOWING EXAMPLES MAY BE USED:

DFORMAT C:  QUAD

DFORMAT A:  QUAD

256 BYTE SECTORS -

DOUBLE DENSITY DISKS FORMATTED WITH 256 BYTE SECTORS ARE IN STANDARD

IBM FORMAT. 26 SECTORS OF 256 BYTES. TRACK ZERO IS SINGLE DENSITY 3740 FORMAT.

THERE IS NO FLAG BYTE IN SECTOR 1 OF TRACK ZERO

512 BYTE SECTORS -

DOUBLE DENSITY DISKS FORMATTED WITH 512 BYTE SECTORS HAVE 16 SECTORS

PER TRACK. TRACK ZERO IS STANDARD 3740 FORMAT. NO FLAG BYTE ON TRACK ZERO

THE OFFSET AND SKEW FUNCTIONS WORK AS BEFORE ALTHOUGH THE EFFECT OF

FURTHER SKEWING THE ALREADY INTERLACED 128 BYTE PATTERN MAY BE CONFUSING.

COPYING DISKS DOUBLE DENSITY IS ALMOST EXACTLY THE SAME AS COPYING

THEM SINGLE DENSITY. TWO SIMPLE RULES MUST BE OBSERVED.

1. BOTH DISKS MUST BE OF THE SAME DENSITY.

2. BOTH DISKS MUST HAVE THE SAME SECTOR SIZE.

A VIOLATION OF THES RULES MAY CAUSE THE CONTROLLER TO "HANG" REQUIRING THE

COMPUTER TO BE RESET TO RECOVER.

IT WILL BE NOTED THAT COPY OPERATIONS ON DOUBLE DENSITY DISKS HAVING

51 SECTORS IS MUCH SLOWER THAN OTHER COPY OPERATIONS. THIS IS BECAUSE THE

SECTORS ON THESE DISKS ARE WRITTEN SO CLOSE TOGETHER THAT THE 1791 CONTROLLER

DOES NOT HAVE TIME TO WRITE CONSECUTIVE SECTORS ALTHOUGH IT IS ABLE TO

READ CONSECUTIVE SECTORS IN THIS FORMAT. COPY OPERATIONS ON DISKS IN THIS

FORMAT READ AND WRITE EVERY OTHER SECTOR THUS REQUIRING TWICE AS MANY DISK

REVOLUTIONS AND TWICE THE TIME.

DOUBLE SIDED OR "QUAD" DISKS ARE NOT AUTOMATICALLY COPIED BY DFOCO

IN A SINGLE OPERATION BUT REQUIRE TWO COPY OPERATIONS. TO COPY A "QUAD"

DISK YOU MIGHT TYPE:

DCOPY A: TO C:		(COPY THE FRONT SIDE)

DCOPY B: TO D:		(COPY THE BACK SIDE)

A SINGLE "QUAD" COPY OPERATION WILL BE ADDED TO DFOCO LATER.

Known bugs:

DFOCO WON'T COPY ON A SINGLE DISK SYSTEM. IT ALSO DOESN'T

COPY TRACK ZERO CORRECTLY IF THE DISK IS FORMATTED IN OTHER

THAN 128 BYTE SECTORS. MANY SECTIONS WERE JUST COPIED FROM THE

OLD FOCO AND HAVEN'T EVEN BEEN TRIED NEVER MIND DEBUGGED.

I JUST FIXED THE PARTS I ACTUALLY USE MYSELF NOW AND

DISTRIBUTED IT TO OTHERS WHO MIGHT LIKE TO USE THOSE FEATURES.

STRAIGHT FORMAT COPY AND VALIDATE WORK FOR THREE SECTOR SIZES

WITH THE EXCEPTIONS NOTED ABOVE.