***  North Star BIOS and Support Programs  ***

***

*** These routines copyright (c) 1980, 1981, 1982 by Steve Bogolub,

*** 2338 S. Scoville Ave., Berwyn, IL 60402.  Jade Double D (tm)

*** disk handlers contributed to the public domain for use on Jade

*** equipment by special permission of Jade Computer Products.

*** This software may be used freely for non-commercial purposes

*** only, and may not be sold.

***

In response to the lack of source routines available in the public domain for

driving the North Star floppy disk controllers, a full set of routines for

supporting these controllers under CP/M 2.2 (tm) was developed.  The original

intent was to allow convenient modification of the driver software when

non-standard drives or additional controllers were desired under the control

of a single CP/M 2.2 system.  The resulting routines fulfill these goals, and

at the same time provide format compatibility with the industry-standard

Lifeboat Associates double density and single density single-sided North Star

CP/M versions 1.4 and 2.2.

The author's system includes a North Star MDS-A double density controller

with two Shugart Associates SA-400 double density, single-sided disk drives,

packaged in a standard North Star Horizon (tm) computer.  Additional hardware

includes a Jade Double D (tm) double density disk controller with two Shugart

Associates SA-800R double density, single-sided disk drives.  The primary

serial I/O devices are provided by a QT IO+ (tm) board, with two serial Intel

8251-type devices and programmable baud rates, among other features.  The

primary list device is a Teletype (tm) Model 40 printer, driven through the

Horizon right serial port with the Model 40 simplified EIA interface (minimal

handshake lines).  A total of 64K of static RAM memory is in the machine, of

which 56K is utilized directly by CP/M 2.2, 4K is overlapped (and thus

unavailable) by the two controllers and the boot PROM, and 4K is reserved in

the address space above the controllers for special-purpose programs.  Every

effort has been made to maintain compatibility with a standard Horizon

configuration.	Therefore, while the author's system is fully supported, a

system disk generated by these routines will boot up unmodified on a standard

Horizon with a minimum of 56K of memory, double density controller, and at

least one SA-400 disk drive or equivalent.

The routines required to support CP/M 2.2 on a North Star controller include

a BIOS module for CP/M 2.2, a system generation utility for writing out the

system to the system tracks of a North Star diskette, and a format utility for

initializing a diskette in the CP/M format.  Each of these routines is included

in a separate module in the author's system.  All three modules are coded using

the Intel 8080 assembler mnemonics and source format compatible with the

assembler program ASM, which is provided as a standard utility with all copies

of Digital Research CP/M 2.2.  With one exception, all 8080-compatible code is

used.  The Z-80 (tm) "LD A,I" instruction is used on entry to the disk driver

routines in BIOS to determine the state of the Z-80 CPU interrupt enable

flip-flop for the purpose of saving and restoring it.  All other code in the

North Star routines is 8080-compatible, and even the single Z-80 instruction

should execute harmlessly on an 8080.

The three primary source modules of the author's system are listed below:

(1) NBIOSxx.ASM 	This is the BIOS module.  It supports both the North

Star and the Jade controllers, with the North Star

drives being A: and B:, and the Jade drives being C: and D:.  The Jade Double D

is a soft-loaded buffered controller containing its own resident Z-80.	The 1K

of control code is stored on the North Star system tracks, and loaded out into

the Jade Double D memory on a cold boot.  This BIOS provides the ability to

read and write Lifeboat-compatible single and double density, single-sided

5 1/4" diskettes, as well as industry-standard IBM 3740-compatible single

density, single-sided 8" diskettes, Altos double density Mode 2 8" format,

Morrow Designs Disk Jockey (tm) 2D 1024 bytes/sector double density,

single-sided 8" diskettes, and all standard Jade Double D formats for

single-sided or double-sided 8" drives.  Due to the large amount of code

required to drive the North Star double density controller, NBIOSxx requires

an additional 2K of space beyond the 1.5K allocated in a standard Digital

Research CP/M 2.2 single density 8" system.  This compares favorably with

the memory requirements of the Lifeboat CP/M 2.2, which requires only 256

bytes less space.  Furthur documentation is provided within the source module.

Due to the excellent design of the North Star double density controller,

NBIOSxx is booted directly into its normal position in memory by the controller

boot routine, using a small cold boot section in NBIOSxx itself to read the

balance of the code into memory.  This scheme avoids the need for the separate

"track zero" boot module required by most other controllers, including the

North Star single density controller.

(2) NSGEN.ASM		This is the system generation utility.	It is used to

read or write the system area of a 5 1/4" diskette in a

format compatible with NBIOSxx, and transfers from or to the TPA the CP/M CCP

and BDOS modules, along with NBIOSxx and the Jade Double D control code.  The

format of the system tracks is described furthur in this source module, and is

intimately connected with corresponding code in NBIOSxx.  Only standard CP/M

2.2 BDOS and BIOS calls are used, allowing bootstrap operation under the

normal Lifeboat CP/M 2.2 operating system.

(3) NSCOPY.ASM		This utility provides the format function.  It will

standardly format 35-track single-sided drives in

single or double density formats that are Lifeboat-compatible.	In addition,

NSCOPY will standardly validate all formats by reading each track, format

80-track double-sided drives in double density, copy all tracks, the system

tracks only, or the data tracks only from one 35-track drive to another, warm

boot the CP/M 2.2 system, or cold boot the North Star controller.  In addition,

NSCOPY can be modified with a one-byte configuration patch to support 40-track

drives instead of 35-track, and provide all the same features.	NSCOPY contains

its own dedicated North Star controller driver routines, and will utilize all

contiguous memory up to the base of CP/M 2.2 BIOS, resulting in highly

efficient and speedy operation noticeably faster than other North Star copy

utilities.  Normal CP/M 2.2 BIOS calls are used for console I/O, allowing this

program to run under the normal Lifeboat CP/M 2.2 operating system if desired.

The above modules provide everything needed along with a standard Digital

Research or Lifeboat CP/M 2.2 to run.  Bootstrapping is obviously easier if a

Lifeboat CP/M 2.2 is available, although the author was able to bootstrap the

system using the standard North Star DOS, and Cromemco CDOS, along with a

standard 8" release disk purchased directly from Digital Research.  This method

of bootstrapping was not exceedingly difficult, but was certainly much harder

than bootstrapping from an existing North Star CP/M 2.2 system.  Needless to

say, in any case, an extensive knowledge of CP/M 2.2 is required, along with

reasonable 8080 assembly language programming proficiency.

Two additional configurations evolved from the routines described above. The

first differs only in the BIOS module.	This new BIOS, designated MBIOSxx to

distinguish it from the author's normal BIOS, supports two Wangco 82 40-track

double density, single-sided drives as A: and B:, and supports two MPI 92

80-track double density, double-sided drives as C: and D: (or the equivalent

Tandon double-sided drive if the step rate is adjusted). The rest of the

target configuration included the normal Horizon left serial port as the

console device, and the Horizon right serial port with an Epson MX-80

printer attached as the list device. In addition, a single-byte change

to the system image allows this BIOS to fully support a PMMI 600 baud

MM-103 modem. If the contents of location MODEM in the BIOS are set to

zero when the BIOS is cold-booted, then instead of the normal local

device handlers, the console, list, reader, and punch are attached to

the PMMI modem, just like the public domain program BYE. In this

configuration, the BIOS waits for the phone to ring. When it does, the

phone is answered, and carriage returns must be struck for the BIOS to

determine the baud rate (110,300,450,600, or 710). When the baud rate

has been determined, a password is solicited. If the password is correct,

a normal warm boot is done, and operation of CP/M will proceed as if

the operator was local, except that console input will be accepted from

either the PMMI or the Horizon left serial port, and console output will

be sent to both the PMMI and the Horizon left serial port. This operation

will stay in effect until either the phone is hung up, or a jump is made

to the BIOS cold boot vector. A special trivial MBYE program is provided

to do the latter for convenience.

To accommodate the different step rates of the drives, a special step

rate table is employed that provides timing loop counts for the step

routine. The timings included in the BIOS are for a 4 MHz Z80 with no

wait states.

MBIOSxx supports all the North Star formats that NBIOSxx does, with the

exception of Lifeboat 2.0, which used the now-obsolete 2k group size. This

format was left out because it was retrofitted to NBIOSxx, and the author

thought it was sufficiently rare that it was not worth the effort or drive

table space to include it in MBIOSxx. It could certainly be added, using

the drive select routine of NBIOSxx as a guide, but the memory space in

MBIOSxx is very tight due to the PMMI support, and something would probably

have to be removed to make the new drive table fit.

Given the above slight limitation, and the fact that there is no support

for a second disk controller in this BIOS, it is still certainly the best

choice for a user with only a North Star controller, since it can be easily

modified for 35- or 40-track drive usage, supports a standard Horizon with

a fairly-standard printer (Epson MX-80), and can easily be modified for a

variety of different 5 1/4" drives, both double and single sided, such as the

standard North Star quad-type disks. However, such modifications require an

intimate knowledge of how to configure CP/M 2.2 disk parameter blocks and

skewing tables, and a knowledge of the stepping requirements of the target disk

drive.	The normal NSCOPY and NSGEN utilities are used to support MBIOSxx,

with the 1K space normally occupied by the Jade control code reserved in the

TPA and on the disk by NSGEN but not normally used by MBIOSxx.  Note that

because of the PMMI support, MBIOSxx uses about 3.5k of memory, just like

NBIOSxx.  There is no significant advantage in attempting to remove the

PMMI drivers to shrink the BIOS, since an entire 1k would have to be

removed, and these drivers only account for about .5k of code.

The other additional configuration was created to support the author's North

Star single density disk system.  That system consists of a North Star single

density disk controller with up to two SA-400 drives, a 16x64 SSM VB1

memory-mapped video display as the console output device, a parallel keyboard

as the console input device, a Teletype Model 43 (or other compatible) serial

printer as the list device, and a skimpy 21K of static RAM main memory.  I/O

devices on this system are memory-mapped.  As pointed out above, a separate

boot module is required by this hardware.  This configuration is incomplete,

in that no format utility corresponding to NSCOPY is provided that will run

with the single density controller.  The author's main Horizon system running

NSCOPY is used to format or copy diskettes for the single density system.  On

another user's system, the standard North Star DOS disk-copy and initialize

functions should suffice for those functions, and a disk utility such as Ward

Christensen's excellent DU can be used for clearing the disk directory before

use with CP/M.

On the author's system, only 20K of main memory is contiguous.  Taking a page

out of Lifeboat's book, the 1K of non-contiguous memory lends itself well to a

user I/O area scheme.  Therefore, this system is implemented with the BIOS

functions separated into two parts, roughly corresponding to the disk I/O

functions in SDNBIOS, and the console and list functions in SDNUSER, resulting

in the four modules described below:

(1) SDNBOOT.ASM 	This routine resides on track zero, in North Star

physical sector 4.  It is read from there by the boot

routine on the single density controller into memory at the fixed location

2000H.	When control is passed to SDNBOOT, it reads the various sections of

SDNBIOS and SDNUSER into memory from the system tracks of the diskette.  Three

such tracks are required for CP/M 2.2 on a single density disk.

(2) SDNBIOS.ASM 	All disk I/O routines reside here.  Lifeboat-compatible

single density format is supported, but with a CP/M 2.2

parameter block, rather than the Lifeboat single density pseudo-8" disk method.

Disks can be read and written interchangeably by SDNBIOS, NBIOSxx, MBIOSxx,

Lifeboat single density CP/M 1.4, and Lifeboat double density CP/M 2.2.  To

overcome the severe speed loss due to lack of sector skewing, full track

buffering is supported standardly, allowing quite respectable operation, with a

penalty of a loss of 2.5K of system memory.

(3) SDNUSER.ASM 	All console and list I/O routines reside here.	The VB1

is used as a glass teletype for console output, with

the single concession of a backspace function on the current line.  Due to the

extremely tight space considerations on this minimal machine, some functions

actually reside in SDNBIOS, to utilize all possible space.

(4) SDNSGEN.ASM 	This is the specialized system generation utility

required for generating the single density system,

corresponding to NSGEN for the double density system. Standard CP/M 2.2 BDOS

and BIOS calls are used, allowing this program to be run on any

Lifeboat-compatible system, single or double density.  Normally, the single

density operation of NBIOSxx or MBIOSxx is very slow, due to the lack of

skewing or track buffering under those BIOS'es.  To overcome this, SDNSGEN

accesses alternate physical sectors, allowing respectable transfer speeds on

the double density systems, with no penalty on the track-buffered single

density system.

These modules were developed using the pertinant North Star controller

documentation, along with a bit of fiddling around with some of the timing

where the documention was not explicit enough to point out the problem areas

(especially on the single density controller!).  They are placed in the public

domain by the author, and may be freely distributed, but may not be sold.

Sections of NBIOSxx pertaining to the Jade Double D were developed from

routines made available to Jade Double D owners by Jade Computer Products, and

appear now in the public domain for use with Jade Double D controllers by

special permission from Jade Computer Products.

Author: 	Steve Bogolub

Date:		May 14, 1982