SPECTRUM ANALYSIS SOFTWARE

The programs in this package were written by:

Victor DePinto

2627-148th ave. S.E. Apt. 10

Bellevue, Washington  98007

(206) 641-7591

The last revision date was January 27, 1982.

These programs provide the capability to perform  the  FFT

and   inverse   FFT   and   to   perform  various  arithmetical

manipulations on signals as well as to display the results.

The data processed by these programs is contained in  disk

files. Data  is also output to disk files except when displayed

on CRT or printer.  The data is in the form of  a  sequence  of

1024  complex  numbers  representing 1024 data points in either

the time or frequency domain.

INSTRUCTIONS FOR USE OF THE FFT PROGRAM

This program performs the FFT  or  inverse  FFT  on  files

containing   a   sequence  of  1024  complex  values. The  user

specifies the input and output filenames.  Filetypes  are .DAT.

Enter  only  the  filename,  not the filetype.  It is necessary

that the file FFT.COM be on the disk.

To operate this program, type "FFT". The program will  ask

for  source  and  destination filenames and then will ask for a

number specifying FFT or inverse FFT.  The  program  will  then

perform  a  1024  point  FFT  on  the source file and write the

result to the destination file.

INSTRUCTIONS FOR USE OF THE UTILITY PROGRAM

This is a general utility program which allows examination

and  plotting  of  file  data  and  performs certain arithmetic

operations on the data, such as  summing,  multiplication,  and

windowing. In the event that other arithmetic manipulations are

required, the user may write FORTRAN programs which operate  on

1024  point complex arrays read and written to unformatted disk

files.  It is necessary that the files UTIL.COM and ARIT.COM be

on the disk.

To run the utility program, type "UTIL". The program  will

respond with a menu. Following is a list of the menu items with

a brief description of each:

-1-

1  WRITE FILE TO CRT

Write real and imaginary parts of all values in  file  to

CRT.  The  user  first enters the filename then the first

point to be displayed,  and  types  "Q"  to  quit.  Three

points are displayed on each line. Each point consists of

a real and imaginary part. The first number is the  index

of the first point displayed.

2  CREATE REAL PULSE FILE

Creates a file containing a  real  constant  in  a  user-

specified range of points. All other points are zero, and

all imaginary parts are zero.

3  WINDOW FUNCTIONS

This will bring up another menu offering  to  multiply  a

range  of points by any one of a number of popular window

functions.

4  ARITHMETIC FUNCTIONS

This  brings  up  a  menu  offering  to  add,   subtract,

multiply,  or  divide two files and place the result in a

third file. It will also multply a file by a constant and

find the conjugates or magnitude squared of a file.

5  PRINT FILE ON PRINTER

Prints real and imaginary parts  of  of  a  file  on  the

system list device.

6  RMS VALUE OF REAL PARTS OF A FILE

Computes the root mean square of he real parts of a  file

and writes this value to the CRT.

7  MODIFY FILE

This allows the user to enter or modify the  data  points

on a point by point basis.

8  TRANSLATE

This rotates (shifts) all data points in a file by  user-

specified  amount.  I  must apologize for the slowness of

this routine. No doubt a clever programmer could make  it

faster.

9  PLOT FUNCTIONS

This brings up a menu offering to make graphic  plots  of

time  or  frequency domain files. The exact nature of the

functions depends on the type of plotting device used.  A

version is supplied which will make plots on any printer.

Another version will make high resolution  plots  on  the

Houston Instruments DMP2 plotter.

10 RETURN TO OPERATING SYSTEM

Return to CPM.

-2-

THE PLOTTER INTERFACE

The TTY version of UTIL.COM makes character plots  on  any

standard  printer.  The  DMP2  version  plots  on  the  Houston

Instruments DMP2 plotter. The plotter should be wired  for .005

inch  per  step  and 9600 baud serial RS-232C interface.  Lower

baud rates are permissible, but will slow the pen  motion.  The

plotter  serial port must be interfaced to the CP/M BIOS as the

PUNCH device.

NOTES ON UTILIZATION

Certain types of data to be processed by this software may

be generated by the utility program itself. (ie. pulses, double

pulses, truncated spectra, etc.) Other data will  be  in  files

written  by  user-generated  programs or possibly acquired from

the real world. A lot of things  can  be  done  with  a  little

imagination.

Here are  some  possibilities:  To  truncate  a  spectrum,

multiply by a real pulse. To filter a signal, multiply it's FFT

by the frequency domain filter function. This may be the FFT of

the  impulse  response  of  the  filter. A  double pulse may be

created by summing two single pulses. Signals may be added  and

subtracted  in either the time or frequency domain. The program

may be used to examine the spectra of waveforms in contemplated

circuit  design,  to  determine  design suitability without lab

measurements. The inverse FFT may be used  as  a  tool  in  the

design of FIR filters.

One version of my software had  a  provision  for  uniform

rate data acquisition from an analog to digital converter. This

is not provided  in  the  package  because  of  it's  hardware-

dependent  nature,  but  it  could easily be implemented by the

user.  In order to get the ADC data into  a  format  compatible

with  this  software,  write  a Microsoft FORTRAN program which

defines both an integer array and a floating  point  array.  An

assembly   language  subroutine  is  linked  with  the  FORTRAN

program. By means of data area declared  COMMON,  the  assembly

language  routine  routine  fills  the  integer array with 1024

samples from the ADC.  The FORTRAN program will  then  transfer

the  data  from  the  integer  array  to  the real parts of the

floating point array and write it to disk. The integer  numbers

will  automatically  be  converted to floating point.  The data

would then be  processed  by  this  software.  Thus,  with  the

addition  of  an  analog  to  digital  converter,  the computer

becomes a Fourier Transform spectrum analyzer.

-3-

FILE STRUCTURE

This software processes data in arrays consisting of  1024

complex  data  points. Each  complex data point consists of two

floating point numbers, the real part and the  imaginary  part.

When  in  memory,  these  numbers  are  stored in the following

sequence: real part of point 0, imaginary part of point 0, real

part  of  point  1, imaginary part of point 1, and so on. Since

each floating point number occupies four bytes, a complex  data

point occupies 8 bytes.

These arrays, or signal vectors, are stored in disk  files

in  internal  machine  format  (not  ASCII). This  permits much

faster access and more compact storage. In  general  a  program

will  get  a 1024 point signal vector to be processed from such

an unformatted disk file, and then write the processed data  to

a  similar  file. Each  disk  file  occupies  8 kilobytes. Disk

filenames are specified by the user. The filetypes  are  always

DAT.

If  the  user  writes  programs  generating  data  to   be

processed  by this software, it should be stored in memory in a

2 by 1024 array.  Element  (1,N)  is  real;  element  (2,N)  is

imaginary. The  data  should  then be written to disk using the

FORTRAN unformatted write statement.

DISK I/O WITH BASIC

Compatible data files can be also be read and  written  to

disk  using  Microsoft  BASIC. Since the data must be stored in

internal binary form, you must use only random access files.  A

file   must   contain   1024  complex  values.  A  file  called

DISKIO.BAS is included in this package, as an example.

COMPILING AND LINKING

In order to produce executable  object  programs,  compile

the  FOR  files  with  the  Microsoft  FORTRAN-80  compiler and

assemble  the  MAC  files  with  the  MACRO-80  assembler.  The

resulting  REL  files must then be linked using the L80 linking

loader. The final developement work on this  package  was  done

with FORTRAN-80 version 3.4.

The objective is to produce three COM files:

UTIL.COM

ARIT.COM

FFT.COM

FFT.COM performs the 1024 point FFT and inverse FFT by means of

a  radix-2  Cooley-Tukey algorithm. UTIL.COM is a program which

performs various computation and display functions. ARIT.COM is

-4-

not intended to be run by itself, but is chained by UTIL.COM.

A note of  explaination  is  needed  with  regard  to  the

plotting  capabilities  of this package. It is possible to make

two different versions of UTIL.COM.  They are identical  except

for  their  plotting functions. The TTY version will make plots

on any printer. The DMP2 version produces high resolution plots

on  the  Houston  Instruments DMP2 plotter. You will be able to

modify the plotting modules  of  this  software  to  accomodate

other  types  of  plotting devices. The chart below shows which

source files are needed to produce each COM file.

TASK            SOURCE FILE     MODULE NAMES IN FILE

-------------------------------------------------------------

(DMP2 version)

UTIL.COM        UTIL.FOR        UTIL

NAME.FOR        READ, NAME

WIND.FOR        WIND, RECTAN, COSBEL, HAFSIN

TRIANG, HANNIN, HFSIN3

HAMMIN, COSSQR, PARZEN

QCHEK.MAC       QCHEK

PLOT.FOR        PLOT

HI.FOR          DRAW, HOME, VECTOR

GRID, BORDER, SPEED

HIA.MAC         PRINT, STTIM, PENUP

PENDN

(TTY version)

UTIL.COM        UTIL.FOR        UTIL

NAME.FOR        READ, NAME

WIND.FOR        WIND

QCHEK.MAC       QCHEK

TTYPLOT.FOR     PLOT, TTDRAW

ARIT.COM        ARIT.FOR        ARIT

NAME.FOR        READ, NAME

FFT.COM         FFT.FOR         FFT1K

NAME.FOR        NAME

-5-

To facilitate linking of the runtime  tasks,  four  submit

files are provided. Before these submit files are used, the FOR

and MAC files must be compiled and assembled.

To link                 Type

UTIL.COM (TTY vers.)    SUBMIT UTIL#T

UTIL.COM (DMP2 vers.)   SUBMIT UTIL#H

ARIT.COM                SUBMIT ARIT#

FFT.COM                 SUBMIT FFT#

THE TWIDDLE PROGRAM

For speed, the FFT program has the needed sine and  cosine

values  initialized in data memory. These values were generated

by a FORTRAN program which wrote them to disk  in  ASCII  form.

The  sines and cosines were then edited into the DATA statement

in the FFT.FOR program.  The twiddle factor  generator  program

is  provided  in a file named TWIDDLE.FOR for those who wish to

use it to make other FFT programs.

-6-

FILES SUPPLIED

Following is a list of files supplied in this package. Two

versions  of the executable utility program are supplied. UTIL-

TTY.COM plots on any standard printer. UTIL-HI.COM plots on the

Houston  Instruments  DMP2  plotter. Rename the one you want to

use as UTIL.COM.

FILENAME        COMMENTS

ARIT.FOR        Arithmetic functions.

ARIT.COM        Executable arithmetic functions.

ARIT#.SUB       Submit file.

DISKIO.BAS      Disk file I/O in BASIC.

FFT.FOR         1024 point FFT.

FFT.COM         Executable FFT program.

FFT#.SUB        Submit file.

HI.FOR          DMP2 driver.

HIA.MAC         DMP2 driver.

NAME.FOR        Filename getter.

PLOT.FOR        Plotter plot routine.

QCHEK.MAC       Checks for abort command.

SPECTRUM.DOC    This documentation.

SPECTRUM.TEX    TEX input file.

TTYPLOT.FOR     Printer plot routine.

TWIDDLE.FOR     Generates twiddle factor table.

UTIL.FOR        Utility program.

UTIL#H.SUB      Submit file.

UTIL#T.SUB      Submit file.

UTIL-HI.COM     DMP2 version of UTIL.COM.

UTIL-TTY.COM    TTY version of UTIL.COM.

WIND.FOR        Window functions.

-7-