UNLZH.REL and CRLZH.REL Documentation

for Version 2.0 (with RUNTIME buffer allocation)

July  1991

Abstract

----------

UNLZH.REL and CRLZH.REL contain assembly language routines for the decoding

and generation of LZH-encoded files, respectively.  The routines need only

be supplied with a pointer to a large scracth area and linkages to character

input and character output routines to be used.  There are a few easily met

functional requirements for the calling routine and I/O routine.

No Z80 opcodes are used, so these routines may be used on 8080/8085/V20/Z80

based machines.

The coding contained within UNLZH.REL and CRLZH.REL are Copyright (c) 1989,

1991 by Roger Warren and may not be used or reproduced on a for-profit

basis.  Non-profit use is freely permitted.  The author will not be

responsible for any damage, loss, etc.  caused by the use of these programs.

Version History and Compatibility

---------------------------------

Version 1.1 was released in Sept. '89 and was the first public offering of

LZH encoding for CP/M.

Version 2.0 (July '91) introduces several improvements/changes:

More efficient encoding

Greater speed

More compact object code

There are NO interface changes from the 1.x version.

Of greatest importance is the encoding improvement.  This change, while

generating even smaller output files, means that files compressed with

version 2.x programs cannot be decoded by old 1.x programs.

However, the version 2.x UNLZH module DYNAMICALLY ADJUSTS for 1.x encoded

input files.  Thus, version 2.x of UNLZH can be used on all LZH-encoded

files regardless of which algorithm version was used to encode the files.

By extensive rewriting for size and speed, a 20% improvement in performance

was achieved.  Since the LZH algorithm is intrinsically slow, this will be

of great interest to many.  The recoding project allowed the incorporation

of version 2.x extensions to the original algorithm while not appreciably

affecting the code module size.

Care and Feeding

------------------

The infomation that follows documents both CRLZH.REL and UNLZH.REL.  One or

both may be in the library this file is in, depending upon the nature of

the program(s) it's bundled with...so ignore the superfluous information

(if any).

UNLZH.REL performs LZH decoding.  It's progamming interface is similar to

the UNCR.REL it was made to mimic.  The programmer must provide the program

with 8k of buffer space.  If RUNTIME buffer allocation is selected (it IS

selected in the version supplied with LT, FCRLZH, CRLZH, and UCRLZH), a

pointer to the buffer must be supplied in the H/L register pair when the

routine is invoked.  If RUNTIME buffer allocation is not selected, the user

must supply a PUBLIC symbol, UTABLE, which is the base of the provided

buffer area.

Once invoked, the routine allocates its own stack and 'stays in control'

until the de-compression is completed (or an error is encountered).  The

programmer must supply two routines GLZHUN and PLZHUN, via which UNLZH

'GETS' bytes from the input stream and 'PUTS' bytes to the output stream,

respectively.

UNLZH *DOES NOT* compute/process checksums, etc. on the  input file.  Any

support of such features must be handled externally.

GLZHUN and PLZHUN should save all registers except the A register and

flags. GLZHUN must return the next character from the input stream in the A

register.  GLZHUN should return with the CARRY flag RESET for a valid

character, or with the CARRY flag SET when the end of the input stream is

encountered (the content of the A reg should be zero in that case).

Upon exit (return to the caller), UNLZH returns the following information:

Carry reset (or A reg = 0) - Success

Carry set,  A reg = 1      - Newer version required

Carry set,  A reg = 2      - File not LZH endocoded

Carry set,  A reg = 3      - Bad or corrupt file

Carry set,  A reg = 4      - Insufficient memory

UNLZH has 2 entry points, to be used as the programmer needs:

UNLZH is the 'normal' entry point which expects the file to be completely

REWOUND.  At this entry point, the entire file is processed - the

standard header is examined, but not reported or acted upon.  By

examining the return code, the programmer can discern if the file was,

indeed, an LZH-encoded file and act accordingly.

UNL is a secondary entry point which can be used when the programmer

needs to process the standard header information (file name and stamp)

and cannot (or doesn't want to) rewind the file.  When this entry point

is invoked, the header (down to and including the stamps/comment

terminating zero) must have been processed (so the next byte in the input

stream will be the revision level).

The revision level of UNLZH.REL performs is at the byte at UNLZH-1.  A hex

value of 11 indicates version 1.1, etc.

CRLZH.REL performs LZH encoding.  It's progamming interface is similar to

the CRUNCH.REL it was made to mimic.	The programmer must provide the

program with 20k of buffer space.  If RUNTIME buffer allocation is selected

(it IS selected in the version supplied with LT, FCRLZH, CRLZH, and

UCRLZH),a pointer to the buffer must be supplied in the H/L register pair

when the routine is invoked.  If RUNTIME buffer allocation is not selected,

the user must supply a PUBLIC symbol, CTABLE, which is the base of the

provided buffer area.

In addition, at invocation time the A register must contain a value for

CRLZH to install in the 'CHECKSUM FLAG' portion of the file header (see

below).  This byte, to be semi-compatible with C.B. Falconer's version of

CRN for the 8080, is a subset of CRN's strategy byte:

value (hex)   meaning

----------------------------------------------------

00       Standard modulo 65536 checksum is used

10       CRC16 is used

20,30    Unassigned

SUPPORT FOR CHECK INFORMATION MUST BE EXTERNALLY PROVIDED IN THE

USER-SUPPLIED I/O ROUTINES (see below).  THIS IS ALSO TRUE OF CRN...BUT WAS

NOT EMPHASIZED!  CRLZH merely provides the support for posting the value in

the output stream since it happens to 'follow' some of the information

posted by CRLZH (see the header description, below).

CRLZH supports no other features of the CRN's strategy byte, all other bits

are ignored.

Once invoked, the routine allocates its own stack and 'stays in control'

until the de-compression is completed (or an error is encountered).  The

programmer must supply two routines GLZHEN and PLZHEN, via which CRLZH

'GETS' bytes from the input stream and 'PUTS' bytes to the output stream,

respectively.

CRLZH *DOES NOT* compute/process checksums, etc. on the  input file.  Any

support of such features must be handled externally.  Specifically, the

GLZHEN routine must provide for the accumulation of check information and

the caller must write that check information to the output stream when

CRLZH returns to the caller.

GLZHEN and PLZHEN should save all registers except the A register and

flags. GLZHEN must return the next character from the input stream in the A

register.  GLZHEN should return with the CARRY flag RESET for a valid

character, or with the CARRY flag SET when the end of the input stream is

encountered (the content of the A reg should be zero in that case).  As a

service to the user's output processor, every 256th call to PLZHEN is made

with the Z flag set (for monitoring).  All other times the Z flag is reset.

Upon exit (return to the caller), CRLZH returns the following information:

Carry reset (or A reg = 0) - Success

Carry set,  A reg = 1      - File already LZH-Encoded,CRUNCHed or

SQueezed

Carry set,  A reg = 2      - File empty

Carry set,  A reg = 3      - Insufficient memory

CRLZH has a single entry point at the label CRLZH.  The user must have

placed the standard header information in the output stream and must have

the input stream REWOUND prior to invoking CRLZH.

The revision level of CRLZH.REL performs is at the byte at CRLZH-1.  A hex

value of 11 indicates version 1.1, etc.

Since CRLZH and UNLZH allocate their own stacks, the user is reminded not

to make too large a use of that stack in the user-supplied I/O routines.

In addition, if the user-supplied I/O routines decide to abort the CRLZH or

UNLZH operation (due to operator keystrokes, for example), the user must

take steps to restore his own stack.  Upon a normal (or error) return from

CRLZH or UNLZH the user's stack is properly restored.

STANDARD HEADER information

-----------------------------

LZH encoding follows Steve Greenberg's CRUNCH file format.  The header

contains information identifying compression format, original file name,

etc:

field  size     value    Purpose

-----------------------------------------------------------------------

1   1 byte    076h     Signifies compressed form

2   1 byte    0FDh     Signifies LZH encoding (0ff is for squeezed and

0feh is for CRUNCHED)

3   variable  User     Original file name in the form name.ext Trailing

supplied   blanks on the name portion should be suppressed,

but a full 3 characters following the '.' should

be used for the extension (i.e. no blank

suppression).

4   variable  User     OPTIONAL.  Used for file comment/stamp.  If used

the convention is that the comment is placed in

square brackets [Like this].  Other information

may be placed here (e.g., date stamp).  The

logical restriction is that a binary zero must

not be part of the comment and/or other informa-

tion.

5   1 byte     00h     Signifies end of STANDARD HEADER

For use of CRLZH, the user must supply all of the information above.

For UNLZH, use of the UNLZH entry point causes UNLZH to expect to process

the above information.  It will discard the file name and optional

comment/stamp, but will examine the general form (first 2 fields for a

match and general form of the rest of the header).  If the user chooses to

use the UNL entry point, UNL will expect to process the first byte

following the end of the standard header.

What follows is the following:

field  size     value    Purpose

-----------------------------------------------------------------------

6   1 byte  variable   Identifies generating program revision level.

(11H signifies program generated by version 1.1

7   1 byte  variable   Significant revision level.  Indicates major

revision level of algorithm for decoding program

compatability. (10h indicates significant

revision 1.0)

8   1 byte  variable   Check type.  0=checksum, 1=CRC16, others

currently undefined.

9   1 byte     05h     Currently a SPARE, set to 05H by convention.

Following this is the compressed file, itself.

What LZH compression does and how it compares

-----------------------------------------------

FIRST - It's SLOW.  Much slower than CRUNCH.  About even with the old

SQueeze.  It's the nature of the algorithm, but the current implementation

contributes somewhat (more on that later).

The most impressive aspect of the algorithm is that it compresses further

than CRUNCH.  The nature of material being compressed is important - prose

and high level language code will compress further.  Since the algorithm

depends, in part, on patterns within the file being compressed, I was

somewhat surprised to discover that it does a better job (in general) on

.COM files than CRUNCH.  Personally, I was surprised to discover that LZH

compression of CRUNCHed files is possible (but I've disabled that ability

in this release)!

Examples:

CRUNCH of SLR180.COM   106% ratio   (actually made a larger file)

CRLZH  of SLR180.COM    84% ratio

CRUNCH of TYPELZ22.Z80  45% ratio

CRLZH  of TYPELZ22.Z80  40% ratio

CRUNCH of 'C' source    45% ratio   (typical 'C' src selected at random)

CRLZH  of 'C' source    33% ratio   (same file as above)

A small history

-----------------

I am NOT the originator of the LZH encoding.  The program that started my

whole involvement in the introduction of this method of compression to the

8-bit world bears the following opening comments:

/*

* LZHUF.C English version 1.0

* Based on Japanese version 29-NOV-1988

* LZSS coded by Haruhiko OKUMURA

* Adaptive Huffman Coding coded by Haruyasu YOSHIZAKI

* Edited and translated to English by Kenji RIKITAKE

*/

This 'C' program implemented the compression algorithm of the LHARC program

which arrived on the US scene in the spring of '89.

Being of a curious nature, I figured I'd play with the algorithm just to

understand it (the internal comments were, indeed, sparse - leaving MUCH to

the reader's contemplation/reverse engineering) while 'better minds' than I

tackled it in earnest.

Months passed.  I found that I was 'mastering' the algorithm (read that as

demonstrating to myself that I understood it) by converting it piece-wise

to assembly language.  After a while, I was left with a 'C' language main

program, run time library, and I/O with the business end of the compression

and decompression implemented entirely in assembly language.  Since the

expected event of one of the 'heavies' in the PD and/or compression world

releasing a CP/M version of the compression algorithm hadn't come to pass,

I set about making a version myself.

The natural choice was to prepare an analog to the CRUNCH.REL and UNCR.REL

of Mr. Steven Greenberg and Mr. C.B. Falconer and append to/substitute in

the existing, widely known programs for handling SQueezed and/or CRUNCHed

files.

I saw no reason to tamper with the format CRUNCH uses on the output file.

Therefore, with the exception of taking the 'next' file type in sequence

(SQueezed files begin with a 76h,FFh sequence; Crunched files with 76h,FEh;

so LZH encoded files begin with 076h,FDh) and setting the revision levels

in the header to appropriate values , there's no difference in the output

file format.  So, you can probably coax your time/date stamping into

operating on LZH encoded files.

R. Warren

Sysop, The Elephant's Graveyard (Z-Node#9)

619-270-3148 (PCP area CASDI)

####################################