TABLE OF CONTENTS

fpsp.resource/--Background--

fpsp.resource/FPSPMonadic

fpsp.resource/FPSPDyadic

fpsp.resource/--Background--		   fpsp.resource/--Background--

PURPOSE

This resource contains the emulation routines for FPU instructions

which are non-native for the 68040 or the 68060 processor.

This resource is - as all resources - not disk based. Instead, it

is build by the 68040 or 68060.library on startup. The CPU driver

libraries install the "FPSP" code as part of their initialization

process, such that every non-impelemented FPU instruction will be

emulated properly. However, this means that for every such

instruction, the CPU has to go thru exception processing, and will

therefore stop multitasking when emulating a non-implemented in-

struction. Calling the fpsp routines directly thru the new

fpsp.resource will prevent this overhead and will gain some speed.

A separate patch - the FastIEEE program - will patch the system

math libraries to make use of this resource.

The fpsp.resource is re-entrant, interrupt-callable code. The

resource offers only two functions which emulate monadic resp.

dyadic functions of the 68881/68882 FPU on a 68040 FPU.

Currently, only the V40 68040.library builds and provides this

resource, future versions of the 68060.library might offer this

as well.

These functions are designed to be stub functions for assembly

language, they are not designed to be callable from C or any

other high-level language. Typical applications would call these

routines indirectly thru a link library which provides all

requires parameters. Direct calling from within application

programs is not desired.

Moreover, the FPSP routines *do not* generate FPU exceptions them-

selves for speed reasons. If this is desired, the calling routine

has to check the fpcr and the fpsr FPU registers manually and

has to generate the exceptions itself.

fpsp.resource/FPSPMonadic		      fpsp.resource/FPSPMonadic

NAME

FPSPMonadic     -   emulate a monadic 68881/68882 instruction

SYNOPSIS

storeflag = FPSPMonadic ( opword, operand );

d0			   d0      fp0

BYTE FPUControl ( UWORD , IEEEExtended );

FUNCTION

This function emulates the 68881/68882 monadic instruction

passed in in register d0, and performs the desired mathematical

operation on FP0. It returns the result in the FPU register FP0

and FP1.

INPUTS

opword      -   The extension word of a valid 68881/68882

instruction. Valid extension words are found in

the motorola family guide or the 68881/68882

manual.

Note that this is actually the SECOND (!) opword

of the 68881/68882 instruction.

The bits of the opword have to be specified as

follows:

Bits 15 and 13 must be cleared. The fpsp.resource

does not emulate FPU control instructions.

It emulates all transcendential math instructions,

and "fint" and "fintrz". Even though these two

instructions are "native" for the 68060, calling

the resource with these opwords works correctly.

R/M:    Source operand mode,    bit 14.

Must be set to zero.

Source: Source specifier,       bits 12 to 10.

Must be set to zero.

Dest:   Destination specifier,  bits 9 to 7.

Must be set to zero.

Inst:   Instruction field,      bits 5 to 0.

Must be filled with the opcode of the

desired instruction, see the motorola

documents for the list. For the special

case of FPSINCOS, the FPC field, bits

2 to 0, has to be set to 001 to encode

FP1 as secondary result register.

operand     -   A valid IEEE extended precision number as argument

to the function to be performed.

Secondary inputs are delivered in the FPU registers FPCR and FPSR,

namely to select the rounding precision and the behaiviour in case

of invalid arguments or infinite results.

RESULTS

storeflag   -   This is set to 0 in case the FPSP routine could

generate a result. THIS DOES NOT MEAN that the

arguments are valid. In case all exceptions are

disabled, the storeflag will always be returned as

zero, but the result might be infinite or a NAN.

Secondary results are delivered in the FPU registers FP0, FP1 and

FPSR.

FP0 contains the (primary) result of the 68881/68882 instruction

in case the storeflag is 0. *IT MIGHT WELL BE A NAN OR AN INF*

In case the storeflag is non-zero, FP0 is undefined and the FPSP

routines indicate that an exception should be generated by the

caller. This happens only if an exception condition was detected

and the corresponding exception has been enabled in the FPCR

passed in. In all other cases a NAN or a INF will be returned

and the storeflag will indicate a valid result.

FP1 contains a secondary result, if any. The only case were FP1

is used is the FSINCOS instruction where the sine is returned in

FP0 and the cosine in FP1. Otherwise undefined.

FPSR contains the FPU status similar to what a real 68881/68882

would have returned.

NOTES

This function should not be called directly. It should either be

called by the system math libraries, or by compiler link libaries.

Note that a non-zero store flag indicates that an exception

condition was detected. NOTE THAT THE FPSP CODE DOES NOT CAUSE

AN EXCEPTION ITSELF. It is the responsibility of the caller to

check the FPSR, the FPCR and the storeflag to generate this

exception itself, if desireable. In case the storeflag is non-

zero, no useful result is returned in FP0 or FP1.

Useful instruction opwords can be found in the motorola

documentation, either the MC68K family guide, or in the 68881/

68882 manual.

This function does not require a6 to be loaded with the FPSP

resource base. It is interrupt-callable.

BUGS

SEE ALSO

libraries/68040.h, FPSPDyadic,

the Motorola 68881/68882 manual, the MC68K family guide.

fpsp.resource/FPSPDyadic		       fpsp.resource/FPSPDyadic

NAME

FPSPDyadic      -   emulate a dyadic 68881/68882 instruction

SYNOPSIS

storeflag = FPSPMonadic ( opword, operand1, operand2 );

d0			   d0      fp0     fp1

BYTE FPUControl ( UWORD , IEEEExtended , IEEEExtended );

FUNCTION

This function emulates the 68881/68882 dyadic instruction

passed in in register d0, and performs the desired mathematical

operation on FP0 and FP1. It returns the result in the FPU

register FP0.

INPUTS

opword      -   The extension word of a valid 68881/68882

instruction. Valid extension words are found in

the motorola family guide or the 68881/68882

manual.

Note that this is actually the SECOND (!) opword

of the 68881/68882 instruction.

The bits of the opword have to be specified as

follows:

Bits 15 and 13 must be cleared. The fpsp.resource

does not emulate FPU control instructions.

It emulates all transcendential math instructions,

and "fint" and "fintrz". Even though these two

instructions are "native" for the 68060, calling

the resource with these opwords works correctly.

R/M:    Source operand mode,    bit 14.

Must be set to zero.

Source: Source specifier,       bits 12 to 10.

Must be set to zero.

Dest:   Destination specifier,  bits 9 to 7.

Must be set to zero.

Inst:   Instruction field,      bits 5 to 0.

Must be filled with the opcode of the

desired instruction, see the motorola

documents for the list. For the special

case of FPSINCOS, the FPC field, bits

2 to 0, has to be set to 001 to encode

FP1 as secondary result register.

operand1    -   A valid IEEE extended precision number as argument

to the function to be performed. This is the source

operand of dyadic instructions.

operand2    -   The destination operand of the dyadic 68881/68882

instructions.

Secondary inputs are delivered in the FPU registers FPCR and FPSR,

namely to select the rounding precision and the behaiviour in case

of invalid arguments or infinite results.

RESULTS

storeflag   -   This is set to 0 in case the FPSP routine could

generate a result. THIS DOES NOT MEAN that the

arguments are valid. In case all exceptions are

disabled, the storeflag will always be returned as

zero, but the result might be infinite or a NAN.

Secondary results are delivered in the FPU registers FP0, FP1 and

FPSR.

FP0 contains the (primary) result of the 68881/68882 instruction

in case the storeflag is 0. *IT MIGHT WELL BE A NAN OR AN INF*

In case the storeflag is non-zero, FP0 is undefined and the FPSP

routines indicate that an exception should be generated by the

caller. This happens only if an exception condition was detected

and the corresponding exception has been enabled in the FPCR

passed in. In all other cases a NAN or a INF will be returned

and the storeflag will indicate a valid result.

FP1 contains a secondary result, if any. As there is currently no

dyadic opcode with a secondary result, this register should be

ignored.

FPSR contains the FPU status similar to what a real 68881/68882

would have returned.

NOTES

This function should not be called directly. It should either be

called by the system math libraries, or by compiler link libaries.

Note that a non-zero store flag indicates that an exception

condition was detected. NOTE THAT THE FPSP CODE DOES NOT CAUSE

AN EXCEPTION ITSELF. It is the responsibility of the caller to

check the FPSR, the FPCR and the storeflag to generate this

exception itself, if desireable. In case the storeflag is non-

zero, no useful result is returned in FP0 or FP1.

Useful instruction opwords can be found in the motorola

documentation, either the MC68K family guide, or in the 68881/

68882 manual.

This function does not require a6 to be loaded with the FPSP

resource base. It is interrupt-callable.

BUGS

SEE ALSO

libraries/68040.h, FPSPMonadic,

the Motorola 68881/68882 manual, the MC68K family guide.