Welcome to IDEfixPrefs!!!

---------------------------------------------------------------------------

LEGAL STUFF

IDEfixPrefs  Elaborate Bytes, Oliver Kastl

All rights reserved.

---------------------------------------------------------------------------

IDEfixPrefs is a preferences editor.

The installation program will copy it to your SYS:Prefs directory.

Items marked with (*) are only of importance, if IDEfix is started via the

LoadIDE mechanism and has been made reset resident and the machine is

rebooted.

MENUS:

PROJECT -> ABOUT

Shows informations about IDEfixPrefs.

PROJECT -> QUIT

Leaves IDEfixPrefs.

EDIT -> RESET TO DEFAULTS

Restores default settings.

EDIT -> LAST SAVED

Restores last saved settings.

EDIT -> RESTORE

Restores currently active settings.

SYSTEM PARAMETERS (*):

Boot Graphics Mode -> Default (*)

If you have a PAL Amiga, the Amiga will boot in PAL.

If you have a NTSC Amiga, the Amiga will boot in NTSC.

Boot Graphics Mode -> NTSC (*)

The Amiga will boot in NTSC.

Boot Graphics Mode -> PAL (*)

The Amiga will boot in PAL.

Disable Cache (*)

Will disable all processor caches.

No FastMem (*)

Will disable all FastMem.

CONTROLLER PARAMETERS:

TIP:

If you have a VOB Adapter, set 4-Drive Adapter -> VOB

BEFORE you install any ATAPI device!!!

If devices are invisible after a reset, increase BootDelay.

Connector Type -> Auto Detect

Will try to automatically detect if you have an A4000, or something else.

Usually, you can leave this setting.

Under rare circumstances, Auto Detection might not work or could cause the

machine to hang. In this case you can skip auto detection and force IDEfix

into one of the following operation modes:

Connector Type -> A600/A1200/A500

Internal IDE port of A600 & A1200, IDE port of A500 turbo boards.

Connector Type -> A4000/A4000T

A4000 and A4000T internal IDE ports.

WARNING! Setting the wrong connectot type could lead to crashes or lock-ups!

4-Drive Adapter -> EB Standard

Will try to automatically detect an "Elaborate Bytes Style" 4-drive adapter,

if present.

Any Adapter we know of are "Elaborate Bytes Style", only exception is the

Speed-Up Adapter from VOB, which works differnt.

Examples of "Elaborate Bytes Style" Adapters are the IDEfix '97 Adapter by

Individual Computers, the AlfaQuattro from AlfaData, the GI-Quattro from

Golden Image, EZ-IDE Adapter from Eyetech, the Winner Adapter from Vesalia,

the 4-Drive Adapter from Micronik.

Unfortunately the RandyROM from MLC, too (grr!).

You can usually leave this setting, even if you don't have such

an Adapter. If you have a VOB SpeedUp Adapter, you MUST set this to VOB!

4-Drive Adapter -> VOB

You MUST use this setting for the VOB SpeedUp Adapter.

4-Drive Adapter -> NONE

Disable 4-Drive support. You should take this choice, if you don't use a

4-drive adapter at all.

Boot Delay (*):

Sets a delay time before any IDE or ATAPI units are accessed on system

startup in seconds. A delay can be very helpful, if drives are not properly

recognised

ATAPI Enabled (*):

If not checked, the ATAPI driver inside IDEfix will be completely disabled.

However, you can still use the disk based atapi.device, but booting from

an ATAPI ZIP or LS-120 drive won't be possible.

UNIT PARAMETERS:

TIP:

If you have a CD-ROM, set Unit Mount -> CD Mount

If you have a ZIP or LS-120, set Unit Mount -> Floppy Mount

Unit:

Selects the particular unit.

AtapiSpeed:

Sets the speed of ATAPI bus transfers, where 0 is slowest and 3 is fastest.

Default is currently identical to 3.

If you have problems with CD-ROM drives (read-errors, crashes, etc.) you

should reduce this value.

Unit Type -> Auto Detect

IDEfix will automatically detect, if this unit is an IDE or ATAPI unit.

Unit Type -> ATAPI Unit

Forces IDEfix to see this unit as an ATAPI unit.

Unit Type -> IDE Unit

Forces IDEfix to see this unit as an IDE unit.

Unit Type -> Disabled

Will completely disable this unit.

It may speed up the boot process, if you disable all units numbers, where no

device is physically connected.

The unit will be completely disabled, if someone program tries to open this

unit, it will fail.

Unit Mount -> Disable Mount

Will disable automatic mounting for this unit. Speeds up the boot process.

The unit will be still accessible, if someone opens the device directly

(e.g. CacheCDFS can still be mounted).

Unit Mount -> HD Mount

Will always try to mount this unit as a harddisk, even if it is something

else. Yes, you CAN add an RDSK Block to a CD-ROM! If this option is set,

IDEfix will try to mount the CD-ROM as a Harddisk.

This setting can also be used to override the "Super-Floppy" mechanism of

IDE-fix.

Unit Mount -> Floppy Mount

Will mount this unit as a "Super-Floppy" - regardless what it actually is.

Unit Mount -> CD Mount

Reserved for future enhancements. Currently equivalent to

"Unit Mount->Disable Mount", the best setting for CD-Roms.

Unit Mount -> Auto Detect

Will try to do its best in mounting this unit on the following strategy:

- If the unit is a CD-ROM or CD-Writer, it will try to mount it as a CD.

However, as the CD Mount is not operational yet, nothing will happen.

If you have a CD-Rom, it is suggested to set CD Mount or Disable Mount

manually, as this speeds up things up a lot.

- If the unit is a fixed disk, it will read the RDSK blocks and try to mount

all partitions.

- If the unit is removeable and has no or an unformatted medium inserted, it

will be mounted as a "Super Floppy".

If a medium is inserted which contains a valid RDSK Block, all partitions

will be mounted regarding to the RDB.

END OF DOCUMENT