RocketPort(TM) and RocketModem(TM)

Driver Installation for Novell NetWare

==================================================================

Driver Requirements

This document discusses installing and configuring the

RocketPort/RocketModem series driver for the Novell operating

system.

-  NetWare 4.1x (MPR 3.1A or higher and/or NetWare Connect 2.0.30

or higher)

-  IntranetWare I with NIAS (Support Pack V1.0)

-  NetWare 5.0

==================================================================

Locating Information

For hardware specific information or the product overview, see the

Hardware Installation documentation that is available on the

Comtrol CD or you can download the current version from the ftp/web

site.

For detailed software installation information, download the

document associated with this driver.

Comtrol documents are available in electronic form on the Comtrol

web/ftp site. Driver

updates can be downloaded at no charge from the Comtrol web/ftp

site. Always check the web or ftp sites to make sure that you have

the current driver and documentation.

==================================================================

Installation Overview

The driver installation process follows this basic outline:

1. Prepare the installation media. If you received the driver in

zipped format (as a single file with a .zip extension), extract

the driver files and create an installation diskette or copy

the files to the /system directory on the server.

Note:  For information about extracting files, see extract.htm.

2. If you created a diskette, copy the driver files into the

/system directory on the server.

3. Initialize and configure the driver.

4. If you use NetWare Connect or a similar third-party product,

use the load command to initialize the driver, then use nwccon

(4.1.x) or niascfg (5.x) to configure the driver and external

devices.

Note:  If you use nwccon for 5.x, you get a modified version of

nwccon.

5. If you use NIAS or MPR, use inetcfg (4.1.x) or niascfg (5.x)

to initialize and configure the driver and external devices.

6. Down and restart the server.

7. Resume normal operations.

==================================================================

Installing the Driver

This discussion assumes that you have installed the RocketPort or

RocketModem in the server.

You must have NetWare Connect or MPR installed before installing

the driver files. If NWC or MPR are not already installed, do

so before continuing.

To install the AIORCKT.NLM driver:

1. Login from a client system with administrator rights.

2. Copy the driver files to the /system directory on the server,

which is found in the sys volume. The /aio directory is created

during NWC/MPR installation. The drive letter for the volume

varies, depending on network configuration and drive mapping.

3. If NetWare Connect (NWC):

a. Copy the AIORCKT.NLM file to the /system directory.

b. Copy the AIORCKT.ADI file to the /system/aio directory.

c. Copy other files as desired.

4. If NetWare Internet Access Server or Multi-Protocol Routing

(NIAS/MPR):

a. Copy the AIORCKT.NLM and AIORCKT.ADI files to the /system

directory.

b. Copy the AIORCKT.ADI file to /system/aio directory.

Note:  While this may appear redundant, the file is needed in both

locations in order for MPR configuration to work correctly.

c. Copy other files as desired.

You are now ready to load the driver.

==================================================================

Loading the Driver

You can load AIORCKT.NLM either manually, by command line from the

server console, or automatically, through AUTOEXEC.NCF when the

server boots.

-  If you are using NetWare Connect, you must load the driver

manually before configuring NetWare Connect.

-  If you are using NIAS/MPR, the driver is loaded as part of the

configuration process, and you must not load the driver

manually. Instead, go to the appropriate discussion:

-  NIAS/MPR Installation and Configuration for NetWare 4.x

-  NIAS/MPR Installation and Configuration for NetWare 5.x

To load the driver manually, enter this command line for each

board. The AIORCKT.NLM Parameters table provides a description

of each parameter.

load AIORCKT port={IOADDR} {PCI} [{FINDPCI} node={NNN} name={SSS}

nports={PP} {fastbaud} {hyperbaud} {mcc}=nnn or cccc]

Note:  Parameters within the [brace set] are optional. If you have

a combination of PCI and ISA boards installed, load the

drivers for the PCI boards before loading the drivers for

the ISA boards.

The following is an example of a PCI load command for a RocketPort

Octacable:

load AIORCKT PCI

ISA Example:

The following is an example of an ISA load command for a RocketPort

Quadcable:

load AIORCKT port=180 nports=4

AIORCKT.NLM Parameters

==================================================================

Parameter                     Description

------------------------------------------------------------------

port=IOADDR     IOADDR is required for ISA boards and specifies

(Required       the hexadecimal I/O address. If you install two

for ISA)     or more ISA boards, the IOADDR for the first ISA

board must match the DIP switch setting on the

board (for example, 180).

For the second, third, and fourth boards, the DIP

switches must be set as described in the Hardware

Installation documentation, but the IOADDR can be

any valid address above the first board's address.

Valid ISA-bus addresses are: 100, 140, 180, 200,

240, 280, 300, 340, and 380.

------------------------------------------------------------------

PCI             PCI is required for PCI boards. If it is not

(Required       supplied, the driver assumes that the board is an

for PCI)     ISA board.

------------------------------------------------------------------

hyperbaud       This optional parameter applies to RocketPort PLUS

(Required for   boards only, and is required to operate a

RocketPort     RocketPort Plus. The parameter remaps 50 baud to

Plus)          230.4K bps, 75 baud to 460.8K bps, and 110 baud to

921.6K bps.

------------------------------------------------------------------

mcc=nnn or      This parameter is required for RocketModem cards

mcc=cccc       that support country code selection. The value can

(Required for   be any of the following:

RocketModem)

05  France, 06  Germany, 08  Italy, 14  Sweden,

16  UK, 22  USA

Leading zeroes are not required.

The following country names can be used instead

(case-insensitive):

France, Germany, Italy, Sweden,

UK (United Kingdom), GBR (Great Britain same

as UK), US, USA (same as US), Default (ATZ

reset init string).

Make sure the RocketModem installed is appropriate

for the specified country code. All RocketModems

on the board will assume the selected country's

telephone network attributes.

Entering an mcc parameter generates additional

modem status messages, and enables the use of the

MR.NLM modem reset utility.

------------------------------------------------------------------

nports=PP       This optional ISA parameter sets the number of

ports. If not used, AIORCKT.NLM defaults to

nports=8.

(ISA boards,

only)    In addition, this parameter allows the count of

the actual number of ports on a board to be

overridden with a different value. You can not

override with a greater number than are actually

present on the board, but you can specify a

smaller number.

------------------------------------------------------------------

FINDPCI         The FINDPCI parameter is optional for PCI boards

(PCI boards,    and lists all Comtrol RocketPort and RocketModem

only)   series boards and their address assignments.

------------------------------------------------------------------

fastbaud        This optional parameter remaps 50 baud to 230.4K

bps and 75 baud to 460.8K bps. After you

(Optional)      initialize a board with this parameter, if you

select 50 or 75 baud in an application, the actual

baud rate used is 230.4K or 460.8K.

------------------------------------------------------------------

name=SSS        The name parameter sets the text name for a board.

If omitted, the text name is blank. Some

(Optional)      applications require a board name and access boards

by name rather than by number.

------------------------------------------------------------------

node=NNN        The node parameter overrides the default node

(Optional)      number passed to AIO for a particular board. Some

applications require that the first board have a

node number of 0.

==================================================================

ISA-bus boards only (This does NOT apply to PCI boards):

You can manually enter load aiorckt without any command line

parameters. The driver will prompt you for required information.

Do not modify the AUTOEXEC.NCF file if you use this procedure.

After you have initialized all boards, go to the NWC/Third-Party

Initialization and Configuration discussion.

==================================================================

NWC/Third-Party Initialization and Configuration

Follow these steps:

1. Load the driver manually, using the command line discussed in

Loading the Driver.

2. Add the load commands to the AUTOEXEC.NCF file, so that they

are executed each time the system is restarted.

3. At the command prompt, type load install. The Installations

Options menu displays.

4. Select NCF files options. The Available NCF Files Options menu

displays.

5. Select Edit AUTOEXEC.NCF file.

6. Scroll down until you find the load commands.

7. At the end of the load commands, add a load line for each

board, using the syntax in the AIORCKT.NLM Parameters table.

Remember, lines initializing PCI boards must be executed

before lines initializing ISA boards.

8. Save the changed file, then press Esc until you exit.

9. Go to the appropriate procedure in the software installation

documentation for your installation:

-  NetWare Connect Configuration for NetWare 4.x

-  NetWare Connect Configuration for NetWare 5.x

==================================================================

Troubleshooting

If you are still unable to resolve the problem, try the following:

-  Check to make sure that you are using the correct cables, and

that the cables are connected properly.

-  If using ISA boards, check to make sure the DIP switches are

set to the desired addresses.

-  Reseat the board in the expansion slot, or try moving the

board to another slot of the same bus type.

-  Make sure that the expansion slot screw is replaced after

inserting or moving the board.

-  Use RCKTTERM to determine whether the ports are functioning

properly.

If you are still unable to resolve the problem, contact Technical

Support. When you contact Comtrol, please have the following

information available:

-  NetWare Connect or MPR revision number

-  SPACK (Support Pack) revision number

-  AIORCKT.NLM version number

To determine the AIORCKT.NLM version number:

1. At the server console prompt, enter:

modules aiorckt.nlm

2. If necessary, scroll down the list until the aiorckt.nlm

version number displays.

To generate a system Configuration Report:

1. At the server console prompt, enter load techwalk

2. This generates a Configuration Report (techwalk.out) that is

stored in the /etc directory. This report contains much useful

and valuable information and may be requested by Comtrol

Technical Support.

==================================================================

RCKTTERM Overview

RCKTTERM is a simple AIO application similar to the Novell

AIOTERM.NLM, which has been modified to work with the RocketModem

and RocketPort boards and the AIORCKT.NLM driver. RCKTTERM

supports up to four RocketModem/Port boards with up to 32 ports on

each board. RCKTTERM displays information on the currently loaded

AIORCKT driver and the boards.

RCKTTERM has the following capabilities:

-  You can specify the board and port you want to test, review, or

reconfigure.

-  View the port's capabilities and current configuration.

-  Reconfigure ports.

-  Perform simple data transfers.

-  Send data out to a RocketPort port that has a loopback plug

connected to verify that the port is functioning properly when

it is echoes back to the screen.

-  Connect two ports together with a NULL MODEM cable, which

allows data to be sent out one of the ports from the keyboard.

Then you can select the receiving port and it can be verified

on the screen.

-  Test modem operations.

------------------------------------------------------------------

Testing Modem Operations

You can use RCKTTERM to test modem operations by selecting a port

and issuing AT modem commands. Responses can be verified as they

echo back and display on the screen.

If two modems are connected to phone lines or a line simulator,

the dial command can be issued from the first modem and the

second modem should answer the call. The modem must be configured

with the number of rings to answer using the ATS0=X command and

negotiate a connection. Selecting the port for the second modem

you should be able to see the RINGs displayed on the screen. Once

this connection has been established, any data entered on the

keyboard for one port displays on the screen when looking at the

second port. The connection can be terminated by entering the

+++ escape sequence on one of the ports. This allows that modem

to recognize AT commands again. At this point if the ATH command

is issued to the modem, it hangs up and the connection terminates.

------------------------------------------------------------------

Using RCKTTERM

Use the following procedure to use RCKTTERM:

1.  Load the AIORCKT driver for each of the installed RocketPort

and RocketModem boards.

2.  Load the RCKTTERM NLM.

3.  Select the desired board from the list of boards.

4.  Select a port from the available ports on the board.

5.  Make a selection from the menu that displays the port

capabilities, statistics, current configuration, reconfigure

the port or go to terminal mode to preform data transfers.

Note:  The ESC key allows you to jump up a level in the menu and

exit if you decide to NOT select another port.

==================================================================

RocketModem Manual Reset

There are times when it is useful to be able to issue a hardware

reset to a RocketModem:

-  When an application is incapable of resetting a RocketModem

-  If an invalid or illegal initialization string (AT command) has

been sent to the RocketModem

-  If the modem appears unresponsive to the application or an

incoming call

International-type RocketModems have a hardware reset capability.

Note:  You can tell if your RocketModem has this capability by

looking at the model number on the packaging or on a tag on

the RocketModem circuit board. The following models do NOT

support reset: 96860-3 and 96850-4.

------------------------------------------------------------------

Reset an Inactive Modem on a Specific Board

To reset an inactive modem on a specific board, enter the command:

load MR b m

where: b is the physical board number (1-4) m is the physical

modem number (1-8)

This resets the particular inactive RocketModem.

------------------------------------------------------------------

Reset All Inactive Modems on a Specific Board

To reset all inactive modem on a specific board, enter the command:

load MR b all

where: b is the physical board number (1-4)

This resets all inactive RocketModems on the board specified.

------------------------------------------------------------------

Reset an Inactive Modem by Node Number

To reset an inactive modem by node number, enter the command:

load MR AIO_130nppp

where: n is the board AIO NetWare node number (0-3) and ppp is

the AIO port number (0-7)

This resets the particular inactive RocketModem.

------------------------------------------------------------------

Reset All Inactive Modems by Node Number

To reset all inactive modem on by node number, enter the command:

load MR AIO_130n

where: n is the board AIO NetWare node number (0-3)

This resets all inactive RocketModems on the AIO NetWare node

specified.

------------------------------------------------------------------

Forced Reset

The force option should be used for specific modems that have

already been allocated (acquired) by an application. The

application must be able to recognize call termination, and issue

the appropriate modem initialization strings, as defined in the

AIORKT.MDC descriptor file. Only individual modems can be forced

down.

To force reset a modem by board number, enter the command:

load MR b m force

where: b is the physical board number (1-4) and m is the

physical modem number (1-8)

To force reset a modem by AIO node number, enter the command:

load MR AIO_130nppp force

where: n is the board AIO NetWare node number (0-3) and

ppp is the AIO port number (0-7)

==================================================================

Technical Support

Comtrol has a staff of support technicians available to help you.

Before you call, please have the following information

available:

=========================================================

Item                    Your System Information

---------------------------------------------------------

Model number

---------------------------------------------------------

Serial number

---------------------------------------------------------

Interface type

---------------------------------------------------------

I/O address and IRQ

---------------------------------------------------------

Operating system type

and release

---------------------------------------------------------

Device driver version

---------------------------------------------------------

PC make, model, and speed

---------------------------------------------------------

List other devices in the

PC and their addresses

=========================================================

==========================================================

Comtrol    Headquarters                Europe

----------------------------------------------------------

Phone         (763) 494-4100         +44 (0)1869 323220

----------------------------------------------------------

FAX           (763) 494-4199         +44 (0)1869 323211

----------------------------------------------------------

Email         support@comtrol.com    support@comtrol.co.uk

----------------------------------------------------------

Web support   Searchable Solution Database and FAQs

----------------------------------------------------------

Web site      www.comtrol.com        www.comtrol.co.uk

----------------------------------------------------------

FTP site                  ftp.comtrol.com

==========================================================

==================================================================

Trademark Notices

RocketPort is a registered trademark of Comtrol Corporation.

RocketModem is a trademark of Comtrol Corporation.

NetWare and Novell are registered trademarks of Novell. Other

product names mentioned herein may be trademarks and/or

registered trademarks of their respective owners.

Comtrol Corporation makes no representations or warranties with

regard to the contents of this document or to the suitability of

the Comtrol product for any particular purpose. Specifications

subject to change without notice. Some software or features may

not be available at the time of publication. Contact your reseller

for current product information.

DOCUMENT NUMBER: 8000047