#[1mAGMSRecordSound#[m

AGMSRecordSound is a Modula-2 program by Alexander G. M. Smith which

records sound samples to disk, using a standard parallel port audio

digitizer.  AGMSRecordSoundPS3 is a modified version that works with

version 3.0+ PerfectSound hardware.

To get you started, try this command:

AGMSRecordSound ram:test verbose

Type a control-C to stop it.  You can play the resulting file with

AGMSPlaySound or some other sound player.

#[4mPropoganda#[m

Since AGMSRecordSound doesn't need to fit the whole sample into one

contiguous memory block, you can record samples longer than your memory

size (up to #[1m2 gigabytes#[m).  It also multi-tasks so you can see the screen

and do other things while recording, unlike all other sound digitizer

programs I know of.

#[4mAnti-propoganda#[m

The downside of this is crummy audio quality.  Other digitizers take

over the computer while recording since that is the best way to get

better quality.  The trouble comes from code that temporarily disables

interrupts, making AGMSRecordSound's interrupt routine miss a sound

sample.  This happens all time time - whenever memory is allocated or

deallocated, interrupts are disabled to prevent other programs from

interfering.  Hopefully Commodore will put in some DMA circuits for

digitizing so that we can get good quality #[1mand#[m multitasking.

#[4mRequirements#[m

AGMSRecordSound requires sound sampler hardware plugged into the

parallel port.  I've tested it with #[3mAMAS #[m(records from the right

channel) and it should work with most other parallel port digitizers

(try left if right doesn't work).  If you have SunRize Industries'

#[3mPerfectSound 3 #[mdigitizer then you have to use a different version of

this program: AGMSPlaySoundPS3.  You also need to have the arp.library

file (version 39 or 1.3) in your LIBS: directory.  If you have ARexx

installed, AGMSRecordSound will take advantage of it.  Other than that, it

should work on all Amigas.

#[4mHow It Works#[m

An interrupt routine is set up for audio channel 1 (right side) that

samples the parallel port, copies the data to the audio register and

copies it to a buffer in memory.  Every time audio channel 1 needs a new

sound sample, this interrupt routine is called.  When a buffer becomes

full, it moves to the next buffer and signals the main program.  If

there are no empty buffers to be filled, the interrupt routine will slow

down the audio channel to a lower sampling rate (around 440 Hz) so that

the processor will have more CPU time for running tasks (less used by

the interrupt code).  You can still hear the sounds but they won't be

stored in the buffer (not recorded in other words) and they will sound

strange (like a dog growling under water) due to the lower sampling

rate.  When an empty buffer becomes available, it will resume filling at

normal speed.

The other half of AGMSRecordSound is a process that dumps filled buffers

to a disk file.  If it is faster at emptying buffers than the interrupt

routine is at filling them, it will go to sleep and wait for a signal:

control-C (a signal from the user to stop recording) and control-F (a

signal from the interrupt routine that another buffer is full).  In the

PerfectSound version, it also responds to control-E (steps up the volume

control).  After dumping each buffer, it can optionally test for a

couple of stop conditions: reaching the maximum number of bytes or the

sound being quiet for a long enough time.

#[4mSound Quality#[m

The problems with disabled interrupts cause pops and similar noises -

great if you are trying to simulate an old scratchy record (or to record

one), but otherwise annoying.  The trick for getting "good" sound

quality is to set the sampling rate to minimize this problem.

A really #[1mlow sampling rate#[m means that the disabled interrupt won't last

long enough to cause trouble.  After interrupts are re-enabled, the

pending audio interrupt will get through and the sample will only be

delayed in time for a short while.  Samples after the delayed one will

be back on schedule.  Unfortunately, most people don't like to record at

1kHz :-).  However, #[1mif you have a faster computer#[m (I only have an A2000)

then the disabled times will be shorter and sound quality should go up.

A really #[1mhigh sampling rate#[m will suck up so much CPU time that only the

interrupt routine will be running; every time it finishes sampling,

another interrupt comes in.  This is actually quite useful (more later).

You can tell that this is happening by the lack of mouse pointer

movement when you move the mouse.  However, as soon as all the buffers

are full, the interrupt routine will go into idle mode.  This frees up

CPU cycles for running other tasks - like the mouse pointer movement and

buffer saving.  As soon as one buffer has been saved, the interrupt

routine will pick up speed and things will be frozen.

At high sampling rates, no other task is running and disabling

interrupts.  This means high quality sound, almost as good as those

dedicated non-multitasking sampler programs.  The catch is that nothing

is recorded while the buffer is being dumped to disk.  So, if you can

put up with brief pauses then you can use this mode.

Also, you can use AGMSRecordSound as an #[1mordinary sound digitizer#[m.  Just

set the buffer size to fill up your whole memory (many smaller buffers

are preferable since the interrupt routine can fill all but the one

buffer being emptied - try buffers 20 size 100000 if you want to fill up

2 meg of memory).  Then set the sampling rate to one that uses all the

CPU time for sampling (12kHz is good on my A2000, you will need larger

rates on faster computers).

At #[1mridiculously high sampling rates#[m, the interrupt routine will be

running flat out at a speed less than the one you requested.  This will

result in a sample that is higher pitched than reality when you play it

back.

#[4mEmbedded IFF Information#[m

The output file is an IFF file.  You have lots of command line options

available for specifying optional information (like sample name, author

etc).  Unfortunately some other sound sample processing programs can't

handle the extra info (but AGMSPlaySound can!).  If you find that you

want to type in more information than will fit on a command line, put

the long command line into a script file and execute it.

#[4mHelp#[m

Most of the arguments to the program are obvious if you know audio

terminology and computer terminology.  Anyways, you really don't have to

specify any arguments except the name of the file to be created.  For

on-line help, just type a ? as the argument to get the standard ARP /

AmigaDOS command template.  A second ? typed at the template prompt will

print the program credits.  The Verbose option also explains in more

detail what the current settings are.  They are intuitively obvious

(well, to me :-), but just in case you aren't intuitive about audio

stuff, I'll describe the parameters in detail near the end of this

message.

#[4mStolen Channels#[m

This program won't let other programs steal its sound channel (channel 1

to be specific).  It allocates it with maximum priority and thus hangs

on to it.  If you are using AGMSPlaySound at the same time as you are

recording, AGMSPlaySound will be bumped off channel 1 and will reappear

on whatever other channel is free.  If someone else has channel 1 with

the highest priority then AGMSRecordSound won't try recording.

#[4mError Handling#[m

The main source of errors is from the file system.  In particular, you

may encounter a disk full error once in a while :-).  When an error

occurs, AGMSRecordSound stops whatever it is doing and shuts down.  It

will return a return code of 10 when something goes wrong.

Now, more about those #[1mdisk full errors#[m.  I've set things up so that the

file you are recording to is closed as if you had normally ended

recording.  Since it is an IFF file, this means going back and filling

in the size of the sample in previously written data near the beginning

of the file.  It seems to work when using the old file system (OFS) but

doesn't work under the fast file system (FFS).  Well, I tried.

#[4mTechnical Notes#[m

Well, I found out a bit while writing this program.  Ok, two bits, make

that three or four after I figured out the PS3 hardware :-).  For one

thing, I found out how to use a digitizer: set CIAA port B to be all

inputs (the parallel port).  Set CIAB port A pins POUT and SEL to be

outputs.  The sampler spews out bytes to the parallel port with no

handshaking, at some rate that I assume is pretty fast.  Some digitizers

use a high level on SEL to connect the right audio input to the sampler

and a high level on POUT to connect the left channel.  #[3mAMAS #[mjust

seems to use the POUT bit to select left or right and seems to use right

instead of left.  Whatever.  Once you have the desired channel, you can

get a sample byte by reading the parallel port byte.  Subtract $80 from

it to make it into a signed number and store it away.  Allow a small

amount of time between switching from left to right (or vice versa) to

let the sampler update its value (the time for a few instructions should

be enough).

The PerfectSound 3.0+ hardware is a different story.  The sample byte is

split between the parallel port (CIAA port B) and the miscellaneous bits

port (CIAB port A).  The low 6 bits of parallel data (PB5-PB0 in CIAA)

are actually the high six bits of the sample (shift them left by two).

The low two bits of the sample are in the printer POUT (paper out) and

BUSY signals (PA1-PA0 in CIAB).  As before, SEL (PA2 in CIAB) controls

the left / right (zero is left).  So, what do PB6 and PB7 in CIAA do? A

lot! The PerfectSound digitizer has a latched output, PB6 triggers the

latch.  Set PB6 to zero to latch a value, read the sample, and set PB6

back to one to start digitizing the next sample (I assume that's what

it's doing).  PB7 is the gain control.  Normally you run the hardware

with PB7 set to one.  If you toggle PB7 to zero and back, the volume

control will be stepped up or down one notch of 16 levels.  SEL controls

the volume direction (as well as left/right), zero for decreasing the

volume.  The volume gain control wraps around from minimum to maximum

and as far as I could tell, there isn't any way of telling what the

current gain setting is or of setting it to minimum or maximum.

I experimented a bit with ways of generating interrupts before I clued

in that no interrupts happen while interrupts are disabled.  CIA timers

running continously are good.  Audio channels themselves are also good

for making a periodic interrupt (and at exactly the same speed as

playback too).  I even tried using an audio channel to generate an

interrupt slightly before the audio channel I was using for timing would

signal the next time period.  The early interrupt routine would then

busy wait for the timing channel interrupt.  The "slightly" grew to over

800 CPU clock cycles before I got slightly improved sound (hey - a way

of determining how long those disabled times are), slightly improved at

a very slow sampling rate that made it useless.

#[4mFeedback#[m

If you have any feature requests or find any bugs, please send me a

message.  I'm on several of the Ottawa BBX's (a BBS written by SteveX)

and my commercial info service names are listed in the program's second

help message (agmsmith@BIX.com and 71330.3173@CompuServe.com).

#[4mDistribution#[m

AGMSRecordSound is FreeWare.  Copyright (c) 1993 by Alexander G. M. Smith.

That means that you can use it freely, can't blame me for anything that goes

wrong (there are probably a few bugs left), and you shouldn't expect more

from it than you paid for it.  Also, don't blame me if your hard drive fills

up and then has problems with invalid directory structures - it happens to

me too.

#[4mFile/A#[m

Names the file where the sample will be stored.  Any existing file by

this name will be erased.  A required keyword.  You can specify NIL: if

you just want to monitor your digitizer without recording the sound to a

file.  Only works in versions of AmigaDOS that support NIL:.

#[4mVerbose/S#[m

When this command switch is used, lots of interesting messages will be

displayed.  All the parameter settings will be shown.  During recording

you can see what buffers are being dumped.

#[4mBuffers/K#[m

This parameter specifies the number of buffers (any old ram is used) to

use for recording the sound.  You need at least 2.  While the system is

recording one buffer, the others can be emptied to disk.  Try the

verbose command switch to watch the buffers being emptied and to see how

changing the number of buffers affects performance.  Gee, I sure like

these reuseable documentation paragraphs - I just changed "play" to

"record" :-)

#[4mBufferSize/K#[m

This parameter sets the size of each sound buffer.  The default is

100000 bytes, the maximum is about 2000000000 bytes :-).  As mentioned

elsewhere, AGMSRecordSound doesn't need to use any chip ram at all.

Buffers use general purpose ram.  For best performance, use large

buffers.  That is because DOS can write a large chunk of data faster

than several small chunks (less overhead).

#[4mHz/K#[m

This parameter controls the record rate.  It is in units of samples per

second, or bytes per second since each sample is one byte long (compact

disks have 2 byte samples and thus sound better).  The default is 7781

hz, less for the PerfectSound version due to extra overhead in reading

each sample.  The highest rate on my A2000 that lets you record and

simultaneously save to disk (no gaps) is about 8500 hz.  A speed of

12000 hz is good for better quality sound with small gaps between

buffers.  See the discussion on sound quality about picking values for

hz.

The Hz setting is internally translated into a code value for the

hardware.  Since the hardware doesn't have all that many code values,

only a few frequencies are actually available.  A frequency near the one

you specified will be picked.  Note that this is half the playback code

value since playback plays words (pairs of bytes) while we only record a

byte at a time.

#[4mStopSize/K#[m

This parameter specifies the maximum size of the sound sample file.  The

default is two billion.  When this many bytes of data have been written,

the sampling process will be stopped.  Because the interrupt routine may

have filled up a few buffers before this much data has been written, you

will get a bit more than the stop size in your file.

#[4mStopQuiet/S#[m

This switch triggers a neat feature requested by Hans Kerkhof who wanted

something to record people calling in to a TV show.  If you specify

StopQuiet then the sampling will also stop when the input is quiet for a

long enough period of time.  This could also be useful for those of you

making telephone answering machines.  The next few command line

arguments control the definition of "quiet" and how much quiet is

needed.

#[4mQuietTime/K#[m

This parameter lets you specify the integer number of seconds of quiet

that will make the program stop recording.  The number is approximate

and will be converted into a count of quiet sound sample buffers (see

BufferSize) that would cover roughly the time you request.  The minimum

time possible is the time it takes to record one sound sample buffer.

If you specify the Verbose option, you can see the quiet time shown as

the number of quiet buffers and the amount of real time that corresponds

to.  The default value is zero (that becomes the time it takes to record

one buffer).  Note that QuietTime is spelt with two "t"s in the middle,

I often misspell it and get the error "Bad positional argument."

#[4mQuietVolume/K#[m

If you have StopQuiet turned on, then each buffer will be examined after

it has been dumped to determine if it is "quiet".  A number of samples

(adding up to 32 per second) will be picked out from the buffer and

averaged together (or rather, the absolute value of each picked out

sample will be averaged).  The resulting value, from 0 to 127, is a

statistical estimate volume level of that buffer.  If you turn on the

verbose option, it will show the buffer volume estimates.  If the volume

is less than or equal to the QuietVolume you specify then that buffer

counts as being "quiet".  If you get a certain number (determined by

QuietTime) of quiet buffers in a row, then the program will stop.

#[4mName/K#[m

This lets you specify the name of the song or whatever you have

recorded.  If it is more than one word, put it in double quotes.

#[4mCopyright/K#[m

This is for the IFF copyright notice inside your sound sample.  Specify

something like "1991, Alexander G. M. Smith".

#[4mAuthor/K#[m

The author of this sample.  If it is music then that is relatively

obvious.  I don't know who is the author of the sound of a toilet

flushing, maybe the toilet manufacturer?  Don't specify it if you don't

want an author.

#[4mAnno=Annotation/...#[m

An endless list of annotation.  You can use the short form of the

keyword (Anno) or the long form (Annotation).  The values are strings

(as many as you want) of annotation text.  It can be whatever you want,

usually something that describes the sample: anno "This is my first hour

long sound sample" "The sound of the dripping tap was created by using

the kitchen sink." "Use at your own risk."

#[4mARexx: StopRecording#[m

If you have ARexx installed, AGMSRecordSound will open an ARexx port

called "AGMSRecordSound" (note the case of the letters) and respond to

commands.  Currently, the only command is "StopRecording" which will

stop the recording, much like a Control-C does.  Have a look at the

Test.rexx script included with the archive to see how to use

AGMSRecordSound from ARexx.  You can thank Al Villarica for requesting

this feature.

#[4mControl-C Etc.#[m

Control-C will abort the program as soon as possible.  This can take a

while if it is busy dumping a buffer.

Control-E will step up the volume on the PerfectSound hardware, if you

are using AGMSRecordSoundPS3 (does nothing otherwise).  Don't hit the

key too fast, when AGMSRecordSoundPS3 is running the rest of the

operating system is slowed down and can't process your signals fast

enough.  In fact, it's so slow that you can overflow something (probably

the supervisor stack) by typing a lot of Control-CDEF's, even in another

program's window.  The result is a visit from the guru (crash #3

usually).

Control-F is used by the interrupt routine to tell the main program that

there is another full buffer.  You can use it to if you want to wake up

a poor little sleepy program (doesn't do any harm, unless you type it

too quickly :-).

- Alex