Linux/68k for the Commodore Amiga

The Complete Installation Guide

Written by Odd-Jarle Kristoffersen

History

-------

6th June 1995   First Release

Introduction

------------

Welcome to the world of UN*X operating system. A powerfull OS that is

widely used around the world for network purposes. Linux is a very

space friendly UN*X OS that is available for a lot of different machine

platform such as Atari, IBM PC's and Macintosh.

The first releases of Linux was made for the IBM PC clones and as it

came available to other machines the documentation didn't follow up for

these as good as the PC documentation. This is the reason I decided to

make a complete (well as close as possible) documentation to describe

the basic setup and use of Linux for the Amiga.

I am by no means a UN*X expert but I wanted to make it easier for the

people new to Linux to install this system on their machines. Reading

on is at your own risk. Please refer to the chapter about The Author.

Index

-----

History

Introduction

System Requirements

Getting the required Linux files

Where to get the Linux files

Testing Linux

Preparing files for harddrive installation

Preparing suitable harddrive partitions

AmigaDOS partition

Partitioning

Preparing fstab

Booting Linux for installating

Booting from installed harddrive

Failed to boot in Multi-User mode

Cleaning up the usr-partition

System overview

Example of Bourne Again Shell (bash) configuration files

The editor "vi"

The password file

Other files in /etc

Sources

Credits

Special thanks

The Author

Future plans

System Requirements

-------------------

For Linux to work you will need a harddrive with at least a 22 MB

partition. Note that you CANNOT use a AmigaDOS partition as UN*X uses

it's own HFS (hierarcic filesystem). It's recommended that you have one

partition for / (root) and another for /usr (additional files).

The root-partition must be approx 3 MB and is the boot-partition. However

I recommend that you leave some more space to edit and add files, and for

logfiles. The /usr partition should be at least 19 MB, but you will need

more if you want to install other programs such as X11 windows and editors.

This is not much compared to NetBSD 1.0 which will require around 200MB

for the complete installation, and around 80MB minimum.

SCSI and IDE controllers are supported.

My setup was on a separate 49MB SCSI drive partitioned into one 10 MB

partition for root (/) and one 39 MB partition for additional files (/usr).

This leaves room for installing additional software.

Linux should work on all processors from the 68000 to 68040. I haven't been

able to test this but will verify as I get to do so. My machine is an A3000

25MHz 68030 based Amiga. I would recommend you have at least an 68030 to

get usefull speed from the Linux OS.

You will need at least 4MB of FAST memory and 1MB of CHIP memory in your

Amiga. Again, it's strongly recommended that you have as much memory as

possible to run Linux.

Linux runs on default on a 640x400 interlaced screen, so a VGA or Multisync

monitor would be recommended. Use of other screenmodes is currently unknown

to me but I will do a own documentation on this when I've done research on

it. It should possible to recompile the kernel to use other screens and

add-on graphicboards.

You will also need some free space on an AmigaDOS harddrive partition for

the required Linux files to be unpacked onto. Temporary free space must be

at least 25 MB. When Linux has been installed you will need under 1MB

for the boot-files.

Getting the required Linux files

--------------------------------

You will need some files to install Linux from. Specific for Amiga:

A bootstrap program             amiboot-1.9.gz

A RAMdisk image                 filesys.gz

A kernel                        vmlinux-0.9.3.gz        or newer

GZip for Amiga to unpack files

GZip can be found on AmiNet. I've used version 1.23 (24.Jan 93).

To unpack the GZip'ed files (*.gz) use the syntax:

gzip -d <filename>.gz

GZip will unpack to the same directory and delete the *.gz file when done

to save space.

The bootstrap program loads the kernel file, and you have to make sure you

get a Amigaspecific bootstrap program. Look under the "amiga-directory" of

Linux on Linuxsites. You should always get the latest bootstrap program

unless other is specified in the ANNOUNCE files. Version 1.9 works with

version 0.93 of the kernel.

The kernel should also be Amiga specific, and I'm using version 0.93. Newer

version has been released (0.96 and 0.97) but I've not been able to boot

with these yet.

The RAMdisk image is a 1MB (1024k) Linux bootable RAMdisk image you will

need to install Linux from. You will need to boot from this when installing

Linux. It can usually be found under the "filesys-directory" on Linuxsites.

This is not an Amiga specific file.

Additional files you will need are:

A root-disk image               root.tar.gz     (~730 kB)

A usr-disk image                usr.tar.gz      (~5.7 MB)

These are Linuxspecific binaries compressed with full paths and all

directories needed to make a root- and usr-partition on your harddrive.

Where to get the Linux files

----------------------------

Several Linuxsites on Internet offer these files. They are public domain,

and there are mirrors of the mainsite that distribute them for you.

The developersite for Linux is:

tsx-11.mit.edu

And the directory where the 680x0 specific files can be found in is:

/pub/linux/680x0

Here you will find several subdirectories with the required files.

Archivenames mentioned above might vary. Please notes the ANNOUCE

files found in this directory as they contain important new-release

information as Linux is under constant development.

A comprehensive list of sites and where to find these files will be made

available.

Testing Linux

-------------

If you just want to see how Linux performs from a mimimum RAMdisk boot you

will not require the root.tar.gz and usr.tar.gz files. You will only need

2 MB of free AmigaDOS space and NO harddrive partition.

Unpack the GZip'ed files

amiboot-1.9.gz          (or newer)

filesys.gz              (1024k image file!)

vmlinux-0.9.3.gz        (or newer)

in a AmigaDOS directory.

You should now have these files:

bootstrap               (amiga executable)

RAMdisk-image           (linux RAMdisk root)

vmlinux-0.9.3           (or newer versionnumber)

To test Linux by booting from the RAMdisk image:

bootstrap -k vmlinux-0.9.3 -r RAMdisk-image

Linux will now take over the AmigaOS and a grey screen telling you that

Linux is starting up should appear. After a few seconds when Linux has

checked what hardware you are using and loaded the necessary files, a

prompt should appear. Something like:

bash#

And a flashing cursor. You can now try various UN*X commands like "ls"

and "cd". You will not be able to perform any usefull UN*X activites

but it will let you know that Linux DOES work on your Amiga. If running

Linux from the RAMdisk works you can be sure it will work from a

harddrive too. This can be usefull before getting all the Linux files

needed for installation.

To shutdown Unix and reboot your Amiga enter:

/sbin/shutdown -r now

After a few seconds when Linux has updated all files it will automatically

shutdown and reboot your Amiga. As UN*X uses it's own HFS it is VERY

important when running from a harddrive that you use the "shutdown" command

and preferably "sync" before too to make sure all data is saved. It is

very easy to wreck the whole Linux OS if you don't follow this procedure.

All data will then be lost (from the Linux partitions) and can NOT be

restored.

Preparing files for harddrive installation

------------------------------------------

Assuming you downloaded all the required files you are now ready to

start installing Linux on your machine.

Put all files into a directory and unzip the GZip'ed files:

gzip -d <filename>.gz

You will need about 25MB of AmigaDOS space for the unziped archives.

These should be:

filename                description                     approx size

-------------------------------------------------------------------

bootstrap               amiga executable                ~12 kB

RAMdisk-image           Linux filesys image             1 MB

vmlinux-0.9.3           or newer version for amiga      ~650-700 kB

root.tar                Linux / (root) image            ~2.7 MB

usr.tar                 Linux /usr (additiona) image    ~18 MB

Preparing suitable harddrive partitions

---------------------------------------

You are now ready to make the required Linux harddrive partitions. Note that

these steps must be followed CAREFULLY as one mistake might RUIN all or

some of your AmigaDOS partitions. If you are not 100% sure how to do the

following steps please consult help from a more experienced Amiga user or

your Amiga reseller. Please read these steps carefully before you start.

I would recommend that you put Linux on a own harddrive, but there is

nothing wrong by having it on the same harddrive as your other AmigaDOS

partitions. It's merely for safety when doing this for the first time.

I also recommend that you use Commodores HDToolBox supplied with AmigaDOS

2.0 or newer. Note that functions might vary slightly from those specified

in this documentation. I am using HDToolBox v40.3, and steps will be

described according to that versions functions. HDToolBox can be found

in your "Tools" directory of your Workbench bootpartition or disk.

If you own a GVP, WarpEngine or other SCSI/IDE-controller you will need to

start HDToolBox from CLI with the commandline:

HDToolBox <device>

Where device is:

scsi.device             Commodore internal SCSI (A3000)

gvpscsi.device          GVP internal/external SCSI/IDE controller

or other supplied device that came with your SCSI/IDE controller. Please

consult your SCSI/IDE controller manual for more information.

You can also edit the info for HDToolBox to use it from Workbench.

Click once on the HDToolBox icon and select Icon/Information (hotkey AMIGA-I)

from the Workbench menu. Enter NEW item with the following line:

device=<device>

as specificed above. HDToolBox uses Commodores scsi.device as default.

The required device should be found in the DEVS: directory of your boot

partition or disk.

You should be able to use any supplied HD-partitioning software that came

with your controller. But as it might NOT tell you all the information you

need I recommend you use HDToolBox to be sure. Consult your manual.

After starting up HDToolBox it will read all your connected SCSI/IDE drives

and show a list of them in a window. Select the desired harddrive you want

to install Linux on. Make a note of which drive you selected. First drive

is listed first (DO NOT note down SCSI ID).

If HDToolBox finds newly added drives or unknown drive types it will

ask if you want to save the drive type to a lookup-table on your

Workbench disk. Refer to the AmigaDOS manuals on more information !

AmigaDOS partition

------------------

Later in the installation you will need to access the AmigaDOS volume

that contains the archived Linux files (root.tar and usr.tar). Therefor

you will need to find out what drive and partition these are located on.

In HDToolBox:

- Select the harddrive that contains your AmigaDOS volumes

- Click on PARTITION DRIVE

Find out which partition NUMBER (leftmost in graph = 1st partition, last

is shown as rightmost partition in graph) you have put the archived files

on and note down it's number and which drive number it is on. Drivenumber

is deteremined by counting from the top and down in HDToolBox's listing

of drives.

For example you put the files in DH3:Linux/Archive. DH3: is refered

to as the 2nd partition in the partitioning setup graph. Note that DH3

not necessary has to refer to the 3rd partition because of the number.

You have to check to find what drive has the name DH3 in HDToolBox's

partitioning setup.

Partitioning

------------

Before we can proceed we need to get some information about the desired

harddrive you want to use. Note what drivetype and manufactor the desired

drive has.

- Click on CHANGE DRIVE TYPE

- Select the correct drivetype from the list by clicking on it

and choose EDIT OLD.

A list of the harddrive data will be shown. You will need tonote down how

many BLOCKS PER CYLINDER this drive has as you will need it later.

Perform this operation on all the drives you want to use for Linux. If

you want both partitions on the same drive it will only be necessary to

do it once. MAKE SURE YOU SELECTED THE CORRECT DRIVE. If no data is

available on your drive select NEW instead of EDIT OLD and then

READ DATA FROM DRIVE to obtain the correct values.

DO NOT TRUST LEAFLETS that came with your drive. USE HDTOOLBOX !

Exit by clicking CANCEL or OK. Refer to the AmigaDOS manuals.

- Click on PARTITION DRIVE

Do NOT change ANY AMIGADOS partitions unless you are SURE you want to

change them. Chaning the AmigaDos partitions will ERASE ALL DATA that

was on that partition.

Please refer to your AmigaDOS manuals for more specific information on

how to use HDToolBox if in doubt!

- Click on ADVANCED OPTIONS to turn them ON

Select the desired partition from the graphic view above the buttons.

If you want to make a new partition click on NEW PARTITION and select

a empty space to use.

Adjust the size of the partition to the required amount using the arrow

below the partition graph.

- At least 3MB for the root partition

- At least 19MB for the usr partition

- Click on the DEVICE NAME BOX marked "CHANGE_ME" and alter the

name of the partition. Avoid using existing names on other drives.

It is not important what this name is set to.

- Select BOOTABLE to NO (box not checked)

- Click on CHANGE FILE SYSTEM and select CUSTOM FILE SYSTEM

- Select AUTOMOUNT to NO (box not checked)

- Set FILE SYSTEM BLOCK SIZE to 512

- Enter new IDENTIFIER :

root:   0x42534452

usr:    0x42534453

These are not important when using version 0.93 of the kernel.

- Select RESERVED BLOCK AT BEGINNING and END to 0

- Select USE CUSTOM BOOT CODE to NO (not checked box)

NUMBER OF CUSTOM BOOT BLOCKS should be 0 too.

Exit by clicking on OK to return to the partitioning setup.

Make a note of TOTAL CYLINDERS for each partition. Also note WHICH

partition on the drive it is. First partition is shown as the leftmost

partition in the graph, and the last one is the most rightmost.

For example if you made a new partition that is shown as the 4th

partition from left in the graph and it contains 419 total cylinders

note down:

root:   Drive 1, partition 4, 420 cylinders, 60 blocks per cyl.

or

usr:    Drive 3, partition 1, 1600 cylinders, 49 blocks per cyl.

You will need these figures later!

Make another partition following the steps above until you have two

partitions. One at least 3MB and the other at least 19 MB. Now exit the

partitioning setup after making notes on which drive, what partition and

how many cylinders each partition uses as described above.

- Click on OK

- Click SAVE CHANGES TO DRIVE

- Select CONTINUE if you are 100% SURE you want to save changes.

- Click on EXIT

Preparing fstab

---------------

You are now ready to install Linux on your partitions. But first you need

to extract one file from the "root.tar" archive and edit it's contents to

suit your drive setup.

tar xf root.tar etc/fstab       (amigados)

fstab (filesystem table) contains information on your connected harddrives

by using the following syntax:

/dev/sdXY       /partition      filesystem      options

where:

"X" is a letter for which drive ID

(a being first drive on controller,

b the second drive etc.)

"Y" is the number of the partition on that drive

(1 being first partition, 4 the 4th partition etc)

"partition" is the name of the partition in Linux

(for example / or /usr)

"filesystem" is what type of filesystem this partition has

(for example minix, ext2, proc, affs etc).

"options" can be bits to show if partition is read/writable etc.

Each entry must be separated with at least one space and be on the same line.

A # in from of a line means the line is commented out.

Now you will need to use an editor to edit this file to fit your setup. This

is where the information obtained during partitioning the drive is needed.

If your root-partition was created on drive 1, as partition 4 this would be

entered as:

/dev/sda4       /               minix   default

And if your usr-partition was created on drive 3, as partition 1 it would be

entered as:

/dev/sdc1       /usr            minux   default

In other words the first partition on the lowest numbered drive on your

controller is /dev/sda1.

Edit the "fstab" file to your setup and comment out previous setups. Save it

when you are done.

Booting Linux for installating

------------------------------

Now you can start Linux using the RAMdisk-image to prepare your harddrive

partitions and install the necessary files on them.

bootstrap -k vmlinux-0.9.3 -r RAMdisk-image

^^ ^^^^^^^^^^^^^ ^^ ^^^^^^^^^^^^^

name of          name of ramdisk

kernel to use    image file

-k option specifies kernel filename, default "vmlinux".

-r option specifies to use ramdisk-image as specified.

When Linux has booted and the prompt has appeared on your screen you are

ready to install the minix filesystem on your harddrive partitions.

Before you commence you will need to calculate the size of each partition

in 1kB blocks. AmigaDOS specifies them as 512 bytes.

size = blocks per cylinder * number of cylinders / 2

Calculate using the above formula for both the root and usr partitions.

Note down these figures and keep them with the rest in case you will

need to change anything later or reformat any partition.

Create a minix filesystem on both partitions using the following syntax:

/sbin/mkfs /dev/sdXY <size>

X is the drive number and Y is the partition number on that drive. size is

the calculated size above. For example to prepare root as in example above

where root is on drive 1, partition 4 and has 420 cylinders with 60 blocks

per cylinder:

size = 60 * 420 / 2 = 12600

/sbin/mkfs /dev/sda4 12600      (linux commandline)

You have to be 100% sure before you perform this operation. Formatting the

wrong drive and partition could destroy an AmigaDOS partition !

For the usr partition in the example:

size = 49 * 1600 / 2 = 39200

/sbin/mkfs /dev/sdc1 39200      (linux commandline)

You have now formatted the two partitions you need for installing Linux onto.

Because of Linux's HFS filesystem you have to mount drives as directories

under the root-directory. This is because the usr-partition is refered

to as /usr. The fstab file tells Linux how to and where mount additional

drives to your Linux system. This design makes it possible to mount AmigaDOS

partitions as own directories in Linux. We will need to do so now to access

the archives containing the required Linux root files.

First we need to create a temporary directory where we will mount the correct

AmigaDOS volume that contains the archived files.

mkdir /mnt2                             (linux commandline)

mount -t affs /dev/sdXY /mnt2           (linux commandline)

where X is the drive number containing the AmigaDOS volume and Y is the

number of the desired partition where the archived files reside.

Now we need to mount the root partition. We will put it in the /mnt directory:

mount /dev/sdXY /mnt                    (linux commandline)

where X is the drive number where your Linux root harddrive partition is, and

Y is the number of that drives partition you created root on. This is the same

as above where you formatted the Linux drives.

Go to the Linux root directory and unpack the root-archive from your AmigaDOS

partition:

cd /mnt                                 (linux commandline)

tar xvf /mnt2/path-to/root.tar          (linux commandline)

For example if you put the "root.tar" archive in /Linux/Archive enter:

tar xvf /mnt2/Linux/Archive/root.tar    (linux commandline)

Note that Linux is CASE SENSITIVE !

To make sure everything is saved onto the disk:

sync                                    (linux commandline)

Now we need to mount the usr-partition and unpack the usr.tar to it:

mount /dev/sdXY /mnt/usr                (linux commandline)

where X is the drive number where your Linux usr harddrive partition is, and

Y is the number of that drives partition you created usr on. This is the same

as above where you formatted the Linux drives.

cd /mnt/usr                             (linux commandline)

tar xvf /mnt2/path-to/usr.tar           (linux commandline)

sync                                    (linux commandline)

Now we need to put the new "fstab" in the correct directory:

cd /mnt/etc                             (linux commandline)

cp /mnt2/path-to/fstab .                (linux commandline)

sync                                    (linux commandline)

We are now ready to reboot and use the harddrive partition, but first we need

to unmount the mounted drives:

umount -a                               (linux commandline)

umount /mnt                             (linux commandline)

/sbin/shutdown -r now                   (linux commandline)

Now you're Amiga will reboot AmigaDOS again. You might have to press

CTRL-AMIGA-AMIGA manually if "shutdown" doesn't reboot after a few

seconds.

Booting from installed harddrive

--------------------------------

Assuming your Amiga reboots and you managed to install Linux correctly you

can now try and boot Linux from the harddrive partition:

bootstrap -k vmlinux-0.9.3 root=/dev/sdXY

where X is the drive number where your Linux root harddrive partition is, and

Y is the number of that drives partition you created root on. This is the same

as above where you formatted the Linux drives.

If everything goes well you should be prompted with a login-prompt if

Linux managed to boot in multi-user mode. Don't be alarmed if you do not

get this prompt and instead a bash-prompt as when you booted with the

ramdisk-image earlier. This only means that something in the configuration

isn't correct.

At the login-prompt you should be able to login as root without any password.

If this is successfull accomplished you may skip the next chapter.

Failed to boot in Multi-User mode

---------------------------------

If the login prompt doesn't show it means something in your current

configuration is wrong. Most likely it is your "fstab" file. If you

get the error "fchk.mkfs /dev/sdXY failed, wrong filesystem" or

similar during Linux boot you need to edit the fstab file, or copy

the new you made earlier from AmigaDOS. To copy the new file in:

mount -t affs /dev/sdXY /mnt2

cd /mnt2/path-to-fstab

cp fstab /etc

Remember to use the correct drivenumber (X) and partition number (Y)

for the correct AmigaDOS volume.

If you wish to edit the "fstab" file you can do this in several ways.

Currently the editor "ed" is only available after you booted. The

documentation for "ed" is found on Internet by ftp to:

tsx-11.mit.edu   /pub/linux/packages/utils/util-linux.1.1.src.tar.gz

To use "ed":

cd /etc

ed fstab

If you don't feel confident to use "ed" you can use "cat". It can be usefull

to first view the fstab file before editing it, to have a reference to

the syntax of it:

cd /etc

more fstab

cat > fstab

"cat" copies from the console to the file fstab. You will probably not be

able to correct any typing errors if you make any. End with CTRL-D. Refer

to earlier information on editing fstab to find out what to write in this

file.

Cleaning up the usr-partition

-----------------------------

The usr.tar archive supplied with Linux requires that you will have to

move some files to their correct paths. In general this is done by moving

an entire directory-tree one step up towards the root-directory. After

your installation the contents of / would be:

/usr/usr

/usr/usr/bin

/usr/usr/lib

...

This is wrong, as the directory-tree should be:

/usr/bin

/usr/lib

...

To correct this you will have to move all the directories one directory up,

EXCEPT for usr. Delete usr and mv the other files one directory up.

MAKE SURE YOU ARE LOCATED in /usr/usr by using the "pwd" command. It prompts

what directory you are in.

cd /usr/usr

command rm -rf usr

mv * ..

Now "mv" won't move complete directories so these have to be moved one by

one. There aren't that many. Use "ls -la" to list all files in current

directory (works similiar to dir):

ls -la

mv bin ..

mv lib ..

Continue to move files using "mv" until all the listed directories have

been moved. Check by using "ls -la":

ls -la                  (should not list any dir's except . and ..)

cd /usr

command rm -rf usr      (delete the empty /usr/usr directory)

Now Linux should be correctly installed and ready to be configured to

suit your network setup. Check out all the files in /etc and edit them

to suit your network. The editor "vi" should now be available, if not

try and logout and relogin:

exit

Now relogin using the login root and no password.

System overview

---------------

Overview of directories:

/                       root directory

/bin                    binaries

/dev                    devices such as console, harddrives etc.

/etc                    setup files

/home                   directory for login users or link to it

/lib                    libraries for compiling and more

/mnt                    empty directory. can be deleted

/mnt2                   empty directory. can be deleted

/root                   login directory for root user

/sbin                   system binaries (only for root)

/usr                    harddrive partition 2 (userfiles)

/usr/bin                additional binaries

/usr/etc                additional setup (users/applications etc)

/usr/lib                additional libraries

/var                    various files (logs etc)

Example of Bourne Again Shell (bash) configuration files

--------------------------------------------------------

Each user has it's own "home" directory. These are usually located in

/home/loginname/. There are no rule for where you want the users home-

directories but it's wise to collect them in one common place. I made

a symbolic link called /home which points to /users. Here I make

directories for each user of my system and edit the /etc/passwd file

to suit this setup.

By default Linux comes with bash. However you can compile csh, tcsh or

any other shell as you like. I will here just describe some VERY basic

functions of the bash configuration files.

Files in users home-dir and /root:

.profile                executed first at login

.bashrc                 user-configurable bash sourcefile

.bash_history           log of last actions in bash

.bash_profile           executed first when bash is started

Example .bashrc file:

#

# Userdefined aliases

#

# Run after .profile

#

alias dir='ls -la'

alias ren='mv'

alias rd='rm -rf'

alias del='rm'

alias ll='ls -l'

alias move='mv'

alias copy='cp'

alias md='mkdir'

Bash commands:

EDITOR=editor           Set default editor (ie. vi)

TERM=termtype           Set default terminaltype (ie. vt100)

PATH=paths              Set default searchpath (ie. /bin:/sbin)

export VARIABLE         Let system know of change (ie. export TERM)

alias this='do that'    Make alias for "do that" (ie. dir='ls -la')

It's recommended that you do further research on how to use the different

shells that are available.

The editor "vi"

---------------

vi is widely used in the UN*X communitity as it's available to most

systems there is. Here is a quick breif userguide to vi. To find out

more use "man vi".

Syntax:         vi filename

Commands:       :d      delete line

:x      save and exit

:quit!  abort edit and exit

a       append mode, starting after current charcter

i       enter insert mode, starting before current

character on line.

esc     enter command mode (to use :d etc)

The password file

-----------------

Users passwords and an overview of the login accounts on the system are

found in /etc/passwd. This file has the following syntax:

loginname:password:groupid:userid:Users Name:homedir:startup-shell

Example:

root::0:0:System Maintainer:/root:/bin/bash

When no password is specified there is no need to enter a password at login.

You can use the command "passwd" in UN*X to change your current password.

This password will of course be encrypted so no other users can view

/etc/passwd and find out your login password.

To add new users simply edit /etc/passwd and add a new line with the

user information. DON'T FORGET to make a home-directory for this user !

Other files in /etc

-------------------

This directory contains all the system configurations and all files should

be made only writable by the root-user. Here's a quick overview of the

files that can be found here:

bootptab        Information on how to start up remote machines

exports         Tells what NFS filesystems are available to others

fstab           File System Table (list of automounting drives)

gateways        List of available network gateways

group           List of available usergroups

host.conf

hosts           List of IP-to-address lookups

hosts.allow     Which hosts are allowed to login

hosts.deny      Which hosts are denied to login

hosts.equiv     Hosts trusted to do rsh logins

hosts.lpd       Hosts trusted to use local printer services

inet.conf       Services available from this server

inittab         Initiziation table for devices

issue           Login message (server information - see rc.local)

magic           Table of magicnumbers (filetypes)

NETCONF         System network configuration file

networks        List of address-to-IP lookups (if no nameserver)

profile         Global profile for login

protocols       List of protocols and functions

rc              Boot script, executes rc.local

rc.local        User-defined boot-script

rc.net          Network bootscript

resolv.conf     address-to-IP for gateway?

securetty       Which terminals are allowed root login??

services        Network services available

shells          List of shells and their binary-directories

syslog.conf

termcap         List of how to handle different terminallogins

Sources

-------

This document was made based on the following sources:

Linux PC documentation          tsx-11.mit.edu:/pub/linux

Amiga HowTo install textfile    tsx-11.mit.edu:/pub/linux/amiga

NetBSD beta installation        sun-lamp.cs.berkeley.edu

Linux diskimage file descriptions

Credits

-------

The people on the channel #Amiga on IRC (Internet Relay Chat)

The Commodore Amiga team for making the Amiga

The authors of Linux for the Amiga

Special thanks to

-----------------

The Author

----------

The author will not take any responsibility for errors or fault in this

documentation, or faults to your system(s) for using this information.

However the author has done his best to ensure the correctness of the

included information.

Contributions to this documentation can be sent the author at the

following addresses:

Email:          oddjk@samson.ingok.hitos.no

WWW/URL:        http://linux.ingok.hitos.no/~oddjk

Snail mail:     Odd-Jarle Kristoffersen

Gammelgaardvegen 42

N-9020 Tromsdalen

Norway

Please don't request any help from either addresses as I will not have

time to follow up that. But you are free to report problems, updates or

any other contribution. These will be used to make a FAQ helpguide.

Feel free to report any distribution sites for Amiga Linux.

Future plans

------------

- How to get X11 windows to run on your Linux setup

- Supported hardware overview

- Complete FTP site list of Amiga Linux distributors

- Internet Online help for Linux