##############################################################################

###  ####  ####   ####  ##  ###  ###   ###  ###   ###   ##   ## ##### ##   ##

## ## ## ## ## ##  ## ## ## ##   ## ## ##   ##    ## ##  ### ### ##    ### ###

##### ## ## ## ##  ####  ## ##   #####  ###  ###  ## ##  ####### ####  #######

## ## ## ## ## ##  ##    ## ##   ## ##    ##   ## ## ##  ## # ## ##    ## # ##

## ## ####  ####   ##    ##  ### ## ##  ###  ###   ###   ##   ## ##### ##   ##

##############################################################################

VERSION 33.03

This  is  the second public release of AddPicassoMem (APM for short) a program

to use the memory of the VillageTronic Picasso-II graphics card as public fast

memory.

WHY I DID IT

I  know it's a shame to waste the graphics capabilities of this great card and

use just its memory, but my A3000 has still V36 ROM's and sometimes I fallback

to  kickstart 1.3 to play some game...  Under 1.3 The Picasso-II can't be used

as graphics card, and its memory stays there doing nothing!

I  have  2  Mb  chip  and  8 Mb fast, and the Zorro2 bus performance is not so

exciting:   Zorro2  memory cards are much slower than chip ram on A3000's, but

one night I was thinking to all the A2000ers who maybe may like a utility like

this:  how about a 1Mb or 2Mb fast ram expansion for free?

I strongly recommend you to use your Picasso-II to e.g. show Hajime Sorayama's

or Boris Valejo's JPEGs rather than to contain some million numbers, anyway if

you really need more memory this may be a cheap solution.

CONFIGURING A CARD

"Configure a card" means tell AmigaDOS what the card is made for.

Once  a  card is configured via the BindDrivers command it can't be configured

again  (until you reset, naturally), so the first step to make APM work is NOT

to run the BindDrivers command.

If  you  configure the card using APM, BindDrivers will not try to reconfigure

it  until you don't remove the Picasso-II memory chunk from public list.

This  means  that  when  Picasso-II  memory  is  available for public use, the

village.library  will  not  be loaded and the AmigaDOS thinks that the card is

just an expensive fast memory expansion.

Configuring  the Picasso-II using APM gives you one advantage:  if you need to

use  the  card  for its graphics capabilities, you can still backup a move and

dismount the memory (if nobody is using it).

Naturally  to  reconfigure  the  card  as graphics engine you will need to run

BindDrivers  and then you will not be able to use APM again, but you can mount

and  dismount  Picasso-II  memory  as  many times as you like, until you don't

run the BindDrivers command.

USAGE

APM  is  a stand alone CLI/Shell-only program that can run under kickstart 1.2

and later releases.

If you give it no arguments it will try to mount the memory chunk, if you give

it  the  "remove"  switch argument it will try to remove the memory chunk, any

other  argument  will  show  you  a  help  line.   All  the rest is automagic:

starting  address  and  size  of  the card are fetched directly from the card,

priority will be the lowest fast ram priority less one, or zero if no fast ram

is installed.

Return codes on program exit may give a rough idea of what happened:

OK    (0) = All right, memory has been added/removed to/from the public list

WARN (10) = Error in the command line arguments

FAIL (20) = APM is not able to add/remove memory to/from the public list

Every  return  code  is  followed by a standard output message. Warn status is

followed  by  "REMOVE/S: TRY TO REMOVE PICASSO-II PUBLIC MEMORY",  okay status

can be followed by one of this two messages:

ADDING MEMORY $aaaaaaaa-$bbbbbbbb, PRI: (-)ccc

memory chunk has successfully been added to the public list.

card is now in ram only mode and can't be used as graphics engine.

aaaaaaaa is the chunk's starting address

bbbbbbbb is the chunk's ending   address

ccc      is the chunk's priority

PICASSO-II MEMORY REMOVED

memory chunk has successfully been removed from the public list.

card is now ready for a new configuration as graphics engine or again as a ram

only card.

Finally, the following are the fail status messages:

CAN'T OPEN EXPANSION.LIBRARY

expansion  is  a resident library and must always be available to any program,

so  this  error message should never appear.  If it does appear, maybe you are

trying to run APM on a 5000 MegaMips Amiga.  Just load the last (I know, it is

20 years old) 680x0 version of the operating system and try again! ;-)

PICASSO-II DRIVER ALREADY IN USE

cannot add memory when village.library is loaded.

village.library is the video driver of the card, if it is found in memory, the

card  is already configured as graphics board.  this is due to the BindDrivers

mambo-jambo (see "configuring a card").

PICASSO-II CARD IS IN SEGMENTED MODE

cannot add memory if the card has been installed in segmented mode.

this  will  happen  only  on 68000 or 68010 Amiga's with 8Mb of fast ram other

than  the  one on the Picasso-II.  The largest segment is 1/16th of the memory

on the card, I think adding 64Kb to the public list makes no sense.

PICASSO-II CARD NOT CORRECTLY INSTALLED

cannot  add  memory if the card is not correctly installed or not installed at

all. for my own information: does the card work in graphics engine mode?

PICASSO-II CARD ALREADY IN USE

cannot  add  memory  if the card is currently configured either in ram only or

graphics  engine  mode.   if  the  card  is  working  in ram only mode you can

dismount  the  memory  and  re-mount  it  (why?).   if  the card is working in

graphics  engine mode there is nothing to do other than resetting the Amiga if

you want to use the card as memory expansion (BindDrivers mambo-jambo).

CAN'T WRITE TO PICASSO-II MEMORY

cannot add memory if the card is write protected.

when  you  boot, the memory on Picasso-II can't be written, but after a bit of

initialization it can be write enabled.

since  I  don't  have any technical doc on waking up this card, I wrote my own

initialization  routine  which  can  have  some wrong timing.  I think it will

always work thanks to Buster, if it does not, it's a programmer's fault :(

CAN'T FIND PICASSO-II MEMORY ADDRESS

cannot add memory if the software coordinates of the card are unknown.

when you boot, each card can be put anywhere in memory.  If there's not enough

addressing  space  for  all the cards, some card may be left off.

this error should appear if you didn't set correctly the segmented mode jumper

on  the  card,  in other words the card is in normal mode when it should be in

segmented mode.

PICASSO-II MEMORY ALREADY IN PUBLIC LIST

cannot add memory twice.

you  already  added  the memory to the public list (now the only thing you can

do, if you wish, is to remove it).

PICASSO-II MEMORY NOT IN PUBLIC LIST

cannot remove the memory chunk if it hasn't been added.

this  message  is  shown when you didn't add the memory to the public list but

it  also appears if you renamed the memory chunk.  actually the case dependent

chunk name is "picasso2 memory", you can restore this name and try again.

MANGLED PICASSO-II MEMORY HEADER

cannot remove the memory chunk if it was modified by someone else.

maybe  someone  moved  the address of the memory header node.  then APM can do

nothing legal to rescue the original structure.

PICASSO-II MEMORY IN USE

cannot remove the memory chunk while it is in use.

someone allocated part of the Picasso-II memory for its purposes, so the whole

chunk  cannot  be freed until all the memory has been released.  if Picasso-II

memory  is not free, APM tries to flush all the resident modules before giving

up.  I flagged Picasso-II memory in a way that system friendly reset resistant

modules can't be allocated in its space (fast, public, 24bitdma).

DISTRIBUTION

This  package  is  released under the concept of freeware, the package must be

distributed  as one whole.  The distributor may charge a fee up to the cost of

the medium for the entire package.

This  package, or part of it, cannot be included with commercial distributions

without written permission from the author, however this document is a written

permission to include this package in the Picasso-II firmware disks and in any

CD-ROM collection.

NO WARRANTY

This package is provided as is, without warranty of any kind, either expressed

or implied.

Should  the  package  prove  defective,  you  assume  the  entire  cost of all

necessary  servicing,  repair or correction even if I have been advised of the

possibility of such damages.

I'm not responsible of the results of the use of the package.

HISTORY

33.01 (02-May-94) First public release

33.02 (24-Oct-94) Testers only release

33.03 (06-Nov-94) BUGFIX: APM 33.01 returns "already in public list" if there

are other Zorro2 expansion boards AND if the Picasso-II is

not in the first slot (I accidentally swapped the board

address and the board size variables in a subroutine).

Thanks to:

Gabriele Bonora for the bugreport,

Gabriele Greco  for the bugreport and the hint for bugfix

(Thanks again: That bug would be very hard to find

without your source-code level help).

SUPPORT

If  you  have  any  suggestions, bug reports, or wish to let me know something

about the package feel free to contact me at nsoggia@telnetwork.it

Have fun,

/\/ / /-<