TABLE OF CONTENTS

ros.library/--Overview--

ros.library/--History--

ros.library/ROSAddTimerInt

ros.library/ROSAllocAudio

ros.library/ROSAllocMem

ros.library/ROSAwakeSystem

ros.library/ROSChipsetCheck

ros.library/ROSCPUCheck

ros.library/ROSCPUClock

ros.library/ROSDeleteFile

ros.library/ROSDisableReq

ros.library/ROSEnableReq

ros.library/ROSFreeAudio

ros.library/ROSFreeMem

ros.library/ROSGetDMA

ros.library/ROSGetInt

ros.library/ROSGetKey

ros.library/ROSKillSystem

ros.library/ROSQueryKeys

ros.library/ROSReadExe

ros.library/ROSReadFile

ros.library/ROSRemTimerInt

ros.library/ROSRequester

ros.library/ROSScreenToBack

ros.library/ROSScreenToFront

ros.library/ROSSetCache

ros.library/ROSSetCopper

ros.library/ROSSetDMA

ros.library/ROSSetExitHandler

ros.library/ROSSetInt

ros.library/ROSSetIntVec

ros.library/ROSSetTimerVec

ros.library/ROSStartTimer

ros.library/ROSStopTimer

ros.library/ROSSysCall

ros.library/ROSSysCallEnd

ros.library/ROSWaitVBlank

ros.library/ROSWriteFile

ros.library/--Overview--                              ros.library/--Overview--

This section describes the way how to use the RETIRE Operating System within

your own applications:

The ros.library is an enhanced lowlevel.library. It's compatible with currently

all available machines and CPU's (upto '060) and OS1.3 compatible too!

The ROS provides additional functions to take over the entire system so it

should be useful for demo & game coders.

Another advantage is the ability to call important system functions (memory

allocation, read, write) from a killed system.

ROS offers a way to run your hardware bashing application within multitasking.

And finally a ROS prefs file is provided to support custom settings.

On startup ROS tries to load the ROS.prefs from your ENV: directory. You may

edit this file to configure the library for your needs.

But remember: It's impossible to open the ros.library from two applications at

the same time. The second application will fail to open the library!

Programming guidelines:

The ros.library is a standard Amiga library so normal restrictions applies to

function calls:  - a6 loaded with _ROSBase

- d0/d1/a0/a1 scratch registers (destroyed by libcall)

- functions can't be called from within interrupts

Some ROS functions are special in these cases; they need no scratch registers

or they may be called from within interrupts. Look at the appropriate 'note'

comment for further information.

Do not call any ROS function from supervisor mode. The machine will crash!!!

I think there is no need to switch to supervisor mode. Think of the '060 CPU;

its supervisor model differs from the '040 CPU and a few basic commands will be

emulated by software. Executing such a command from supervisor mode may crash

the machine!!!

Call ROSKillSystem(ROSKSF_DEATHMODE) to take over the entire system. But do not

modify DMA & interrupt stuff by yourself! Use the appropriate ROS routines!!!!!

ROSKillSystem() opens a PAL-LowRes screen so all 'special' AGA registers are

flushed (FMODE may not!?!) and it forces graphics boards to switch to the Amiga

custom chip display.

After taking over the system you may use the blitter. You should finish your

blits before calling ROS functions which may invoke system calls. After the

function returns you may bash the blitter again.

Remember: Use a WaitBlit before freeing some chip memory!!!

In a multitasking environment your vblank code may be called after the display

is started depending on the speed of higher prioritized vblank interrupts.

If you want to synchronize custom chip modifications with the display you

should use the copper.

And remember: The vblank frequency may vary (not always 50 Hz) if ROS screen

behind other screens!

The registers you may trash:

- the display custom chip registers after taking over the system, in multi-

tasking mode only if the ROS screen is the frontmost

- the audio custom chip registers (if channels allocated)

- the blitter registers (only if taken over the system)

- the CIA registers (if successfully allocated)

You should never touch:

- DMACON and INTENA registers

- CIA ICR registers

- CPU interrupt vectors

- serial registers

- disk related registers

Please read the following autodocs carefully!

If you have questions, problems, idea's or bug reports feel free to contact me:

TIK/RETIRE via email:  Nico.Schmidt@Student.Uni-Magdeburg.de

or write to:           Nico Schmidt

M�hlauer Str. 9

06773 J�denberg

GERMANY

Thanks to:  TODI/RETIRE for hints and useful idea's

PABLO/RETIRE for beta testing

ros.library/--History--                                ros.library/--History--

ROS HISTORY

*******************************************************************************

RELEASE 1.0, 29.12.95 (Library version 1.0)

The first release.

*******************************************************************************

RELEASE 1.1, 05.01.96 (Library version 1.1)

Fixed two minor bugs:

- ROSCPUClock() shouldn't display a requester if no free CIA available,

but it does

- user vblank code was called from vblank server with priority 9, this

may confuse the timer.device (priority 0) and thus all CIA interrupts

(e.g. level6 p61) if it steals to much rastertime;

now user vblank code is executed from a priority 0 vblank server

*******************************************************************************

RELEASE 1.2, 08.01.96 (Library version 1.2)

Library name changed from 'ROS.library' to 'ros.library' because a library

name should always be a lower-case string (thanks to Thomas Kessler)

*******************************************************************************

ros.library/ROSAddTimerInt                          ros.library/ROSAddTimerInt

NAME

ROSAddTimerInt -- add an interrupt that is executed at regular

intervals

SYNOPSIS

intHandle = ROSAddTimerInt(Flags, intRoutine);

D0                         D0     A0

ULONG AddTimerInt(ULONG, APTR);

FUNCTION

Calling this routine causes the system to allocate a CIA timer

depending on 'Flags' and set up 'intRoutine' to service any interrupts

caused by the timer.

Although the timer is allocated it is neither running, nor enabled.

ROSStartTimer() must be called to establish the time interval and

start the timer.

The routine is called from within an interrupt, so normal

restrictions apply. On entry A5 holds the custom base ($dff000) and

A6 holds _ROSBase. Your code may trash all registers.

The CIA timer used by this routine is not guaranteed to always be

the same. This routine utilizes the CIA resource and uses an

unallocated CIA timer.

You should remove the timer with ROSRemTimerInt().

Closing the ros.library removes this timer interrupt automatically.

INPUTS

Flags - specify the timer to allocate:

TIMF_ANY  - try to allocate any timer

TIMF_LEV2 - try to allocate level 2 timers (CIA A)

TIMF_LEV6 - try to allocate level 6 timers (CIA B)

intRoutine - the routine to invoke upon timer interrupts.

RESULT

intHandle - a handle used to manipulate the interrupt, or NULL

if it was not possible to attach the routine.

NOTE

If allocation failed a Retry/Cancel-Requester appears to inform the

user about the problem (if not suppressed and if not in Death-Mode).

This function interrupts the Death-Mode state with ROSSysCall()!

SEE ALSO

ROSRemTimerInt(), ROSStartTimer(), ROSStopTimer(), ROSSetTimerVec()

ros.library/ROSAllocAudio                            ros.library/ROSAllocAudio

NAME

ROSAllocAudio -- allocate audio channels

SYNOPSIS

success = ROSAllocAudio()

D0

BOOL ROSAllocAudio(VOID);

FUNCTION

This function allocates all audio channels. Audio DMA is updated

so that matches with current DMA settings (former ROSSetDMA() calls).

You should free the audio channels with ROSFreeAudio().

Closing the ros.library frees audio channels automatically.

RESULT

success - boolean

NOTE

If allocation failed a Retry/Cancel-Requester appears to inform the

user about the problem (if not suppressed and if not in Death-Mode).

This function interrupts the Death-Mode state with ROSSysCall()!

SEE ALSO

ROSFreeAudio()

ros.library/ROSAllocMem                                ros.library/ROSAllocMem

NAME

ROSAllocMem -- allocate memory and keep track of the size

SYNOPSIS

memoryBlock = ROSAllocMem(byteSize, attributes, alignmask)

D0                        D0        D1          D2

VOID *ROSAllocMem(ULONG, ULONG, ULONG);

FUNCTION

This function works identically to Exec's AllocMem(), but tracks the

size of the allocation and provides additional memory alignment.

See the AllocMem() documentation for details.

You should free the memory with ROSFreeMem().

Do _not_ use exec.library functions!!!!!!!

Closing the ros.library frees allocated memory automatically.

INPUTS

byteSize - # of bytes to allocate

attributes - memory requirements

alignmask - used to allocate memory at special boundaries, if NULL

Exec's standard alignment applies

--> newaddr = memaddr AND NOT(alignmask)

e.g. mem align: even by 8 bytes -> alignmask = $00000007

RESULT

memoryBlock - a pointer to the newly allocated memory block or NULL

NOTE

This function interrupts the Death-Mode state with ROSSysCall()!

SEE ALSO

ROSFreeMem(), exec.lib/AllocMem()

ros.library/ROSAwakeSystem                          ros.library/ROSAwakeSystem

NAME

ROSAwakeSystem -- close ROS screen and awake from Death-Mode

SYNOPSIS

ROSAwakeSystem()

VOID ROSAwakeSystem(VOID);

FUNCTION

This function deallocates all things allocated with ROSKillSystem().

It closes the opened screen and sets the display related DMA to the

appropriate system values.

Calling this function without a matching ROSKillSystem() is harmless.

Closing the ros.library calls automatically ROSAwakeSystem().

SEE ALSO

ROSKillSystem()

ros.library/ROSChipsetCheck                        ros.library/ROSChipsetCheck

NAME

ROSChipsetCheck -- check if desired custom chipset available

SYNOPSIS

result = ROSChipsetCheck(chipsetbit)

D0                       D0

UWORD ROSChipsetCheck(UWORD);

FUNCTION

Use this routine to obtain the available chipset.

As input you may specify the chipset requirement (bitnumber) and if

_not_ found a requester appears to inform the user about his lack of

hardware (if not suppressed and if not in Death-Mode).

INPUTS

chipsetbit - the required chipset; currently defined are

ROSCSB_ECS for at least HR-AGNUS rev3 and 8373 DENISE

ROSCSB_AGA for at least ALICE rev2 and LISA,

or -1 for only obtaining chipset (no requester appears)

RESULT

result - chipset flags (ROSCF_xxx, AGA includes the ECS flag) or NULL

if required chipset not found

ros.library/ROSCPUCheck                                ros.library/ROSCPUCheck

NAME

ROSCPUCheck -- check if desired CPU/FPU available

SYNOPSIS

result = ROSCPUCheck(cpubit)

D0                   D0

UWORD ROSCPUCheck(UWORD);

FUNCTION

Use this routine to obtain the available CPU/FPU.

As input you may specify the processor requirement (bitnumber) and if

_not_ found a requester appears to inform the user about his hardware

problem (if not suppressed and if not in Death-Mode).

Call this routine twice if you need a CPU (AFB_680x0) check and a FPU

(AFB_6888x/FPU40) check.

INPUTS

cpubit - the required CPU/FPU -> AttnFlags AFB_xxxxx

or -1 for only obtaining CPU/FPU (no requester appears)

RESULT

result - AttnFlags or NULL if required chipset not found

NOTE

This code handles the FPU40 flag correct: it's only valid if AFB_68040

set. Keep this in mind if you examine the RESULT (AttnFlags)!!!

ros.library/ROSCPUClock                                ros.library/ROSCPUClock

NAME

ROSCPUClock -- CPU/FPU clock calculation

SYNOPSIS

CPU/FPUclock = ROSCPUClock()

D0  D1

ULONG/ULONG ROSCPUClock(VOID);

FUNCTION

Calculate CPU/FPU clock. The values returned should be accurate at

least 1/10 MHz, so it's enough for everyone.

This routine works with all (currently) available processors and it

takes only a few milliseconds!!!

In some cases the returned clock is NULL because of the lack of

free CIA timers (or other problems, see BUGS) so it's best to call

this routine first (e.g. before any timer function).

RESULT

CPUclock - the CPU clock in kHz or NULL on error (see also BUGS)

FPUclock - the FPU clock in kHz or NULL on error

NOTE

This function interrupts the Death-Mode state with ROSSysCall()!

BUGS

In some rare cases the function returns NULL:

- on 68000/10 with only chipmem

- on _really_ slow machines (e.g. a 10MHz 68040?!?)

- on 68040/60 systems where it's impossible to turn on instruction

cache (e.g. A4000 with only chipmem and 68040.library loaded)

The FPU clock is _always_ NULL for 68881/2 systems because I'm

(currently) unable to find a reliable algorithm (blame on me).

It's hard to deal with the asynchronous data transfer between CPU

and FPU.

On 68040/60 systems with FPU the returned value is the same like

CPU clock.

ros.library/ROSDeleteFile                            ros.library/ROSDeleteFile

NAME

ROSDeleteFile -- Delete a file or directory

SYNOPSIS

success = ROSDeleteFile(flags, name)

D0                      D0     A0

ULONG ROSDeleteFile(ULONG, STRPTR);

FUNCTION

This attempts to delete the file or directory specified by 'name'.

Note that all the files within a directory must be deleted before the

directory itself can be deleted.

If ROSRWF_DISKWAIT flag set the routine assumes that you want to delete

'name' from a floppy. This means:

- waiting for the disk to be inserted (can't be cancelled!!!)

(implementation: DOS error codes DEVICE_NOT_MOUNTED, NOT_A_DOS_DISK,

NO_DISK force retry until any other error occurs)

- waiting until disk drive stopped

(implementation: polling disk.resource until drives are ready)

Since OS2.0 it's possible to delete files from your program path

instead of the current directory using PROGDIR:xxx.

On OS1.3 the "PROGDIR:" string is automatically removed by ROS so you

don't have to worry about it. The only thing the user have to do is a

"CD" to the program directory before starting (only on OS1.3). ;-)

WARNING: It may be dangerous to use this routine within Death-Mode!

Because of the AMIGA's multitasking facilities it isn't

possible (isn't it???) to wait till the final end of a delete

operation. That's why I use a simple delay of 2 seconds after

any delete operation in Death-Mode. If your filesystem is to

slow to write all the data within this time your partition

will be _not_ validated!!!

You've been warned!!!

INPUTS

flags - use ROSRWF_DISKWAIT if you want to delete from a floppy

name - pointer to a null-terminated string

RESULT

success - DOS error code on failure or NULL for success

NOTE

This function interrupts the Death-Mode state with ROSSysCall()!

SEE ALSO

DOS.lib/DeleteFile()

ros.library/ROSDisableReq                            ros.library/ROSDisableReq

NAME

ROSDisableReq -- suspend requesters for our task

SYNOPSIS

ROSDisableReq()

VOID ROSDisableReq(VOID);

FUNCTION

Suspend all upcoming requesters for our task (ROS- and DOS-Requesters).

By default requesters are enabled.

SEE ALSO

ROSEnableReq(), ROSRequester()

ros.library/ROSEnableReq                              ros.library/ROSEnableReq

NAME

ROSEnableReq -- permit requesters for our task

SYNOPSIS

ROSEnableReq()

VOID ROSEnableReq(VOID);

FUNCTION

Permit all upcoming requesters for our task (ROS- and DOS-Requesters).

By default requesters are enabled.

SEE ALSO

ROSDisableReq(), ROSRequester()

ros.library/ROSFreeAudio                              ros.library/ROSFreeAudio

NAME

ROSFreeAudio -- free audio channels

SYNOPSIS

ROSFreeAudio()

VOID ROSFreeAudio(VOID);

FUNCTION

This function removes probably installed audio interrupt handlers and

resets the audio DMA (former system values). Then channels are freed.

Freeing channels without allocation is harmless.

Closing the ros.library frees audio channels automatically.

NOTE

This function interrupts the Death-Mode state with ROSSysCall()!

SEE ALSO

ROSAllocAudio()

ros.library/ROSFreeMem                                  ros.library/ROSFreeMem

NAME

ROSFreeMem -- return allocated memory to the system

SYNOPSIS

ROSFreeMem(memoryBlock)

A0

VOID ROSFreeMem(VOID *);

FUNCTION

Free an allocation made by the ROSAllocMem() call. The memory will be

returned to the system pool from which it came.

WARNING: Before freeing some chip memory be sure that the blitter has

finished its work within this memory block!!!

The same applies to closing the ros.library.

Closing the ros.library frees allocated memory automatically.

NOTE

Passing a wrong pointer is harmless ;-)

This function interrupts the Death-Mode state with ROSSysCall()!

INPUTS

memoryBlock - pointer to the memory block to free, or NULL

SEE ALSO

ROSAllocMem()

ros.library/ROSGetDMA                                    ros.library/ROSGetDMA

NAME

ROSGetDMA -- obtain DMA settings

SYNOPSIS

DMACONR = ROSGetDMA()

D0

UWORD ROSGetDMA(VOID);

FUNCTION

This function returns current DMA settings (set via ROSSetDMA()).

Do _not_ use "move.w $dff002,d0" !!!!!!!

This may return wrong bits if you are in Non-Death-Mode (e.g. copper

DMA -> the OS may change the bit during screen switching).

But ROSGetDMA() returns the right value.

Closing the ros.library reset former system DMA.

RESULT

DMACONR - DMA settings set via ROSSetDMA() with additional BBUSY/BZERO

bits (unsupported (disk) & unallocated (audio, ...) DMA bits

are always 0)

NOTE

This call is guaranteed to preserve all other registers and is save

to call from interrupts.

SEE ALSO

ROSSetDMA()

ros.library/ROSGetInt                                    ros.library/ROSGetInt

NAME

ROSGetInt -- obtain interrupt settings

SYNOPSIS

INTENAR = ROSGetInt()

D0

UWORD ROSGetInt(VOID);

FUNCTION

This function returns interrupt enable bits (set via ROSSetDMA()).

Additional bits are defined in "ros.i".

Do _not_ use "move.w $dff01c,d0" !!!!!!! This may return wrong bits.

Closing the ros.library reset former system interrupt bits.

RESULT

INTENAR - Interrupt bits set via ROSSetInt()

Note: level 1/2/5/6 bits (disk, serial, timer) are not

supported!!!

Instead additonal bits are defined:  - KEYB

- CIAATIMA/B

- CIABTIMA/B

NOTE

This call is guaranteed to preserve all other registers and is save

to call from interrupts.

SEE ALSO

ROSSetInt(), libraries/ros.i

ros.library/ROSGetKey                                    ros.library/ROSGetKey

NAME

ROSGetKey -- returns the currently pressed rawkey code and qualifiers

SYNOPSIS

key = ROSGetKey()

D0

ULONG ROSGetKey(VOID);

FUNCTION

This function returns the currently pressed non-qualifier key and

all pressed qualifiers.

The values returned by this function are not modified by which

window/screen currently has input focus.

RESULT

key - key code for the last non-qualifier key pressed in the low

order word ($0..$67). If no key is pressed this word will be $ff.

The upper order word contains the qualifiers which can be found

within the long word as follows (lowlevel.library like):

Qualifier               Key

LLKB_LSHIFT             Left Shift

LLKB_RSHIFT             Rigt Shift

LLKB_CAPSLOCK           Caps Lock

LLKB_CONTROL            Control

LLKB_LALT               Left Alt

LLKB_RALT               Right Alt

LLKB_LAMIGA             Left Amiga

LLKB_RAMIGA             Right Amiga

NOTE

This call is guaranteed to preserve all other registers and is save

to call from interrupts.

SEE ALSO

lowlevel.lib/GetKey(), libraries/lowlevel.i

ros.library/ROSKillSystem                            ros.library/ROSKillSystem

NAME

ROSKillSystem -- kill the system and open a PAL-LowRes screen

SYNOPSIS

success = ROSKillSystem(mode)

D0                      D0

BOOL ROSKillSystem(ULONG);

FUNCTION

This function opens a PAL-LowRes screen, installs your copper list and

sets additional DMA (copper, bitplane, sprite).

By default all DMA will be cleared.

Whenever this screen is the frontmost you may bash all display related

custom chip registers (preferably via copper list).

If the ROS is unable to open the screen or if it's not PAL-LowRes

(promoted screen) a Retry/Cancel-Requester appears (if not suppressed).

If you specify the flag KILLF_DEATHMODE the multitasking will be turned

off. Now ROS has taken over the entire system. But interrupts and DMA

should only be set with ROS functions!!!

The blitter is owned via OwnBlitter() so you may use it. But before

using you should do a WaitBlit because it may still working.

You may interrupt the Death-Mode via ROSSysCall(). At this time the

blitter will be disowned so you do _not_ use it during a SysCall!!!

Call ROSKillSystem() with KILLF_SYSMODE to run your application within

multitasking. But be polite and bash the hardware as less as possible.

The blitter is under system control so you should use OwnBlitter()/

DisownBlitter() if you need it. But allocate the blitter only for a

short time because the system uses it too!

Also check out the ROS_KillSysFlags (the current ROS state).

You may call ROSKillSystem() multiple times to switch between Death-

and SysMode.

A switch from Sys- into Death-Mode automatically makes the ROS screen

the frontmost so you don't have to call ROSScreenToFront().

You should return to the system with ROSAwakeSystem().

Closing the ros.library calls ROSAwakeSystem() automatically.

INPUTS

mode - KILLF_SYSMODE for a multitasking environment

KILLF_DEATHMODE to take over the entire system

RESULT

success - FALSE if ROS was unable to open the screen

SEE ALSO

ROSAwakeSystem(), ROSSysCall(), ROSSysCallEnd()

ros.library/ROSQueryKeys                              ros.library/ROSQueryKeys

NAME

ROSQueryKeys -- return the states for a set of keys

SYNOPSIS

ROSQueryKeys(queryArray, arraySize)

A0          D0

VOID ROSQueryKeys(struct KeyQuery *, UBYTE);

FUNCTION

Scans the keyboard to determine which of the rawkey codes

listed in the 'queryArray' are currently pressed. The state for each

key is returned in the array.

The values returned by this function are not modified by which

window/screen currently has input focus.

INPUTS

queryArray - an array of KeyQuery structures. The kq_KeyCode fields

of these structures should be filled with the rawkey

codes you wish to query about. Upon return from this

function, the kq_Pressed field of these structures

will be set to TRUE if the associated key is down,

and FALSE if not.

arraySize - number of key code entries in 'queryArray'

NOTE

This function is save to call from interrupts.

SEE ALSO

lowlevel.lib/QueryKeys(), libraries/lowlevel.i

ros.library/ROSReadExe                                  ros.library/ROSReadExe

NAME

ROSReadExe -- scatterload a loadable file

SYNOPSIS

seglist/errorcode = ROSReadExe(flags, name)

D0      D1                     D0     A0

BPTR/ULONG ROSReadExe(ULONG, STRPTR);

FUNCTION

This function tries to scatterload the file 'name' using DOS.library's

LoadSeg().

If ROSRWF_DISKWAIT flag set the routine assumes that you want to load

'name' from a floppy. This means:

- waiting for the disk to be inserted (can't be cancelled!!!)

(implementation: DOS error codes DEVICE_NOT_MOUNTED, NOT_A_DOS_DISK,

NO_DISK force retry until any other error occurs)

- waiting until disk drive stopped

(implementation: polling disk.resource until drives are ready)

Since OS2.0 it's possible to load files from your program path instead

of the current directory using PROGDIR:xxx.

On OS1.3 the "PROGDIR:" string is automatically removed by ROS so you

don't have to worry about it. The only thing the user have to do is a

"CD" to the program directory before starting (only on OS1.3). ;-)

You should unload the file with ROSFreeMem(seglist)!

Do _not_ use DOS.library's UnLoadSeg()!!!!!!!

Closing the ros.library unloads file automatically.

INPUTS

flags - use ROSRWF_DISKWAIT if you want to load from a floppy

name - pointer to a null-terminated string

RESULT

seglist - BCPL pointer to a seglist or NULL if error

errorcode - DOS error code on failure (may be NULL!!!)

NOTE

This function interrupts the Death-Mode state with ROSSysCall()!

SEE ALSO

ROSFreeMem(), DOS.lib/LoadSeg()

ros.library/ROSReadFile                                ros.library/ROSReadFile

NAME

ROSReadFile -- read a number bytes from a file

SYNOPSIS

actLength/errorcode/actBuffer = ROSReadFile(flags, length, offset,

D0        D1        A0                      D0     D1      D2

memtype, memalign, name, buffer)

D3       D4        A0    A1

ULONG/ULONG/VOID *  ROSReadFile(ULONG, ULONG, ULONG,

ULONG, ULONG, STRPTR, VOID *);

FUNCTION

This function tries to load 'length' bytes from the file 'name' using

DOS.library's Read(). Set 'length` = 0 to load the whole file.

A1 points to a buffer where the data should be placed, or set A1=0

to force an automatic memory allocation. In this case you must specify

'memtype' and 'memalign' (see also ROSAllocMem()).

If ROSRWF_DISKWAIT flag set the routine assumes that you want to load

'name' from a floppy. This means:

- waiting for the disk to be inserted (can't be cancelled!!!)

(implementation: DOS error codes DEVICE_NOT_MOUNTED, NOT_A_DOS_DISK,

NO_DISK force retry until any other error occurs)

- waiting until disk drive stopped

(implementation: polling disk.resource until drives are ready)

Since OS2.0 it's possible to load files from your program path instead

of the current directory using PROGDIR:xxx.

On OS1.3 the "PROGDIR:" string is automatically removed by ROS so you

don't have to worry about it. The only thing the user have to do is a

"CD" to the program directory before starting (only on OS1.3). ;-)

You should free the automatically allocated memory with ROSFreeMem()!

Closing the ros.library unloads file automatically (if automatic

memory allocation).

INPUTS

flags - use ROSRWF_DISKWAIT if you want to load from a floppy

length - number of bytes to load or NULL for the whole file

offset - number of bytes to skip before reading from file

memtype - see ROSAllocMem(), only neccessary if 'buffer' == NULL

memalign - see ROSAllocMem(), only neccessary if 'buffer' == NULL

name - pointer to a null-terminated string

buffer - pointer to buffer or NULL to force an automitic memory

allocation

RESULT

actLength - number of bytes read from file or -1 if error

errorcode - DOS error code on failure (may be NULL!!!)

actBuffer - points to loaded data, only valid on success (D0 <> -1)!!!

NOTE

This function interrupts the Death-Mode state with ROSSysCall()!

SEE ALSO

ROSAllocMem(), ROSFreeMem(), DOS.lib/Read()

ros.library/ROSRemTimerInt                          ros.library/ROSRemTimerInt

NAME

ROSRemTimerInt -- remove a previously installed timer interrupt

SYNOPSIS

ROSRemTimerInt(intHandle);

A1

VOID AddTimerInt(ULONG);

FUNCTION

Removes a timer interrupt routine previously installed with

ROSAddTimerInt().

Closing the ros.library removes this timer interrupt automatically.

INPUTS

intHandle - handle obtained from ROSAddTimerInt(), if NULL nothing

happens

NOTE

This function interrupts the Death-Mode state with ROSSysCall()!

SEE ALSO

ROSAddTimerInt(), ROSStartTimer(), ROSStopTimer(), ROSSetTimerVec()

ros.library/ROSRequester                              ros.library/ROSRequester

NAME

ROSRequester -- Display a requester

SYNOPSIS

result = ROSRequester(TextFormat, ArgList, GadgetFormat)

D0                    A0          A1       A2

LONG ROSRequester(STRPTR, APTR, STRPTR)

FUNCTION

This procedure automatically builds a requester for you using

intuition's EasyRequestArgs() (AutoRequest() on OS1.3) and then

waits for a response from the user.

The requester appears only if it is not disabled via ROSDisableReq()

or your tasks pr_WindowPtr = -1.

Do not set this pointer directly. Use instead ROSDisable()/ROSEnable().

By default requesters are enabled.

In Death-Mode requesters are always disabled.

If requesters are disabled the 'result' will be always FALSE (NULL).

INPUTS

TextFormat - Format string, a la RawDoFmt(), for message in

requester body. Lines are separated by the newline character ($0A).

Formatting '%' functions are supported exactly as in RawDoFmt().

Note: on OS1.3 max 5 textlines

GadgetFormat - Format string for gadgets.  Text for separate

gadgets is separated by '|'. You MUST specify at least one gadget.

Note: max 2 gadgets on OS1.3

gadget format functions only supported on OS2.0+.

argList - Arguments for format commands. Arguments for gadgets

follow arguments for bodytext

RESULT

0, 1, ..., N - Successive GadgetID values, for the gadgets

you specify for the requester.  NOTE: The numbering

from left to right is actually: 1, 2, ..., N, 0.

This is for compatibility with AutoRequest(), which has

FALSE for the rightmost gadget.

BUGS

Do not rely on the 'result' of an one-button requester!

Due a bug in intuitions AutoRequest()/EasyRequestArgs() the 'result'

may be TRUE (1) or FALSE (0) because it's possible to satisfy an one-

button requester with both short-cuts: AMIGA-B and AMIGA-V.

SEE ALSO

intuition.lib/AutoRequest(), intuition.lib/EasyRequestArgs(),

exec.lib/RawDoFmt(), ROSDisableReq(), ROSEnableReq()

ros.library/ROSScreenToBack                        ros.library/ROSScreenToBack

NAME

ROSScreenToBack -- Send ROS screen to the back of the display

SYNOPSIS

ROSScreenToBack()

VOID ROSScreenToBack(VOID);

FUNCTION

Sends the ROS screen to the back of the display if it is open and

if _not_ in Death-Mode.

SEE ALSO

ROSScreenToFront()

ros.library/ROSScreenToFront                      ros.library/ROSScreenToFront

NAME

ROSScreenToFront -- Make ROS screen the frontmost

SYNOPSIS

ROSScreenToFront()

VOID ROSScreenToFront(VOID);

FUNCTION

Brings the ROS screen to the front of the display if it is open and

if _not_ in Death-Mode.

SEE ALSO

ROSScreenToBack()

ros.library/ROSSetCache                                ros.library/ROSSetCache

NAME

ROSSetCache - Instruction & data cache control

SYNOPSIS

oldBits = ROSSetCache(cacheBits, cacheMask)

D0                    D0         D1

ULONG ROSSetCache(ULONG, ULONG);

FUNCTION

This function provides global control of any instruction or data

caches that may be connected to the system.  All settings are

global (until ros.library closed) -- per task control is not provided.

Closing the ros.library reset former system cache settings.

INPUTS

cacheBits - new values for the bits specified in cacheMask.

use -1 to reset to system defaults (ignores cacheMask)

cacheMask - a mask with ones for all bits to be changed

Note: cacheBits/cacheMask may be overridden by values specified in

ROS.prefs (to support individual cache settings)

RESULT

oldBits - the complete prior values for all settings.

NOTE

As a side effect, this function clears all caches.

This function interrupts the Death-Mode state with ROSSysCall()!

SEE ALSO

exec/execbase.i, exec.lib/CacheControl()

ros.library/ROSSetCopper                              ros.library/ROSSetCopper

NAME

ROSSetCopper -- install copperlist for ROS screen

SYNOPSIS

ROSSetCopper(flags, coplist)

D0     A0

VOID ROSSetCopper(ULONG, APTR);

FUNCTION

This function installs a new copper list for our ROS screen.

Whenever this screen is the frontmost this copper list becomes the

active.

Do _not_ use "move.l #mycoplist,$dff080" or "move.w #0,$dff088" !!!!!!!

This may crash the display if you are in Non-Death-Mode and the ROS

screen is not yet available or behind others.

ROSSetCopper() takes this into account and displays this copper list as

soon as possible.

A screen switch forces always copper1 starting in a long frame!

INPUTS

flags - COPF_COPPER1/2: modify copper1 or copper2

Remember: copper1 is always started during vertical blank

COPF_STROBE: force an immediate copper start, move.w #0,$dff088

set 'coplist' to NULL for only strobe

COPF_LOF/SHF: forces a long/short frame while starting copper

(to display interlaced screens correctly)

coplist - copper list pointer, if NULL the old copper pointer remains

NOTE

This call is guaranteed to preserve all registers (except D0!!!) and is

save to call from interrupts.

SEE ALSO

libraries/ros.i, ROSKillSystem()

ros.library/ROSSetDMA                                    ros.library/ROSSetDMA

NAME

ROSSetDMA -- modify DMA bits

SYNOPSIS

ROSSetDMA(DMACON)

D0

ROSSetDMA(UWORD);

FUNCTION

This function acts like a write to the DMACON register.

Do _not_ use "move.w #xxx,$dff096" !!!!!!!

This may interfere with the system you are in Non-Death-Mode, but

ROSSetDMA() handles all exceptions correctly.

DMA bit modification:

- DMAF_BLTPRI: only in Death-Mode, but during a ROSSysCall() the

system DMA bit will be restored

- DMAF_DMAEN: same like BLTPRI

- DMAF_BPLEN: only if ROS screen in front

- DMAF_COPEN: same like BPLEN

- DMAF_BLTEN: same like BLTPRI

- DMAF_SPREN: same like BPLEN

- DMAF_DSKEN: never modified!!!

- DMAF_AUDxEN: only if audio channels allocated

A currently forbidden DMA modification is automatically updated as

soon as possible (e.g. bitplane DMA if ROS screen behind others).

Closing the ros.library reset former system DMA.

INPUTS

DMACON - DMA bits

NOTE

This call is guaranteed to preserve all registers (except D0!!!) and is

save to call from interrupts.

SEE ALSO

ROSGetDMA(), hardware/dmabits.i

ros.library/ROSSetExitHandler                    ros.library/ROSSetExitHandler

NAME

ROSSetExitHandler -- add handler that is executed at exit key condition

SYNOPSIS

ROSSetExitHandler(handler);

A0

ROSSetExitHandler(APTR);

FUNCTION

Calling this routine causes the ROS to install an exit key conditon

handler. Whenever this special key is pressed your handler is invoked

(should be only once). Now it's time to finish your application.

The routine is called from within an interrupt, so normal

restrictions apply. On entry A5 holds the custom base ($dff000) and

A6 holds _ROSBase. Your code may trash all registers.

The default exit key code is SHIFT ESC but may be changed in ROS.prefs.

You should remove this handler with ROSSetExitHandler(NULL).

Closing the ros.library removes this timer interrupt automatically.

INPUTS

handler - the routine to invoke upon exit key condition or NULL to

detach the handler

NOTE

This call is guaranteed to preserve all registers (except A0!!!) and is

save to call from interrupts.

The handler may be called more than once if your application isn't

fast enough to exit!!!

SEE ALSO

ros.library/ROSSetInt                                    ros.library/ROSSetInt

NAME

ROSSetInt -- modify interrupt settings

SYNOPSIS

ROSSetInt(INTENA)

D0

ROSSetInt(UWORD);

FUNCTION

This function acts like a write to the INTENA register.

Do _not_ use "move.w #xxx,$dff09a" !!!!!!!

This may interfere with the system you are in Non-Death-Mode, but

ROSSetInt() handles all exceptions correctly.

Only those bits are affected which belongs to previous ROSSetIntVec()

calls. Look at ROSSetIntVec() for further information.

In Non-Death-Mode the master interrupt bit (INTB_INTEN) is emulated to

support enable/disable of all ROS interrupts at once (system interrupts

are still active!!!)

Closing the ros.library reset former system interrupt bits.

INPUTS

INTENA - Int bits

Note: level 1/2/5/6 bits (disk, serial, timer) are not

supported!!!

Instead additonal bits are defined:  - KEYB

- CIAATIMA/B

- CIABTIMA/B

Look at ROSSetIntVec() for further details

NOTE

This call is guaranteed to preserve all registers (except D0!!!).

Do _not_ call this routine from interrupts if you want to modify the

CIA interrupts (CIAATIMA/B, CIABTIMA/B)!!!!

SEE ALSO

ROSGetInt(), ROSSetIntVec(), libraries/ros.i

ros.library/ROSSetIntVec                              ros.library/ROSSetIntVec

NAME

ROSSetIntVec -- set a new handler for an interrupt vector

SYNOPSIS

success = ROSSetIntVec(intNum, handler)

D0                     D0      A0

BOOL ROSSetIntVec(UWORD, APTR);

FUNCTION

This function attaches a new handler to the interrupt specified with

'intNum'. Detach a handler with A0 = NULL.

The routine is called from within an interrupt, so normal

restrictions apply. On entry A5 holds the custom base ($dff000) and

A6 holds _ROSBase. Your code may trash all registers.

Your handler won't be invoked until you enable it (and the master int)

via ROSSetInt(). After enabling your handler it's always invoked

whether you taken over the systen or not.

Interrupts are executed as follows:

� INTB_BLIT

- only in Death-Mode, during a ROSSysCall() the system routine

will be restored so you do _not_ use the blitter while the system

is (partially) active

- returncode is always TRUE

� INTB_VERTB

- always invoked, also without a call to ROSKillSystem()

- in Non-Death-Mode your vblank code may be called after display

is started depending on the speed of higher prioritized vblank

interrupts. If you want to synchronize custom chip modifications

with the display you should use the copper.

- but remember: in Non-Death-Mode the vblank frequency may vary

(not always 50Hz) if ROS screen behind others!

- returncode is always TRUE

� INTB_COPER

- only if ROS screen in front

- returncode is always TRUE

� INTB_NMI (bit 15)

- always invoked

- returncode is always TRUE

� INTB_AUD0..3

- only if channels allocated, else the returncode will be FALSE

Further 5 new int definitions are supported:

� INTB_CIAATIMA/CIAATIMB/CIABTIMA/CIABTIMB

- used to attach a handler to the specified CIA timer

- returncode may be FALSE if CIA allocation failed, in this case a

Retry/Cancel-Requester appears (if not suppressed)

- if returncode TRUE the specified timer bits are set to 0 (timer

stopped) and now you may modify the contents of the CIA control

register (CRA/CRB)

- never read or write the CIA interrupt control register (ICR),

use instead ROSSetInt() to enable/disble CIA interrupts!!!!!!

� INTB_KEYB

- used to track keyboard events (see also ROSGetKey())

- only invoked if ROS screen in front!!!

- your handler is called with additional register D0 setup:

* the lower word contains the keycode ($0..$67) with the

additional key_up/down flag (bit7 set for key-up condition)

* the upper word contains the qualifier bits (shift, alt, ...)

- in Non-Death-Mode it's up to you passing this key to other system

components (like intuition.lib, commodities.lib, ...)

* set the ZERO-flag at the end of your handler to pass the key

to other system components, e.g.  moveq #0,d0

* else clear the ZERO-flag, e.g.  moveq #1,d0

* but remember: if you catch all key events screen switching

and other system hotkeys dosn't work!!!

* by default all keys without left-amiga or without control

qualifiers are catched to prevent background menu activities

and other undesirable things

- returncode is always TRUE

All other interrupt bits are _not_ supported!!!

Closing the ros.library detaches interrupt handlers automatically.

INPUTS

intnum - the Paula interrupt bit number (0 through 14). Processor

level seven interrupts (NMI) are encoded as intNum 15.

Note: level 1/2/5/6 bits (disk, serial, timer) are not

supported!!!

Instead additonal bits are defined:  - KEYB

- CIAATIMA/B

- CIABTIMA/B

handler - the routine to invoke or NULL to detach the handler

RESULT

success - TRUE if handler successfully attached

NOTE

This call is guaranteed to preserve all other registers.

Do _not_ call this routine from interrupts if you want to add a CIA

handler for the first time!!! After a successful call you may change

this vector with ROSSetIntVec() also from interrupts because the

desired CIA is already allocated.

SEE ALSO

ROSSetInt(), ROSGetInt(), libraries/ros.i

ros.library/ROSSetTimerVec                          ros.library/ROSSetTimerVec

NAME

ROSSetTimerVec -- change the timer interrupt entry point

SYNOPSIS

ROSSetTimerVec(intRoutine, intHandle);

A0          A1

VOID ROSSetTimerVec(APTR, ULONG);

FUNCTION

This routine changes the interrupt entry point that is assocatied with

a timer interrupt created by ROSAddTimerInt().

No timer modification is done (no restart, no stop). The next interrupt

will execute your new 'intRoutine'.

INPUTS

intRoutine - the routine to invoke upon timer interrupts.

intHandle - handle obtained from ROSAddTimerInt().

NOTE

This call is guaranteed to preserve all registers (except A1!!!) and is

save to call from interrupts.

SEE ALSO

ROSAddTimerInt(), ROSRemTimerInt(), ROSStartTimer(), ROSStopTimer()

ros.library/ROSStartTimer                            ros.library/ROSStartTimer

NAME

ROSStartTimer -- start the timer associated with the timer interrupt

SYNOPSIS

ROSStartTimer(timeInterval, continuous, intHandle);

D0            D1          A1

VOID ROSStartTimer(ULONG, BOOL, ULONG);

FUNCTION

This routine starts a stopped timer that is assocatied with a

timer interrupt created by ROSAddTimerInt().

INPUTS

timeInterval - number of micoseconds between interrupts. The

maximum value allowed is 90,000. If higher values

are passed there will be unexpected results.

continuous - FALSE for a one shot interrupt. TRUE for multiple

interrupts.

intHandle - handle obtained from ROSAddTimerInt().

NOTE

This call is guaranteed to preserve register A0 and is save to call

from interrupts.

SEE ALSO

ROSAddTimerInt(), ROSRemTimerInt(), ROSSetTimerVec(), ROSStopTimer()

ros.library/ROSStopTimer                              ros.library/ROSStopTimer

NAME

ROSStopTimer -- stop the timer associated with the timer interrupt

SYNOPSIS

ROSStopTimer(intHandle);

A1

VOID ROSStopTimer(ULONG);

FUNCTION

Stops the timer associated with the timer interrupt handle passed.

This is used to stop a continuous timer started by

ROSStartTimer().

INPUTS

intHandle - handle obtained from ROSAddTimerInt()

NOTE

This call is guaranteed to preserve all registers (except A1!!!) and is

save to call from interrupts.

SEE ALSO

ROSAddTimerInt(), ROSRemTimerInt(), ROSSetTimerVec(), ROSStartTimer()

ros.library/ROSSysCall                                  ros.library/ROSSysCall

NAME

ROSSysCall -- interrupts Death-Mode to enable system calls

SYNOPSIS

ROSSysCall()

VOID ROSSysCall(VOID)

FUNCTION

This function provides a method to call system functions from within

Death-Mode.

YOU SHOULD NEVER NEED THIS FUNCTION! IT SHOULD BE A PRIVATE ROS CALL.

DO NOT USE THIS CALL WITHOUT GOOD JUSTIFICATION.

In order to restore normal Death-Mode, the programmer must execute

exactly one call to ROSSysCallEnd() for every call to ROSSysCall().

During a SysCall all ROS interrupts continue executing (look at

ROSSetIntVec() for details).

But remember: system interrupts are also active and additional system

DMA will be set!

The blitter will be disowned so you do _not_ bash the blitter until

ROSSysCallEnd() is called.

Do _not_ call system functions during a SysCall which may cause any

visual modifications (like OpenScreen(), ScreenToBack(), ...).

These functions will never return as long as the Death-Mode is the

active one!!!

Calling ROSSysCall() without a Death-Mode killed system is harmless.

RESULT

A partially active system.

NOTE

This call is guaranteed to preserve all registers.

SEE ALSO

ROSKillSystem(), ROSSetDMA(), ROSSetIntVec(), ROSSysCallEnd()

ros.library/ROSSysCallEnd                            ros.library/ROSSysCallEnd

NAME

ROSSysCallEnd -- return from a SysCall

SYNOPSIS

ROSSysCallEnd()

VOID ROSSysCallEnd(VOID)

FUNCTION

Call this function to return from a SysCall after a matching

ROSSysCall() has been executed.

RESULT

A Death-Mode killed system.

NOTE

This call is guaranteed to preserve all registers.

SEE ALSO

ROSSysCall()

ros.library/ROSWaitVBlank                            ros.library/ROSWaitVBlank

NAME

ROSWaitVBlank -- Wait for the vertical blank to occur

SYNOPSIS

ROSWaitVBlank()

VOID ROSWaitVBlank(VOID);

FUNCTION

Waits for vertical blank to occur and than returns to the caller.

NOTE

This call is guaranteed to preserve all registers and is save to call

from interrupts.

This call does not require A6 to be loaded with the library base.

BUGS

Currently this routine polls the VPOSR register so you should only use

this call if the ROS screen is the frontmost.

It doesn't work while displaying a 'foreign' screen on a graphics board.

ros.library/ROSWriteFile                              ros.library/ROSWriteFile

NAME

ROSWriteFile -- Write a number bytes to a file

SYNOPSIS

actLength/errorcode = ROSWriteFile(flags, length, offset, name, buffer)

D0        D1                       D0     D1      D2      A0    A1

ULONG/ULONG ROSWriteFile(ULONG, ULONG, ULONG, STRPTR, VOID *);

FUNCTION

This function writes 'length' bytes to the file 'name' using

DOS.library's Write().

Set 'offset' to NULL to overwrite an existing file (file opened with

MODE_NEWFILE). In the other case the data is written to the file 'name'

with an 'offset' from the beginning (file opened with MODE_READWRITE).

This can be used to append/replace data to/in an existing file. If the

file doesn't exists or if filelength shorter than 'offset' a seek error

occurs.

A1 points to a buffer from where the data is taken from.

Remember: On error a corrupt or a 0-byte-file may still exists!!!

It's up to you to delete this.

If ROSRWF_DISKWAIT flag set the routine assumes that you want to write

'name' to a floppy. This means:

- waiting for the disk to be inserted (can't be cancelled!!!)

(implementation: DOS error codes DEVICE_NOT_MOUNTED, NOT_A_DOS_DISK,

NO_DISK force retry until any other error occurs)

- waiting until disk drive stopped

(implementation: polling disk.resource until drives are ready)

Since OS2.0 it's possible to write files to your program path instead

of the current directory using PROGDIR:xxx.

On OS1.3 the "PROGDIR:" string is automatically removed by ROS so you

don't have to worry about it. The only thing the user have to do is a

"CD" to the program directory before starting (only on OS1.3). ;-)

WARNING: It may be dangerous to use this routine within Death-Mode!

Because of the AMIGA's multitasking facilities it isn't

possible (isn't it???) to wait till the final end of a write

operation. That's why I use a simple delay of 2 seconds after

any write operation in Death-Mode. If your filesystem is to

slow to write all the data within this time your partition

will be _not_ validated!!!

You've been warned!!!

INPUTS

flags - use ROSRWF_DISKWAIT if you want to write to a floppy

length - number of bytes to write

offset - NULL for overwrite mode, else a number of bytes to skip

before writing into an existing file

name - pointer to a null-terminated string

buffer - pointer to buffer where data is taken from

RESULT

actLength - number of bytes written or -1 if error

errorcode - DOS error code on failure (may be NULL!!!)

NOTE

This function interrupts the Death-Mode state with ROSSysCall()!

SEE ALSO

DOS.lib/Write(), ROSDeleteFile()