November 29th 1996

R a y S t o r m   3mv1.70m

by Andreas Heumann and Mike Hesser

0m1m1. Introduction0m

3m Introduction       0m     What is RayStorm?

3m Requirements       0m     What do I need to run it?

3m Features           0m     What can Scenario do?

3m Limitations        0m     What limitations has the demo version?

0m1m2. Installation0m

3m Installation       0m     How can I install it?

0m1m3. General information0m

3m Raytracing         0m     What is raytracing?

3m Octree             0m     What is an octree?

3m Antialiasing       0m     What is antialiasing?

3m Depth of field     0m     What is Depth of Field?

3m Soft shadows       0m     What are soft shadows?

3m Motion Blur        0m     What is Motion Blur?

3m Surfaces           0m     What meaning have the surface parameters?

3m Directory Structure0m     Some words to directory and project structures.

3m Virtual Memory     0m     How do I use virutal memory?

3m Internals          0m     Some internal details.

3m Tips & Tricks      0m     Useful tips,tricks and hints (FAQ)

3m Textures           0m     How do I use textures?

0m1m3. Scenario Usage0m

3m Usage              0m     How do I use Scenario?

3m Menus              0m     Description of the menus.

3m Buttons            0m     Description of the buttons.

3m Object Menus       0m     Description of object orientated menus.

3m Keyboard Control   0m     How do I use Scenario with the keyboard?

0m1m4. Script Usage0m

3m ARexx interface    0m     Which commands doe it have?

3m Examples           0m     How do I use the examples?

3m Tutorials          0m     Some tutorials

0m1m5. Registration0m

3m Register           0m     What must I do to register?

0m1m6. Misc0m

3m Known Bugs         0m     Bugs

3m Credits            0m     Thanks go to...

3m Authors            0m     Who wrote it?

3m Literature         0m     Which books do we use?

3m PC-Version         0m     Where can I get the PC-version?

3m Homepage           0m     Where to find us on the World Wide Web.

3m History            0m     What happened in the past?

3m Future             0m     What is planned for the future?

0m1m7. Legal Stuff0m

3m Legal Stuff        0m     Legal stuff

0m1mINTRODUCTION0m

This document describes the raytracing package RayStorm. This package consists

of two parts:

- The scene modeler 0m1mScenario0m which is the graphical user interface

(GUI) of the RayStorm package.

- The script interface 0m1mRayStorm Script0m, a program wich allowes one

to use the raytracer through a script language.

- The heart of RayStorm: 0m1mRayStorm Raytracer0m, wich generates the

picture from a scene description (this part is not visible for the user

but nevertheless it's the main part).

Here are some additional informations to the parts:

3mRayStorm Scenario0m

3mRayStorm Script0m

3mRayStorm Raytracer0m

0m1mINTRODUCTION TO SCENARIO0m

As we developed Scenario we tried to make the user interface to use as

easy as possible to enable even absolute beginners to work with it. Therefore

almost all actions can be performed with the mouse. For advanced users we

implemented keyboard shortcuts for almost all functions.

0m1mINTRODUCTION TO RAYSTORM SCRIPT0m

The script language that contains the description of the scene is easy to

learn and offers some powerful capabilities. In addition, the script language

has a full support for key frame animation.

A RayStorm script file (.ray files) is basically divided into the following

sections:

- Various settings of the camera, the world and the lights,

- Definitions of the different textured surfaces,

- Different actors that set the Motion Blur and animation parameters,

- Objects that are associated with surfaces and actors

- Settings for the final rendered graphic.

For a more detailed description click 3mhere0m.

0m1mINTRODUCTION TO RAYSTORM RAYTRACER0m

The raytracer itself is a part to RayStorm which is not directly visible.

All features of the raytracer are packed into a shared library. Both Scenario

and RayStorm Script use this library to render their scenes.

The raytracer is based on a fast octree algorithm. It has some powerfull

features such as 3mMotion Blur0m and 3mDepth of Field0m.

0m1mGENERAL0m

Raytracing makes it possible to generate fotorealistic pictures of objects.

A raytracer casts a ray form the position of the viewer through a scene

and calculates possible intersections with the objects in that scene. If an

intersection is found, the raytracer decides which color the object

at this position has. If the object is reflective or transparent, the

raytracer casts new rays from this positon and tests the intersections

again and so on.

To make the surfaces of the objects more realistic, textures which

simulate marble, wood other surfaces can be used.

0m1mOCTREE0m

Simple raytracers determine the intersections with objects by testing all

objects. This can lead to long rendering times if there are a lot of

objects in the scene.

One solution of this problem is the Octree algorithm.

This algorithm divides the scene into eight child-cells recursively until

there is less than oe object in the cell or the maximum depth of the tree is reached.

Division of space with the octree algorithm:

_

/| z

/

/-------/-------/|

^ y/       /       / |

| / left  / right / b|

|/       /       / |a|

|-------/-------| f|c|

|               | r|k|

|      up       | o| /

|---------------| n|/

|               | t|

|     down      | /  x

|---------------|/-->

0m1mANTIALIASING0m

RayStorm uses a algorithm called 'Adaptive Supersampling' to do antialiasing.

For each pixel with a high contrast against its four neighbours, the algorithm

casts new rays which are close to the ray used for the pixel itself. The new

color of the pixel is calculated with the supersampled pixels and the gaussian

filter.

Supersampling is also used to do 3mdepth of field0m and 3msoft shadows0m. So if

you want to use this features you have to set a antialiasing value greater

than one. (-> 3mANTIALIAS0m)

Example:

Settings: squareroot of number of samples per pixel: 3

|--------|--------|--------|

| super- | super- | super- |

| sampled| sampled| sampled|

|        |        |        |

|--------|--------|--------|

| super- |        | super- |

| sampled| pixel  | sampled|

|        |        |        |

|--------|--------|--------|

| super- | super- | super- |

| sampled| sampled| sampled|

|        |        |        |

|--------|--------|--------|

|- Gaussian filter width  -|

The rendering time increases dramatically if you use antialiasing. The

values below depend on the contrast of the picture.

Samplesetting     rendering time

average case     worst case

1                 x1                x1

2                 x2                x4

3                 x4                x8

4                 x8               x16

...               ...              ...

Setting higher than 3 are not leading to significant better results.

0m1mDEPTH OF FIELD0m

Objects in computer graphics are normally rendered in an image plane

using a pinhole camera model. That is to say, no matter how far or how

near the objects are from the camera, they are always in sharp focus.

Depth of field means that only objects at a certain distance from the

camera lens are in sharp focus. Further and nearer objects produce a

blurred image on the film plane.

[From 'Advanced Animation and Rendering Techniques']

To use depth of field you have to set 3mANTIALIAS0m to a value bigger

than one.

3mExample for DOF0m

0m1mSOFT SHADOWS0m

Real Light sources never have a zero size, therefore the shadows behind

objects are never hard edged, they are soft. RayStorm generates this

shadows by jittering the position of the light source. To use soft shadows

you must set 3mDISTRIB0m to a value bigger than one.

0m1mMOTION BLUR0m

Motion blur is temporal anti-aliasing. In animated sequences, the normal

rendering process functions like a camera that possesses an infinitely

short exposure time and this eliminates the blurring of the image due

to relative motion between an object and the film plane. When a series of

images, generated without motion blur, is displayed as an animated

sequence, the illusion of smooth motion is diminished by strobing effects.

As human beings we expect to see loss of detail in moving images.

Motion blur is accounted for in distributed ray tracing by extending the

distributed sampling and jittering into the time domain and computing a

solution that extracts information from the scen over the duration of the

shutter exposure time. Objects are moved as required in the time period

and visibility consequently changes over this time intervall. This method

ensures that highlights and shadows are blurred or not, depending on the

nature of the motion.

[From 'Advanced Animation and Rendering Techniques']

0m1mSURFACES0m

Ambient (set with AMBIENT)

This determines the color of the object in sections which are in shadow.

Diffuse reflection (set with DIFFUSE)

The diffuse reflection falls off as the cosine of the angle between

the normal and the ray to the light. Diffuse reflection determines the

main color of the object (color in Imagine).

Specularly reflected highlights (set with SPECULAR)

Specularly reflected highlights fall off as the cosine of the angle

between the reflected ray and the ray to the light source (specular in

Imagine)

Specular reflection exponent (set with REFEXP)

Determines the size of the specularly reflected highlights, the higher

the smaller the highlight (hardness in Imagine)

Diffuse transmission (set with DIFFTRANS)

Same as diffuse reflection, but only used if the lightsource is on

opposite side of surface. Only applied if tranlucency is not 0.

Specular transmission (set with SPECTRANS)

Same as specular reflection, but only used if the lightsource is on

opposite side of surface. Only applied if tranlucency is not 0.

Specular transmission exponent (set with TRANSEXP)

Same as specular reflection exponent, but only used if the lightsource

is on opposite side of surface.

Specular transmittance (set with TRANSLUC)

Specular transmittance.

Transparency (set with TRANSPAR)

Transparent color (filter in Imagine).

Reflectivity (set with REFLECT)

Reflective color (reflect in Imagine).

Fog lenght (set with FOGLEN) (fog in Imagine).

Index of refraction (set with REFRINDEX)

Determines how the ray through transparent objects is refracted, the higher

the more (index of refraction in Imagine).

Is calculated with the formula

lightspeed in vacuum

index = ----------------------

lightspeed in object    .

0m1mDIRECTORY AND PROJECT STRUCTURE0m

scenario             the root directory of scenario

|-brushes            the brush directory (you can create subdirectories here)

| |- brush 1

| |- brush 2

| |- ...

| |- brush n

|-objects            the objects directory (you can create subdirectories here)

| |- object 1

| |- object 1

| |- ...

| |- object n

|-textures           the textures directory (you can create subdirectories here)

| |- texture 1

| |- texture 2

| |- ...

| |- texture n

|-projects           the projects directory (Scenario creates for every project a subdirectory)

|-project 1        the first project

| |- project.scn   the file where the scene is stored (has always this name)

| |-brushes        the directory for brushes which are local to this project

| | |- ...

| |-objects        the directory for objects which are local to this object

| | |- ...

| |- pic.0001      the rendered pictures

| |- pic.0002

| |- ...

| |- pic.n

|- project 2       another project

|- ...

|- project n

The default project directory can be set with the 3mpreferences0m.

0m1mVIRTUAL MEMORY0m

RayStorm has been tested succesfully with VMM 3.1 from Martin Apel. If you

want use RayStorm with virtual memory notice follwing hints:

- set Minimum VM allocation to 100 bytes if you define large scenes with

many objects, because RayStorm only allocates small pieces of memory for

single objects (less then 200 bytes). If you're loading Imagine objects

RayStorm allocates big blocks of memory so you don't have to set Minimum

VM allocation to 100.

- use a partition or a pseudo-partition for VMM, this is faster

0m1mINTERNALS0m

The development of RayStorm has started in march 1995.

0m1mCompiler0m

- Amiga

Originally, RayStorm has been developed on Amiga using Maxon C++ 3.0

Developer. The final Amiga version was compiled with StormC 1.1.

- PC

The PC version was compiled with WATCOM C++ 10.5.

0m1mMemory requirements of the raytracer0m

Triangle:      156 Bytes (flat shaded)

192 Bytes (Phong shaded)

Sphere:         70 Bytes

Plane:          78 Bytes

Surface:       122 Bytes + length of name

Screenbuffer:    4 Bytes per pixel

Memory requirements of the octree depends on the scene.

0m1mSource Code0m

About 70.000 lines of C++-Code in more than 250 modules (about

1.6 MByte of source).

0m1mREQUIREMENTS0m

(1) You will need at least Kickstart 2.0.

(2) 040-version: 68040 processor (with internal FPU)

(3) 881-version: 68020 processor or better and a mathematical coprocessor)

(4) 020-version: 68020 processor (no math coprocessor needed)

(5) 2MB RAM minimum

(6) RayStorm was written using MUI. Therefore you need muimaster.library

V3.3 or better to run RayStorm.

recommended: 68040, 8MB RAM, Harddisk, GFX-Board

The faster the better :-).

Tested with:

A1200 68030/50, 6MB, 200MB HD

A2000 68040/30, 9MB, 250+250MB HD, Merlin Gfx-board

A2000 68030/14, 68882/20, 4MB, 730+52MB HD

A4000 68030/25, 68882/57, 10MB, 730+80MB HD, Cybervision 64 Gfx-board

0m1mFEATURES0m

0m1mScenario0m

- create spheres, planes, lightsources and cameras

- create mesh objects: cubes, spheres, planes, tubes and cones

- unlimited amount of cameras to view scene from different directions

- define surfaces for all objects, including brushes and textures

- load external objects (RayStorm object files, Imagine TDDD files,

AutoDesk 3DS files and RAW files)

- save and load scenes

- save and load objects

- group and ungroup objects

- preview of surfaces in material requester

- render scenes with RayStorm Raytracer

- preview of renderd scenes

- quadview

0m1mRayStorm Script0m

- ARexx port with 38 commands

0m1mRayStorm Raytracer0m

- Up to 30% faster than Imagine (in trace mode).

- Octree algorithm used for rendering.

- Motion blur for realistic simulations of moving objects.

- Color, reflectivity, filter, altitude and specular mapping.

- Flat, cylinder and sphere mapping.

- Soft brush mapping.

- Mathematical textures: wood, marble, bumps, checker, linear, radial and

stars.

- Tranparency and physically correct refractions.

- 8 levels of antialiasing (adaptive supersampling).

- Rendering box.

- Three builtin object types: sphere, plane and triangle.

- Three light types: ambient, point and spot.

- Depth of field with adjustable focal distance and aperture.

- Soft shadows.

- Backdrop picture.

- Global fog and foggy objects.

- Material attributes for realictic objects: ambient color, diffuse color,

specular color, specular reflection exponent, diffuse transmission color,

specular transmission color, specular transmission exponent, specular

transmittance, transparent color, reflective color, index of refraction,

foglength.

- Bright objects.

- Quick rendering.

- Global reflection map.

- Image formates: IFF-ILBM, PNG, TGA and Datatypes.

- Object format: RayStorm object file, Imagine-TDDD, Autodesk 3DS and RAW

- New image- and object-formats can be easily included because of the

modular concept.

0m1mLIMITATIONS0m

The demo version is limited to a resolution of 160x128. The registered version

does not have any limitations.

Click 3mhere0m for information on how to register RayStorm.

0m1mINSTALLATION0m

There is a installation script included in the archive which uses the

Commodore Installer. Run it to install RayStorm Scenario.

0m1mUSAGE0m

After you have started Scenario you see the worksheet.

At the top of the sheet are the buttons to control view mode, display mode,

edit mode and currently active directions. Here you also can see the

coordinate display.

Below this button bar is the work area. At startup you can see a grid in

the middle and the camera (note: there must always remain at least one camera

in the scene).

All objects, cameras and light sources have a axis to handle it. The axis

consist of a rectangular area - the hot spot or handle - and three lines

which show the orientation of the axis. The axis can be selected by clicking

with the left mouse button at the handle. If it is selected the color of

the axis and the object changes.

Clicking with the left mouse button invokes the modify mode. According to the

slected mode (move, rotate or scale) the world or the selected object(s) can

be modified.

Clicking with the right mouse button opens object specific menus (see

3mObject Menus0m).

0m1mMENUS0m

Project

3m New               0m

3m Open...           0m

3m Save              0m

3m Save As...        0m

3m Render settings...0m

3m Render            0m

3m About...          0m

3m Quit              0m

Edit

3m Cut               0m

3m Copy              0m

3m Paste             0m

3m Delete            0m

3m Select all        0m

3m Deselect all      0m

3m Select next       0m

3m Select previous   0m

3m Mode              0m

3m Active            0m

View

3m Redraw            0m

3m Focus             0m

3m Zoom in           0m

3m Zoom out          0m

3m Zoom fit          0m

3m View              0m

3m Camera to viewer  0m

3m Grid              0m

3m Snap to Grid      0m

3m Gridsize...       0m

3m Show              0m

3m Global settings...0m

Object

3m Attributes...     0m

3m Settings...       0m

3m Group             0m

3m UnGroup           0m

3m Load...           0m

3m Save...           0m

3m Create            0m

3m Mesh              0m

3m Active Camera...  0m

3m Browse...         0m

Settings

3m Prefs...          0m

3m MUI...            0m

3m Load              0m

3m Save              0m

0m1mNEW PROJECT0m

Deletes all objects and creates the default camera.

0m1mOPEN PROJECT0m

Load a new scene.

0m1mSAVE PROJECT0m

Save the current scene.

0m1mSAVE PROJECT AS0m

Save the current scene with the specified name.

0m1mRENDER SETTINGS0m

Opens the 3mrender requester0m. At top of the requester you can select the render

resolutions with a listview or enter the values to the numeric fields. The

values range from 0 to 65535. Here you also can set the rendering field.

RayStorm renders only inside this field. The field is shown in the perspective

window if camera view is switched on.

The 'Show' flag sets enables the direct showing of the picture while the

render process is running (needs KS 3.0 or better).

The 'Quick' flag selects the quick rendering mode. In this mode RayStorm don't

calculates shadows, transparency or reflections.

The 'Octree depth' slider sets the maximum depth of the octree which RayStorm

uses. For almost all scenes a vlaue of three is the best, but in complex scenes

it may be better to use higher values. The higher this value is the longer takes

the initialazion phase and the more memory is needed.

The 'Filename' string gadget sets the filename of the generated picture and the

cycle gadget sets the fileformat. You can show the rendered picture with the

view button.

The 'Ok' button closes the requester and stores the current settings, whereas the

'Cancel' button don't changes the settings.

The 'Render' button starts the rendering of the current scene.

0m1mRENDER PROJECT0m

Renders the current scene.

0m1mAbout0m

Shows the about requester with the version number and the current Arexx port

name.

0m1mQUIT0m

Quits Scenario.

0m1mCUT0m

Moves the currently selected objects to the paste buffer.

0m1mCOPY0m

Copys the currently selected objects to the paste buffer.

0m1mPASTE0m

Copys the currently selected objects from the paste buffer to the scene.

0m1mDELETE0m

Deletes the currently selected objects. The active camera can't be deleted.

0m1mSELECT ALL0m

Selects all objects.

0m1mDESELECT ALL0m

Deselects all objects.

0m1mSELECT NEXT0m

Selects the next object.

0m1mSELECT PREVIOUS0m

Selects the previous object.

0m1mMODE0m

Sets the edit mode:

Move object

Rotate object

Scale object

Move world

Rotate world

Zoom world

0m1mACTIVE0m

Sets the active directions:

X, Y, Z.

0m1mREDRAW0m

Redraws the work area.

0m1mFOCUS0m

Sets the zoom factor so that all selected objects are visible.

0m1mZOOM IN0m

Zooms out by factor two.

0m1mZOOM OUT0m

Zooms in by factor two.

0m1mZOOM FIT0m

Sets the zoom factor so that the whole scene is visible.

0m1mVIEW0m

Sets the view mode:

Front, Right, Top, Perspective, Four, Camera.

0m1mCAMERA TO VIEWER0m

Moves the camera to the the position of the virtual camera, roatates the

camera and sets the opening angles (horizontal and vertial field of view).

0m1mGRID0m

Switches grid on/off.

0m1mSNAP TO GRID0m

Switches grid snapping on/off.

0m1mGRID SIZE0m

Sets grid size.

0m1mSHOW0m

3m Coordinates      0m

3m Names            0m

3m Brushes          0m

3m Textures         0m

0m1mSHOW COORDINATES0m

Switches coordinate display on/off.

0m1mSHOW NAMES0m

Sets the show names mode on/off.

0m1mSHOW BRUSHES0m

Sets the show brushes mode on/off.

0m1mSHOW TEXTURES0m

Sets the show textures mode on/off.

0m1mGlobal0m

Opens the 3mglobal settings requester0m. Here you can change the global scene

settings.

The 'Background' colorfield sets the color of the background of the rendered

picture.

With the 'Backdrop' string gadget you can enter a picture which will replace

the background.

The 'Reflectionmap' string gadget sets the name of a picture which will be

reflected by objects instead of the background. The reflectionmap is even

applied in quick mode.

The 'Ambient' colorfield sets color of the ambient light which is the base

brightness of all objects. In other words: surfaces which are not illuminated

by a light source get the ambient color.

The next group of gadgets set the 3mantialiasing0m parameters. The

antialiasing value ranges from 1 (no antialiasing) to 8 (very high

antialiasing). This value is the squareroot of rays per pixel. Filter width

sets the maximum distance from the main ray to the additional generated rays

in pixels. The 'Contrast' colorfield sets the minimum contrast. If the contrast

of the main pixel against its four neighbours is higher than this color,

RayStorm performs antialiasing at this pixel.

The global fog parameters are controlled with the gadgets of the next group.

'Length' sets the thickness of the fog. The 'Color' colorfield determines the

color of the fog. 'Height' sets the height at which the fog ends. Global fog

always starts at negative infinity in y direction and and at the specified y

position. For more informations about fog click 3mhere0m.

The 'Motionblur level' slider is not activated in this version of Scenario,

because motionblur needs animated objects.

The 'Softshadow level' slider sets the squareroot of the amount of additional

casted rays for softshadow. Soft shadows are caused from light sources with

a size bigger than 0.

The 'Random jitter' flag switches between normal and random sampling. With

random sampling the sampling position is calculated randomly.

0m1mATTRIBUTES0m

Opens the attribute requester for the currently selected object, if no object

is selected no requester is opened.

Here you can set the parameters which determine the apperance of the surface.

The requester is divided into three groups:

- 3mSurface0m

At this page you can set the base apperance of the surface.

- Diffuse

The diffuse reflection falls off as the cosine of the angle between

the normal and the ray to the light. Diffuse reflection determines

the main color of the object.

- Ambient

This determines the color of the object in sections which are in

shadow.

- Specular

Specularly reflected highlights fall off as the cosine of the angle

between the reflected ray and the ray to the light source.

- Diffuse Transmission

Same as diffuse reflection, but only used if the lightsource is on

opposite side of surface. Only applied if tranlucency is not 0.

- Transparence

Transparent color.

- Specular Transmission

Same as specular reflection, but only used if the lightsource is on

opposite side of surface. Only applied if tranlucency is not 0.

- Reflectivity

Reflective color.

- Translucency

Specular transmittance.

- Reflection exponent

Determines the size of the specularly reflected highlights, the higher

the smaller the highlight.

- Index of refraction

Determines how the ray through transparent objects is refracted, the

higher the more.

- Transmission exponent

Same as specular reflection exponent, but only used if the lightsource

is on opposite side of surface.

- Fog length

- Bright

The brightness of the surface is everywhere the same. Light sources don't

affect the surface, no shadows are visible on the surface, but the object

casts shadows.

- 3mBrush0m

Brushes are pictures which are projected to the surface. They can replace

different surface parameters as color, transparency, reflectivity and

specularity. Additional with altitude mapping brushes can change the normals

of the surface to simulate bumps or other structures which go 'in' or come

'out' of the surface.

There are three wrap methods available:

- flat (Flat)

The brush is projected to X-Y plane.

- cylindrical around x- or y-axis (WrapX and WrapY)

The brush is wrapped around the x or y-axis, like on a cylinder. The left

edge of the brush begins at the posititve x-axis and wraps the brush around

the cylinder from 'west' to 'east'.

- spherical (WrapXY)

Wrapping both: around x- and y-axis. It is assumed, that the object is a

sphere. The y-axis is the north/south pole of the spherical mapping. The

left edge of the brush begins at the positive x-axis and wraps the brush

around the sphere from 'west' to 'east'. The brush covers the sphere

exactly once.

Additional brushes can be repeated and mirrored (only for flat or cylindrical

mapping). If 'Soft' is selected, the color between two points of the brush is

softly interpolated.

- 3mTexture0m

Textures are mathematical generated patterns. Textures can change color,

reflectivity, transparency and normals of a surface. Textures are controlled

with a set of 16 parameters.

0m1mPreview0m

Renders a preview of the surface on a sphere (only for KS 3.0 and better).

The light source is always placed right top before the sphere.

All surface attributes can be viewed here except brushes and fog.

If reflectivity is set the RayStorm logo is reflected on the surface.

0m1mSave0m

Saves the surface data.

0m1mLoad0m

Loads surface data.

0m1mSettings0m

Opens the 3msettings requester0m for the currently selected object.

On the first page you can set the name, position, alignment and size of an object.

With the second page you can change object settings. The contents of

this page depends on the object type.

3mCamera   0m

3mPointight0m

3mSpotLight0m

3mMesh     0m

0m1mCAMERA SETTINGS0m

With the 3mcamera settings requester0m you can set various paramaters, which

influence the camera.

With 0m3mTrack0m yue can set the name of an object the camera will track to.

This means that the camera will always point to the axis of the track object.

0m3mFocal distance0m and 0m3mAperture0m set the parameters for

3mDepth of Field0m. If you select 0m3mFocus Track0m and the camera tracks

to an object the focal distance of the camera is automatically set to the

distance between camera and track.

With the values of 0m3mHorizontal0m and 0m3mVertical0m field of view you

can set the opening angle of the camera lens. If you activate the button

0m3mTake VFOV from resolution0m Scenario calculates the vertical field of

view (VFOV) from the resolution and the horizontal field of view (HFOV). An

Example will demonstrate this. You selected a HFOV of 30 degree and a

resolution of 800x600 pixels. This means that the VFOV is

600 / 800 * 30 = 22.5 degree.

0m1mLIGHT SETTINGS0m

With the 3mlight settings requester0m you can set various paramaters which

influence the light.

0m3mColor0m sets the color of the light source.

0m3mFalloff0m is the distance where the intensity of the light source is

zero. A value of zero disables it.

0m3mCast shadows0m enables the light source to cast shadows.

0m3mLens flares0m enables the 3mlens flare settings0m.

0m3mVisible light0m enables the 3mvisible light settings0m.

0m1mLIGHT SETTINGS0m

With the 3mlight settings requester0m you can set various paramaters which

influence the light.

With 0m3mTrack0m you can set the object a spotlight points to. See

3mcamera settings0m for a more detailed explanation of

tracking.

0m3mColor0m sets the color of the light source.

0m3mFalloff0m is the distance where the intensity of the light source is

zero. A value of zero disables it.

0m3mOpening angle0m sets the opening angle of a spotlightsource in degree.

0m3mCast shadows0m enables the light source to cast shadows.

0m3mLens flares0m enables the 3mlens flare settings0m.

0m3mVisible light0m enables the 3mvisible light settings0m.

0m1mLENS FLARES0m

With the 3mlens flare settings0m you can create and change lens flares.

Lens flares only appear when the light source is visible. They appear on an

axis which runs from the center of the light source through the center of the

screen. The position of the flare is relative to the light source it belongs

to. If the position value is zero, the flare appears at the position of the

light source. Higher values of pos move it towards the center of the screen.

Here a some example values and their meaning:

0 : flare has the same position as the light source

100 : flare is in the middle of the screen

200 : flare is on opposite side of the light source

You can also set pos to negative values, so the flare will appear behind the

light source.

The radius of the flare is relative to the horizontal render resolution.

For example: A flare with radius of 20 % renderd with a resolution of 320x200

will have a radius of 0.2*320 = 64 pixels.

0m1mType0m can be one of the following identifiers:

Disc

A flare which looks like a disc

Polygon

A flare which looks like a polygon. The number of edges can be set with

edges.

0m1mFunction0m can be one of the following identifiers:

Plain

the color of the flare is uniform

Ring

the flare looks like a ring

Radial

varies the color of the flare radial around the flare center

0m1mVISIBLE LIGHT0m

With the 3mvisible light settings0m you can change the appearance of the

light source.

In reality, a light source looks like a star with several spikes getting out

of the center of the light source. Usually there's a halo around it too.

The radius (length of the longest spike) of the star is relative to the

horizontal render resolution (see 3mLens Flares0m).

The radius of the halo is relative to the radius of the star itself. A value

of 100 % means, that the halo has the same radius as the star.

The range value determines the difference between the longest and the shortes

spike of the star. A value of 0 % means that all spikes have the same length,

whereas 100 % means that the shortest spike has zero length.

0m1mHow are the spikes computed ?0m

The four spikes in the main directions (0, 90, 180, 270 degrees) always have

maximum length. Spikes between them (45, 135, ... degrees for example) are

shorter and again spikes between them are shorter and so on.

If you enable random, the length of the spikes is randomly changed.

0m1mMESH SETTINGS0m

With the 3mmesh settings requester0m you only can change external objects, the

values are disabled for normal mesh objects.

0m3mObject filename0m sets the the name of the external object which is

attached with the mesh object.

If 0m3mApply surface to chils0m is activated the surface of the mesh

object is applied to the surface of the attached external object.

0m1mGROUP0m

Groups the selected objects.

0m1mUNGROUP0m

Ungroups the selected objects.

0m1mLOAD0m

Loads an external object. Currently Imagine,3DS and RAW objects are supported.

Scenario generates an axis and adds the object as childs of it. It's currently

impossible to change the position, alignment or size of this child objects

directly, nevertheless changing of the parent axis also affects it's childs.

After loading the name of the object file can be changed with the 3mmesh settings0m

requester.

0m1mSAVE0m

Saves an object and it's childs in RayStorm object file format. You can only

save objects from the root level of the object tree. This means you cannot

save child objects. You also cannot save external objects.

0m1mCREATE0m

3m Sphere           0m

3m Plane            0m

3m Pointlight       0m

3m Spotlight        0m

3m Camera           0m

0m1mSPHERE0m

Creates a new sphere at position <0,0,0> with radius 1.

0m1mPLANE0m

Creates a new plane at position <0,0,0> with the normal pointing in positive

y direction.

0m1mPOINTLIGHT0m

Creates a new point light source at position <0,0,0>.

0m1mSPOTLIGHT0m

Creates a new spot light source at position <0,0,0> with a opening angle of

45 degree pointing in negative y direction.

0m1mCAMREA0m

Creates a new pinhole camera at position <0,0,0> with a horizontal and

vertical field of view of 25 degree witch is pointing in positive z

direction.

0m1mMESH0m

3m Axis             0m

3m Cube             0m

3m Sphere           0m

3m Plane            0m

3m Tube             0m

3m Cone             0m

3m Torus            0m

0m1mAXIS0m

Creates a new axis at position <0,0,0> with size <1,1,1>. An axis can be used

to track a camera or a spot lightsource on it.

0m1mCUBE0m

Creates a cube at position <0,0,0>. You can set the size of the cube with a

requester.

0m1mSPHERE0m

Creates a spere at position <0,0,0>. You can set the size, the divisions and

the slices with a requester.

0m1mPlane0m

Creates a plane at position <0,0,0> with normal pointing in positive y-

direction. You can set the divisions with a requester.

0m1mTUBE0m

Creates a tube at position <0,0,0>. You can set the size, the height, the

divisions, the slices and if it's a closed tube with a requester.

0m1mCONE0m

Creates a cone at position <0,0,0>. You can set the size, height, divisions,

slices, and if the cone is closen with a requester.

0m1mTORUS0m

Creates a torus at position <0,0,0>. You can set the inner and outer size, the

division and slices with a requester.

0m1mACTIVE CAMERA0m

You can select the active camera.

0m1mBROWSE0m

Opens the object browser. There are two different selectable behaviours:

- Normal

View by selecting an entry and pressing the view button.

Select object with double click.

- Alternative

View by selecting any entry.

Select object with double click.

The behaviour is selectable in the 3mpreferences requester0m.

0m1mPREFS0m

Opens the preferences requester.

The 3m'Paths'-page0m let you set various paths.

0m1mViewer0m

Sets the path and name of the viewer. This viewer is used to display the

images generated by RayStorm. The filename of the image is appended to the

end of the string. You have specify the whole path, for example not

'multiview' but 'sys:utilities/multiview'.

0m1mProject path0m

Sets the default path for projects

0m1mObject path0m

Sets the default path for objects.

0m1mTexture path0m

Sets the default path for textures.

0m1mBrush path0m

Sets the default path for brushes.

The 3m'Flags'-page0m let you set various flags.

0m1mAlternative browser mode0m

See 3mBrowse0m.

0m1mAlways show coordinates0m

The 3d coordinates of the mouse pointer are displayed for front, right and

top. If a object is transformed, the relative coordinates, rotation angles

or scale fctors are displayed

0m1mMUI0m

Opens the MUI preferences.

0m1mLOAD0m

Loads the preferences from a file called 'scenario.prefs'.

0m1mSAVE0m

Saves the preferences to a file called 'scenario.prefs'.

0m1mBUTTONS0m

The first six buttons are for selecting the view (front, right, top,

perspective, quad and camera view).

The following three buttons select the display type. (bounding box,

wireframe and solid display).

After this are six buttons to select the edit mode (move, rotate, scale object

and move, rotate, scale world).

The next three buttons enable the X, Y and Z-axis.

0m1mOBJECT MENUS0m

Sceneario supports object oriented menus. These menus are shown when you click

the right mouse button inside the work area and hold it. Scenario then pops

up a menu. The contents of this menu depends on the object which is found at

this position.

0m1mWorld menu0m

The world menu is shown if you click on a place where no object is placed.

The menu contents follwing points:

Move:       switch to move world

Rotate:     switch to rotate (the virtual camera is rotated around the view

point (only perspective mode))

Zoom:       zoom in or out

Settings:   opens the general settings requester

0m1mObject menu0m

The object menu pops up if you click on the handle of the object.

The menu contents follwing points:

Move:       switch to mode move object

Rotate:     switch to rotate object (the object is rotated around it's local

axis)

Scale:      switch to scale object

Settings:   opens the 3msettings0m requester

Attributes: opens the surface 3mattributes0m

requester

0m1mCamera menu0m

The camera menu pops up if you click on the handle of a camera

The menu contents follwing points:

Move, Rotate, Scale, Settings: see object menu

Settings:   opens the 3mcamera settings0m requester

Active:     sets this camera as the active camera (the scene is viewed with

this camera)

0m1mLight menu0m

Opens the light popup menu.

The menu contents follwing points:

Move, Rotate, Scale, Settings: see object menu

Settings:   opens the 3mlight source settings0m requester

0m1mMesh menu0m

Opens the mesh popup menu.

The menu contents follwing points:

Move, Rotate, Scale, Settings: see object menu

Settings:   opens the 3mMesh settings0m requester

Attributes: see object menu

To scene:   convert external to scene objects

0m1mKEYBOARD CONTROL0m

ESC          break redraw

cursor keys  Function depends on the curren edit mode (move, rotate, scale of

world or object).

'+'          zooms in

'-'          zooms out

'm'          switches to move object action

'r'          switches to rotate object action

's'          switches to scale object action

'M'          switches to move world action

'R'          switches to rotate world action

'S'          switches to scale world action

't'          opens settings requester

'1'          switches to front view

'2'          switches to right view

'3'          switches to top view

'4'          switches to perspective view

'5'          switches to quad view

'6'          switches to camera view

'x'          enable x axis

'y'          enable y axis

'z'          enable z axis

'F1'         render settings

'R-Amiga' - '.'	Center view

0m1mTIPS & TRICKS0m

0m1mGeneral0m

- RayStorm renders faster if you don't use planes, because intersections

with planes cannot be calculated with the octree (helpfull in scenes with

motion blur or soft shadows).

0m1mScenario0m

- 0m1mI have problems with the screen refresh, the scene is only refreshed

behind the requester!0m

Open the 3mMUI-settings0m requester and select from the window

part for "Refresh" the "Smart"-mode.

- 0m1mHow can I change the texture and brush names without removing and adding

them again?0m

Select 3mView->Show->Show Textures0m or 3mView->Show->Show Brushes0m

and select the texture or brush. Now open the 3msettings-requester0m and change

the name of the brush or texture.

- 0m1mHow can I change the name of an Imagine/3DS/RAW-object without deleting

and reloding it?0m

Select 3mSettings0m from the object menu of the parent axis. Here you

can change the name of the object.

- 0m1mHow can I load my old Imagine attribute files to use it for RayStorm

objects?0m

Load the attributes file with 3mload object0m. You will get an axis

named 'Mesh' with another axis attached to it. The surface of this axis is set

to the attributes of the file. Click with the right mouse button to the parent

axis and select 3mTo scene0m. The parent axis is no deleted

and can now open the attributes requester for the child axis. Select save from

the 3mattributes requester0m and save the surface as a RayStorm material file.

0m1mRayStorm Script0m

- The commands '3mTEXTUREPATH0m', '3mOBJECTPATH0m' and '3mBRUSHPATH0m' are relative

to the directory RayStorm is started from.

- If RayStorm crashes with scenes with reflections and transparence, try

to start RayStorm with a larger stack (e.g. 8192 Bytes).

0m1mAREXX INTERFACE0m

3mIntroduction0m

3mAddress     0m

3mParameters  0m

3mCommands    0m

3mErrors      0m

0m1mAREXX INTRODUCTION0m

RayStorm is completly controled through it's ARexx interface. We recommend

that you have a look at the 3mtuturials0m and the 3mexample0m

script files in the 'ARexx' directory.

These examples cover most of the features of RayStorm. Further encourage you

to create your own files and make them available for the public. You can send

them to us and we might add them as an example files in the next version of

RayStorm or we include them to our 3mHomepage0m.

In one of the next versions of RayStorm we'll create a more powerful

language, which has a similar syntax to C++.

It's the same if you write the the commands in upper case or lower case.

But it's important to enclose all commans in quotes because ARexx tries

to interpret the line before it sends it to ARexx. It may happen that the

line is changed and RayStorm don't do this what you want.

A typical structure of a scene file is:

/* title, comments, ... */

/* setting resolution, world, camera, lightsources */

'SETSCREEN 160 128'

'SETWORLD [0,0,0] [40,40,40]'

'SETCAMERA <0,0,80> <0,0,0> <0,1,0> 25 20'

'POINTLIGHT <10,-10,100> [255,255,255] SHADOW'

/* define surfaces and actors */

'NEWSURFACE TEST1'

'AMBIENT [255,0,0]'

'DIFFUSE [255,0,0]'

'SPECULAR [255,255,255]'

'NEWSURFACE TEST2'

'AMBIENT [0,0,255]'

/* creating objects */

'SPHERE TEST1 <0,0,0> 10'

'SPHERE TEST2 <0,0,0> 10'

/* finally start to render the scene */

'STARTRENDER'

/* save the image */

'SAVEPIC "test.iff"'

'CLEANUP'

0m1mADDRESS0m

The ARexx-address of RayStorm is 'RAYSTORM'.

0m1mAREXX PARAMETERS0m

The parameters of a command can be FLOATs, INTEGERs, VECTORs, COLORs,

STRINGs, and IDENTIFIERs.

FLOAT     An FLOAT is a floating point number with single precision

NUMBER    A NUMBER is a simple integer number

VECTOR    A VECTOR is embedded in '<' '>' and consists of three FLOATs

COLOR     A COLOR is embedded in '[' ']' and consists of three INTEGERs

which range normally from 0 to 255, but you can also set negative

values or values above of 255.

STRING    A STRING consists of characters

KEYWORD   An KEYWORD is a switch and consists of uppercase characters

0m1mPARAMETER CONVENTIONS0m

/S - Switch.

/N - Number.

/A - Required.

All other numeric parameters are floating point numbers.

0m1mAREXX COMMANDS0m

3m General         0m

3m Objects         0m

3m Attributes      0m

3m Animation       0m

0m1mAlphabetically sorted0m

0m1m-A-0m

3mALIGNMENT  0m

3mAMBIENT    0m

3mANTIALIAS  0m

0m1m-B-0m

3mBRUSH      0m

3mBRUSHPATH  0m

0m1m-C-0m

3mCLEANUP    0m

0m1m-D-0m

3mDIFFTRANS  0m

3mDIFFUSE    0m

3mDISPLAY    0m

3mDISTRIB    0m

0m1m-F-0m

3mFLARE      0m

3mFOGLEN     0m

0m1m-G-0m

3mGETERRORSTR0m

0m1m-I-0m

3mIMTEXTURE  0m

0m1m-L-0m

3mLOADOBJ    0m

0m1m-N-0m

3mNEWSURFACE 0m

0m1m-O-0m

3mOBJECTPATH 0m

0m1m-P-0m

3mPLANE      0m

3mPOINTLIGHT 0m

3mPOSITION   0m

0m1m-Q-0m

3mQUIT       0m

0m1m-R-0m

3mREFEXP     0m

3mREFLECT    0m

3mREFRINDEX  0m

0m1m-S-0m

3mSAVEPIC    0m

3mSETCAMERA  0m

3mSETSCREEN  0m

3mSETWORLD   0m

3mSIZE       0m

3mSPECTRANS  0m

3mSPECULAR   0m

3mSPHERE     0m

3mSPOTLIGHT  0m

3mSTAR       0m

3mSTARTRENDER0m

0m1m-T-0m

3mTEXTUREPATH0m

3mTRANSEXP   0m

3mTRANSLUC   0m

3mTRANSPAR   0m

3mTRIANGLE   0m

0m1m-W-0m

3mWINTOFRONT 0m

0m1mGENERAL AREXX-COMMANDS0m

3mANTIALIAS  0m  sets antialiasing parameters

3mBRUSHPATH  0m  sets brush path

3mCLEANUP    0m  cleanups scene

3mDISPLAY    0m  displays scene

3mDISTRIB    0m  sets parameters for distributive sampling

3mFLARE      0m  add a flare to a lightsource

3mGETERRORSTR0m  gets a error string for a given number

3mOBJECTPATH 0m  sets object path

3mPOINTLIGHT 0m  creates point lightsource

3mQUIT       0m  quits RayStorm

3mSAVEPIC    0m  saves rendered picture

3mSETCAMERA  0m  sets camera parameters

3mSETSCREEN  0m  sets screen parameters

3mSETWORLD   0m  sets world parameters

3mSPOTLIGHT  0m  creates spot lightsource

3mSTAR       0m  add a light star to a lightsource

3mSTARTRENDER0m  starts rendering

3mTEXTUREPATH0m  sets texture path

3mWINTOFRONT 0m  brings window to front

0m1mANTIALIAS0m

0m1mTemplate:0m

SAMPLES/N/A,WIDTH,CONTRIB

0m1mArguments:0m

NUMBER SAMPLES

squareroot of number of samples per pixel (max. 8)

FLOAT WIDTH

width of gaussian filter. The range is infinite but values between 1 and

3 are recommended.

COLOR CONTRIB

max. allowed contrast

0m1mDescription:0m

Sets antialiasing parameters (see 3mAntialiasing0m)

0m1mDefault:0m

ANTIALIAS 1 1.3 [51,38,76]

0m1mBRUSHPATH0m

0m1mTemplate:0m

PATH/A

0m1mArguments:0m

STRING PATH

pathname

0m1mDescription:0m

Defines the path where to search brushes. More than one path may be included

as long as they are separated by semi-colons.

0m1mExample:0m

BRUSHPATH 'path1;path2'

0m1mCLEANUP0m

0m1mTemplate:0m

none

0m1mArguments:0m

none

0m1mDescription:0m

Deletes all defined objects, lightsources, surfaces and actors

0m1mDISPLAY0m

0m1m!!! CAUTION !!!

THIS COMMAND ISN'T RELEASED IN THIS VERSION YET

!!! CAUTION !!!0m

0m1mTemplate:0m

FLOYD/S

0m1mArguments:0m

KEYWORD FLOYD/S

dither with Floyd-Steinberg algorithm

0m1mDescription:0m

Displays rendered pic on screen

0m1mDISTRIB0m

0m1mTemplate:0m

SAMPLES/N,SOFTSHADOW/N

0m1mArguments:0m

NUMBER SAMPLES/N

squareroot of number of samples per pixel for motionblur

NUMBER SOFTSHADOW/N

squareroot of number of samples per pixel for softshadows

0m1mDescription:0m

Sets number of samples per pixel for distributive sampling (used for

0m1mDefault:0m

DISTRIB 1,1

0m1mGETERRORSTR0m

0m1mTemplate:0m

ERRNUM/N/A

0m1mArguments:0m

NUMBER ERRNUM

error number

0m1mDescription:0m

Returns the error string for the given error number

0m1mOBJECTPATH0m

0m1mTemplate:0m

PATH/A

0m1mArguments:0m

PATH

pathname

0m1mDescription:0m

Defines the path where to search objects. More than one path may be included

as long as they are separated by semi-colons.

0m1mExample:0m

OBJECTPATH 'path1;path2'

0m1mPOINTLIGHT0m

0m1mTemplate:0m

POS/A,COLOR,SIZE,SHADOW/S,ACTOR,FALLOFF

0m1mArguments:0m

VECTOR POS

position of pointlight

COLOR COLOR

color of light

VECTOR SIZE

size of light source (used for 3msoft shadows0m)

KEYWORD SHADOW/S

lightsource casts shadows

STRING ACTOR

name of actor

FLOAT FALLOFF

distance where the brightness of the light is zero

0m1mDescription:0m

Creates a point lightsource. The lightsource casts shadows, if the keyword SHADOW is given

0m1mDefault:0m

POINTLIGHT <0,0,0> [255,255,255] 0 ?? ?? 0

0m1mQUIT0m

0m1mTemplate:0m

none

0m1mArguments:0m

none

0m1mDescription:0m

Quits Raystrom

0m1mSAVEPIC0m

0m1mTemplate:0m

NAME/A,FORMAT

0m1mArguments:0m

STRING NAME

the picture is saved under that name

STRING FORMAT

image format (TGA, PNG, ILBM; default ILBM)

0m1mDescription:0m

Saves rendered picture 24-Bit IFF-ILBM-file, 24 Bit TGA or as PNG file. If

an error occures the command returns an error string.

0m1mExample:0m

SAVEPIC 'path\name with extension'

0m1mSETCAMERA0m

0m1mTemplate:0m

POS/A,VIEWPOINT,VIEWUP,FOVX,FOVY,FOCALDIST,APERTURE,POSACTOR,VIEWACTOR

0m1mArguments:0m

VECTOR POS

position of camera

VECTOR VIEWPOINT

position to which the camera point to

VECTOR VIEWUP

view up vector

FLOAT FOVX, FOVY

field of view (in degree) (20 degree creates camera like Imagine

default camera)

FLOAT FOCALDIST

distance from eye to focal plane

FLOAT APERTURE

aperture width (0 = pinhole) (-> 3mdepth of field0m)

STRING POSACTOR

name of position actor

STRING VIEWACTOR

name of look_at_actor

0m1mDescription:0m

Sets the parameters of the camera

0m1mDefault:0m

SETCAMERA <0,0,-10> <0,0,0> <0,1,0>  45 45 1. 0.

0m1mSETSCREEN0m

0m1mTemplate:0m

RESX/N/A,RESY/N/A,COLORS/N

0m1mArguments:0m

NUMBER RESX, RESY

resolution

NUMBER COLORS

number of colors (not yet implemented)

0m1mDescription:0m

Sets the resolution of the rendered picture. Note that in the demo-version

the resolution is limited to 160x128!

0m1mDefault:0m

SETSCREEN 128 128

0m1mSETWORLD0m

0m1mTemplate:0m

BACK/A,AMBIENT,RANDJIT/S,BACKDROP,FOGLEN,FOGHEIGHT,FOGCOLOR,REFLMAP

0m1mArguments:0m

COLOR BACK

backgroundcolor

COLOR AMBIENT

ambientcolor

KEYWORD RANDJIT

use random jitter for 3mdepth of field0m and 3msoft shadows0m

STRING BACKDROP

name of backdrop picture

FLOAT FOGLEN

global fog length

FLOAT FOGHEIGHT

highest fog y-coordinate

COLOR FOGCOLOR

fogcolor

STRING REFLMAP

name of reflection map

0m1mDescription:0m

Sets world parameters. The backdrop picture is automatically scaled to

the screen resolution.

0m1mDefault:0m

SETWORLD [0,0,0] [0,0,0] ?? 32 0 [255,255,255] ??

0m1mSPOTLIGHT0m

0m1mTemplate:0m

POS/A,COLOR,LOOKPOINT,ANGLE,SIZE,SHADOW/S,ACTOR,LOOKP_ACTOR,FALLOFF

0m1mArguments:0m

VECTOR POS

position of the spotlight

COLOR COLOR

color of light

VECTOR LOOKPOINT

point to which the spotlight shines at

FLOAT ANGLE

opening angel (in degree max. 180)

FLOAT SIZE

size of light source (used for 3msoft shadows0m)

KEYWORD SHADOW

lightsource cats shadows

STRING ACTOR

name of position actor

STRING LOOKP_ACTOR

name of look_at_actor

FLOAT FALLOFF

distance where the brightness of the light is zero

0m1mDescription:0m

Creates a spotlight. The rays emitted from a spotlight are constrained

by a cone. The LOOKPOINT vector gives the center of the illuminated area.

0m1mDefault:0m

SPOTLIGHT <0,0,0> [255,255,255] <0,0,1> 45 0 ?? ?? ?? 0

0m1mFLARE0m

0m1mTemplate:0m

POS/A,RADIUS/A,COLOR,TYPE,FUNCTION,EDGES/N,TILT

0m1mArguments:0m

FLOAT pos

position of flare on light axis (in percent)

FLOAT radius

radius of flare (in percent)

COLOR color

color of flare

KEYWORD type [DISC|POLYGON]

type of flare

KEYWORD function [PLAIN|RING|RADIAL]

look of flare

NUMBER edges

number of edges of polygonal flare (3 - 8 edges possible)

FLOAT tilt

tilt angle of flare (only useful when polygonal flare)

0m1mDescription:0m

A new lens flare is added to the last defined light source. Lens flares only

appear when the light source is visible. They appear on an axis which runs

from the center of the light source through the center of the screen. The

position of the flare is relative to the light source it belongs to. If the

position value is zero, the flare appears at the position of the light source.

Greater values of pos move it towards the center of the screen. Here a some

example values and their meaning:

0 : flare has the same position as the light source

100 : flare is in the middle of the screen

200 : flare is on opposite side of the light source

You can also set pos to negative values, so the flare will appear behind the

light source.

The radius of the flare is relative to the horizontal render resolution.

For example: A flare with radius of 20 % renderd with a resolution of 320x200

will have a radius of 0.2*320 = 64 pixels.

0m1mtype0m can be one of the following identifiers:

DISC

A flare which looks like a disc

POLYGON

A flare which looks like a polygon. The number of edges can be set with

edges.

0m1mfunction0m can be one of the following identifiers:

PLAIN

the color of the flare is uniform

RING

the flare looks like a ring

RADIAL

varies the color of the flare radial around the flare center

0m1mDefault:0m

FLARE 50 10 [50,50,50] DISC PLAIN 5 0

0m1mSTAR0m

0m1mTemplate:0m

SPIKES,RADIUS,HALORADIUS,INNERHALORADIUS,RANGE,RANDOM_SEED/N

0m1mArguments:0m

NUMBER spikes

number of spikes (4 - 256)

FLOAT radius

radius of star (in percent)

FLOAT haloradius

radius of star halo (in percent relative to star radius)

FLOAT innerhaloradius

radius of inner star halo (in percent relative to halo radius)

FLOAT range

range between shortest and longest spike (in percent)

NUMBER random_seed

Random seed value (0-32767)

0m1mDescription:0m

This command makes a light source visible. In reality, a light source looks

like a star with several spikes getting out of the center of the light source.

Usually there's a halo around it too.

The radius (length of the longest spike) of the star is relative to the

horizontal render resolution (see 3mFLARE0m).

The radius of the halo is relative to the radius of the star itself. A value

of 100 % means, that the halo has the same radius as the star.

The range value determines the difference between the longest and the shortes

spike of the star. A value of 0 % means that all spikes have the same length,

whereas 100 % means that the shortest spike has zero length.

0m1mHow are the spikes computed ?0m

The four spikes in the main directions (0, 90, 180, 270 degrees) always have

maximum length. Spikes between them (45, 135, ... degrees for example) are

shorter and again spikes between them are shorter and so on.

If you specify a value for random_seed, the length of the spikes is randomly

changed.

0m1mDefault:0m

STAR 4 30 20 40 0 0

0m1mSTARTRENDER0m

0m1mTemplate:0m

QUICK/S,DEPTH/N,FROM,TO/N,LEFT/N,TOP/N,RIGHT/N,BOTTOM/N

0m1mArguments:0m

KEYWORD QUICK

render quick (no shadows, reflections and transparency)

NUMBER DEPTH

depth of generated 3moctree0m

FLOAT FROM,TO

time code (default 0,0). If you want 3mmotion blur0m you have to

set FROM and TO to different values, else only set FROM.

NUMBER LEFT,TOP,RIGHT,BOTTOM

coordinates for rendering box. Picture is renderd only inside of

rectangle.

0m1mDescription:0m

Starts rendering process. If you set QUICK shadows, reflections and

transparency are not calculated. In very complex scenes it is useful to

increase the octree depth in order to reach a better performance during

the rendering process. But this can only be done with enough memory!

Here are values for the file "chess.ray" renderd with a resolution of

160x128 on a A4000 with 25Mhz 68030 and 57MHz 68882:

depth  memory [MByte]  init [mm:ss]  cleanup [mm:ss]  render [mm:ss]  total [mm:ss]

2          1.38         00:12           00:02           09:24          09:38

3          1.13         00:14           00:04           03:58          04:16

4          1.55         00:22           00:08           01:50          02:20

5          2.35         00:41           00:26           01:15          02:22

6          5.70         01:35           02:38           01:06          05:19

0m1mDefault:0m

STARTRENDER 3 0 0

0m1mTEXTUREPATH0m

0m1mTemplate:0m

PATH/A

0m1mArguments:0m

PATH

pathname (format: 'path1;path2;...;pathn')

0m1mDescription:0m

Defines the path where to search textures. More than one path may be included

as long as they are separated by semi-colons.

0m1mExample:0m

TEXTUREPATH 'path1;path2'

0m1mWINTOFRONT0m

0m1mTemplate:0m

none

0m1mArguments:0m

none

0m1mDescription:0m

Brings RayStorm window in front

0m1mAREXX-COMMANDS FOR CREATING OBJECTS0m

3mLOADOBJ 0m  loads an Imagine TDDD-file

3mPLANE   0m  creates a plane (ground in Imagine)

3mSPHERE  0m  creates a sphere

3mTRIANGLE0m  creates a triangle

0m1mLOADOBJ0m

0m1mTemplate:0m

NAME/A,POS,ALIGN,SCALE,ACTOR,SURFACE

0m1mArguments:0m

STRING NAME

filename

VECTOR POS

position

VECTOR ALIGN

alignment (in degrees)

VECTOR SCALE

size factor

STRING ACTOR

name of actor

STRING SURFACE

name of surface to replace object surface

0m1mDescription:0m

Loads an

- Imagine TDDD-file

loads attributes, triangles (with correct handling of sharp edges),

perfect spheres, planes, brushes and textures

- Autodesk 3D-Studio file

loads attributes, triangles and generates sharp edges

If you specify a surface, all surfaces of the object will have that surface

(overriding any surfaces defined in the object file).

0m1mWhere to get Imagine object files?0m

Look on FTP-servers which support AMINET. For example try out

ftp.uni-paderborn.de

Path: ftp/aminet/pub/gfx/3dobj/

0m1mWhere to get 3D-Studio object files?0m

Try out these WWW-Pages:

For Star wars fans: http://www.loop.com/~hhc/

Mesh Mart: http://cedar.cic.net/~rtilmann/mm/index.htm

Objects Archive: http://sunserver1.rz.uni-duesseldorf.de/~pannozzo/3ds.html

0m1mDefault:0m

LOADOBJ ??? <0,0,0> <0,0,0> <1,1,1>

0m1mPLANE0m

0m1mTemplate:0m

SURF/A,POS,NORM,ACTOR

0m1mArguments:0m

STRING SURF

name of surface

VECTOR POS

position

VECTOR NORM

normal of the plane

STRING ACTOR

name of actor

0m1mDescription:0m

Creates an infinite plane.

0m1mDefault:0m

PLANE ??? <0,0,0> <0,1,0>

0m1mSPHERE0m

0m1mTemplate:0m

SURF/A,POS/A,RADIUS/A,ACTOR

0m1mArguments:0m

STRING SURF

name of surface

VECTOR POS

center of sphere

FLOAT RADIUS

radius of sphere

STRING ACTOR

name of actor

0m1mDescription:0m

Creates a sphere

0m1mDefault:0m

SPHERE ?? <0,0,0> 1

0m1mTRIANGLE0m

0m1mTemplate:0m

SURF/A,P1/A,P2/A,P3/A,N1,N2,N3,ACTOR

0m1mArguments:0m

STRING SURF

name of surface

VECTOR P1

first corner

VECTOR P2

second corner

VECTOR P3

third corner

VECTOR N1

normal at first corner

VECTOR N2

normal at second corner

VECTOR N3

normal at third corner

STRING ACTOR

name of actor

0m1mDescription:0m

Creates a triangle with corners at position P1, P2 and P3. If you specify

the normals, a phong shaded triangle otherwise a flat triangle is created.

Computing the normals by hand is a difficult task, and should be done by

utility programs.

0m1mAREXX-COMMANDS FOR SETTING ATTRIBUTES0m

Every object must have a surface definition. With the following commands

you can set the attributes of a surface. First you have to define the

current surface with 'NEWSURFACE <name>'. Raystorm will set the attributes

of the new surface to default values. Every following command such as

AMBIENT or DIFFTRANS refers to the current surface and will override

the corresponding default values.

The following examples define two surfaces:

NEWSURFACE RED

AMBIENT [255,0,0]

DIFFUSE [255,0,0]

NEWSURFACE WATER

DIFFUSE [0,0,255]

REFRINDEX 1.333

List of surface commands:

3mNEWSURFACE0m  creates a new surface

3mAMBIENT   0m  sets ambient color

3mBRUSH     0m  adds a brush

3mDIFFTRANS 0m  sets diffuse transmission color

3mDIFFUSE   0m  sets diffuse color

3mFOGLEN    0m  sets the foglength

3mIMTEXTURE 0m  adds a Imagine texture

3mREFEXP    0m  sets the specular reflection exponent

3mREFLECT   0m  sets the specular reflectivity

3mREFRINDEX 0m  sets the index of refraction

3mSPECTRANS 0m  sets the specular transmission

3mSPECULAR  0m  sets the specular color

3mTRANSEXP  0m  sets the specular transmission exponent

3mTRANSLUC  0m  sets the specular transmittance

3mTRANSPAR  0m  sets the diffuce transmittance

0m1mAMBIENT0m

0m1mTemplate:0m

COLOR/A

0m1mArguments:0m

STRING COLOR

color

0m1mDescription:0m

Sets the ambient color of surface. Determines the color of the object in

sections, which are in shadow.

0m1mDefault:0m

AMBIENT [255,255,255]

0m1mBRUSH0m

0m1mTemplate:0m

NAME/A,TYPE/A,WRAP/A,POS/A,ALIGN/A,SIZE/A,REPEAT/S,MIRROR/S,SOFT/S,ACTOR

0m1mArguments:0m

STRING NAME

filename of brush

KEYWORD TYPE [COLOR|REFLECT|FILTER|ALTITUDE|SPECULAR]

type of brush

KEYWORD WRAP [FLAT|WRAPX|WRAPY|WRAPXY]

brush wrapping method

VECTOR POS

position

VECTOR ALIGN

alignment

VECTOR SIZE

size of brush

KEYWORD REPEAT

if set, brush is repeated like a tile

KEYWORD MIRROR

if set, brush is mirrord (when REPEAT is specified)

KEYWORD SOFT

if set, brush color is interpolated softly

STRING ACTOR

name of actor

0m1mDescription:0m

Adds a brush to surface definition. A brush is a bitmap which is wrapped

around an  object. The specified file will be searched for in the current

directory. If it wasn't found there and a 3mBRUSHPATH0m

be searched there.

If an error occures the command returns an error string. Supported formats

are: IFF-ILBM, PNG, TGA and Datatypes.

Constants for type:

COLOR

Replaces the surface color of the object with the image (sets 3mDIFFUSE0m and

3mAMBIENT0m color).

REFLECT

Map covers the surface and reflects environment (see 3mREFLECT0m)).

FILTER

Uses the white color to pass colors and the black area to hold back color

with a variance between two colors (like with 3mTRANSPAR0m).

ALTITUDE

The red values of the brush are used to give the surface an appearence of

bumpiness.

SPECULAR

The rgb values set the specular color of the surface (see 3mSPECULAR0m).

Constants for wrap :

FLAT

The brush is projected to X-Y plane, the axis is in the middle of the

brush area, length is the distance from the middle to the border.

WRAPX

The brush is wrapped around the x-axis, like on a cylinder. The left edge

of the brush begins at the posititve X axis and wraps the brush around

the cylinder from 'west' to 'east'.

WRAPY

Same as WRAPX, but wrapping is around the y-axis.

WRAPXY

Wrapping both: around X and Y axis. It is assumed, that the object is a

sphere. The Y axis is the north/south pole of the spherical mapping. The

left edge of the brush begins at the positive X axis and wraps the brush

around the sphere from 'west' to 'east'. The brush covers the sphere

exactly once.

0m1mExample:0m

BRUSH "earth.iff" COLOR, WRAPXY <0,0,0> <0,0,0> <0.1,0.1,0.1>

0m1mDIFFTRANS0m

0m1mTemplate:0m

COLOR/A

0m1mArguments:0m

COLOR COLOR

color

0m1mDescription:0m

Sets the diffuse transmission color of surface. Same as diffuse reflection,

but only used if the lightsource is on opposite side of surface. Only

applied if tranlucency is not set to zero.

0m1mDefault:0m

DIFFTRANS [0,0,0]

0m1mDIFFUSE0m

0m1mTemplate:0m

COLOR/A

0m1mArguments:0m

COLOR COLOR

color

0m1mDescription:0m

Sets the diffuse color of surface. The diffuse reflection falls off as

the cosine of the angle between the normal and the ray to the light.

Diffuse reflection determines the main color of the object (color in

Imagine).

0m1mDefault:0m

DIFFUSE [255,255,255]

0m1mFOGLEN0m

0m1mTemplate:0m

VALUE/A

0m1mArguments:0m

FLOAT VALUE/A

foglength

0m1mDescription:0m

Sets the foglength of the surface. The fog color is set with 3mTRANSPAR0m.

0m1mDefault:0m

FOGLEN 0

0m1mIMTEXTURE0m

0m1mTemplate:0m

NAME/A,POS,ALIGN,SIZE,P1,P2,P3,P4,P5,P6,P7,P8,P9,P10,P11,P12,P13,P14,P15,P16,ACTOR

0m1mArguments:0m

STRING NAME

name of Imagine texture file

VECTOR POS

position

VECTOR ALIGN

alignment

VECTOR SIZE

size of texture axis

FLOAT P1,P2,P3,P4,P5,P6,P7,P8,P9,P10,P11,P12,P13,P14,P15,P16

texture parameters

STRING ACTOR

name of actor

0m1mDescription:0m

Adds a Imagine texture to surface

0m1mDefault:0m

defaults are taken from texture if not all paramters are given

0m1mExample:0m

IMTEXTURE "checker.itx" <0.1,0.1,0.1> <0,0,0> <2,2,2>

0m1mNEWSURFACE0m

0m1mTemplate:0m

NAME/A,BRIGHT/S

0m1mArguments:0m

STRING NAME

name

KEYWORD BRIGHT

if set, the brightness of the surface is everywhere the same

0m1mDescription:0m

Defines a new surface.

0m1mREFEXP0m

0m1mTemplate:0m

VALUE/A

0m1mArguments:0m

FLOAT VALUE

specular reflection exponent

0m1mDescription:0m

Sets the specular reflection exponent of surface. Determines the size of

the specularly reflected highlights, the higher the smaller the highlight

(hardness in Imagine).

0m1mDefault:0m

REFEXP 12.

0m1mREFLECT0m

0m1mTemplate:0m

COLOR/A

0m1mArguments:0m

COLOR COLOR

color

0m1mDescription:0m

Sets the specular reflectivity of surface

0m1mDefault:0m

REFLECT [0,0,0]

0m1mREFRINDEX0m

0m1mTemplate:0m

VALUE/A

0m1mArguments:0m

FLOAT VALUE

index of refraction

0m1mDescription:0m

Sets the index of refraction of surface. Determines how the ray is refracted

through transparent objects, the higher the more (index of refraction in

Imagine).

0m1mDefault:0m

REFRINDEX 1.

0m1mExamples:0m

MATERIAL                      Index

-------------------------------------

Vacuum ...................... 1.00000 (exactly)

Air (STP).................... 1.00029

Acetone ..................... 1.36

Alcohol ..................... 1.329

Amorphous Selenium .......... 2.92

Calspar1 .................... 1.66

Calspar2 .................... 1.486

Carbon Disulfide ............ 1.63

Chromium Oxide .............. 2.705

Copper Oxide ................ 2.705

Crown Glass ................. 1.52

Crystal ..................... 2.00

Diamond ..................... 2.417

Emerald ..................... 1.57

Ethyl Alcohol ............... 1.36

Flourite .................... 1.434

Fused Quartz ................ 1.46

Heaviest Flint Glass ........ 1.89

Heavy Flint Glass ........... 1.65

Glass ....................... 1.5

Ice ......................... 1.309

Iodine Crystal .............. 3.34

Lapis Lazuli ................ 1.61

Light Flint Glass ........... 1.575

Liquid Carbon Dioxide ....... 1.20

Polystyrene ................. 1.55

Quartz 1 .................... 1.644

Quartz 2 .................... 1.553

Ruby ........................ 1.77

Sapphire .................... 1.77

Sodium Chloride (Salt) 1 .... 1.544

Sodium Chloride (Salt) 2 .... 1.644

Sugar Solution (30%) ........ 1.38

Sugar Solution (80%) ........ 1.49

Topaz ....................... 1.61

Water (20 C) ................ 1.333

Zinc Crown Glass ............ 1.517

0m1mSPECTRANS0m

0m1mTemplate:0m

COLOR/A

0m1mArguments:0m

COLOR COLOR

color

0m1mDescription:0m

Sets the specular transmission color of surface. Same as specular

reflection, but only used if the lightsource is on opposite side of

surface. Only applied if tranlucency is not 0.

0m1mDefault:0m

SETSPECTRANS [255,255,255]

0m1mSPECULAR0m

0m1mTemplate:0m

COLOR/A

0m1mArguments:0m

COLOR COLOR

color

0m1mDescription:0m

Sets the specular color of surface. Specularly reflected highlights fall

off as the cosine of the angle between the reflected ray and the ray to the

light source (specular in Imagine).

0m1mDefault:0m

SPECULAR [255,255,255]

0m1mTRANSEXP0m

0m1mTemplate:0m

VALUE/A

0m1mArguments:0m

FLOAT VALUE

specular transmission exponent

0m1mDescription:0m

Sets the specular transmission exponent of surface. Same as specular

reflection exponent, but only used if the lightsource is on opposite

side of surface.

0m1mDefault:0m

TRANSEXP 12.

0m1mTRANSLUC0m

0m1mTemplate:0m

VALUE/A

0m1mArguments:0m

FLOAT VALUE

specular transmittance

0m1mDescription:0m

Sets the specular transmittance of surface

0m1mDefault:0m

TRANSLUC 0

0m1mTRANSPAR0m

0m1mTemplate:0m

COLOR/A

0m1mArguments:0m

COLOR COLOR

color

0m1mDescription:0m

Sets the diffuse transmittance of surface

0m1mDefault:0m

TRANSPAR [0,0,0]

0m1mAREXX-COMMANDS FOR ANIMATION CONTROL0m

3mALIGNMENT0m  sets alignment

3mNEWACTOR 0m  creates a new actor

3mPOSITION 0m  sets position

3mSIZE     0m  sets size

0m1mALIGNMENT0m

0m1mTemplate:0m

FROM/A,TO/A,ALIGN/A,TYPE

0m1mArguments:0m

FLOAT FROM,TO

time interval

VECTOR ALIGN

alignment at the end of the interval

KEYWORD TYPE [LINEAR]

interpolation type (currently only linear)

0m1mDescription:0m

Sets the alignment of the object. At time to, the actor will have this

alignment.

'TYPE' can be one of the following identifiers:

LINEAR  the object moves on a straight line between the positions

SPLINE  the object moves on a spline curve between the positions. (NOT

IMPLEMENTED YET)

0m1mNEWACTOR0m

0m1mTemplate:0m

NAME/A,POS,ALIGN,SIZE

0m1mArguments:0m

STRING NAME

name of actor

VECTOR POS

initial position of actor

VECTOR ALIGN

initial alignment (in degrees)

VECTOR SIZE

initial size

0m1mDescription:0m

Creates a new actor. Usually a NEWACTOR definition is followed by one or more

POSITION, ALIGNMENT, SIZE commands which define the properties of the actor

at a certain time.

For more information on NEWACTOR, please click 3mhere0m.

0m1mDefault:0m

NEWACTOR ??? <0,0,0> <0,0,0> <1,1,1>

0m1mPOSITION0m

0m1mTemplate:0m

FROM/A,TO/A,POS/A,TYPE

0m1mArguments:0m

FLOAT FROM,TO

time interval

VECTOR POS

position at the end of the interval

KEYWORD TYPE [LINEAR]

interpolation type (currently only linear)

0m1mDescription:0m

Sets the position of the object. At time to, the actor will be at this

position.

'TYPE' can be one of the following identifiers:

LINEAR  the object moves on a straight line between the positions

SPLINE  the object moves on a spline curve between the positions. (NOT

IMPLEMENTED YET)

0m1mSIZE0m

0m1mTemplate:0m

FROM/A,TO/A,SIZE/A,TYPE

0m1mArguments:0m

FLOAT FROM,TO

time interval

SIZE

size at the end of the interval

KEYWORD TYPE [LINEAR]

interpolation type (currently only linear)

0m1mDescription:0m

Sets the size of the object. At time to, the actor will have this size.

'TYPE' can be one of the following identifiers:

LINEAR  the object moves on a straight line between the positions

SPLINE  the object moves on a spline curve between the positions. (NOT

IMPLEMENTED YET)

0m1mWHAT IS AN ACTOR?0m

The statement:

NEWACTOR name, position, alignment, size

defines a "virtual object" that can be associated with "real" objects,

including the Camera and  the Lights objects.  Position, alignment, and

size values are the initial values of an actor.

The object that is associated with a named actor behaves exactly like the

actor. When the actor is sized, rotated, or moved; the associted object is

sized, rotated, or moved accordingly.  It is as though the actor and the

object that is associated with it are connected by a rod.

While the coordinates of the object that is associated with an actor are

always defined in an absolute way in reference to the set world; its

movements - when associated with an actor - takes place with respect to

the coordinates of the actor.  The coordinates of the actor are defined in

the NEWACTOR statment and can have default values.

The position, alignment, and size values that follow the NEWACTOR statement

are the final values of an actor.  By defining time from and to values, a time

increment is defined by which the actor is resized, rotated, or moved from

the initial values to the final values.  By associating objects to an actor,

and rendering a picture at a particular time, or through a period of time,

animation and/or motion blur effects are achieved.

0m1mAREXX-ERRORS0m

These values are returned when something went wrong, you can get the

error string with the command 3mGETERRORSTR0m.

0m1mApplication and parser errors0m

Here are the errors returned from the command parser and the application

itself.

100   Wrong screen resolution

Both components of the screen resolution have to be higher than one.

101   Not enough memory

Allocation of memory failed.

102   Limitations of demo version reached

The demo version is limited to a resolution of 160x128.

103   Unknown brush mapping type

You specified a unknown mapping method for the 3mBRUSH0m command.

104   Unknown brush wrapping method

You specified a unknown wrapping method for the 3mBRUSH0m command.

105   Invalid time intervall

One component of a time intervall was negative or the beginning time

was later than the end.

106   Unknown interpolation method

You specified a unknown interpolation method for the 3mPOSITION0m,

3mALIGNMENT0m or 3mSIZE0m command.

107   No picture renderd

There is no picture for 3mSAVEPIC0m to save because you renderd none

or called 3mCLEANUP0m before.

108   Can't open screen

The 3mDISPLAY0m command was unable to open the screen

(!!! THIS COMMAND ISN'T RELEASED IN THIS VERSION YET !!!).

109   Can't open raystorm.library (Only one copy of RayStorm can run at a time)

RayStorm failed to open raystorm.library

110   Can't open intuition.library

RayStorm failed to open intuition.library (at least version 37 is

needed)

111   Can't open window

RayStorm failed to open the window.

112   Can't open muimaster.library

RayStorm failed to open muimaster.library (at least version 8 is

needed)

113   Invalid vector definition

The specified vector has the wrong format (must be '<x,y,z>').

114   Invalid color definition

The specified color has the wrong format (must be '[r,g,b]').

115   Invalid region definition

The specified region is out of range.

116   No light defined

You tried to add lens flares or a light star and there is no previously

defined light source.

0m1mInternal errors0m

This are errors of the renderer.

1   Not enough memory

Allocation of memory failed.

2   Limitations of demo version reached

The demo version is limited to a resolution of 160x128.

3   Wrong screen resolution

Both components of the screen resolution have to be higher than one.

4   Error in triangle definition

It's impossible to generate a triangle with the specified coordinates

(see 3mTRIANGLE0m).

5   The view and up directions are identical?

You specified a view-up-vector for the 3mCAMERA0m command which is identical

to the view direction.

6   Not enough memory for screen buffer

The allocation of the screen buffer failed.

8   Can't open Imagine texture file

RayStorm failed to open the specified Imagine texture file, check

filename and path.

9   Can't open brush file

RayStorm failed to open the specified brush file, check

filename and path.

10   Error initializing Imagine texture

An error occured as RayStorm tried to initialize a Imagine texture.

11   Can't open picture

RayStorm failed to open the specified picture file, check filename

and path.

12   Error reading picture

An error occured while RayStorm read a picture file.

13   Can't open picture type file ('modules/picture/types')

RayStorm failed to open the typefile. The typefile is needed to

identify the filetypes of the pictures. RayStorm was unable to open

the file 'modules/picture/types'.

14   Error reading picture type file

An error occured while RayStorm read the picture type file, maybe the

file is damaged.

14   Unknown picture format

RayStorm was unable to recognize the format of the picture file.

16   An error occcured while invoking picture handler

The used picture handler returned a error.

17   Can't open object

RayStorm failed to open the specified object file, check filename

and path.

18   Error reading object

An error occured while RayStorm read a object file.

19   Can't open object type file ('modules/object/types')

RayStorm failed to open the typefile. The typefile is needed to

identify the filetypes of the objects. RayStorm was unable to open

the file 'modules/object/types'.

20   Error reading object type file

An error occured while RayStorm read the object type file, maybe the

file is damaged.

21   Unknown object format

RayStorm was unable to recognize the format of the object file.

22   An error occcured while invoking object handler

The used object handler returned a error.

23   Actor not defined

The specified actor name does not exist.

24   Surface not defined

The specified surface name does not exist.

25   Depth of octree too big (max. 6)

The octree depth is limited to a depth of 6.

26   Antialiasing value too big (max. 8)

The value of the 3mANTIALIAS0m command is limited to 8.

27   Invalid time intervall

One component of a time intervall was negative or the beginning time

was later than the end.

28   No picture renderd

There is no picture for 3mSAVEPIC0m to save because you renderd none

or called 3mCLEANUP0m before.

29   Distribution value too big (max. 8)

The value of the 3mDISTRIB0m command is limited to 8.

30   Wrong error number

The error number for 3mGETERRORSTR0m is out of range.

31   Unknown Parameter

A RSI-function was called with an unknown parameter (should nerver occur).

0m1mEXAMPLES0m

We have included several demos in the directories 'arexx' and 'examples'

to show how to use RayStorm.

In the 'arexx' directory are examples scripts which show the usage of

RayStorm with ARexx. Start them simply by typing 'rx ???.ray' in a shell

(???.ray is the name of the script).

In the 'examples' directory are C-programs which show the usage of

RayStorm directly with a program. They can only be run from a shell.

These programs are producing a couple of pictures no animation, which must

be glued together with a utility like MainActor.

0m1mTUTORIALS0m

If you use RayStorm for the first time, then it would be a good idea to do

the following tutorials:

3mSimple scene 0m

3mBouncing ball0m

3mMotion blur  0m

0m1mTutorial: Simple scene0m

Now we will create a very famous scene. A sphere over a checkerboard! Ok, it

may be boring, but it's good for the absolute beginner to get an

impression of building a scene.

Here we go:

1. In the  drawer 'ARexx' of the RayStorm directory there is a file named

'default.ray'. This is a default form for RayStorm ARexx scripts. You can

use this form to write your own scripts.

We'll use this file as a default for our animation script. Copy this file

to the file 'simple.ray'. After this load the file 'simple.ray' to your

favorite text editor (e.g GoldEd or CygnusEd).

2. To view the scene, we need a camera. Insert after the line 'address

RAYSTORM' the line:

0m3m   '3mSETCAMERA0m <6,1.5,-1.5> <0,0,0> <0,1,0>'0m

This sets the camera to position <6,1.5,-1.5>. The camera points to

<0,0,0> and the view-up vector is <0,1,0>. Note that you don't have to

specify every single parameter. Every command has default values. Refer

to the description of a command to find out the default values.

3. Nothing can be seen without a lightsource.

Go to the next line and type:

0m3m   '3mPOINTLIGHT0m <0,50,0> [255,255,255] SHADOW'0m

The sphere is illuminated from above with white light.

4. Before placing the objects in the scene, you have to define their

surfaces.

Insert this line:

0m3m   '3mNEWSURFACE0m planesurf'0m

This creates a surface with name planesurf. The plane has a checkered

surface, so insert:

0m3m   '3mIMTEXTURE0m /textures/checker.itx <0.1,0.1,0.1> <0,0,0> <2,2,2>'0m

5. That was the plane texture. Let 's go over to sphere texture.

Add:

0m3m   '3mNEWSURFACE0m spheresurf'0m

The sphere has a mirrored surface. To simulate a perfect mirror, type

0m3m   '3mREFLECT0m [255,255,255]'0m

6. Now we can add the objects to the scene:

0m3m   '3mSPHERE0m spheresurf <0,0.5,0> 1'0m

This creates a sphere on position <0,0.5,0> and radius 1.

Add the plane:

0m3m   '3mPLANE0m planesurf'0m

The default values for the position and the normal vector fit to our

scene, so we can take them over.

7. Let's make an end to the definitions and render the scene!

Type:

0m3m   '3mSTARTRENDER0m'0m

8. Finally we may not forget to save the picture, so add:

0m3m   '3mSAVEPIC0m simple.iff'0m

which will save the renderd picture in the current directory as a

IFF-ILBM file.

The last step is to free all the memory with the command 'CLEANUP'. Add:

0m3m   3mCLEANUP0m'0m

9. Start the script from a shell-window with the sequence 'rx simple.ray'.

RayStorm will now generate your picture. When RayStorm finished the work

start your favourite viewer-program, load the file and have a look at

it.

Looks very monochrome!!

To make the world colorful, we make a red checker and set the sky to

blue. A blue sky can be done by setting the world's background color.

10. Before 'SETCAMERA' insert:

0m3m   '3mSETWORLD0m [30,30,255]'0m

Add

0m3m   '3mDIFFUSE0m [155,0,0]'0m

to the surface planesurf (this defines one checker color), the other one

must be set in the 'IMTEXTURE' command, so change it to

0m3m   '3mIMTEXTURE0m /textures/checker.itx <0.1,0.1,0.1> <0,0,0> <2,2,2> 255 0 0'0m

(Note that '255 0 0' describes a color, but is not embedded in < >,

because the checker color belongs to the texture parameters which are

all floats.)

11. Render the scene once again, and view it.

That's the end of the tutorial! Make some changes to the scene file and

play around with the parameters to see their effects.

0m1mTutorial: Bouncing ball0m

The goal of this tutorial is to show you how to generate little animations.

At the end of this tutorial you'll have a animation where the earth rotates

and bounces on a rotating plane with a white checker texture on the top and

a red checker on the bottom. If you have a fast computer you can also

generate the animation with motion blur.

O.k. here we go:

1. In the  drawer 'ARexx' of the RayStorm directory there is a file named

'default.ray'. This is a default form for RayStorm ARexx scripts. You can

use this form to write your own scripts.

We will use this file as a default for our animation script. Copy this file

to the file 'bounce.ray'. After this load the file 'bounce.ray' to your

favorite text editor (e.g GoldEd or CygnusEd).

2. First we define some values: the acceleration of the ball and the amount

of frames to generate.

RayStorm has three commands to set the paths where it searchs the files

it needs. We use a brush for the surface of the ball and a texture for

the surface of the ground.

To do this we have to insert after the command 'ADDRESS RAYSTORM' the

lines:

0m3m   g = .2

frames = 17

'3mBRUSHPATH0m /brushes'

'3mTEXTUREPATH0m /textures'0m

It's the same if you write the commands in upper case or lower case.

But it's important to enclose all commans in quotes because ARexx tries

to interpret the line before it sends it. It may happen that the line is

changed and RayStorm don't do the things you want.

3. Next we set the screen resolution. For the first experiments we choose

a low resolution of 160x128 pixels. Insert the line:

0m3m   '3mSETSCREEN0m 160 128'0m

4. Now we set the camera parameters. The first three values determine the

position of the camera. We want to place it so that we can see the ball

all over the time. The next values set the viewpoint of the camera, this

is the point the camera aims to. The next values determine the view up

vector. And the last two values determine the field of view. To get a

pixel aspect of 1:1 we have to set them to 25 and 20 degree.

0m3m   '3mSETCAMERA0m <0,10,40> <0,5,0> <0,1,0> 25 20'0m

5. We want to have a bright blue background for our animation. The

background and the global ambient color is set with the 'SETWORLD'

command. We want to set the ambient color to a dark gray, if this color

is to bright the scene will look washed out and the objects appear flat.

Insert the line:

0m   '3mSETWORLD0m [10,30,200] [10,10,10]'0m

6. The illumination is an important part of a scene. We want to place a

pointlight near the camera. Add the line:

0m   '3mPOINTLIGHT0m <5,10,50>'0m3m

7. Now we define the actor for the plane. We want to rotate it around the

Z-axis. Insert the lines:

0m3m   '3mNEWACTOR0m groundactor'

'3mALIGNMENT0m 0 ' frames+2 ' <0,0,360>'0m

7. Now we define the surface for the plane and the plane itself. We make it

a little reflective and apply a checker texture. The surface 'groundtop'

is for the top of the plane and the surface 'groundbottom' is for the

bottom of the surface. The plane itself consits of four triangles. Two

fo the top and two for the bottom. Insert the lines:

0m3m   '3mNEWSURFACE0m groundtop'

'3mDIFFUSE0m [255,255,255]'

'3mSPECULAR0m [0,0,0]'

'3mREFLECT0m [50,50,50]'

'3mIMTEXTURE0m checker.itx <0,-1,0> <0,0,0> <10,10,10> ACTOR groundactor'

'3mNEWSURFACE0m groundbottom'

'3mDIFFUSE0m [255,0,0]'

'3mSPECULAR0m [0,0,0]'

'3mREFLECT0m [50,50,50]'

'3mIMTEXTURE0m checker.itx <0,-1,0> <0,0,0> <1.5,1.5,1.5> ACTOR groundactor'

'3mTRIANGLE0m groundtop <-2,0,-2> <2,0,-2> <2,0,2> ACTOR groundactor'

'3mTRIANGLE0m groundtop <-2,0,-2> <-2,0,2> <2,0,2> ACTOR groundactor'

'3mTRIANGLE0m groundbottom <-2,-.01,-2> <2,-.01,-2> <2,-.01,2> ACTOR groundactor'

'3mTRIANGLE0m groundbottom <-2,-.01,-2> <-2,-.01,2> <2,-.01,2> ACTOR groundactor'0m

8. Next we define the motion of the ball. It starts at a height of 10 and

accelerates until it bounces on the plane, changes it's direction and

the motions ends as the ball is back at he start point. Additional the

ball rotates around the Y-axis. Add the following sequence to your script:

0m3m   speed = -g

pos = 10

'3mNEWACTOR0m ballactor <0,'pos',0>'

do i=0 to frames

'3mPOSITION0m ' i i+1 '<0,'pos',0>'

pos = pos+speed

if pos<=1 & speed<0 then

speed = -speed

else

speed = speed-g

end

'3mALIGNMENT0m 0 ' frames+2 ' <0,360,0>'0m

9. Now we define the surface for the ball and the ball itself. The only thing

we must do is to map a earth styled brush map to a sphere. To reach this

goal the position of the brush must be set to the middle of the sphere

and the size must be small enough to be completely inside the sphere.

This are the lines to define the ball:

0m3m   '3mNEWSURFACE0m ball'

'3mBRUSH0m earth.iff COLOR WRAPXY <0,10,0> <0,0,0> <.1,.1,.1> ACTOR ballactor'

'3mSPHERE0m ball <0,10,0> 1 ACTOR ballactor'0m

10. If your computer is fast enough you can insert the follwing lines:

0m3m   '3mANTIALIAS0m 2'

'3mDISTRIB0m 2'0m

'ANTIALIAS' improves the quality of the picture; 2 or 3 are normal values,

higher values don't improve the quality significant.

A value higher than one for 'DISTRIB' switches {"motion blur" link Motion Blur} on.

11. At this the we have finished the definitions and now can render the single

frames. If youn want the reflections of the ball on the plane you have

to delete the keyword 'QUICK', because RayStorm renders no reflections in

quick mode. The frame time is set with 'FROM' and 'TO'. We save the frames

as IFF-ILBM pictures with the names 'bounce0001.iff' ... 'bounceXXXX.iff'.

The last step is to free all the memory with the command 'CLEANUP'. Add

these lines:

0m3m   do i=0 to frames

'3mSTARTRENDER0m QUICK FROM 'i' TO 'i+1

'3mSAVEPIC0m bounce' || RIGHT(i,4,0) || '.iff'

end

'3mCLEANUP0m'0m

12. Start the script from a shell-window with the sequence 'rx bounce.ray'.

RayStorm will now generate your frames. When RayStorm finished the work

you must glue the pictures together to get the animation.

That's all. Have fun!

0m1mTutorial: Motion blur0m

In this tutorial we show you how to make animations and how to use motion

blur.

To animate objects we need ACTORs. An ACTOR can be seen as a virtual object

which can have a certain position/alignment/size at a certain time. ACTORs can

be used for keyframe animation by giving control points which RayStorm can

interpolate (only linear for now.  Spline interpolations will be implemented

later).

You can assign an ACTOR to one or more real objects. An object with an actor

assigned to it will follow all actions the actor does. Let's take the sphere

from the second tutorial and move it to direction of the camera.

Type following:

0m3m'3mSETCAMERA0m <6,1.5,-1.5> <0,0,0><0,1,0>'

'3mPOINTLIGHT0m <30,50,30> [255,255,255] SHADOW'0m

you can add POSITION, ALIGNMENT and SIZE commands after NEWACTOR

(similar to the surface commands after NEWSURFACE)

0m3m'3mNEWACTOR0m actor'

'3mPOSITION0m 0 1 <3,0,0.4>'

'3mNEWSURFACE0m planesurf'

'3mDIFFUSE0m <255,30,30>'

'3mIMTEXTURE0m "checker.itx" <0.1,0.1,0.1> <0,0,0> <2,2,2> 155 25 0'

'3mNEWSURFACE0m spheresurf'

'3mREFLECT0m <255,255,255>'

'3mSPHERE0m spheresurf <0,1,0> 0.7 actor'

'3mPLANE0m planesurf'0m

You can make an animation by a series of STARTRENDER/SAVEPIC combinations. Our

little movie shall consist of 6 frames, so we will subdivide our time intervall

(1 unit) and make a photo every 0.2 time units. The STARTRENDER command has the

option to render a picture within a certain time intervall. That can be

compared with the shutter time of a real camera. The camera of RayStorm records

all movements of the objects in the scene within that time intervall which

results in a blurred scene. At the beginning we do no motion blur, so we set

the start and the end time to the same value. (Which means no shutter time).

Add this code and execute it:

0m3mdo i=0 to 1 step 0.2

'3mSTARTRENDER0m QUICK FROM 'i' TO 'i

'3mSAVEPIC0m sphere' || RIGHT(i*5,4,0) || '.iff'

end

'3mCLEANUP0m'0m

You can make a movie out of it using an animator program such as MainActor.

Now we introduce one of the advanced features of RayStorm: Motion Blur

Replace the commands above by the following lines:

0m3m/* when doing motion blur you *MUST* add the distrib command +/

'3mDISTRIB0m 3'

/* shutter time is 0.2 */

do i=0 to 0.8 step 0.2

'3mSTARTRENDER0m QUICK FROM 'i' TO 'i+0.2

'3mSAVEPIC0m sphere' || RIGHT(i*5,4,0) || '.iff'

end

'3mCLEANUP0m'0m

For each frame, the camera opens the shutter 0.2 time units long and records

what's happening. As you can see, an animation with motion blur gives a better

visual effect.

Again feel free to change the parameters.

0m1mTEXTURES0m

Textures are mathematically generated patterns which can be applied to the

surface of an object.

There are several textures in the directory 'textures'.

3m Bump    0m

3m Checker 0m

3m Linear  0m

3m Marble  0m

3m Radial  0m

3m Stars   0m

3m Wood    0m

0m1mBUMP0m

This texture applies bumps to the surface.

Size of texture determines size of the bumps.

0m1mParameters:0m

1: X bump size

2: Y bump size

3: Z bump size

set the 'depth' of the bumps.

Example:

sphere with radius 1

0m3mIMTEXTURE bump.itx <0,0,0> <0,0,0> <.3,.3,.3> 1 1 10m

3mPicture0m

0m1mCHECKER0m

This texture applies thw ell known checker pattern to the surface.

0m1mAttention!

If you apply a checker texture to a plane, the plane may not be at the same

position on which the checker changes its color. Otherwise you get a noisy

texture due to rounding errors.0m

0m1mParameters:0m

1: Color Red

2: Color Green

3: Color Blue

Color of the checkers, other color is taken from object.

4: Reflect Red

5: Reflect Green

6: Reflect Blue

Reflect color of the checkers.

7: Filter Red

8: Filter Green

9: Filter Blue

Filter color of the checkers.

0m1mExample:0m

0m3mIMTEXTURE "checker.itx" <0,0.1,0> <0,0,0> <2,2,2> 255 0 00m

3mPicture0m

0m1mLINEAR0m

This texture varies the color of the object in the y-direction of the

texture.

0m1mParameters:0m

1: Color Red

2: Color Green

3: Color Blue

color to interpolate to.

4: Reflect Red

5: Reflect Green

6: Reflect Blue

reflection to interpolate to.

7: Filter Red

8: Filter Green

9: Filter Blue

filter to interpolate to.

0m1mExample:0m

0m3mIMTEXTURE "linear.itx" <0,0.1,0> <0,0,0> <2,2,2> 0 0 2550m

3mPicture0m

0m1mWOOD0m

This texture applies a wood like texture to the surface.

Size of texture determines size of wood.

0m1mParameters:0m

1: Color Red

2: Color Green

3: Color Blue

Color. Other color is taken from object.

4: Reflect Red

5: Reflect Green

6: Reflect Blue

Reflection color.

7: Filter Red

8: Filter Green

9: Filter Blue

Filter color.

10: Octave

The higher the octave the noisier are the wood rings.

11: Frequency

The higher the frequency the smaller the wood rings.

0m1mExample:0m

cube with size 2

0m3mIMTEXTURE wood.itx <0,0,0> <0,0,0> <1,1,1> 255 255 50 0 0 0 0 0 0 2 40m

3mPicture0m

0m1mMARBLE0m

This texture applies a marble like texture to the surface.

Size of texture determines size of bumps.

0m1mParameters:0m

1: Color Red

2: Color Green

3: Color Blue

Color. Other color is taken from object.

4: Reflect Red

5: Reflect Green

6: Reflect Blue

Reflection color.

7: Filter Red

8: Filter Green

9: Filter Blue

Filter color.

10: Octave

The higher the octave the noisier is the texture.

0m1mExample:0m

cube with size 2

0m3mIMTEXTURE marble.itx <0,0,0> <0,0,0> <.5,.5,.5> 150 50 50 0 0 0 0 0 0 70m

3mPicture0m

0m1mRADIAL0m

This texture varies the color of the object radial around the texture axis.

0m1mParameters:0m

1: Start radius

Interploation start radius.

2: End radius

Interploation end radius.

3: Color Red

4: Color Green

5: Color Blue

Color to interpolate to.

6: Reflect Red

7: Reflect Green

8: Reflect Blue

Reflection to interpolate to.

9: Filter Red

10: Filter Green

11: Filter Blue

Filter to interpolate to.

0m1mExample:0m

0m3mIMTEXTURE "radial.itx" <0,0,0> <0,0,0> <1,1,1> 1 2 255 0 00m

3mPicture0m

0m1mSTARS0m

This texture applies randomly stars to the surface. Cannot be used in

animations!

0m1mParameters:0m

1: Color Red

2: Color Green

3: Color Blue

Color of the stars.

4: Density

Star density. The higher the more stars (0. - 1.).

0m1mExample:0m

0m3mIMTEXTURE "stars.itx" <0,0,0> <0,0,0> <1,1,1> 255 255 255 0.10m

3mPicture0m

0m1mKNOWN BUGS0m

0m1mDISCLAIMER0m

THERE IS NO WARRANTY FOR THIS PROGRAM TO THE EXTENT PERMITTED BY APPLICABLE

LAW.  EXCEPT WHERE OTHERWISE STATED IN WRITING THE COPYRIGHT HOLDER AND/OR

OTHER PARTIES PROVIDE THE PROGRAM "AS IS" WITHOUT WARRANTY OF ANY KIND,

EITHER EXPRESSED OR IMPLIED, INCLUDING, BUT NOT LIMITED TO, THE IMPLIED

WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE.  THE

ENTIRE RISK AS TO THE QUALITY AND PERFORMANCE OF THE PROGRAM IS WITH YOU.

SHOULD THE PROGRAM PROVE DEFECTIVE, YOU ASSUME THE COST OF ALL NECESSARY

SERVICING, REPAIR OR CORRECTION.

IN NO EVENT UNLESS REQUIRED BY APPLICABLE LAW OR AGREED TO IN WRITING WILL

ANY COPYRIGHT HOLDER, OR ANY OTHER PARTY WHO MAY REDISTRIBUTE THE PROGRAM

AS PERMITTED ABOVE, BE LIABLE TO YOU FOR DAMAGES, INCLUDING ANY GENERAL,

SPECIAL, INCIDENTAL OR CONSEQUENTIAL DAMAGES ARISING OUT OF THE USE OR

INABILITY TO USE THE PROGRAM (INCLUDING BUT NOT LIMITED TO LOSS OF DATA OR

DATA BEING RENDERED INACCURATE OR LOSSES SUSTAINED BY YOU OR THIRD PARTIES

OR A FAILURE OF THE PROGRAM TO OPERATE WITH ANY OTHER PROGRAMS), EVEN IF

SUCH HOLDER OR OTHER PARTY HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH

DAMAGES.

0m1mCOPYRIGHT0m

The RayStorm package is Copyright 1995,96 by Andreas Heumann and Mike Hesser.

All Rights Reserved. It is released under the concept of `Shareware'.

The archive of RayStorm may only be distributed in unmodified form.  No files

may be added, changed or removed. You may not charge for this archive, other

than the cost of the media and duplication fees. Distribution is allowed in

all forms, such as BB systems, floppy or compact disks, and ftp sites.

0m1mCREDITS0m

We want to thank the following persons:

- Bernhard Moench - chairman of Plasma Pictures (a great Amiga club)

Address:

Plasma Pictures

Reality and Imagination!

Bernhard Moench

Regensburger Strasse 28c

10777 Berlin

GERMANY

- Stephan Dorenkamp - for testing

- Maan Hamze - for testing, hundrets of suggestions and bugreports...

... and many many E-Mails

- Klaus Melchior for his listtree class

- Stefan Stunz for MUI

- Thanks to all registrated users for the support of the shareware concept.

0m1mREGISTER0m

If you like RayStorm use the registration programm to register.

Fill out the registration form and press the 'Print' button.

If the printer is installed correctly, the registration is printed out.

You can get information about the current agreements by pressing the 'Info'

button.

0m1mAUTHORS0m

For bug reports, comments, suggestions ... you can contact us at the

following addresses (E-mail prefered).

Amiga-version:

Andreas Heumann

E-mail:  calvin@sol.wohnheim.uni-ulm.de (or heumann@hugo.rz.fh-ulm.de)

S-mail:  Heilmeyersteige 105

89075 Ulm

Germany

PC-version:

Mike Hesser

E-mail:  calvin@sol.wohnheim.uni-ulm.de

S-mail:  Heilmeyersteige 105

89075 Ulm

Germany

0m1mPC-VERSION0m

The PC version is available on the Internet from our 3mhomepage0m.

0m1mHISTORY0m

3mRayStorm Scenario 0m

3mRayStorm Script   0m

3mRayStorm Raytracer0m

0m1mHISTORY0m

version 1.0b (27-August-96)

- initial release.

version 1.1 (04-October-96)

- save and load materials in attribute requester

- create cube, sphere, plane, tube, cone and torus mesh objects

- fixed bug with color requesters (colors changed every call)

- string gadgets now advance to next/prev gadget when return is pressed

- improved zooming with the mouse

- added enable button for render area

- fixed problems with KS-versions less than 3.0 (<39)

- key 't' opens settings requester

- added new preview resolutions (NTSC 80x50,PAL 80x64,VGA 80x60)

- if more than one object is selected the object for attributes can be

selected by the browser

- implemented two browser modes

- added center view with 'R-Amiga - .'

- joined settings and transform requester

- several bugfixes

- improved object scaling with mouse

- can now scale external objects

version 1.2 (29-November-96)

- forgot to scale child objects of external objects

- added ok button to render requester

- added 3mcamera to viewer0m

- added hierarchical object browser

- ask for rendering of scenes with no light sources

- bugfix: it was impossible to track and to set the opening angle of a

spotlight

- convert external to scene objects

- added RayStorm object file format (RSOB)

- fixed problem with vertical stripes with 020-version

- fixed problem with screen update

- coordinate display even if no object is translated

- focal distance of camera can automatically be set to camera-track distance

- added preview to attributes requester

- improved color selection for rendering preview window

- added show and save button to registration program

- added some materials

- added lens effects (flares, light star)

0m1mHISTORY0m

version 1.02 (16-August-95)

- added BRIGHT-flags for surface

- deleted TRANSATTU

version 1.081 (08-October-95)

- added listview for history

version 1.13 (01-November-95)

- now more than one path with PATH-commands possible

version 1.16 (09-January-96)

- added 'SOFTSHADOW' to 'DISTRIB'

- added 'FALLOFF' for 'POINTLIGHT' and 'SPOTLIGHT'

version 1.2 (29-February-96)

- bugfix 'ANTIALIAS' command: parameter CONTRIB produced error 30

version 1.22 (25-April-96)

- added 'SURFACE' to 'LOADOBJ' to replace objects surface with own surface

version 1.26 (04-October-96)

- fixed a bug which caused the program to crash on exit

version 1.3 (29-November-96)

- added 'FLARE' and 'STAR'

0m1mHISTORY0m

version 1.0 (09-July-95)

- initial release.

version 1.01 (15-August-95)

- added soft shadows

- added random jitter

- added brush repeat and mirror

version 1.02 (16-August-95)

- bugfix: altitude mapping -> black object: fixed

- bugfix: loading of TDDD-objects with brushes crashed: fixed

- added backdrop picture

- added BRIGHT-flags for surface

- added fog

version 1.03 (17-August-95)

- bugfix: sphere intersection test: fixed

version 1.04 (21-August-95)

- added global fog

version 1.05 (28-August-95)

- added animation commands

version 1.06 (01-September-95)

- added motion blur

version 1.07 (10-September-95)

- added specular brush mapping

version 1.08 (11-September-95)

- added rendering box

version 1.081 (08-October-95)

- added global reflection map

- changed error messages

version 1.082 (11-October-95)

- improved memory management for Imagine objects

version 1.083 (12-October-95)

- changed spotlight direction to lookpoint and added actor for lookpoint

- new form for vectors '<x,y,z>'

- new form for colors '[r,g,b]'

version 1.1 (18-October-95)

- next offical release

version 1.11 (19-October-95)

- bugfix: Imagine fog objects are now loaded properly

- added parameter check for field rendering

version 1.12 (21-October-95)

- speedup of motion blur

version 1.13 (01-November-95)

- bugfix: spotlight look point changed camera view point

- added soft interpolation of colors for brushmapping

- bugfix: objects behind light sources casted shadows

version 1.14 (03-November-95)

- changed default gaussian filter width from 1.8 to 1.3

- bugfix: problem with global fog

- plane can now be animated

- changed axis position in flat brush mapping

- added 'Time spend' and 'Time left'

version 1.15 (28-November-95)

- added PNG- and ILBM-modules

- added radial texture

version 1.16 (09-January-96)

- bugfix in PNG-module: had problems with palette pictures

- bugfix motion blur: had a problem with voxel calculations

- bugfix motion blur: had no motion blur in scenes with planes

- added TGA-module

- added 'SOFTSHADOW' to 'DISTRIB'

- optimized octree (up to 10% faster)

- added 'FALLOFF' for 'POINTLIGHT' and 'SPOTLIGHT'

- bugfix can now load plane

- bugfix can now load multiple planes or perfect spheres

version 1.17 (18-February-96)

- object loading is now done in modules

- added 3DS module

- bugfix in TDDD-module: hardness is now used to set REFEXP

- brushes and textures are no applied to surfaces which lie

between the light source and the illuminated surface

- bugfix antialiasing: there where some bright pixel trash in the picture

if antialiasing with fog was used

- rewrote bump, wood and marble textures

- global reflections map is now applied in 'QUICK' mode too

version 1.2 (29-February-96)

- bugfix: last line of picture has been always black

- bugfix: had problems with Imagine 4.0 TDDD objects

version 1.21 (25-March-96)

- TDDD objects are now rotated and scaled relative to axis of first object

- bugfix: since 1.2 shadows in motion blur scenes were calculated false

- bugfix: in some cases there were vertical and horizontal stripes in

rendered pictures

- bugfix: triangles which were exactly in one plane (XY,XZ,YZ) disapeared

version 1.22 (25-April-96)

- bugfix: had problems with sharp edges of TDDD objects

- bugfix: altitude brush mapping produced ugly results

- bugfix: fixed some motion blur bugs

- added 'SURFACE' to 'LOADOBJ' to replace objects surface with own surface

- added Datatype support for all commands which load brushes

version 1.25 (08-September-96)

- the RayStorm kernel is now a shared library

- added support of new TDDD TXT4 chunk for textures from Imagine 4.0

- changed the world axis orientation (x left, y up, z in)

version 1.26 (04-October-96)

- fixed error in marble texture

- bugfix: had problems with sharp edges of TDDD objects (once again)

version 1.3 (29-November-96)

- recompiled raystorm.library with StormC -> upto 30% faster (68030/881)

- backdrop pictures are now automatically scaled

- added lens effects (flare, light star)

0m1mHOMEPAGE0m

Come and visit our RayStorm-Homepage! There you can always get the latest

version of RayStorm and can see some example pictures.

The address:

http://sol.wohnheim.uni-ulm.de/~calvin/raystorm.html

0m1mFUTURE ADDITIONS0m

0m1mRayStorm Scenario0m

- modify triangle objects

- animation control

- Arexx port

- undo

- solid display (z-Buffer) (why is there no useable (fast) version of OpenGL for

Amiga available?)

- use Imagine staging files

0m1mRayStorm Raytracer0m

- more objects (torus, cylinder, ...)

- JPEG-saver

- animation language (ALAN)

- shadow caching

- more textures

- don't allocate whole picture buffer at once

- diffuse reflectivity

- diffuse transparency

- better light FX (lens flares)

- log file

- apply post-2D-FX

- spline interpolation for actors

- load Lightwave format

- CSG (Constructive Solid Geometry)

- metaballs

- some extra programs (e.g. terrain and plant generation)

0m1mAdvanced Animation and Rendering Techniques:

Theory and Practice0m

by

Alan Watt, Mark Watt, Addison-Wesley

0m1mTexturing and Modeling:

A procedural Approach0m

by

Ebert, Musgrave, Peachy, Perlin, Worley

0m1m-A-0m

3mAbout           0m

3mActive          0m

3mALIGNMENT       0m

3mAltitude brush  0m

3mAMBIENT         0m

3mANTIALIAS       0m

3mAntialiasing    0m

3mARexx           0m

3mAttributes      0m

3mAuthor          0m

0m1m-B-0m

3mBackdrop        0m

3mBrowse          0m

3mBrushes         0m

3m - show         0m

3mBRUSH           0m

3mBRUSHPATH       0m

3mBugs            0m

3mBump            0m

3mButtons         0m

0m1m-C-0m

3mCamera          0m

3m - Active       0m

3m - Settings     0m

3m - to viewer    0m

3mChecker         0m

3mCLEANUP         0m

3mCLEANUP         0m

3mColor brush     0m

3mCommands        0m

3mCoordinates     0m

3mCopy            0m

3mCopyright       0m

3mCredits         0m

3mCut             0m

3mCylinder mapping0m

0m1m-D-0m

3mDelete          0m

3mDepth of field  0m

3mDeselect All    0m

3mDIFFTRANS       0m

3mDIFFUSE         0m

3mDISPLAY         0m

3mDISTRIB         0m

0m1m-E-0m

3mErrors          0m

3mExamples        0m

0m1m-F-0m

3mFeatures        0m

3mFilter brush    0m

3mFLARE           0m

3mFlat mapping    0m

3mFocus           0m

3mFog             0m

3mFOGLEN          0m

3mFuture          0m

0m1m-G-0m

3mGeneral         0m

3mGETERRORSTR     0m

3mGlobal          0m

3mGrid            0m

3m - Size         0m

3m - Snap to      0m

0m1m-H-0m

3mHistory         0m

3mHomepage        0m

0m1m-I-0m

3mIMTEXTURE       0m

3mInternals       0m

3mInstallation    0m

3mIntroduction    0m

3m - Raytracer    0m

3m - Scenario     0m

3m - Script       0m

0m1m-L-0m

3mLegal Stuff     0m

3mLens flares     0m

3mLimitations     0m

3mLinear          0m

3mLoad            0m

3mLOADOBJ         0m

3mPointLight Set. 0m

3mSpotLight Set.  0m

0m1m-M-0m

3mMarble          0m

3mMenus           0m

3mMesh            0m

3m - Axis         0m

3m - Cube         0m

3m - Sphere       0m

3m - Plane        0m

3m - Tube         0m

3m - Cone         0m

3m - Torus        0m

3mMesh Settings   0m

3mMode            0m

3mMotion Blur     0m

3mMUI Prefs       0m

0m1m-N-0m

3mNew             0m

3mNEWACTOR        0m

3mNEWSURFACE      0m

0m1m-O-0m

3mOBJECTPATH      0m

3mOpen            0m

0m1m-P-0m

3mParameters      0m

3mPaste           0m

3mPC-Version      0m

3mPlane           0m

3mPLANE           0m

3mPointlight      0m

3m - Settings     0m

3mPOINTLIGHT      0m

3mPOSITION        0m

3mPrefs           0m

3mPrefs Load      0m

3mPrefs Save      0m

0m1m-Q-0m

3mQuit            0m

3mQUIT            0m

0m1m-R-0m

3mRadial          0m

3mRedraw          0m

3mREFEXP          0m

3mREFLECT         0m

3mReflect brush   0m

3mReflection map  0m

3mREFRINDEX       0m

3mRegister        0m

3mRender          0m

3m - Settings     0m

3mRequirements    0m

0m1m-S-0m

3mSave            0m

3mSave As         0m

3mSAVEPIC         0m

3mSelect All      0m

3mSelect Next     0m

3mSelect Previous 0m

3mSettings        0m

3mSETCAMERA       0m

3mSETSCREEN       0m

3mSETWORLD        0m

3mSIZE            0m

3mShow            0m

3m - Coordinates  0m

3m - Names        0m

3m - Brushes      0m

3m - Textures     0m

3mSoft shadows    0m

3mSPECTRANS       0m

3mSPECULAR        0m

3mSpecular brush  0m

3mSphere          0m

3mSPHERE          0m

3mSphere mapping  0m

3mSpotlight       0m

3m - Settings     0m

3mSPOTLIGHT       0m

3mSTAR            0m

3mStars           0m

3mSTARTRENDER     0m

3mSurface         0m

0m1m-T-0m

3mTEXTUREPATH     0m

3mTextures        0m

3m - description  0m

3m - show         0m

3mTips & Tricks   0m

3mTRANSEXP        0m

3mTransform       0m

3mTRANSLUC        0m

3mTRANSPAR        0m

3mTRIANGLE        0m

3mTutorials       0m

0m1m-U-0m

3mUsage           0m

0m1m-V-0m

3mView            0m

3mVirtual Memory  0m

3mVisible Light   0m

0m1m-W-0m

3mWINTOFRONT      0m

3mWood            0m

3mWWW             0m

0m1m-Z-0m

3mZoom fit        0m

3mZoom in         0m

3mZoom out        0m