TMUF

TMUF was originally written in response to a  request  from  VS6DX

for  a  IBM  PC  program  that  would replace the commercial APPLE

program  he  had  been  using for predicting HF propagation. There

seemed to be a noticeable lack of Ham Radio Programs for  the  IBM

PC,  and no graphical MUF plotting program could be located. Hence

this program was written. Version 1.00 of TMUF has  been  finished

and  in  use since February 1986. Due to requests of many hams who

have seen it in operation, I am Placing it in Public  Domain,  and

encouraging  its  FREE distribution. TMUF may be shared with other

people under one condition;  that  there  is  NO  CHARGE  for  the

copy(s).

There is obviously much room for improvement of the Program, and I

have since developed better ways of doing many of the functions in

TMUF. However, as the program fills my needs at the moment, I have

little incentive to rewrite it.

TMUF  is  a  User Supported program. I am asking for a donation of

$20.00 for its use. If there is enough interest and support shown,

I will be updating and improving it. Otherwise,  it  does  what  I

need and will remain as is. Donations should be sent to:

James W. Harrison W4HSR

1904 Duke Drive

Richardson, Texas 75081

DESCRIPTION

This  is  TMUF,  version  1.00, placed in the Public Domain by the

Author, W4HSR. It is a Turbo Pascal  version  of  MINIMUF  3.5  as

Described  in  the  December  1982  QST  article.  In  addition to

calculating and displaying the Maximum Usable Frequency (MUF)  and

the  Optimum  Working  Frequency  (FOT),  it permits you to easily

choose and change the Date and Flux value for the Calculation,  to

select the Transmit and receive Stations from a database which you

can control and customize,  and  to  plot  the  calculated  values

graphically  if  you  have  a  color graphics adapter. You may add

stations  to  the  supplied database using the program, or you may

use any ASCII Text Editor to add to, modify, or rearrange the data

base.

TMUF was written in Turbo Pascal for an IBM PC or close compatible

with 256 K or more of RAM.  If  equipped  with  a  Color  Graphics

adapter, you will be able to display the calculated values  on  an

X-Y graph very similar to those published monthly in QST.  If  you

have  only  the monochrome adapter don't despair, you will instead

receive  a Table of values when requesting a graph. The Program is

able to determine if you have graphics compatible hardware. Credit

is  hereby  given  to  to  the following persons who have provided

previous  programs,  source  code, and the testing upon which TMUF

has been built.

Robert B. Rose, K6GKU for the original MINIMUF

program in BASIC published  in  QST  magazine,

December 1982.

Dennis    Allen,   WA5LXS  who  converted  the

original MINIMUF to  IBM  BASIC  and  included

the  conversion  algorithm for Flux to Sunspot

Number.

J.  Scott  Douglass, K2SD for his BEAMHEAD.BAS

which  formed  the  basis  of  the  Data  File

included with TMUF.

Ed  Nance, VS6DX and the Ham community in Hong

Kong which tested the preliminary  version  of

TMUF and offered constructive comments.

Ed  Bachmann for his EBMENU.PAS which was used

to implement the Menus for TMUF.

Donald  R.  Ramsey  and Larry Romero for their

TURBO-UT.PAS which was used as  the  basis  of

the formatted inputs.

If  desired,  the program may be initiated with up to two optional

parameters on  the  command  line  to  automatically  set  desired

selections. This will bypass the introductory screen and result in

an automatic calculation/plot. The  first  parameter  is  normally

solar  flux  value  and  the second is the desired receive Station

Call Letters or Prefix. Either or both of these parameters may  be

specified by typing them on the command line immediately after the

program name separated by a space. If no parameters are specified,

the  program  will  default  to  a  solar  Flux of 80 and with the

Receive  Station set to the second record in the Data base. If you

wish  to  specify  the  receive  station without specifying a flux

value you may do so. The Transmit Station will always  be  set  to

the first entry in the Data Base.

If  either  of  the two parameters are specified, the program will

automatically go into the Plot mode upon entry, and  will  shortly

after  display the graph of the specified data. If at any time you

specify or select a station which is not in  the  data  base,  you

will be so informed, and may make another selection.

For  anyone  who may not know how to obtain the value of the Solar

Flux, it is broadcast by the National Bureau  of  Standards  radio

station  WWV  at 18 minutes after each hour. WWV broadcasts in the

high frequency bands may be received on 2.5, 5.0, 10.0,  15.0  and

20.0 mHz.

The program features a Lotus style menu for  all  selections.  You

may make your selections by typing the first letter of the desired

selection, or by moving with the cursor keys or space bar  to  the

desired selections and pressing Return. The Menu selections are:

Plot           Plots  the  MUF/FOT  of the current settings if you

have a color graphics adapter. If not a table  will

be displayed as described below.

Table          Displays  a  table  of  the  MUF/FOT of the current

settings. All information available on the Plot  is

also  displayed in the table. For some applications

the table may be preferable, and the table does not

require the color graphics adapter.

Add_a_Station  Permits  adding  new  Stations  to  the  Data base.

Stations added in this manner are appended  to  the

end of the data base file. You will  be  asked  for

the  Call or Prefix of the station, the city, state

or  country,  and the latitude and longitude. Refer

to the  description  of  the  data  file  for  more

information.

Date           Permits  changing  the Date of the Calculation. The

default date is the DOS date as read by the program

upon  entry.  You  may change the date to any valid

date.  TMUF will not accept invalid dates. The year

portion  is  used only in checking validity of Leap

years  and  has  no  bearing  on  the  actual   MUF

calculations!  The  primary  use  of the date is to

calculate  the  solar zenith angle at the mid-point

of  the  path.  This has a very important effect on

the MUF as you will find  if  you  experiment  with

various dates. You will find a marked difference in

the behavior of the  path  during  the  winter  and

summer periods.

Flux           Permits changing the value of the solar flux. Solar

Flux  may  range  between  65  and  245.  This   is

converted  to Smoothed International Sunspot Number

in the TMUF program. Flux is  used  instead  of

sunspot number as the input unit due to the readily

available flux data from WWV.

Select         Allows  you  to  Select a new station from the data

base as either the Transmit or the Receive Station.

You  will  first  be  asked for the Station Call or

Prefix. You may type over the data in the field, or

you  may clear the field by pressing the <ESC> key.

Your entry need  not  be  complete.  If  you  can't

remember the complete call or prefix, enter as much

of  the leading part as you are sure of. The select

function is not case sensitive, so you  don't  have

to  worry  about  whether you are in upper or lower

case  mode.  Partial  matches  are   allowed,   but

remember that the program  will  select  the  FIRST

entry which matches your input.  After  entering  a

value  in the Call field, press return and you will

be asked to enter  either  a  R  to  designate  the

Selected  call  as  the  Receive station, or a T to

designate it as the Transmit station.

End           Ends the Program and returns you to DOS

The  Data  Base  itself  is  a  simple sequential file with fields

delimited by  commas  in  the  manner  of  a  BASIC  program  DATA

statement.  In  Fact  this  file  was  generated  from  BASIC DATA

statements included in the Public Domain Beam Heading  Calculation

Program BEAMHEAD.BAS. The fields are as follows:

Field 1:  Call Sign or Prefix - Free form Upper Case 20 characters

Maximum.

Field 2:  City                - Free form Upper Case 20 characters

Maximum.

Field 3:  State or Country    - Free Form Upper Case 20 characters

Maximum.

Field 4:  Latitude            - Numbers  and sign only, entered as

a decimal number,  North  Latitude

is  positive,  South  Latitude  is

negative, nine characters Maximum,

Absolute Value must not exceed 90,

Plus Sign not Required.

Field 5:  Longitude           - Numbers  and sign only, entered as

a decimal number,  West  Longitude

is  positive,  East  Longitude  is

negative, nine characters Maximum,

Absolute  Value  must  not  exceed

180, Plus Sign not Required.

Being  a  sequential  text  file, the Data Base may be Edited with

almost  any  ASCII Text Editor. The order of entry of the majority

of the file as furnished  here  is  alphabetical.  This  is  quite

arbitrary and may be changed by you if you desire. Only two of the

entries are important as to their order, and these are  the  first

and second. The program will always use the  first  entry  in  the

Data File for the default  transmit  station.  Hence,  you  should

place  your  own station as the first entry in the Data File. Also

the program will use the second entry in  the  Data  File  as  the

default  receive  station unless it receives contrary instructions

via the command line. Therefore the second entry in the Data  File

should  be selected as the station of your primary interest. Other

entries may be in any order, but as  the  search  for  a  call  is

sequential, you might want to move  the  stations  you  frequently

plot  to  the first part of the file, immediately after the second

entry (Receive station). This will speed up the selection process.

Duplicate  Entries  are  allowed,  and  you  will find many in the

supplied data base, but due to the Sequential search method,  only

the  First occurrence can be selected. Also, since partial matches

are allowed in the selection portion of  the  program,  the  first

entry  matching  the partial entry will be selected, i.e., if both

UA1 and UA9 are in the data file and you select Ua, the  first  UA

entry  will  be  chosen. Note that selection in the program is not

case sensitive.

The mathematics of the MUF calculation used in this program are as

nearly as possible exactly the same as the BASIC program described

in  the  QST  article. There's a whole lot of Trigonometric number

crunching going on, for each hop, for each hour of the  selection,

so  be patient. Actually it's not too bad, requiring at most about

30 seconds to calculate a new 2 hop path (All paths are treated as

either one or two hops). If you wish to  compare  the  results  of

TMUF with the original MINIMUF, the two stations used in  the  QST

article example are included as TEST1 & TEST2 in the data Base. Be

sure  and  set  the  Flux  to  156  and the date to 17 Oct for the

comparison.  As supplied, TEST1 and TEST2 are the first and second

entries, and will default to the transmit and receive stations.

The  calculated  data  including  the  selected  Parameters,  Path

Distance, Beam Heading from the  Transmit  Station,  and  computed

number of Hops are displayed in either Graphical or Tabular format

by choosing PLOT or TABLE from the Menu. If no changes  have  been

made  to  Station,  Date,  or Flux since the last calculation, the

Display is almost Instantaneous, as the data does not have  to  be

recalculated.

If you wish to print the Graphical  Output  of  TMUF,  you  should

issue the DOS command GRAPHICS before  running  the  Program,  and

then  use  the  Print Screen Function (depress the SHIFT and PrtSc

keys  simultaneously)  to produce the printout. Note that the call

to GRAPHICS is is included in the MUF.BAT file.

The TMUF program includes the following Files:

TMUF.COM       This is the Actual Program.

TMUF.DAT       This is the Data Base of Calls and locations.

TMUF.DOC       This File which you are presumably reading!

4X6.FON        This is a font file required by  the  Turbo  Pascal

Graphics  routines.  If  you  are not equipped with

Graphics Hardware, you do not need this file.

8X8.FON        This is also a font file required by  the  Program.

Likewise if you do not have Graphics hardware,  you

will not need this File.

ERROR.MSG      This is the Pascal Error Message File.

MUF.BAT

This is  a  batch  file  used  to  start  TMUF.  It

includes  the  call to GRAPHICS.COM and handles the

command line variables for TMUF.COM.

AUTOEXEC.BAT   Optional, included in case you wish to boot and run

the program from diskette. You  will  need  to  add

this file to a system formatted diskette along with

the TMUF.COM and TMUF.DAT files.  You  should  also

add  the DOS files GRAPHICS.COM and MODE.COM and If

you  have  a clock/ calendar board, you should copy

your  file  for  setting the DOS date/time onto the

diskette  and  replace the DATE and TIME statements

with the proper command for Setting the  DOS  clock

from  your  Clock  board.  This  Batch file ends by

calling the MUF.BAT file described above.

The TMUF Program supports Pathing. In order to make sure that TMUF

can  find  the  necessary files, you should install the files in a

subdirectory  supported  by your SET PATH command. Provided you do

this,  TMUF  should  be  able to find its files regardless of what

directory you are  in  when  you  call  it.  The  files  TMUF.COM,

TMUF.DAT, 4X6.FON, 8X8.FON, and ERROR.MSG must all be located in a

directory or subdirectory which is listed in the PATH  environment

in order for the program to be able to locate the necessary files.

If you fail to do this, you will receive an error message  to  the

effect that the program can  not  locate  the  required  file.  If

unable to locate  the  data  base  file,  TMUF  will  default  the

transmit and receive stations to TEST1 and TEST2.

73 s

Jim Harrison, W4HSR

#