Glacier Bay/Mount Fairweather

The Glacier Bay area in Alaska is famous for its rugged mountains

and tidewater glaciers.  Just two hundred years ago the entire bay

was filled with ice, but the glaciers retreated dramatically in the

late 19th and early 20th centuries.  Now the remaining glaciers are

at the heads of fiords some 50 or 60 miles long.  Between Glacier Bay

and the Gulf of Alaska is what is now a long peninsula with a chain

of towering mountains rising abruptly from the sea.  The highest is

Mount Fairweather at over 15,000 feet, and Mount Crillon is over

12,000 feet.  The center of the peninsula is occupied by the Brady

Glacier, an enormous icefield feeding streams of ice in several different

directions.

A sampling of this spectacular scenery is contained in the DEM files

FairwthW02.dem and FairwthW12.dem, subquadrangles of the Mount Fairweather

West one degree DEM. The highest point of Mount Fairweather itself is just

west of the boundary between the two subquadrangles.  The fiord coming into

the FairwthW12 subquadrangle from the east is an arm of Glacier Bay called

John Hopkins Inlet, and the western and southern part of the FairwthW02

subquadrangle extends into the Gulf of Alaska.

Each one degree by one degree quadrangle is split up, at this

lattitude, into six subquadrangles.  The two digits at the end of the

subquadrangle file name locate the subquadrangle within the one degree

quadrangle.  The first digit varies from 0 at the west edge of the

quadrangle to 1 at the east edge of the quadrangle.  The second digit

varies from 0 at the south edge of the quadrangle to 2 at the north

edge of the quadrangle.  The points in the DEMs are spaced every 3

seconds of arc in lattitude from south to north (about 90 meters) and

every 6 seconds of arc in longitude from west to east, which at this

latitude of about 60 degrees is also about 90 meters.

The Fairwth.DEMset file sets up the rendering parameters for

this area in a way which does a reasonably good job of properly coloring

the glaciers which fill the valleys almost down to sea level.  This is

done by taking advantage of the flatness of the lower reaches of the

glaciers with the SlopeBlend parameter.  Load this file using the

first file requester that appears when you "Save Data" from GeoRama.

Since much of the terrain is covered by ice and snow, you probably will

not want to fill lakes or run rivers in Panorama.