-------------------

STE SAMPLE PLAYING!

-------------------

Written by Paul Bramwell

Those of you with an STE like me will feel the lack of support in

games, this to change soon, you may have a few STE Demos like

Black Cats Demo or whatever, these use the enhanced sound chip

and sample chip the DMA chip.

These routines supplied will also use the DMA chip, which enables

us to get on with something else while we are playing a sample.

The routines supplied are based around the Mastersound playback

routines.

------------

HOW TO DO IT

------------

First of all we need to set up some memory for our sample, we do

this using DIM A%(Value), you should have come across this by

now.

DIM Sample%(5000)               ! Sets up some memory

sample%=VARPTR(b%(0))

The next step is to load the sample into this space, we do this

using the Bmove command.

BLOAD "backbed.sam",sample%

The length of the sample supplied is about 30k, so we setup this

value as follows

length%=30000                    ! Length of sample

dma%=&HFF8901                    ! Sets up DMA value

Now we need to reset the DMA chip so we SPOKE it,

SPOKE dma%,0                     ! Resets DMA

Next we setup the sample rate and a single mono channel.

SPOKE dma%+32,&H81

SPOKE dma%+2,sample%/&H10000      ! Start, count addresses

SPOKE dma%+4,sample%/&H100

SPOKE dma%+6,sample% AND &HFF

SPOKE dma%+8,sample%/&H10000

SPOKE dma%+&HA,sample%/&H100

SPOKE dma%+&HC,sample% AND &HFF

SPOKE dma%+&HE,(sample%+length%)/&H10000

SPOKE dma%+&H10,(sample%+length%)/&H100

SPOKE dma%+&H12,(sample%+length%) AND &HFF

Now you can put your main program here and then recall the sample

using

SPOKE dma%,1

FOR n%=0 TO 50

VSYNC

NEXT n%

And thats all there is to it!

To change the playback rate we just do this:

FOR speed%=0 TO 3

SPOKE dma%+32,&H80+speed%

SPOKE dma%,3                          ! Loop DMA

FOR n%=0 TO 100

VSYNC

NEXT n%

NEXT speed%

SPOKE dma%,0                            ! End sound

End

If you have a long sample, and you want it interupt driven just

take out the SPOKE dma%,0 out, this will make the sample keep

playing.

The listing is on the disk, in the GOODIES\GFA folders.