USEFUL ADDRESSES

No, this isn't a contact service!  Here,  I will list some addresses in

memory which may be useful to  programmers. Unfortunately, I don't know

many, but here they are anyway:

$FF8800  (PSG - You can do 'VU' meters with this one:

poke $FF8800,8 : V1=peek($FF8800)

poke $FF8800,9 : V2=peek($FF8800)

poke $FF8800,10 : V3=peek($FF8800)

You get a value from 0 to 15 depending on the volume of the

particular channel).

$FF8240  colour 0 (Hardware colour registers)

$FF8242  colour 1

$FF8244  colour 2 (Note, each colour is a 'word')

$FF8246  colour 3 (so use DOKE or DEEK)

$FF8248  colour 4

$FF824A  colour 5

$FF824C  colour 6

$FF824E  colour 7

$FF8250  colour 8

$FF8252  colour 9

$FF8254  colour 10

$FF8256  colour 11

$FF8258  colour 12

$FF825A  colour 13

$FF825C  colour 14

$FF825E  colour 15

$FF8203  Produces a rather interesting screen offset thingy.

POKE $FF8203,128 is normal. Try 1,2...

$FF820A  You can use this one to either add that professional

touch to a demo, or just to panic the unsuspecting ST

user. POKE $FF820A,254 is normal. 0 flickers the

screen, 1 slowly fades it, 5 angles the screen and

sometimes H-shifts it. Try others...

$FF820F  This is the width of the screen. 0 is default, so try

others.

$9F8     Checks to see if the disk is write-protected. (Longword)

? LEEK($9F8) returns -1 for protected, 0 for enabled.

$FB0001  Reads the sample cartridge (Byte)

$4D2     A vertical-blank interrupt vector. You assign music to this

so it plays continuously.

That's all I know, I  am  afraid.  But  these  ones  above may keep you

interested for a while!

Article: BLACK EAGLE 1/6/93