0) Configurationon the IO page, the timer board is assumed to be an 8254 style timerWe used the Keithly Metrabyte board DAS 8 PGA,the A/D is at adress x, and the timer at x+4.Thus, the two IO ports should have x and x+4 as the addresses.The Interrupt routine address is ignored IN ANY_INT386 routine.Make sure the A/D board has the timer clock jumpered into theinput interrupt pin to actually get interrupts.Faster times can be achieved by using the N.I. Board,because it can read in 10 usecs.  The Keithely board takes 30usecsjust to read.On a Gateway 2000, no network cards,NI board at hex 220, and Keitly Board at x300 worked fine.1) Startingboot up OS9000gwindows -q  ccalc -x=startup demo_04.calThe display has an XY plot that shows the running mean.(a low bandwidth signal).  The averaging time was currentlyset at 50/(acq_rate_Khz*1000) seconds.Now, you can move a slider between 0 and the max buffer_size - 10.The max buffer size is 50000.   at 5Khz, get up to 1 second.At 1Khz, get up to 5 seconds etc....In simulated IO mode, I have the wrong read-out hooked up.2) stoppingmove window over, find ccalc icon, click on it, /TR to toggle run off.3)  Implementation Notes /SC1  cyclic  ticks=2  main control and xy plot   This routine also scans the the sampling rate, and then   if simulation mode is wanted, because scan page 6 does it at   2 ticks, or 100/2 = 50hz, it takes the  desired_rate/50hz or   desired_rate_Khz * 1000/50 and determines the necessary loop count.   Because it must be an integer, there are a few glitches in   the simulated values for non-integer loop values.  Thus,   in simulation mode, the acq_rate should be 1Khz,2Khz, 3Khz     2  INTerrupt page.  In simulated mode, branches around this page.   The 8254 repeat period is time_count*1e-6.3  init page     has fake_amp_in, fake_bias_in, fake_fsam_over_fin         AD_levels, AD_p2p_volts etc... scale factor settings     fake_fsam_over_fin should be 2 or greater.  Default it 10.     to generate the sinewave values. 4  cyclic ticks = 2 logging5  cyclic strip charts, ticks = 56  cyclic simulated data creation, ticks = 2.   put out a fake sinewave input.    As long as the averaging time is a few cycles in length,   you will get perfect answers for the mean and variance.There needs to be added:a) a power switch on or off.  b) Display of the averaging time.   (running_size/(Acq_rate_Khz*1000)  in seconds.   Should be a slider to set this.c) A more precise way to measure loading.   Currently    using the built  in Gwindows CPU usage indicator to get loading.   Should really have the int handler keep a running count   and compare this count with predicted count over known time   (using a long time) so that get_time can be used.   Over a 1 second interval, should have x hz interrupts   If less than this, lost some due to loading problems.   a) /GHA  hide all gives benchmark w/o graphics   b) gwindows_configure  to pick the best video driver      VGA mode is slowest   c)/GS demo_win to bring window backd) The display label of Data should really be MEAN   The xy plot should say mean, and the x axis should   have units of time on it.4) known bugsa)  you have to /TR /TR stop and run when chaning the averaging timebecause we did not clear the history cells between runs when this slider moves.b)  in simulation mode, for some reason, the xy display of themean does not work right unless the sampling rate is chosento be integer multiples of the simulated data task.need to rewrite this demo to make it clean.took a while to get working, i had to see how Rich Clarke did itonce to see how interrupts really worked, and how timer board work.   With time and more users, larger A/D and DA librarieswill eventually be in place.  Right now, you have to work harderto use these functions, getting down at board level to programthe timer chips Rich Clarke wrote most of the demo to show me how to useany_int386 and the raw 8254.  I did the algorithms.The results is spaghetti code the first time around.Part of the learning curve.5)  Here is how this should be rewritten.algorithms:to map a slider from 0 to 100 to some_min to some_maxslider_value = some_min + (some_max-some_min)*(raw_slider_value/100)on page 1, running_sum and running_sqr are there in raw AD integers.when ever you want to compute a mean and std.do those computations on page 1, and read these out from all other placesfrom page 1. Currently have this stuff calculated in several pages.unscaled_mean = (running_sum/FLOAT(running_size)unscaled_rms2 = (running_sqr)/FLOAT(running_size)unscaled_std  = sqrt( unscaled_rms2 - unscaled_mean*unscaled_mean)std           = unscaled_std*(AD_p2p_volts)/AD_levelsmean          = unscaled_mean(AD_p2p_volts)/AD_levels  + AD_0level_voltsthe averaging time in seconds.running_size/sampling_rateThe sampling rate is read from a slider that takes min and maxcount values from 8254.  There is some tricks for cleanly setting itas you move the slider.The timer control and the simulated rates should all be centralizedoff of one set of variables. The problem is that in simulated mode, the counter is reset afterthe averaging time is printed out.   The final printout of averagingtime should occur at the bottom of page 1.