MHS Tracking and Statistics File Format

Tue 11-Dec-90

-----------------------------------------------------------------

If you enter a directory name (with no filename) in the "Stats"

field of the SET UP TRANSPORT menu, three or more files will be

created when the Connectivity Manager is started.  Each will have

the name of your host and the names of all passive internet hosts

that your host connects to, with the following extensions:

hostname.Cnn

hostname.Tnn

hostname.Rnn

The "C", "T", and "R" in the filename extension refer to the type

of tracking file being generated.  Files whose extensions begin

with a "C" are "Call Record" files.  Those whose extensions begin

with "T" are "Transfer Record" files, and those whose extensions

begin with "R" are "Routing Record" files.

The two digit numbers in the filename extensions refer to the hub

line whose records are being kept in the file.  The first line

will have records whose extensions end in "00".  The second line

will have files whose extensions end in "01", and so forth.

Tracking records are kept in a comma-delimited format, readable

by most database programs.  Details of the record structures are

given below.

The first record in all tracking files is an ID record which

indicates the type of record file, the host for which the file is

being kept, and when the file was created.  The ID record has the

following format:

int rectype

The record type for ID records is 0.

string  filnam

This file's full pathname

string  hostname

This host's name

string  hostsn

This host`s serial number

int dayofweek

Day of the week that the call occured.  This field

allows different billing rates to be used based on

day of week without requiring calculation from date.

Sunday:    1

Monday:    2

Tuesday:   3

Wednesday: 4

Thursday:  5

Friday:    6

Saturday:  7

DATE date

Current date in the format "dd/mm/yy"

int time

Current time in minutes since midnight

CALL Records

A call record is made for every call from or to a host

connectivity manager line including all internet and gateway

sessions, plus every routing session during which messages are

routed, plus every MHS Utilities Session during which a queue has

been reposted or messages in a queue have been reposted or

returned to their sender.

The Call-ID in each call record can be used to correlate calls to

particular messages identified in Transfer records that were

transferred or processed during the call.

int rectype

Call record entries have the number "1" in this field

int dayofweek

Day of the week that the call occured.  This field

allows different billing rates to be used based on

day of week without requiring calculation from date.

Sunday:    1

Monday:    2

Tuesday:   3

Wednesday: 4

Thursday:  5

Friday:    6

Saturday:  7

DATE date

Current date in the format "dd/mm/yy"

int time

Current time in minutes since midnight

string OrgAns

Session Originator:

"O" == Originate

"A" == Answer

Notes:

Internet sessions while the host is sending a queue

to a remote host are logged as "O", and while the

host is bursting a queue received from an internet

remote host is logged as "A".

Gateway sessions are logged as "O".

string SessionID

Session ID of this session

Notes: The session ID is a unique ID generated in the

format of a Message-ID at the beginning of a session.

This number is used only for correlating call records

with Transfer records.

string RemoteHost

Name of the Host that this session is with

string RemoteSerNum

Serial Number of the remote host

int SessionVer

Session Protocol version

Note:

The session protocol used by a host is the same as

the host's version.

int LinkVer

Link Protocol version used during the session.

Note:

The value 0 is used for Gateway and Internet sessions.

int TotDuration

Total session time including call setup and hangup.

Note:

The difference between TotDuration and the sum of

OutDuration and InDuration is the overhead for

establishing the call.

int OutDuration

Total Inbound message transfer time in seconds.

int InDuration

Total Outbound message transfer time in seconds.

int contype

Session type code:

0 = local submission/delivery/routing session

1 = Session with a Host that can be called.

2 = Session with a Host that calls this host.

3 = Session with a Host reached via an Internet

4 = Session transferring messages to/from Gateway

10 = Discard Queue session from MHSU

int speed

Raw data transfer speed:  (examples:)

0    (Internet and Gateway)

300 bps

1200 bps

2400 bps

4800 bps

9600 bps

int rescode

connection result code

0 == All OK

1 == Partial transfer

2 == Failed transfer

127 == Fatal error during transfer

255 == Unknown error during transfer

Notes:

"Failed transfer" means that the initial login

negotiation with the remote host did not succeed.

The record will indicate -Unknown- for the hostname

and serial number when a remote host calls in under

these circumstances.

The following are sample call records including a sample file ID record:

0, F:\tgb\mhs\stats\syscorp.C00, syscorp, HBGNPBBCHL, 6,06/04/90,941

1,6,06/04/90,953,O,35CB1C26017B12F1,host1,HBGAPBBCHL,70,0,0,0,0,3,0,0

1,6,06/04/90,954,A,61CB1C26017B12F1,host2,HBGNEKGMGN,70,64,234,0,210,2,9600,2

1,6,06/04/90,958,O,50CC1C26017B12F1,,,0,0,3,0,0,0,0,0

1,6,06/04/90,959,O,93CC1C26017B12F1,host1,HBGAPBBCHL,70,0,2,2,0,3,0,0

1,6,06/04/90,966,O,37CE1C26017B12F1,host3,HBGNEKGMGN,70,64,318,0,261,1,9600,2

TRANSFER Records

Transfer records record the "appearance" and "disappearance" of

messages at a host.  Each entry reflects a message entry or an

associated attachment entry.  For example, a version 64 queue

with a single message and its attachment will produce two

transfer records.

A transfer record is created when an entry arrives at a host, and

later when the entry is delivered or transfered from the host.

When a version 70 transfer is partially transferred, more than

one instance of the transfer will appear in transfer records,

with smaller than the total volume, and with a code indicating

that each transmission consisted of only part of the transfer.

Version 64 entries, on the other hand, are transfered in their

totality, or not at all.

Transfer records can be correlated to particular calling sessions

using the Session-ID which appears in both Entry and Call

records, and can be correlated to particular routing activities

using the MHS-ID, which appears in both Call and Routing records.

Transfer Records have the following format:

int rectype

Transfer records have a record type 3

string SessionID

Session ID of this session

Notes: The session ID is a unique ID generated in the

format of a Message-ID at the beginning of a session.

This number is used only for correlating call records

with Transfer and Routing records.

string MHS-ID

MHS-ID of entry

Note: The MHS-ID is generated when an entry enters a

version 70 host, and remains associated with all

instances of the entry as long as the entry stays in

a version 70 environment.  In a pure version 70

environment, this ID can be used to track an entry

message through a network.

The MHS-ID field is also used to correlate Transfer

records with Routing records.

A v70 multiple attachment message is associated with

a single transfer record.  If that message is burst

into multiple v64 entries, a single transfer record

continues to specify the sum of the v64 entries with

the same MHS-ID that are now each carrying different

attachments.

int duration

Transfer time in seconds

This number is determined during version 70 sessions,

and is calculated from the review of incoming and

outbound version 64 queues and the total duration of

the session during which those queues were

transfered.

string direction

Direction of transfer of this entry recorded by this

entry record.

"I" indicates INBOUND, local submission, arrived from

inbound transfer.

"O" indicates OUTBOUND, outbound from the host: local

delivery, discarded queue, or non-delivery discard.

long charcnt

Number of bytes in entry transferred.

Notes:

This is the total for the particular instance being

recorded.  There is a transfer record for each

instance of a message and for each of its

attachments.  The transfer record shows the charcnt

of the particular instance of the message or its

attachment being recorded.

int SequenceNumber

0 for message body, and 1 through n for each

attachment.

int rescode

Transfer result code

0 = All OK

1 = Partial transfer

255 = Unknown error

Note: Some errors won't be logged because the system

crashed and the volume to which logging is done can't

be accessed.

The following are sample transfer records including a sample file ID record:

0, F:\tgb\mhs\stats\ati-fast.T00, ati-fast, HBGNPBBCHL, 6,06/04/90,941

3,61CB1C26017B12F1,F2BB1C2601EC0F4A,2,I,672,0,0

3,61CB1C26017B12F1,F8D61C2601EC0F4A,3,I,793,0,0

3,61CB1C26017B12F1,F8D61C2601EC0F4A,204,I,63280,1,1

3,93CC1C26017B12F1,F2BB1C2601EC0F4A,0,O,666,0,0

3,37CE1C26017B12F1,F8D61C2601EC0F4A,0,I,0,0,0

3,37CE1C26017B12F1,F8D61C2601EC0F4A,259,I,83168,1,1

3,27D81C26017B12F1,F8D61C2601EC0F4A,0,I,0,0,0

ROUTING records

Routing Records record the actions generated by routing and queue

maintenance operations.

Routing Records have the following format:

int rectype

Routing records have record type 2

string SessionID

Session ID of the session during which the message

was submitted to this host for routing.

Notes: The session ID is a unique ID generated in the

format of a Message-ID at the beginning of a session.

This number is used only for correlating call records

with Transfer and Routing records.

string MHS-ID

MHS-ID of entry being routed

This ID can be used to correlate routing records to

in- and outbound Transfer records.

string SenderUser

User name of Sender

string SenderApp

Application name of Sender

string SenderWG

Workgroup name of Sender

string SenderEnt

Enterprise name of Sender

Note: This field will always be empty in routing

records produced by MHS 1.5.

string RecipUser

User name of Recipient

string RecipApp

Application name of Recipient

string RecipWG

Workgroup name of Recipient

string RecipEnt

Enterprise name of Recipient

Note: This field will always be empty in routing

records produced by MHS 1.5.

string QueuedTo

Name of Queue to which entry was queues.

Notes:

-Here- in this field means the message was delivered

to a local user.

-ERROR- in this field means the message was discarded

as undeliverable.

-SND- in this field means the message was reposted to

the SND directory.

string XRecipUser

Transformed User name of Recipient.

Note: If no transformation was performed then this

field is empty (represented as two consecutive

commas.

string XRecipApp

Transformed Application name of Recipient.

Note:  If no transformation was performed then this

field is empty (represented as two consecutive

commas.

string XRecipWG

Transformed Workgroup name of Recipient.

Note: If no transformation was performed then this

field is empty (represented as two consecutive

commas.

string XRecipEnt

Transformed Enterprise name of Recipient.

Notes: If no transformation was performed then this

field is empty (represented as two consecutive

commas.

This field will always be empty in routing

records produced by MHS 1.5.

int QtyAtts

Number of Attachment-type entries being routed with

this message entry. (0 through 64)

int V64AttBursting

"1" or "0".  If the value of this field is 1, then

a new message body entry was output for each

attachment specified in QtyAttsIn.  If the value of

this field is 0, then only the original message entry

was output as a result of this routing operation.

int ErrorBurstMultiplier

Number of values in the Addresses-referred-to field,

about which a complete set of messages is being

generated.  This number is 1 if there is no

Addresses-referred-to field or if the message is not

a -Maiser- being burst due to its being a V70 to V64

conversion and multiple Addresses are referred to,

and is equal to the number of Addresses-referred-to

when a Maiser message referring to multiple errors is

burst at a V70 to V64 boundary.

Notes regarding the previous three fields:

Using the fields QtyAttsIn, V64AttBursting, and

ErrorBurstMultiplier, the number of outbound

transfer records due to a particular routing

operation can be calculated:

(1 + QtyAttsIn + ((QtyAttsIn * V64AttBursting)-1)   )

* ErrorBurstMultiplier

The initial "1" accounts for the message input to

the router.

The QtyAttsIn accounts for the number of entries

attributable to attachments associated with the

message -- these will always be output.

The ((QtyAttsIn * V64AttBursting)-1) accounts for

the number of new message body entries output due

to v70 to v64 attachment bursting.

The * ErrorBurstMultiplier gives a multiplier to

the above quantity of entries to account for

messages spawned due to V70 to V64 multiple

error-report bursting.

The following are sample routing records including a sample file ID record:

0, F:\tgb\mhs\stats\ati-fast.R00, ati-fast, HBGNPBBCHL, 6,06/04/90,941

2,37CE1C26017B12F1,F2BB1C2601EC0F4A,MSmith,,WG1,,John,ATC,ati,,host1,,,,,0,0,0

2,37CE1C26017B12F1,F8D61C2601EC0F4A,dmark,ATC,WG1,,tjones,ATC,WG1,,host1,,,,,1,0,0

2,27D81C26017B12F1,F8D61C2601EC0F4A,dmark,ATC,WG1,,tjones,ATC,WG1,,host1,,,,,1,0,0