Microsoft MS-DOS CD-ROM Extensions 2.1

Product Overview Version 2.10 Beta

The Microsoft MS-DOS CD-ROM Extensions are an extension to the MS-DOS

operating system which permit reading CD-ROM disks which conform to both the

High Sierra May 28th format and the ISO-9660 version of the High Sierra

format. The CD-ROM disc appears just like a magnetic disk to the user and to

applications software, ensuring compatibility with current software.

Microsoft, as creator of the MS-DOS operating system, is able to ensure

compatibility with MS-DOS.

Product Components

The complete product consists of a program supplied by Microsoft and of a

hardware-dependent device driver supplied by an OEM customer. The program

supplied by Microsoft is named MSCDEX.EXE. Technical documentation as well

as a sample driver are also supplied by Microsoft.

Technical Overview

Characteristics

ţ Requires MS-DOS 3.1 or higher or 4.0 (or PC-DOS 3.1 or higher or 4.0)

ţ Implements the High Sierra May 28th format and ISO-9660

ţ Requires a hardware-dependent device driver

This product uses the Microsoft Networks interface to MS-DOS so it requires

MS-DOS version 3.1 or higher or 4.0.  MS-DOS 3.1 virtualizes the interface

to drives. The entire CD-ROM (potentially all 660 megabytes) will appear to

applications as a single MS-DOS drive letter. The extensions overcome the 32

megabyte disk size limitation in MS-DOS. The Microsoft MS-DOS CD-ROM

Extensions provide a high degree of compatibility with applications that

depend on MS-DOS standard interfaces. Applications can access files on the

CD-ROM just as they would on any disk.

The program MSCDEX.EXE is an installable file system driver implemented as a

terminate and stay resident module. The user will load this program upon

booting the computer using AUTOEXEC.BAT. The hardware-dependent device

driver implements basic functions to read the CD-ROM disc and is loaded with

the MS-DOS CONFIG.SYS file.

The Microsoft MS-DOS CD-ROM Extensions implement both the May 28th High

Sierra file format and the ISO-9660 version of that standard. All features

defined in May 28th proposal for Level 1 are implemented. In addition the

following are implemented:

Features Beyond High Sierra Level 1

ţ  Support for Interleaved Files

ţ  Support for 31 Character File Names when possible through truncation

ţ  Support for Hidden Files

ţ  Support for Access to VTOC

ţ  Ignores Higher Level Files and Functions when present on the disk:

-  Associated Files

-  Protection Bits

-  Record Bits

-  File Version Numbers

ţ Support for shift-JIS Kanji (Japanese character) filenames

Hardware-Dependent Device Driver

This product requires a hardware-dependent device driver that interfaces to

a specific OEM drive or drives. A detailed specification for the device

driver as well as a sample driver are included. The driver implements the

basic functions of reading the CD-ROM and is installed using the DOS

CONFIG.SYS conventions. A minimum set of functions that allow reading the

CD-ROM disc are required to be in the device driver; there are optional

additional functions which provide increased performance when the CD-ROM

drive and controller can provide additional functions and these are

implemented in the driver. These functions are detailed in the Microsoft MS-

DOS CD-ROM Extensions Hardware-Dependent Device Driver Specification.

The device driver is written by the OEM customer for the MS-DOS CD-ROM

Extensions. Development of the device driver is estimated to take

approximately 1-3 man-months. This estimate assumes an engineer experienced

in 8086 assembler programming and familiar with MS-DOS and the CD-ROM drive

hardware. If a previous device driver has already been written, less time

will probably be needed to implement the driver for the Microsoft MS-DOS CD-

ROM extensions. There are third party companies who will write the hardware-

dependent device drivers on a consulting basis.

Licensing the Microsoft MS-DOS CD-ROM Extensions

Microsoft will license the MS-DOS CD-ROM Extensions to manufacturers and

marketers of CD-ROM disc drives. The license agreement will allow the use of

the product on a personal computer to which a licensed disc drive is

attached. Developers of CD-ROM disks will not need to acquire any license or

pay any royalty in order to develop or sell CD-ROM discs, and will not be

entitled to distribute the MS-DOS CD-ROM Extensions. The end user will

purchase the driver from drive manufacturer or marketer, not the CD-ROM disc

developer.

The Microsoft MS-DOS CD-ROM Extensions will be delivered to licensees on a

5 1/4" MS-DOS diskette. Licensees are expected to distribute the Extensions

to their customers on a floppy diskette containing both MSCDEX.EXE and the

hardware-dependent device driver written by the licensee. The floppy would

be included in the package containing the CD-ROM drive.

Creating CD-ROM Disks in the High Sierra Format

The Microsoft MS-DOS CD-ROM Extensions provides for reading CD-ROM discs in

the High Sierra/ISO-9660 format on MS-DOS computers. They do not create CD-

ROM disks in the High Sierra/ISO-9660 format. Microsoft does not manufacture

CD-ROM discs, nor provide pre-mastering services. Third party companies can

create CD-ROM discs in the High Sierra/ISO-9660 format and provide other

pre-mastering services. Microsoft can supply a list of companies providing

or planning to provide these services upon request.

Software developers do not need the MS-DOS CD-ROM Extensions in order to

create either applications software which reads CD-ROM discs, or to create

CD-ROM discs. Once the software is ready and a disc has been pressed,

developers will want a copy of the Extensions for testing; however, they are

not needed in order to start development.

Software developers need do nothing special for accessing CD-ROM discs; they

issue the same MS-DOS OPEN and READ calls as for opening any magnetic disks.

Programmers can develop CD-ROM applications using standard MS-DOS tools.

They need to be aware that they cannot create any temporary files or write

any files in either the directory or on the entire CD-ROM disc. Software

developers will want to minimize the number of seeks to the CD-ROM because

of the comparatively long seek times of CD-ROM drives.

ŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰ

Hardware-Dependent Device Driver Specification

Intent

This document (Document Number: 000080010-100-O00-1186) describes the CD-ROM

hardware-dependent device driver and its interface with MSCDEX.EXE, the MS-

DOS CD-ROM Extensions resident program. Differences between CD-ROM drives

and hard- or floppy-disk drives account for the differences in this device

driver specification from the normal MS-DOS block and character device

driver specification. The chapters on device drivers in the MS-DOS

Programmer's Reference Manual (MS-PRM) provide more information.

The MS-DOS operating system reads CONFIG.SYS and installs the device.

MSCDEX.EXE performs an open system call on the device driver name in order

to communicate with it and uses an IOCTL call to ask the device driver for

the address of its device header. From the device header address, MSCDEX.EXE

locates the device driver's interrupt and strategy routines. After that, all

requests the device driver receives come directly from MSCDEX.EXE, not MS-

DOS. To avoid reentrancy problems and allow MSCDEX to monitor all media

changes, all other applications that wish to communicate directly with CD-

ROM device drivers should do so through the Send Device Driver Request INT

2Fh function 10h. MSCDEX.EXE interfaces with MS-DOS so that normal requests

for I/O with files on a CD-ROM drive down to the MS-DOS INT 21h service

layer will work just as they would for a normal MS-DOS device.

Installation

The device driver will be installed in the same way as any other device with

an entry in CONFIG.SYS. The syntax is:

DEVICE=<filename> /D:<device_name> /N:<number of drives>

The following are examples:

DEVICE=HITACHI.SYS /D:MSCD001 /D:MSCD002

DEVICE=SONY.SYS    /D:MSCD003 /N:2

The arguments will be the character device names that will be used on the

command line when starting MSCDEX.EXE so that it can find and communicate

with the device driver.

A device driver may support one or more physical drives or logical disks.

This may be done by having multiple device headers in the device driver file

(in which case it will be necessary to have more than one device_name on the

command line - one for each device header; see the HITACHI.SYS example

above) or through the use of subunits. Each disk handled by a device driver

that supports multiple disks using subunits is addressed by the subunit

field of the request header when a request is made for that disk. A device

driver that supports more than one disk can share code and data instead of

requiring separate device drivers for each disk. A "jukebox" CD-ROM system

would be an example of a CD-ROM device that might wish to support more than

one drive or a disk pack using a single device driver.

Device drivers that use multiple subunits should use the optional switch

/n:<number of drives> to say how many drives are present. If not present,

the default number of drives is 1. If the driver can tell how many drives

are installed without a command line switch, then this argument is not

necessary. Unless there are special considerations, it is better practice to

support multiple drives using subunits than to have multiple device headers

in the same device driver file.

Device Header

The device header is an extension to what is described in the MS-PRM.

DevHdr    DD   -1        ; Ptr to next driver in file or -1 if last driver

DW   ?         ; Device attributes

DW   ?         ; Device strategy entry point

DW   ?         ; Device interrupt entry point

DB   8 dup (?) ; Character device name field

DW   0         ; Reserved

DB   0         ; Drive letter

DB   ?         ; Number of units

The following are the device attributes for MSCDEX.EXE device drivers:

Bit 15         1       - Character device

Bit 14         1       - IOCTL supported

Bit 13         0       - Output 'till  busy

Bit 12         0       - Reserved

Bit 11         1       - OPEN/CLOSE/RM supported

Bit 10-4       0       - Reserved

Bit  3         0       - Dev is CLOCK

Bit  2         0       - Dev is NUL

Bit  1         0       - Dev is STO

Bit  0         0       - Dev is STI

MSCDEX.EXE device drivers will be character devices that understand IOCTL

calls and handle OPEN/CLOSE/RM calls.

The drive letter field is a read-only field for the device driver and is

initialized to 0. The field is for MSCDEX.EXE to use when it assigns the

device driver to a drive letter (A = 1, B = 2...Z = 26). It should never be

modified by the device driver. For drivers that support more than one unit,

the drive letter will indicate the first unit, and each successive unit is

assigned the next higher drive letter. For example, if the device driver has

four units defined (0-3), it requires four drive letters. The position of

the driver in the list of all drivers determines which units correspond to

which drive letters. If driver ALPHA is the first driver in the device list,

and it defines 4 units (0-3), they will be A, B, C, and D. If BETA is the

second driver and defines three units (0-2), they will be E, F, and G, and

so on. The theoretical limit to the number of drive letters is 63, but it

should be noted that the device installation code will not allow the

installation of a device if it would result in a drive letter > 'Z' (5Ah).

All block device drivers present in the standard resident BIOS will be

placed ahead of installable device drivers in the list.

ÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄ

NOTE:

It is important that one set lastdrive=<letter> in CONFIG.SYS

to accommodate the additional drive letters that CD-ROM device drivers

will require.

ÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄ

The number-of-units field is set by the device driver to the number of disks

that are supported. Normal character devices do not support more than one

unit and MS-DOS does not expect a character device to handle more than one

unit or have a nonzero subunit value in the request header. Since these

device drivers are not called by MS-DOS directly, this is not a problem.

Nonetheless, the number of units returned by the device driver in the

number-of-units field during the INIT call must be 0, since MS-DOS makes the

INIT call and does not expect a nonzero value for a character device.

MSCDEX.EXE will never see what is returned anyway, and relies on the number-

of-units field in the device header.

Sample device header:

HsgDrv    DD   -1             ; Pointer to next device

DW   0c800h         ; Device attributes

DW   STRATEGY       ; Pointer to device strategy routine

DW   DEVINT         ; Pointer to device interrupt routine

DB   'HSG-CD1 '     ; 8-byte character device name field

DW   0              ; Reserved (must be zero)

DB   0              ; Drive letter (must be zero)

DB   1              ; Number of units supported (one or more)

As with other MS-DOS device drivers, the code originates at offset 0, not

100H. The first device header will be at offset 0 of the code segment. The

pointer to the next driver is a double word field (offset/segment) that is

the address of the next device driver in the list, or -1 if the device

header is the only one or the last in the list. The strategy and interrupt

entry points are word fields and must be offsets into the same segment as

the device header. The device driver is expected to overwrite the name(s) in

each of its one or more device headers with the <device_name> command line

arguments during its initialization.

MSCDEX.EXE will call the device driver in the following manner:

1.  MSCDEX.EXE makes a far call to the strategy entry.

2.  MSCDEX.EXE passes device driver information in a request header to the

strategy routine.

3.  MSCDEX.EXE makes a far call to the interrupt entry.

Request header

MSCDEX.EXE will call the device's strategy routine with the address of a

request header in ES:BX. The format of the request header is the same as

what is described in the MS-PRM.

ReqHdr    DB   ?         ; Length in bytes of request header

DB   ?         ; Subunit code for minor devices

DB   ?         ; Command code field

DW   ?         ; Status

DB   8 dup (?) ; Reserved

Status

The status word also has the same format as described in the MS-PRM. It is 0

on entry and is set by the device driver.

Bit 15         - Error bit

Bit 14-10      - Reserved

Bit  9         - Busy

Bit  8         - Done

Bit  7-0       - Error code (bit 15 on)

Bit 15, the error bit, is set by the device driver if an error is detected

or if an invalid request is made to the driver. The low 8 bits indicate the

error code.

Bit 9, the busy bit, should be set by the device driver when the drive is in

audio play mode. Device drivers should fail all requests to the physical

device that require head movement when the device is playing and return the

request with this bit and the error bit set and an error code. Requests that

would not interrupt audio play may return without error but will also have

this bit set when the drive is in audio play mode. Play mode can be

terminated prematurely with a reset or STOP AUDIO request and a new request

can be made at that point. Monitoring this bit in each successive request,

an Audio Q-Channel Info IOCTL for example, will tell when play mode is

complete.

Bit 8, the done bit, is set by the device driver when the operation is

finished.

Error codes are the following:

0  Write-protect violation

1  Unknown unit

2  Drive not ready

3  Unknown command

4  CRC error

5  Bad drive request structure length

6  Seek error

7  Unknown media

8  Sector not found

9  Printer out of paper

A  Write fault

B  Read fault

C  General failure

D  Reserved

E  Reserved

F  Invalid disk change

Command Code Field

The following values are valid command codes:

0  INIT

1   MEDIA CHECK (block devices)

2   BUILD BPB (block devices)

3  IOCTL INPUT

4   INPUT (read)

5   NONDESTRUCTIVE INPUT NO WAIT

6   INPUT STATUS

7  INPUT FLUSH

8   OUTPUT (write)

9   OUTPUT WITH VERIFY

10   OUTPUT STATUS

11  OUTPUT FLUSH

12  IOCTL OUTPUT

13  DEVICE OPEN

14  DEVICE CLOSE

15   REMOVABLE MEDIA (block devices)

16   OUTPUT UNTIL BUSY

128  READ LONG                (NEW)

129   Reserved

130  READ LONG PREFETCH       (NEW)

131  SEEK                     (NEW)

132  PLAY AUDIO               (NEW)

133  STOP AUDIO               (NEW)

134  WRITE LONG               (NEW)

135  WRITE LONG VERIFY        (NEW)

136  RESUME AUDIO             (NEW)

Unsupported or illegal commands will set the error bit and return the error

code for Unknown Command. This includes command codes 1, 2, 4, 5, 6, 8, 9,

10, 15, 16, and 129; and 11, 134 and 135 for systems that do not support

writing.

If, in the time since the last request to that device driver unit, the media

has changed, the device driver will return the error code for invalid disk

change and set the error bit. MSCDEX.EXE will then decide whether to retry

the request or abort it.

The minimal CD-ROM device driver will read cooked Mode 1 data sectors using

HSG addressing mode and return appropriate values for the IOCTL calls. Most

other features enhance performance or add useful capabilities.

INIT

Command code = 0

ES:BX = INIT

INIT      DB   13 dup (0); Request header

DB   0         ; Number of units (must be 0)

DD   ?         ; End address

DD   ?         ; Ptr to BPB array

DB   0         ; Block device number

This call is made only once, when the device is installed. INIT and a single

IOCTL call for the device header address are the only device driver calls

that come directly from MS-DOS. Because the INIT function is called from MS-

DOS, the number of units returned is 0, as for normal MS-DOS character

devices. MSCDEX.EXE will get the number of units supported from the device

header.

The device must return the END ADDRESS, which is a DWORD pointer to the end

of the portion of the device driver to remain resident. Code and data

following the pointer is used for initialization and then discarded. If

there are multiple device drivers in a single file, the ending address

returned by the last INIT call will be the one that MS-DOS uses, but it is

recommended that all the device drivers in the file return the same address.

The code to remain resident for all the devices in a single file should be

grouped together low in memory with the initialization code for all devices

following it in memory.

The pointer to BPB array points to the character after the "=" on the line

in CONFIG.SYS that caused this device driver to be loaded. This data is

read-only and allows the device driver to scan the invocation line for

parameters. This line is terminated by a carriage return or a line feed.

During initialization, the device driver must set the device name field in

the device header to the argument provided on the invocation line in

CONFIG.SYS. The device driver must also check that the device_name command

line argument is a legal 8-character filename and pad it out to 8 characters

with spaces (20H) when copying it to the device name field.

The block device number and number of units are both 0 for character

devices.

READ (IOCTL Input)

Command code = 3

ES:BX = IOCTLI

IOCTLI    DB   13 dup (0); Request header

DB   0         ; Media descriptor byte from BPB

DD   ?         ; Transfer address

DW   ?         ; Number of bytes to transfer

DW   0         ; Starting sector number

DD   0         ; DWORD ptr to requested vol ID if error 0FH

The media descriptor byte, starting sector number, and volume ID fields are

all 0.

The transfer address points to a control block that is used to communicate

with the device driver. The first byte of the control block determines the

request that is being made. If the command code is reserved or the function

not supported, then the device driver will return the error code for Unknown

Command. If, for some reason, the device driver is not able to process the

request at that time, it will return the error code for Drive Not Ready.

Number of Bytes

Code       to Transfer                 Function

0             5            Return Address of Device Header

1             6            Location of Head

2             ?            Reserved

3             ?            Error Statistics

4             9            Audio Channel Info

5           130            Read Drive Bytes

6             5            Device Status

7             4            Return Sector Size

8             5            Return Volume Size

9             2            Media Changed

10             7            Audio Disk Info

11             7            Audio Track Info

12            11            Audio Q-Channel Info

13            13            Audio Sub-Channel Info

14            11            UPC Code

15            11            Audio Status Info

16-255        ?             Reserved

Return Address of Device Header

Raddr     DB   0         ; Control block code

DD   ?         ; Address of device header

The device driver will fill the 4-byte field with the address of its device

header. This is used by MSCDEX.EXE to locate the device driver's strategy

and interrupt routines.

Location of Head

LocHead   DB   1         ; Control block code

DB   ?         ; Addressing mode

DD   ?         ; Location of drive head

The device driver will return a 4-byte address that indicates where the head

is located. The value will be interpreted based on the addressing mode. (See

function READ LONG for more information about addressing modes.)

ÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄ

NOTE:

The drive could provide this information by monitoring the Q-channel on

the disk.

ÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄ

Error Statistics

ErrStat   DB   3         ; Control block code

DB   N dup (?) ; Error statistics

The format of the Error Statistics is not yet defined.

Audio Channel Info

AudInfo   DB   4       ; Control block code

DB   ?       ; Input  channel (0, 1, 2, or 3) for output channel 0

DB   ?       ; Volume control (0 - 0xff) for output channel 0

DB   ?       ; Input  channel (0, 1, 2, or 3) for output channel 1

DB   ?       ; Volume control (0 - 0xff) for output channel 1

DB   ?       ; Input  channel (0, 1, 2, or 3) for output channel 2

DB   ?       ; Volume control (0 - 0xff) for output channel 2

DB   ?       ; Input  channel (0, 1, 2, or 3) for output channel 3

DB   ?       ; Volume control (0 - 0xff) for output channel 3

This function returns the present settings of the audio channel control set

with the Audio Channel Control Ioctl Write function. The default settings

for the audio channel control are for each input channel to be assigned to

its corresponding output channel (0 to 0, 1 to 1, etc.) and for the volume

control on each channel is set at 0xff.

Read Drive Bytes

DrvBytes  DB   5          ; Control block code

DB   ?          ; Number bytes read

DB   128 dup (?); Read buffer

Data returned from the CD-ROM drive itself can be read using this function.

The number-bytes-read field returns the length of the number of bytes read,

which will not exceed 128 per call. If more than this needs to be returned,

the call will be repeated until the number returned is 0.

The function and content of these bytes are entirely device and device

driver dependent. This function is provided to allow access to device-

specific features that are not addressed under any other portion of the

device driver spec.

Device Status

DevStat   DB   6         ; Control block code

DD   ?         ; Device parameters

The device driver will return a 32-bit value. Bit 0 is the least significant

bit. The bits are interpreted as follows:

Bit 0     0    Door closed

1    Door open

Bit 1     0    Door locked

1    Door unlocked

Bit 2     0    Supports only cooked reading

1    Supports cooked and raw reading

Bit 3     0    Read only

1    Read/write

Bit 4     0    Data read only

1    Data read and plays audio/video tracks

Bit 5     0    No interleaving

1    Supports interleaving

Bit 6     0    Reserved

Bit 7     0    No prefetching

1    Supports prefetching requests

Bit 8     0    No audio channel manipulation

1    Supports audio channel manipulation

Bit 9     0    Supports HSG addressing mode

1    Supports HSG and Red Book addressing modes

Bit 10-31 0    Reserved (all 0)

Return Sector Size

SectSize  DB   7         ; Control block code

DB   ?         ; Read mode

DW   ?         ; Sector size

The device driver will return the sector size of the device given the read

mode provided. In the case of CD-ROM, the value returned for cooked is 2048,

and the return value  for raw is 2352.

Return Volume Size

VolSize   DB   8         ; Control block code

DD   ?         ; Volume size

The device driver will return the number of sectors on the device. The size

returned is the address of the lead-out track in the TOC converted to a

binary value according to FRAME + (SEC * 75) + (MIN * 60 * 75). A disc with

a lead out track starting at 31:14.63 would return a volume size of 140613.

The address of the lead-out track is assumed to point to the first sector

following the last addressable sector recorded on the disc.

Media Changed

MedChng   DB   9         ; Control block code

DB   ?         ; Media byte

The normal media check function (command code 1) is not performed on

character devices and contains additional semantics that are not needed for

CD-ROM device drivers. This is why there is an IOCTL request for this

function.

When the device driver receives a call to see if the media has changed on

that subunit, it will return one of the following values:

1         Media not changed

0         Don't know if changed

-1 (0FFh)  Media changed

If the driver can assure that the media has not been changed (through a

door-lock or other interlock mechanism), performance is enhanced because

MSCDEX.EXE does not need to reread the VTOC and invalidate in-memory buffers

for each directory access. For drives that do not report if the media has

changed, CD-ROM device drivers can utilize the same solution that has been

applied to floppy disks. In some floppy-disk device drivers, if the MEDIA

CHECK occurs within 2 seconds of a floppy-disk access, the driver reports

"Media not changed." It is highly recommended though that drives be able to

detect and report media changes.

If the drive can enforce a door lock mechanism so that the device driver is

notified when the door lock has been unlocked or the device driver is

requested to do so by MSCDEX.EXE, then to improve performance, the driver

could return that the media has not changed without bothering to communicate

with the physical device.

If the media has not been changed, MSCDEX.EXE will proceed with the disk

access. If the value returned is "Don't know," or "Media changed," then

MSCDEX.EXE will check to see if the disk has changed. It will  continue if

it has not, and reinitialize what it knows about the disk if it has.

It is not necessary for the device driver to do anything for the volume ID

when the media has changed.

Audio Disk Info

DiskInfo  DB   10        ; Control block code

DB   ?         ; Lowest track number

DB   ?         ; Highest track number

DD   ?         ; Starting point of the lead-out track

This function returns TOC (Table of Contents) information from the Q-Channel

in the lead-in track indicating what the first and last track numbers are

and the Red Book address for the lead-out track (PMIN/PSEC/PFRAME when POINT

= A2). The first and last track numbers are binary values and not BCD. It is

recommended that the information for Audio Disk Info and Audio Track Info

should be read by the drive when the disc is initialized and made accessible

to the driver so that when these functions are called, the drive or driver

do not have to interrupt audio play to read them from the TOC. If the TOC is

not made available to the driver and the driver must obtain the information

itself from the lead-in track, the driver should read and and attempt to

cache the disk and track information during the Audio Disk Info command and

invalidate this information only if the media changes.

Audio Track Info

TnoInfo   DB   11        ; Control block code

DB   ?         ; Track number

DD   ?         ; Starting point of the track

DB   ?         ; Track control information

This function takes a binary track number, from within the range specified

by the lowest and highest track number given by the Audio Disk Info command,

and returns the Red Book address for the starting point of the track and the

track control information for that track. The track control information byte

corresponds to the byte in the TOC in the lead-in track containing the two

4-bit fields for CONTROL and ADR in the entry for that track. The CONTROL

information is in the most significant 4 bits and the ADR information is in

the lower 4 bits. The track control information is encoded as follows:

00x00000  - 2 audio channels without pre-emphasis

00x10000  - 2 audio channels with pre-emphasis

10x00000  - 4 audio channels without pre-emphasis

10x10000  - 4 audio channels with pre-emphasis

01x00000  - data track

01x10000  - reserved

11xx0000  - reserved

xx0x0000  - digital copy prohibited

xx1x0000  - digital copy permitted

Audio Q-Channel Info

QInfo     DB   12        ; Control block code

DB   ?         ; CONTROL and ADR byte

DB   ?         ; Track number (TNO)

DB   ?         ; (POINT) or Index (X)

; Running time within a track

DB   ?         ; (MIN)

DB   ?         ; (SEC)

DB   ?         ; (FRAME)

DB   ?         ; (ZERO)

; Running time on the disk

DB   ?         ; (AMIN) or (PMIN)

DB   ?         ; (ASEC) or (PSEC)

DB   ?         ; (AFRAME) or (PFRAME)

This function reads and returns the most up to date Q-channel address

presently available. It should not interrupt the present status of the drive

as one of its intended purposes is to monitor the location of the read head

while playing audio tracks. This function should return valid information

even when no audio tracks are being played and the head is stationary. The

fields returned correspond to the data that is stored in the Q-channel as

described in the Red Book. The values in MIN-SEC-FRAME, AMIN-ASEC-AFRAME and

PMIN-PSEC-PFRAME are converted by the driver from BCD to binary so that

minutes range from 0 to 59+, seconds from 0 to 59, and frames from 0 to 74.

The Control and ADR byte, TNO, and POINT/Index bytes are always passed

through as they appear on the disc and are not converted. If the drive

returns Q-channel information when ADR is not equal to 1, then when ADR is

not equal to 1 all ten bytes of information are passed through unmodified to

the caller.

Audio Sub-Channel Info

SubChanInfo DB   13        ; Control block code

DD   ?         ; Starting frame address

DD   ?         ; Transfer address

DD   ?         ; Number of sectors to read

This function takes a Red Book address for a particular frame (also known as

a block or frame) and copies 96 bytes of sub-channel information per frame

for all the sectors that are requested sequentially at the transfer address

given. Each 96 bytes of information do not include the two sync patterns (S0

and S1) that head the subcoding block but only the the 96 bytes of subcoding

symbols each with one bit of information for the eight different channels

(P-W) that follow them. P is the MSB, W is the LSB of each byte.

The caller is responsible for making sure that 96 *

Number_of_sectors_to_read bytes are available at the transfer address for

the device driver to store the results.

Data definition and integrity restrictions for data received with this

command are interpreted according to the CD-ROM standard (Red and Yellow

Book).

UPC Code

UPCCode   DB   14        ; Control block code

DB   ?         ; CONTROL and ADR byte

DB   7 dup (?) ; UPC/EAN code

; (last 4 bits are zero; the low-order nibble of

; byte 7)

DB   ?         ; Zero

DB   ?         ; Aframe

This function returns the UPC/EAN (Universal Product Code - BAR coding) for

the disc. This information is stored as a mode-2 (ADR=2) Q-channel entry.

The UPC code is 13 successive BCD digits (4 bits each) followed by 12 bits

of zero. The last byte is the continuation of FRAME in mode-1 though in the

lead-in track (TNO=0) this byte is zero. If the CONTROL/ADR byte is zero or

if the 13 digits of UPC code are all zero, then either no catalog number was

encoded on the disc or it was missed by the device driver. If the command is

not supported, then the driver will return an error code of Unknown Command.

If the command is supported but the disc does not have a UPC Code recorded,

then the driver will return an error code of Sector not Found.

Audio Status Info

AudStat DB   15        ; Control block code

DW   ?         ; Audio status bits

; Bit 0 is Audio Paused bit

; Bits 1-15 are reserved

DD   ?         ; Starting location of last Play or for next Resume

DD   ?         ; Ending location for last Play or for next Resume

The Audio Paused bit and Starting and Ending locations are those referred to

in the RESUME command.

WRITE (IOCTL OUTPUT)

Command code = 12

ES:BX = IOCTLO

IOCTLO    DB   13 dup (0); Request header

DB   0         ; Media descriptor byte from BPB

DD   ?         ; Transfer address

DW   ?         ; Number of bytes to transfer

DW   0         ; Starting sector number

DD   0         ; DWORD ptr to requested vol ID if error 0FH

The media descriptor byte, starting sector number, and volume ID fields are

all 0.

The transfer address points to a control block that is used to communicate

with the device driver. The first byte of the control block determines the

request that is being made. The Length of Block is the number of bytes to

transfer.

Length of

Code   Block            Function

0      1              Eject Disk

1      2              Lock/Unlock Door

2      1              Reset Drive

3      9              Audio Channel Control

4      ?              Write Device Control String

5      1              Close Tray

6-255  ?              Reserved

Eject Disk

Eject          DB   0         ; Control block code

The device driver will unlock the drive and eject the CD-ROM disk from the

drive unit. The door will report as being open until the user has inserted

a disk  into the drive unit and closed the door. The status bit for door

open can be monitored to determine when a disk has been reinserted.

Lock/Unlock Door

LockDoor  DB   1         ; Control block code

DB   ?         ; Lock function

When this function is received, the device driver will ask the CD-ROM drive

to unlock or lock the door. If lock function is 0, the device driver will

unlock the door. If lock function is 1, it will lock the door.

Reset Drive

ResetDrv  DB   2         ; Control block code

This function directs the device driver to reset and reinitialize the

drive.

Audio Channel Control

AudInfo   DB   3     ; Control block code

DB   ?     ; Input  channel (0, 1, 2, or 3) for output channel 0

DB   ?     ; Volume control (0 - 0xff) for output channel 0

DB   ?     ; Input  channel (0, 1, 2, or 3) for output channel 1

DB   ?     ; Volume control (0 - 0xff) for output channel 1

DB   ?     ; Input  channel (0, 1, 2, or 3) for output channel 2

DB   ?     ; Volume control (0 - 0xff) for output channel 2

DB   ?     ; Input  channel (0, 1, 2, or 3) for output channel 3

DB   ?     ; Volume control (0 - 0xff) for output channel 3

This function is intended to provide playback control of audio information

on the disk. It allows input channels on the CD-ROM to be assigned to

specific output speaker connections. The purpose of this function is to

allow two independent channels to be recordedÄÄin different languages for

exampleÄÄand to play back only one of them at a time or to be able to

manipulate an audio signal so that the source appears to moveÄÄto make a

sound seem to move from left to right for example.

Output channel 0 is the left channel, 1 is right, 2 is left prime, and 3 is

right prime. The Red Book specification allows for 4 audio channels. The two

"prime" channels (2 and 3) extend stereo to quadrophonic stereo.

An audio volume setting of 0 means off. Drives that don't support 4 output

audio channels may ignore output to channels 2 and 3. Assignment of input

channels 2 and 3 to output channels 0 and 1 may be treated as though the

volume control for that channel is 0.

Drives that do not support variable audio control will treat a setting of 0

as off and 1-0xff as on. Drives that support less than 256 volume settings

will do their best to break up the 256 settings among the settings they can

support. E.g. if there are 16 settings supported, then the first setting

will cover 0x01-0x10, the second 0x11-0x20...the sixteenth 0xf1-0xff. Drives

that can't play a single channel in both must play only that one channel and

try to suppress the other if possible. Drives that can't swap channels

should play the channel that was moved in its normal channel.

Write Device Control String

DrvBytes  DB   4         ; Control block code

DB   N dup (?) ; Write buffer

This function is provided to allow programs to talk directly to the CD-ROM

drive. All remaining bytes are sent uninterpreted to the drive unit.

The function and content of these bytes are entirely device and device

driver dependent. This function is provided to allow access to device-

specific features that are not addressed under any other portion of the

device driver spec.

Close Tray

CloseTray DB   5         ; Control block code

This command is the logical complement to the Eject Disk command. This

command will instructs drives that can do so to close the door or tray.

READ LONG

Command code = 128

ES:BX = ReadL

ReadL     DB   13 dup (0); Request header

DB   ?         ; Addressing mode

DD   ?         ; Transfer address

DW   ?         ; Number of sectors to read

DD   ?         ; Starting sector number

DB   ?         ; Data read mode

DB   ?         ; Interleave size

DB   ?         ; Interleave skip factor

The request block is different from a normal character device READ to

accommodate the larger size and different characteristics of CD-ROM

devices.

The media descriptor byte, which has no meaning for character devices, is

now the addressing mode field. The following values are recognized

addressing modes:

0      HSG addressing mode

1      Red Book addressing mode

2-255  Reserved

The default addressing mode is the HSG addressing mode. Long (DWORD) address

values are treated as logical block numbers, as defined by the High Sierra

proposal. When Red Book addressing mode is on, all disk addresses are

interpreted as Minute/Second/Frame addresses, according to the Philips/Sony

Red Book standard. Each of these fields is 1 byte. The frame byte is the

least significant byte of the address field, the "second" byte the next most

significant, the minute byte the next, and the most significant byte of the

4-byte field is unused. These values are represented in binary rather than

in BCD format. For example, if we are referencing the sector addressed by

minute 36, second 24, frame 12, the hex long value for this would be

0x0024180C. The relationship between High Sierra sectors and Red Book frames

is described by the equation:

Sector = Minute * 60 * 75 + Second * 75 + Frame - 150

The byte/sector count field becomes the number of sectors to read and the

starting sector number expands from one word to two, which  means we can

address up to 4 giga-sectors (over 8 terabytes). The DWORD ptr for requested

volume ID is eliminated and MSCDEX.EXE will keep track of what volume is

needed.

MSCDEX.EXE handles buffering requests, but performance may be improved if

the device driver reads ahead or uses a sector caching scheme, given the

slow seek times of CD-ROM drives. The operating system will use the prefetch

function when it can to give hints to the driver.

The data read mode field will be one of the following:

0      Cooked mode

1      Raw mode

2-255  Reserved

Cooked mode is the default mode in which the hardware typically handles the

EDC/ECC and the device driver returns 2048 bytes of data per sector read.

When raw mode is set, the driver will return all 2352 bytes of user data,

including any EDC/ECC present independent of the actual sector mode (Mode 2

Form 1 vs. Mode 2 Form 2). User programs will have to consider this and

allow enough room for buffer space when reading in raw mode as each sector

returned will take up 2352 bytes of space. Drives that cannot return all

2352 bytes will return what they can and leave blank what they cannot. For

example, drives that can return all 2336 bytes except the 16 byte header

will leave a space in the first 16 bytes where the header would go so that

the sectors align on 2352 byte boundaries. Drivers should do what they can

to return as much of the user data per sector as possible.

The two interleave parameters are for drivers that support interleaved

reading. If the driver does not support interleaving, these fields are both

ignored. If it does, interleave size is the number of consecutive logical

blocks or sectors that are stored sequentially, and the interleave skip

factor is the number of consecutive logical blocks or sectors that separate

portions of the interleaved file.

READ LONG PREFETCH

Command code = 130

ES:BX = ReadLPre

ReadLPre  DB   13 dup (0); Request header

DB   ?         ; Addressing mode

DD   0         ; Transfer address

DW   ?         ; Number of sectors to read

DD   ?         ; Starting sector number

DB   ?         ; Read mode

DB   ?         ; Interleave size

DB   ?         ; Interleave skip factor

This function is similar in form to READ LONG, but control returns

immediately to the requesting process. The device driver is not obligated to

read in the requested sectors but can instead consider the request for these

sectors as hints from the operating system that they are likely to be

needed. It is recommended that at a minimum, the driver seek to the location

provided. The attribute in the device status for prefetching is used to

distinguish drivers that do more than just seek to the given location. The

requests are low priority and preemptible by other requests for service. A

READ LONG PREFETCH with 0 number of sectors to read should be treated as an

advisory seek, and the driver can, if it is not busy, move the head to the

starting sector. Since prefetching requests are advisory, there will be no

functional difference between a device driver that supports prefetching from

one that does not, except in terms of performance. The transfer address is

not applicable for this call as the driver is not meant to transfer any data

into the user address space.

SEEK

Command code = 131

ES:BX = SeekReq

SeekReq   DB   13 dup (0); Request header

DB   ?         ; Addressing mode

DD   0         ; Transfer address

DW   0         ; Number of sectors to read

DD   ?         ; Starting sector number

Control returns immediately to the caller without blocking and waiting for

the seek to be completed. The number of sectors to be read and the transfer

address are ignored. SEEK is used to relocate the head in order to begin

playing audio or video tracks, or in anticipation of reading in a particular

region on the disk. Further requests for disk activity will wait until the

given SEEK is completed. This seek is not advisory and the head must move to

the desired location.

PLAY AUDIO

Command code = 132

ES:BX = PlayReq

PlayReq   DB   13 dup (0); Request header

DB   ?         ; Addressing mode

DD   ?         ; Starting sector number

DD   ?         ; Number of sectors to read

This function will cause the driver to play the selected audio tracks until

the requested sectors have been exhausted or until play is interrupted with

a AUDIO STOP request. Control returns immediately to the caller. Monitoring

the busy bit in the status word will determine if the drive is presently

playing audio and also when the play request is completed.

STOP AUDIO

Command code = 133

ES:BX = StopPlayReq

StopPlayReq    DB   13 dup (0)     ; Request header

This function is included to interrupt the drive unit when it is currently

in play mode. At the next stopping point it reaches, the drive will

discontinue playing and process the next request. If the drive is not

currently playing or does not support playing, this request is ignored.

RESUME AUDIO

Command code = 136

ES:BX = ResumeReq

ResumeReq DB   13 dup (0)     ; Request header

This function is used to resume playing audio tracks when play has been

interrupted with the STOP AUDIO command. Its behavior should correspond to

the following:

RESET, NEW DISC, PLAY/RESUME COMPLETED

playing = FALSE;

paused  = FALSE;

last_startloc = 0;

last_endloc   = 0;

PLAY_AUDIO(startloc, endloc) {

if (play(startloc, endloc) != SUCCESSFUL) {

return error;

playing = TRUE;

paused = FALSE;

last_startloc = startloc

last_endloc = endloc

return no error;

}

STOP_AUDIO() {

if (playing) {

last_startloc = present q-channel location

playing = FALSE;

paused = TRUE;

if (stop() == SUCCESSFUL)

return no error;

return error;

}

else {

playing = FALSE;

paused = FALSE;

last_startloc = 0;

last_endloc = 0;

return no error;

}

}

RESUME_AUDIO() {

if (paused) {

if (play(last_startloc, last_endloc) != SUCCESSFUL)

return error;

playing = TRUE;

paused = FALSE;

return no error;

}

else

return error;

Note that the playing flag corresponds to the state that should be reported

by the busy bit in the status word in the request header when the drive is

in audio play mode. The paused flag corresponds to the Audio Paused bit and

last_startloc and last_endloc correspond to the starting and ending location

in the Audio Status Info IOCTL.

WRITE LONG

Command code = 134

ES:BX = WriteL

WriteL    DB   (dup 13 0); Request header

DB   ?         ; Addressing mode

DD   ?         ; Transfer address

DW   ?         ; Number of sectors to write

DD   ?         ; Starting sector number

DB   ?         ; Write mode

DB   ?         ; Interleave size

DB   ?         ; Interleave skip factor

The device will copy the data at the transfer address to the CD RAM device

at the sector indicated. The media must be writable for this function to

work. Data is written sector by sector, depending on the current write mode

and the interleave parameters. The following values are recognized as valid

write modes:

0      Mode 0

1      Mode 1

2      Mode 2 Form 1

3      Mode 2 Form 2

4-255  Reserved

Writing in Mode 1 is the default and must be supported. If the device driver

supports the other modes, then they can be used. If Mode 0 is used, the

transfer address is ignored and all sectors are written with zeroes. If the

current write mode is Mode 1 or Mode 2 Form 1, each sector will consist of

2048 bytes of data located sequentially at the transfer address. If the

write mode is Mode 2 Form 2, the device driver will expect 2336 bytes of

data per sector at the transfer address.

WRITE LONG VERIFY

Command code = 136

ES:BX = WriteLV

WriteLV   DB   (dup 13 0); Request header

DB   ?         ; Addressing mode

DD   ?         ; Transfer address

DW   ?         ; Number of sectors to write

DD   ?         ; Starting sector number

DB   ?         ; Write mode

DB   ?         ; Interleave size

DB   ?         ; Interleave skip factor

This function is identical to WRITE LONG, with the addition that the device

driver is responsible for verifying the data written to the device.

INPUT FLUSH

Command code = 7

ES:BX = FlushI

FlushI         DB   13 dup (0)     ; Request header

Requests that the device driver free all input buffers and clear any pending

requests.

OUTPUT FLUSH

Command code = 11

ES:BX = FlushO

FlushO         DB   (dup 13 0)     ; Request header

Requests that the device driver write all unwritten buffers to the disk.

DEVICE OPEN

DEVICE CLOSE

Command code = 13,14

ES:BX = DevOpen, DevClose

DevOpen   DB   13 dup (0)     ; Request header

Used by the device driver to monitor how many different callers are

currently using the CD-ROM device driver. All new device drivers should

support these calls even if nothing is done with the information.

ŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰ

Function Requests Specification

There is a need for access to features from the MSCDEX redirector that

transend DOS capabilities. This proposal documents a means that the

application can use to talk directly to MSCDEX to request information or set

parameters that only MSCDEX can provide. This document outlines some of the

features I think MSCDEX should support. Comments and suggestions are

welcome.

Access to these functions is provided through an INT 2Fh interface. AH

contains 15h which is what MSCDEX will use to tell its requests from those

of other INT 2Fh handlers. AL will contain the code of the function to be

performed.

Function Request Command Codes:

Contents of AL   Function

00h              Get Number of CD-ROM Drive Letters

01h              Get CD-ROM Drive Device List

02h              Get Copyright File Name

03h              Get Abstract File Name

04h              Get Bibliographic Doc File Name

05h              Read VTOC

06h              Turn Debugging On

07h              Turn Debugging Off

08h              Absolute Disk Read

09h              Absolute Disk Write

0Ah              Reserved

0Bh             CD-ROM Drive Check

0Ch             MSCDEX Version

0Dh             Get CD-ROM Drive Letters

0Eh             Get/Set Volume Descriptor Preference

0Fh             Get Directory Entry

10h             Send Device Request

11h-0FFh         Reserved

Get Number of CD-ROM Drive Letters

AX   1500h

BX   Number of CD-ROM drive letters used

CX   Starting drive letter of CD-ROM drive letters (A=0, B=1, ...Z=25)

MSCDEX will return the number of CD-ROM drive letters in BX and the starting

drive letter in CX. The first CD-ROM device will be installed at the

starting drive letter and subsequent drives will be assigned the next

greater drive letter. A single device driver may be assigned to more than

one drive letter, such as the case of a device driver that supports multiple

units. MSCDEX keeps track of which sub-unit a particular drive letter is

assigned to.

ÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄ

NOTE:

This function can be used to determine if MSCDEX is installed by setting

BX to zero before executing INT 2Fh. MSCDEX is not installed if BX is

still zero on return.

ÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄ

Also, in a networking environment, one cannot assume that drive letters will

always be assigned contiguously beginning with the starting drive letter.

Use function Get CD-ROM drive letters instead.

Get CD-ROM Drive Device List

AX        1501h

ES:BX     Transfer address; pointer to buffer to copy drive letter

device list

The buffer must be large enough to hold the device list. By calling function

Get Number of CD-ROM Drive Letters, one can find out the number of CD-ROM

drive letters and the buffer size will be a multiple of that. This will be

an absolute maximum of 26. Each drive letter device entry will consist of

one byte for the sub-unit followed by 4 bytes for the address of the device

header assigned to that drive letter. This byte for the sub-unit takes care

of the problem of distinguishing which unit is assigned to which drive

letter for device drivers that handle sub-units.

For example: Suppose there are two installed CD-ROM device drivers, FOO,

which supports 1 sub-unit, and BAR, which supports two sub-units, on a

system with 2 floppy drives (A=0 and B=1) and a hard disk (C=2). Then asking

for the number of CD-ROM drive letters will report that there are 3 drive

letters used starting at drive letter D=3. ES:BX must point to a buffer that

is at least 3 * 5 = 15 bytes long. The buffer will be filled as follows:

ES:BX  = Buffer

Buffer   DB   0                        ; sub-unit of FOO on drive letter D:

DD   <far addr of FOO device header>

DB   0                        ; sub-unit of BAR on drive letter E:

DD   <far addr of BAR device header>

DB   1                        ; sub-unit of BAR on drive letter F:

DD   <far addr of BAR device header>

Get Copyright File Name

AX     1502h

ES:BX  Transfer address; pointer to a 38 byte buffer

CX     CD-ROM drive letter (A=0, B=1, ... Z=25)

MSCDEX will copy the name of the copyright file in the VTOC for that drive

letter into the buffer space provided. The copyright filename is presently

restricted in the High Sierra proposal to 8.3 but we require 38 bytes here

for the possibility at a later date of handling 31 character file names plus

6 bytes for a ';' and 5 digit version number and 1 byte for a NULL at the

end. Carry will be set if the drive letter is not a CD-ROM drive and

error_invalid_drive (15) will be returned in AX.

Get Abstract File Name

AX     1503h

ES:BX  Transfer address; pointer to a 38 byte buffer

CX     CD-ROM drive letter (A=0, B=1, ... Z=25)

MSCDEX will copy the name of the abstract file in the VTOC for that drive

letter into the buffer space provided. The abstract filename is presently

restricted in the High Sierra proposal to 8.3 but we require 38 bytes here

for the possibility at a later date of handling 31 character file names plus

6 bytes for a ';' and 5 digit version number and 1 byte for a NULL at the

end. Carry will be set if the drive letter is not a CD-ROM drive and

error_invalid_drive (15) will be returned in AX.

Get Bibliographic Documentation File Name

AX     1504h

ES:BX  Transfer address; pointer to a 38 byte buffer

CX     CD-ROM drive letter (A=0, B=1, ... Z=25)

ÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄ

NOTE:

This function is provided in advance of the ISO standard. For discs

complying with the May 28th draft from the High Sierra Group, this

function will return a null string as though the field is blank on the

disc.

ÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄ

MSCDEX will copy the name of the bibliographic documentation file in the

VTOC for that drive letter into the buffer space provided. The bibliographic

documentation filename is presently restricted in the High Sierra proposal

to 8.3 but we require 38 bytes here for the possibility at a later date of

handling 31 character file names plus 6 bytes for a ';' and 5 digit version

number and 1 byte for a NULL at the end. Carry will be set if the drive

letter is not a CD-ROM drive and error_invalid_drive (15) will be returned

in AX.

Read VTOC

AX     1505h

ES:BX  Transfer address; pointer to a 2048 byte buffer

CX     CD-ROM Drive letter

DX     Sector index

This function is provided to scan the Volume Descriptors on a disc. A sector

index of 0 will read the first volume descriptor, 1 reads the second, etc.

If there is no error, then AX will return 1 if the volume descriptor read

was the standard volume descriptor, 0FFh if it was the volume descriptor

terminator and there are no more volume descriptors to be read, and 0 for

all other types.

If there is an error in processing the request, the Carry Flag will be set

and AL will contain the MS-DOS error code. These will be either

error_invalid_drive (15) or error_not_ready (21).

Turn Debugging On

AX     1506h

BX     Debugging function to enable

This is used for development and is reserved. It will be non-functional in

the production version of MSCDEX.

Turn Debugging Off

AX     1507h

BX     Debugging function to disable

This is used for development and is reserved. It will be non-functional in

the production version of MSCDEX.

Absolute Disk Read

AX     1508h

ES:BX  Disk Transfer Address; pointer to a buffer to copy data to

CX     CD-ROM Drive letter (A=0, B=1, ... Z=25)

DX     Number of sectors to read

SI:DI  Starting sector

This function corresponds to INT 25h. It will be converted directly into a

READ_LONG device driver request and sent to the correct device driver. There

are no requirements for this call to pop flags as there are with INT 25h. SI

holds the high word and DI the low word for the starting sector to begin

reading from.

If there is an error in processing the request, the Carry Flag will be set

and AL will contain the MS-DOS error code. These will be either

error_invalid_drive (15) or error_not_ready (21).

Absolute Disk Write

AX     1509h

ES:BX  Disk Transfer Address; pointer to buffer to copy data from

CX     CD-ROM Drive letter

DX     Number of sectors to write

SI:DI  Starting sector

This function corresponds to INT 26h. It is not supported at this time and

is reserved. It is intended to be used by authoring systems.

CD-ROM Drive Check

AX     150Bh

BX     Signature word

CX     CD-ROM Drive letter (A=0, B=1,...Z=25)

This function returns whether or not a drive letter is a CD-ROM drive

supported by MSCDEX. If the extensions are installed, BX will be set to

ADADh. If the drive letter is supported by MSCDEX, then AX is set to a non-

zero value. AX is set to zero if the drive is not supported. One must be

sure to check the signature word to know that MSCDEX is installed and that

AX has not been modified by another INT 2Fh handler.

MSCDEX Version

AX     150Ch

BX     MSCDEX Version

This function returns the version number of the CD-ROM Extensions installed

on the system. BH contains the major version number and BL contains the

minor version. Values returned are binary. For example, BX would contain

0x020a for version 2.10. This function does not work on versions earlier

than 2.00 so if BX is zero before and after this function is called, an

earlier version of MSCDEX is installed.

Get CD-ROM Drive Letters

AX     150Dh

ES:BX  Transfer address; pointer to buffer to copy drive letter

device list

The buffer must be large enough to hold a list of drive letters. The buffer

size will be a multiple of the number of drives returned by the Get Number

of CD-ROM Drive Letters function. There are a maximum of 26 drive letters.

Each drive letter entry is a single byte (0=A:, 1=B: .. 25=Z:) that exactly

corresponds each respective entry returned by the command Get CD-ROM Drive

Device List. This command is included to allow applications to locate CD-ROM

drives supported by MSCDEX. CD-ROM drive letters may sometimes be

noncontiguous so this command is necessary.

For example: Suppose there is an installed CD-ROM device driver FOO

supporting 3 sub-units on a system with 2 floppy drives (A=0 and B=1), a

hard disk (C=2) and a network drive (E=4). Note the network drive occupies

one of the drive letters normally taken by a CD-ROM drive. MSCDEX assigns

that CD-ROM drive to the next available drive letter. Asking for the number

of CD-ROM drive letters reports there are 3 drive letters used starting at

drive letter D=3. ES:BX must point to a buffer that is at least 3 bytes long

and will be filled as follows:

ES:BX   = Buffer

Buffer  DB   3                   ; drive letter for CD-ROM (D=3)

DB   5                   ; drive letter for CD-ROM (F=5)

DB   6                   ; drive letter for CD-ROM (G=6)

Get/Set Volume Descriptor Preference

AX     150Eh

BX     0 - Get Preference. 1 - Set Preference

CX     CD-ROM Drive letter (A=0, B=1,...Z=25)

DX     if BX = Get Preference

DX = 0

MSCDEX will return preference settings in DX

if BX = Set Preference

DH = volume descriptor preference

1 - PVD - Primary Volume  Descriptor

2 - SVD - Supplementary Volume Descriptor

DL = Supplementary Volume Descriptor Preference

if DH = PVD

DL = 0

if DH = SVD

1 - shift-Kanji (an unregistered ISO coded

character set)

Normally, MSCDEX will scan for the PVD (Primary Volume Descriptor) when

initializing a CD-ROM. This behavior can be altered for each individual

drive to scan for a SVD (Supplementary Volume Descriptor) instead. A CD-ROM

drive set to scan for an SVD will use the PVD if there is no SVD present.

There can be more than one SVD on a CD-ROM but at present, MSCDEX will only

recognize SVDs for shift-Kanji CD-ROMs. Carry will be set, AX will be set to

error_invalid_function (1) and DX will be set to 0 if the coded character

set is not recognized.

If BX contains Get_Preference, MSCDEX will report the present setting for

that drive. If DX is still zero on return, that version of MSCDEX does not

support this function or reading SVDs. Otherwise DX will contain the

setting.

If the drive letter is not a CD-ROM drive, carry will be set and

error_invalid_drive (15) will be returned in AX. If BX is anything other

than Get/Set_Preference, AX will be set to error_invalid_function (1) and

carry will be set.

Get Directory Entry

AX     150Fh

CX     CD-ROM Drive letter (A=0, B=1,...Z=25)

ES:BX  Pointer to buffer with null-terminated path name

SI:DI  Pointer to buffer to copy directory record information

AX     0 is returned if the disc is High Sierra, 1 is returned if the

disc is ISO-9660

The pathname expected is a null-terminated string e.g. char far *path =

"\\a\\b\\c.txt"; (note: the "\\" characters map to a single '\' character in

C so this would be '\a\b\c.txt' if printed). The path must consist only of

valid High Sierra or ISO-9660 filename characters and must not contain

any wildcards nor may it include entries for '.' or '..'.

The buffer to copy the directory record to can be a maximum of 255 bytes

long including all system use information. The directory record is a direct

copy from the directory file and it is up to the application to choose what

fields to use.

Carry will be set and an error code returned if there were problems with the

request. The error codes will be error_invalid_drive (15) if the drive

letter is incorrect, error_not_ready (21) if the disc didn't initialize

correctly, error_file_not_found (2) if the file was not found and

error_no_more_files (18) if the pattern fails to find a match or if mscdex

failed to allocate buffers.

The format of the directory record for High Sierra discs is:

/* High Sierra directory entry structure */

typedef struct hsg_dir_entry {

uchar      len_dr;        /* length of this directory entry  */

uchar      XAR_len;       /* length of XAR in LBN's          */

ulong      loc_extentI;   /* LBN of data Intel format        */

ulong      loc_extentM;   /* LBN of data Molorola format     */

ulong      data_lenI;     /* length of file Intel format     */

ulong      data_lenM;     /* length of file Motorola format  */

uchar      record_time[6];/* date and time                   */

uchar      file_flags_hsg;/* 8 flags                         */

uchar      reserved;      /* reserved field                  */

uchar      il_size;       /* interleave size                 */

uchar      il_skip;       /* interleave skip factor          */

ushort     VSSNI;         /* volume set sequence num Intel   */

ushort     VSSNM;         /* volume set sequence num Motorola*/

uchar      len_fi;        /* length of name                  */

uchar      file_id[...];  /* variable length name upto 32 chars       */

uchar      padding;       /* optional padding if file_id is odd length*/

uchar      sys_data[...]  /* variable length system data              */

} hsg_dir_entry;

The format of the directory record for ISO-9660 discs is:

/* ISO-9660 directory entry structure */

typedef struct iso_dir_entry {

uchar      len_dr;        /* length of this directory entry  */

uchar      XAR_len;       /* length of XAR in LBN's          */

ulong      loc_extentI;   /* LBN of data Intel format        */

ulong      loc_extentM;   /* LBN of data Molorola format     */

ulong      data_lenI;     /* length of file Intel format     */

ulong      data_lenM;     /* length of file Motorola format  */

uchar      record_time[7];/* date and time                   */

uchar      file_flags_iso;/* 8 flags                         */

uchar      il_size;       /* interleave size                 */

uchar      il_skip;       /* interleave skip factor          */

ushort     VSSNI;         /* volume set sequence num Intel   */

ushort     VSSNM;         /* volume set sequence num Motorola*/

uchar      len_fi;        /* length of name                  */

uchar      file_id[...];  /* variable length name upto 32 chars       */

uchar      padding;       /* optional padding if file_id is odd length*/

uchar      sys_data[...]  /* variable length system data              */

} iso_dir_entry;

The difference between the two forms is the file flag byte moved to account

for an additional byte of date and time used for a Greenwich mean time

offset. See the May 28th draft of the High Sierra proposal or ISO-9660 for a

more complete explanation of the fields. Note that the C structs above are

not syntactically correct; C does not allow variable length arrays as struct

elements.

Send Device Driver Request

AX     1510h

CX     CD-ROM drive letter (A=0, B=1, ... Z=25)

ES:BX  Address of CD-ROM device driver request header

This function has been added to simplify communication with CD-ROM drivers

and help prevent contention between applications that wish to communicate

with the device driver. It is highly recommended that all applications

communicate with device drivers through this function request. Applications

using this function will not have to locate the device driver. The format of

the request header is specified by the Microsoft MS-DOS CD-ROM Extensions

Hardware-Dependent Device Driver Specification.

ŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰ

Networking CD-ROMS

Although it is possible to share CD-ROM drives on a local area network or

LAN, it is not as easy as it should be. While MS-DOS provides a single,

stable platform to develop a file system driver like the Microsoft CD-ROM

Extensions, there are a wide variety of LANs and LAN server implementations

that do not provide a stable platform for which a file system driver like

MSCDEX could be provided. Because each LAN implementation takes a different

approach for server support, the approach for CD-ROM support on a server

depends on what LAN implementation is being used.

This document should help clarify the present situation and help get you

started.

At present, there are several CD-ROM products that allow sharing of CD-ROM

drives on a LAN. LAN support may range from very simple and inexpensive to

not so simple and inexpensive. At present, there are three products

presently available that offer some form of LAN support. These are:

Microsoft      MSCDEX - The Microsoft CD-ROM Extensions

Meridian Data  CD-NET

Online         Opti-Net

Choosing which product depends on your LAN and your needs.

There are some LANs, such as Lantastic by Artisoft, that can share CD-ROM

drives using any version of MSCDEX on a Lantastic server. This is possible

because their servers run as an MS-DOS application and do I/O with standard

MS-DOS INT 21 services. LAN servers like this, that do not make assumptions

about the underlying media or try to bypass MS-DOS and do use standard MS-

DOS INT 21 services to access the drive letter, will likely work with all

versions of MSCDEX.

There are several LAN products based on MS-NET or a similar LAN server model

such as Ungermann-Bass or 3COM. Unfortunately, these products do not access

files on the server using standard INT 21 calls and for several reasons due

to assumptions inside MS-DOS about non-standard calls from the server, you

cannot share CD-ROM drives on MS-NET based servers. Although the server

seems to allow sharing of the CD-ROM drive letter, requests to the server

from workstations do not work correctly.

Fortunately, MSCDEX Version 2.10 has a command line switch (/S) that

instructs MSCDEX to patch the in-memory image of MS-DOS during its

initialization to fix these problems. By including this parameter on the

MSCDEX command line, MSCDEX can be loaded before the network server software

is started and the CD-ROM drive letters can then be shared by MS-NET based

server software and workstations will see the correct behavior. This

solution requires only that the server use MSCDEX Version 2.10 and no

software or hardware changes to the workstation. Only the server runs MSCDEX

or loads any CD-ROM related device drivers. To the workstation, the CD-ROM

server drives are indistinguishable from other server drives.

For LAN products that are not MS-NET based yet have NETBIOS support such as

Novell or IBM PC-NET, both Optinet and Meridian Data have adapted the MSCDEX

and CD-ROM Device Driver model to provide LAN CD-ROM support. Each

workstation runs MSCDEX and a special CD-ROM device driver that accepts

normal CD-ROM driver requests from MSCDEX and uses the NETBIOS to transmit

the command to a network driver on a server that submits the request to a

true CD-ROM device driver on the server and transmits the results back to

the workstation pseudo CD-ROM driver which in turn responds to MSCDEX. So

long as the workstation CD-ROM device driver responds appropriately, MSCDEX

is unaware that the command has passed through the network to a server.

Contact Meridian Data and Online for information for these networks as they

can both describe their products and features best.

Online offers one potential configuration for computer systems that do not

wish to dedicate a machine to be a server. The workstation operates as above

but instead of communicating the workstations driver request to a dedicated

server process, another user's workstation running a special TSR version of

their system can field the driver request, submit it to the CD-ROM driver,

and respond to the requesting workstation. This allows a network of

workstations to share the CD-ROM drives that each computer has connected to

it at the same time all workstations are available to the users. This option

does slow performance of the workstation when outside requests come in and

does use up valuable memory for the TSR system code but for some this option

may work.

At present, there is no available version of the CD-ROM Extensions for OS/2

although there is a way to access CD-ROM data in OS/2 on a network. Since

from the outside, workstations cannot tell MS-DOS server drives that are

shared CD-ROM drives using version 2.10 of MSCDEX from traditional block

drives, even OS/2 machines can access the CD-ROM drive on the server.

Although this does mean including an MS-DOS server on an OS/2 LAN, it does

provide at least an interim way to access CD-ROM data under OS/2 at this

time.

ŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰ

Kanji Support

The Kanji support in MSCDEX presently recognizes High Sierra CD-ROM discs

with a coded character set that has bit 0 set to 1 in the volume flags

indicating at least one escape sequence is not registered according to ISO

2375, and has an escape sequence of three bytes in the coded character set

for descriptor identifiers field of "$+:". This indicates that the character

set is a private multi-byte G3 coded character set and identifies the disc

as having shift-Kanji.

In order to make MSCDEX scan for the SVD (Supplementary Volume Descriptor)

instead of the PVD (Primary Volume Descriptor), there is a new command line

argument /K. If this is present, MSCDEX will use the shift-Kanji SVD if it

is present, otherwise it will use the PVD. All discs are required by ISO-

9660 to have a PVD even if there is an SVD.

In addition, there is an accompanying program SVD that can be used to change

the default preference each CD-ROM drive has for scanning for a SVD or PVD.

The syntax is:

SVD [<drive letter>: <std  svd>]

Running SVD with no arguments will report the current settings. Including a

drive letter and either STD or SVD will change the preference for that drive

from one to the other.

ŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰ

CD-ROMifying Your Software

CD-ROM is the first of what will probably be several alien file structures

that will start appearing in the MS-DOS world primarily with the

introduction of installable file systems under newer versions of DOS. The

following will attempt to outline some guidelines for writing software that

will help in porting your software to these new file systems and for CD-ROM

specifically.

Choice of Filename Characters

On the first Microsoft Test CD-ROM disc, the Codeview demo failed because

certain filename characters that were legal on MS-DOS were not allowed

according to the High Sierra file format. When the software looked for file

'S1.@@@', it wasn't found because the character '@' is illegal for High

Sierra filenames and during High Sierra premastering, the file was renamed

'S1'.

Valid High Sierra filename characters are the letters 'A' through 'Z', the

digits '0' through '9', and the underscore character '_'. All other

characters are invalid. Note that the letters 'a' through 'z' are not

included so that High Sierra file names are not case sensitive. Under DOS,

filenames are mapped to upper case before they are looked up so this is

typically not a problem. When choosing file name characters, keep in mind

the restrictions of the file structure format and the operating systems your

media may be targeted towards.

Depth of Path

The High Sierra format allows for pathnames to be up to 8 levels deep. It's

possible to create a path on MS-DOS that is deeper than that but you won't

be able to transfer it to a CD-ROM.

\one\two\three\four\five\six\seven\eight\file.txt /* Ok */

\one\two\three\four\five\six\seven\eight\nine\file.txt /* Illegal */

Length of Path

The High Sierra format allows for the entire pathname to be a maximum of 255

characters. Since MS-DOS imposes a limit far lower than this, this should

not present a problem. The MS-DOS call to connect to a sub-directory is

limited to a directory string of 64 characters. The length of path

restriction is more a concern for Xenix/Unix than MS-DOS.

Amusingly enough, the MS-DOS call to create a sub-directory allows a

directory string greater than 64 characters which allows you to create sub-

directories that you cannot connect to.

Unfortunately, a CD-ROM may potentially contain a pathname that is much

larger than 64 characters long. This is not a concern here but is discussed

in a related memo - "MS-DOSifying your CD-ROM". As a rule, try to keep the

length of your longest path less than 64 characters and you should be pretty

safe.

Read-only

Even though most people understand that CD-ROM discs are read-only, there's

still a lot of software written by these same people that assumes the

current disk is always writable. For example, the Microsoft Multiplan Demo

assumes that it can create and write temporary files to the presently

connected drive.

In order to avoid this problem, try to provide another means of letting the

user specify where temporary files can be created. Many applications check

the environment for the variables TMP or TEMP which contain the pathname to

use when creating temp files. Most people understand this convention now (or

should anyway) and an added benefit will be the speed improvement that will

be recognized if the temp directory is located on a ram-drive. If the

environment variable is not set, then the application can fall back on the

assumption that the media is writable or ask where temporary files should be

kept.

As a rule, for both temporary and permanent files, if a file creation error

occurs, allow the user to re-specify the pathname used so that he can work

around the error. The last thing that should happen is for work to be lost

because the user was not allowed to store his output in a valid place.

Non-DOS Formatted Disks

Don't depend on the format of data on the disk. CD-ROM's do not have a FAT

so don't even bother looking for one. Do not talk to any media at a physical

level (reading/writing sectors) unless you expect to be media dependent

(such as CHKDSK or FORMAT). MS-DOS INT 21h calls should provide everything

you need to get at the file contents and attributes.

Small Directories

For performance reasons, try to keep directory sizes smaller than about 40

or so. Much beyond this and directory files grow beyond one 2048 byte

sector. Typically this is not a problem but if the number of sector buffers

chosen when MSCDEX is started is small and the directory files are large,

whatever software scanning the directory could potentially thrash badly if

every time the directory is searched for the next entry it has to bring

earlier directory sectors back into memory from the CD-ROM drive.

For certain pathological programs, such as certain implementations of the

Xenix utility find, the penalty is about 1 second per directory sector that

you have to scan to get to the next entry. If the directory is large, say 8

sectors, the time for FIND to scan that one directory could potentially take

a half hour for something that would take less than a second if all the

entries fit in the cache.

The solution for this problem is to make sure that MSCDEX never throws out

of the cache what it will need next. This is accomplished by growing the

cache (very easy - simply change the parameter to MSCDEX) and to make sure

that the largest object that goes through the cache will not clear it out.

There is a balance between having too many directories and too many files in

a few directories but the balance is heavily weighted towards many small to

medium sized directories. Keep this in mind when laying out your files.

Since the penalty for using a file in the lowest sub-directory instead of

the root-directory is virtually nil and as more directories don't cost much,

it's a good idea to break up large directories into several smaller ones.

This will help avoid problems of flushing the disc sector cache. Try to keep

related files close together both in location on the CD-ROM and in the same

directories. Close proximity will reduce seek time when accessing related

files at the same time and having them in the same directory will help

prevent swapping out directory sectors.

Updating CD-ROM Databases and Software

Many people are interested in providing updates to files that are contained

on a CD-ROM disc. They would like to create a directory on their hard disk

with all updated files in them and have the CD-ROM Extensions look there

first before searching the CD-ROM. Unfortunately, by the time the Extensions

get the request, it is very difficult for it to look for updates on the hard

disk so whatever alternative searching that is necessary will have to be

done in the application software.

For this reason, it's a good idea to have your path set so that it looks

through directories on the hard disk first. Another good strategy is to copy

executables to a directory on your hard disk so that they can be updated and

will also start up faster. Also, have the application software itself search

alternative hard disk directories for updates before it searches the CD-ROM.

This way both software updates and updated or commonly used database files

can be stored on a hard disk which will both speed performance and allow

incremental updating.

Search Strategies

Try to avoid relying on the operating system to be part of your search

strategy. If your database is broken up into a hierarchy and your order is

imposed through the file structure by breaking up the database into many

files in a tree, then accessing data in the database is typically going to

require a lot of directory reading and searching.

Usually the time involved in doing this on a hard disk is not large but on a

CD-ROM, the search times can add up. Opening a file can be an expensive

operation simply because the directory must be read before the file can be

opened. At best, seeking to a location on the CD-ROM can take 10 msec or so;

at worst, a seek can run to over a second on some older CD-ROM drives. Some

newer drives have worst case seek of about half a second. Whenever this can

be avoided you will save time. MSCDEX caches as many directory sectors as it

can so that searching the most active directories is very quick but any

operations that search multiple directories once through continually clears

out the cache and renders it ineffective.

The strategy used by Microsoft Bookshelf was to lump the entire database

into a single file and structure the indexing so that searching used a

minimum of seeks. Bookshelf uses packed b-trees with each node holding as

many entries as will fit into a single sector and also cache in memory as

much of the root of the tree as it can.

Combining databases avoids the extra overhead of repeatedly opening and

closing database files. Caching as much of the indexes in memory as possible

allows searching of keywords to be completed typically with a single seek.

In general, identify your software bottlenecks and through judicious use of

faster storage media (either memory or hard disk) you can both have large

storage and respectable performance.

Portability

One of the advantages of the High Sierra format is data interchangeability

with other operating systems. One must take care to chose a subset of the

range of High Sierra features that are presently supported across different

operating systems to be sure you can read the disc in each of them. The

lowest common denominator then (this list is not complete - see also what

must be done to target MS-DOS) would need a logical block size of 512 bytes,

both type L and M path tables and for all fields, single volume sets, at

least one primary volume descriptor and terminator. Be aware that if one of

your goals is data portability, you will have to do some additional research

to see what restrictions on the High Sierra format other operating systems

may impose.

ŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰ

MS-DOSifying Your CD-ROM

Most of the following caveats apply to the present version of the Microsoft

CD-ROM Extensions. Future versions of the extensions are expected to support

many of the features listed below that are at present best avoided. The

behavior of the extensions with fields and records that are presently

ignored may change at any time.

Correctness

Make sure that your disc is in valid High Sierra format. Nothing is

guaranteed if your disc is not in valid format. Surprisingly enough, we have

received several discs that have one or more illegally formatted data areas

ranging from directories being sorted incorrectly, incorrect path table

sizes, incorrect directory file sizes, directories missing from the path

table, invalid directory names, etc. In almost every case, the Extensions

will behave incorrectly and at worst, the system will crash.

In addition to running validation software to verify the High Sierra image,

one should also verify that the Extensions work with your CD-ROM disc and

application software before distributing it. Unfortunately, it may not

matter if your disc is correct and the Extensions are wrong if they don't

work together. Please report any and all problems you think are in the

Extensions to Microsoft so that they can be fixed.

Pathtable and Directory Sizes

This bears repeating because many people have gotten it wrong. Directory

file sizes are always a multiple of the logical sector size - 2 kilobytes.

Path table sizes are always the exact number of bytes that are contained in

the path table which is typically not a multiple of 2k. You must not have

blank directory sectors and the directory length must reflect the correct

length of the directory file. The directory sectors always begin on a

logical sector boundary.

8.3 File Names

MS-DOS cannot handle longer than 8.3 filenames. If the CD-ROM filename is

longer than 8.3, then the filename will be truncated. If this happens, two

files that are not unique within 8.3 characters will map to the same

filename. For example:

filename1.txt will appear as filename.txt

filename2.txt will also appear as filename.txt

Kanji filenames are also limited to 8.3 or 4.1 kanji characters. Only shift-

kanji filenames are recognized at present. To get kanji, you must specify a

supplementary volume descriptor indicating you have kanji filenames. Contact

Microsoft to find out how this is done.

Record Formats

The extensions do not support any record formats so if the RECORD bit is set

in the file flags byte in the directory entry for a file, it will be

ignored.

Interleaving

In the present version, the Extensions do not support interleaving so if the

Interleave size and Interleave factor are non-zero, the file will ignore

these fields and return erroneous data.

Multi-Extent Files

Multi-extent files are not supported in the present version. Each extent of

a multi-extent file will appear as a separate file with the same name.

Multi-Volume

Multi-volume disc sets are not supported in the present version. Directories

that are located on another volume could potentially cause the Extensions to

crash if searched and erroneous data will be returned for files that are

located on another volume.

Coded Character Sets

Only one coded character set or supplementary volume descriptor is

recognized in the latest version. This is for shift-Kanji.

Version Numbers

Version numbers are not supported by the Extensions. The Extensions will

strip the version string off the end of the filename so that two identical

filenames with different versions will appear to have the same name. There

is no way to specifically ask for any but the first instance of that

filename. Two files with the same name and different version numbers have

the same accessing problem as two files with longer than 8.3 filenames that

have been truncated to the same filename.

Protection

Protection bits are not used on MS-DOS. If the protection bit is set in the

file flags byte in the directory entry for a file though, the file will not

show up on any search or open even if the protection bits in the XAR are set

to allow all access.

No XAR Support

At present, the Extensions ignore the contents of any XAR record.

Motorola Format Tables

The additional copies of the path table and any values in "Motorola" format

(most significant bytes using the lowest address values) are ignored at

present. MSCDEX only pays attention to "Intel" formatted values. They should

be included though for portability sake.

Multiple Copies of the Path Table

The Extensions presently only read and use the first copy of the path table.

Later versions may check to see that copies of the path table agree.

Additional Volume Descriptor Records

Boot records and Unspecified volume descriptors are ignored. The first

standard volume descriptor found is the one that is used. Additional copies

are ignored at present.

File Flags

The existence bit is treated the same as the hidden bit on MS-DOS. Some

other operating systems may not handle the existence bit so you may not want

to use it if you are targeting these systems. The directory bit for High

Sierra is treated the same as the directory bit in MS-DOS. Files with the

protection bit set are not found when searched for or opened.

None of the remaining bits, (Associated/Record/Multi-extent/Reserved), are

handled at present. Using files with these bits set will have undefined

behavior.

Unique Volume Identifiers

It is highly recommended that the volume identifier be unique. The

Extensions use the volume identifier to do volume tracking and to double-

check to see if the disc has changed. The more chance that users will have

two discs with the same volume identifier, the more chance that this will

confuse the Extensions and lead it to believe that the disc has not changed

when in fact it has.

It is also highly recommended that application programs use the volume label

to tell if the CD-ROM disc has changed. The volume label for a CD-ROM on MS-

DOS is obtained from the volume identifier field in the primary volume

descriptor. The call to get the volume label is very inexpensive to make

once the CD-ROM has been initialized and will cause no disc I/O to be done

unless the media has changed. This is the best way for an application to

tell if the disc it wants to work with is in the drive. The application

software should not communicate with the driver or drive to determine if the

media has changed or the Extensions may not learn that the disc has changed

and will not reinitialize what it knows about the new disc.

Many Small Directories or A Few Large Directories

As a rule, it is better to have many small directories that contain fewer

files than one very large directory. The answer depends on your

application's behavior because if you try very hard, you can thrash almost

as badly with many small subdirectories as you can with one large

subdirectory. Reading further will help explain.

What makes the difference? For each file open, suppose you have 1000

subdirectories with 40 files, on average you'll read about one sector per

file open and scan 1/2 of it. On the other hand, you could have 1 directory

with 4000 files. On average, each file open in this large directory (about

100 sectors) will involve scanning about 50 sectors to open that one file.

As long as it is very inexpensive to get to each directory through the

pathtable, clearly it is much better to have many small directories.

Further improvements can be made by grouping files that are related and will

be opened together in each of these subdirectories so that as you open each

successive file, the directory sector is very likely in the disc cache and

this will help minimize hitting the CD-ROM disc.  Putting each file in a

separate subdirectory is extreme and will cost you because you will never

gain the benefits of locating the next file in a directory sector that has

already been cached and you will needlessly enlarge the pathtable.

There is a limit though to how many subdirectories you may want because if

there are too many you may end up thrashing on the pathtable sectors. Each

pathtable sector holds pointers to approximately 100 to 200 directory files

depending on the directory name lengths. If you have a pathtable that is 10

sectors long, you will want at least 10 sectors of memory buffers to hold

the pathtable or you may risk re-reading sections of the pathtable on every

file open which will be very costly.

The most important point you can learn is that you can vastly improve your

file open speed by making sure you have enough memory buffers. If you are

repeatedly trying to scan a 10 sector directory file (approximately 400

entries) and you only have 4 sectors in the sector cache, the cache is going

to work against you because you will end up churning it to death. If you

allocate 14 sectors for example (/M:14), then the whole directory file will

find its way into the cache and you will stop hitting the disc. The

difference in speed may be several orders of magnitude. A safe bet is to

recommend reserving as many sectors are in the pathtable plus the number of

sectors for the largest directory plus 2. The last two are reserved for data

sectors and internal dynamic data. This formula is complicated with multiple

drives because the buffers are not tied to specific drives and are shared

and because not all drives are active at the same time.

Another rule, do not rely on the file system to do your searching for you.

If you are performance conscious, finding a chunk of data by looking for it

with a file name through the file system is expensive if speed is a great

concern. 99% of the time, locating data through the file system is fine

because the cost is a single one time operation but if this is repeated

often enough, it may pay to do some of the work yourself. What can be better

is lump everything into one big file and cache your own hierarchy, indexing,

binary trees, or whatever searching scheme you choose to use to get you to

the data you need rather than asking for the file system to tell you where

it is.

ŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰŰ

Questions and Answers

Q: Does the /V option to MSCDEX actually cause a message to be displayed? I

can't make it do anything. I use:

MSCDEX /D:CDDRV /V /M:8

A: Yes, a series of statistics are displayed. MSCDEX uses INT 10 function

0eH to write to the console so if for some reason you are trapping this or

have software that captures this and doesn't display bios output to the

screen the information will not appear. All screen output uses this function

so if any output appears on the screen after loading MSCDEX, It is not clear

why this additional information does not appear.

Newer versions (2.0 and above) use the DOS write handle call for output

which will fix problems of I/O redirection of this output.

As it turned out, the machine that had this problem was not a strict IBM

compatible machine and did not emulate a PC at the BIOS level. Consequently,

the messages written with INT 10h were not displayed.

Q: Is it normal for MSCDEX to hang the system if an error is returned by a

driver's IOCTL function? Wouldn't an error message be better?

A: The only ioctl calls sent to the driver in version 1.01 are to request

the device header address and media check. Media check calls will never hang

no matter what is returned so long as the driver returns. Illegal values may

not do what you want but it won't hang. Request device header address may

hang if the driver fails to set error conditions correctly as DOS expects

them as DOS will return from my ioctl call without error. MSCDEX will then

assume the bogus return values are correct and jump to a random location.

Q: Does MSCDEX do anything that should preclude its working in a non-IBM-

clone machine?

A: Except for the INT 10 problem mentioned earlier, no. If you can identify

any problems whatsoever, we would be happy to learn about it but as yet, we

have heard of no other problems. If your machine runs MS-DOS version 3.X,

it should be capable of running the extensions correctly. As for

driver/drive-controller compatability problems, that may be another matter.

We do not guarantee anything about these because we do not write the device

drivers or design the hardware interface boards and cannot make any claims

concerning them. It is up to the drive manufacturer or device driver writer

to do this kind of compatability testing.

Q: Based on what I read in the spec, I decided to support only HSG type

addressing which seems to be allowed by the IOCTLI function #6 (Device

Status). I return 4 bytes of 00h if that function is called. I would have

thought that would be one of the first calls MSCDEX would make (after "Find

Header") but so far it hasn't called the status function. How will MSCDEX

know enough not to use Red Book addressing if it doesn't check status

first?

A: In version 1.01, ioctl function #6 is not called. This is not to say that

in a future versions it will not (in fact it will). Since all device drivers

must support some default functionality and as MSCDEX only uses the basic

default now (only High Sierra addressing for example), it wasn't a problem

that it didn't call this function to find out about non-default features

that were supported.

Some software is being written that controls audio on a CD-ROM that expects

Red Book Addressing and checks the device status to see that it is

supported. The conversion algorithms are fairly simple and code fragments

are provided.

Q: Can you provide more info on how the READ/WRITE device control string

should work. Does the read device bytes command get information that was

written by the write device control string?

A: As of yet, no one to our knowledge has used this. There are a couple

other features of which this can be said. Again, this is not to say that

they won't be used at some later time.

The purpose of these commands was to allow a standard way of delivering

commands that were not specified in the CD-ROM device driver spec to the

drive. For example, sending SCSI command strings and reading the responses

from the drive. This function is deliberately open-ended and vague because

it was intended to provide a catch-all mechanism for application programs to

communicate requests or request data in ways that were not specified by the

device driver spec. For application programs to use these functions they

have to know the driver supports these functions and also how to communicate

with that specific drive. The mechanism would let the driver do what it does

best and worry about which ports and interrupts to use. This relieves the

application program from these details and allow it to deal with controlling

the device at a higher level.

Right now, if the driver does not support these functions, it should

return an error for Unknown Command. One could test whether these two

function were supported this way.

ÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄ

Note:

If there are commands which you feel should be supported by the device

driver specification, please communicate them to us and we will consider

adding them if they are of sufficient general interest.

ÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄ

Q: The version of MSCDEX I am using is 1.01 dated 3/20/87@11:06am and is

marked Evaluation Copy. Is this any less functional than a "licensed" copy?

A: No. There may be a few more bug fixes in the licensed copy that are not

in the Evaluation copy but none that should prevent correct operation. If

you do find any bugs, please let us know as we will fix any bugs in the

software that are reported. Our policy is to try to reward users that report

bugs with updates of the software that fix their problem.

Q: Would it be possible for me to get a sample source file for a driver?

A: Yes. Licensees are given source code to device drivers for:

SONY    - complete

HITACHI - missing two modules. These are owned by Hitachi so we

cannot supply them, though Hitachi will.

PHILIPS - this driver communicates with the Philips CD-ROM driver

supplied by Philips.

Q: How can an application access CD-ROM drives that are subunits of one

driver? The IOCTL calls do not take an argument for subunit. MSCDEX seems to

handle this OK since when I do a directory of each CD-ROM in turn it

accesses the correct drive. I do not see any clean way for an application

to, for example lock the door on CD-ROM drive G: which is the third drive

handled by the driver.

A: Requests all have a sub-unit field in the request header. Commands that

one would expect to be directed to a specific drive, such as open door, are

targeted at a particular drive through the use of the sub-unit field.

Q: What is the current release version number of MSCDEX? The version I have

is version 1.01 that is marked EVALUATION COPY. When can I get a final

release version of it? Also will that final version include the changes to

do all I/O through MSDOS (i.e. no INT 10h).

A: The most recent released version is version 2.0. You can purchase this

from a number of licensees including all drive manufacturers. An Extensions

availability list is attached.

Q: Why doesn't MSCDEX allow IOCTL access via the drive letter (i.e. DOS

func. 44h subfunc. #4,5), as if the CD-ROM were a drive. I understand that

the driver is not a block device, but this is being handled already in some

way since you allow a user to perform file I/O to a CD-ROM making it appear

to be a block device. It would seem that all that would be necessary to

accomplish this is to intercept IOCTL calls in the same way that file access

calls are being intercepted.

A: MSCDEX doesn't presently hook int 21h, which is what this would involve.

It's doubtful that this will change. It's not that much more difficult to

open the file and send an IOCTL to the handle. File access calls are not

caught at an INT 21h level but are caught from within DOS at another

interface. CD-ROM drives are far more like network drives than traditional

MS-DOS FAT file structure block drives and their drivers. For example, try

to FORMAT a CD-ROM drive and you'll see. Part of all this prevents IOCTL's

to the drive letter from being directed to the appropriate driver.

Q: Why not allow access to the PLAY, STOP and SEEK functions via the INT 2Fh

entry point as is allowed for READ LONG. This would be much simpler than

requiring the application to locate the driver header and then find the

STRATEGY entry point and create request control blocks etc. This is a lot of

code to start the music playing!

A: It's preferable to see MS-DOS provide extensions to the application

program interface for audio/video control (new int 21h calls). The reason we

haven't included play, etc. in the int 2Fh interface is to avoid loading

down MSCDEX with additional functionality that most people don't use. Your

suggestion would only move that code from the CD-playing program into

MSCDEX. It makes your program smaller, but in the whole, doesn't buy much.

As time goes on, this may change and some of the functionality may move into

the int 2Fh interface.

Q: Shouldn't the specification eliminate the need for the application to

OPEN the driver by name, This is especially important in systems where the

driver creates a new driver header for each CD-ROM drive. MSDOS allows so

few file handles to be simultaneously open as it is that requiring

applications to open even more is very bad.

A: Simply close the driver handle after you have located the device header.

You no longer need to communicate through DOS to control it, so free the

handle and make it available for other programs to use. With version 2.10,

it is no longer necessary to OPEN the device driver in order to communicate

with it. Applications can communicate with the device driver using Send

Device Driver Request.

Q: Have you considered adding an addressing mode for the PLAY AUDIO function

that would allow the application to specify the PLAY address by TNO instead

of block number or min/sec/frame?

A: This has been considered but has not been added to keep from complicating

the device drivers unnecessarily. At the moment, most CD-ROM drives are used

without audio so our intent was to put what was needed for audio support in

the audio playing software. In addition, we chose to keep the interface

simple to leave more latitude for changes to the OS/2 API that may include

newer data types like audio and video. Nonetheless, this may be added in the

future. In the meantime, audio playing software has the extra overhead of

reading the audio table of contents and interpreting the tracks itself.

Q: Why is there no CLOSE TRAY function in the driver spec? The CD-ROM drive

we are using (Toshiba SCSI) has this capability but I see no way to use it

via the extensions. Why allow the user to OPEN it without allowing him to

close it?

A: A close tray command has been added.

Q: It seems that there should be bits in the Device Status word to indicate

whether a driver supports Reading/Writing device control strings.

A: Reading and writing device control strings was put it in as a catch-all

for anything that was missed so that application programs could send

specific commands through the device driver to the device if they understood

the device and knew how to communicate to it. A manufacturers CD-ROM

diagnostic program would be an example of a program that might choose to

communicate with the drive in this way. If the driver does not support these

functions, it should return an error. One can test whether these two

function are supported by testing if the error returned is for Unknown

Command.

Q: In the spec, treatment of the BUSY bit in the status word with regard to

the PLAY AUDIO function seems to assume only one CD-ROM drive. What happens

when the user has two or more drives each of which want to be playing music?

How does the application tell whether the desired drive is busy? It would

seem better to use some of the bits in the upper word of Device Status to

indicate BUSY for each drive. Perhaps allowing 8 or 16 drives.

A: Requests all have sub-unit numbers associated with them. A request for

service from one sub-unit may report that the drive is busy at the same time

another sub-unit was not busy. The sub-unit field is used to distinguish

requests between the drives supported by the driver. The busy bit serves as

an indication of drive status for the sub-unit the request is for.

Q: If a CD-ROM file is opened in write-only mode, no error occurs.

A: True. The same happens on a floppy drive with a write-protect tab on it.

If you do have a handle to a CD-ROM file in open_for_write mode, as soon as

you attempt to write, you will get an error. The correct model is to

duplicate the behavior of a file that has been set READ-ONLY. Read-only

files return error_access_denied if you try to open them in open_for_write

or open_for_both modes. MSCDEX has been changed to duplicate this behavior.

Q: What other non-MSDOS calls are issued by MSCDEX besides INT 10h?

A: INT 2Fh - dos callbacks

INT 67h - expanded memory

INT 10h - all INT 10h calls went away with version 2.00 which uses DOS

write handle instead.

Q: Why does MSCDEX do the READ LONG PREFETCH call after it has done a DEVICE

CLOSE call? Is this intentional?

A: MSCDEX version 1.X never issued device open or close. These were issued

by DOS as part of driver initialization. MSCDEX version 2.00 now issues

device open calls and will precede the prefetch call.

Q: In the device driver spec, it says that if more than one unit is

supported by the driver that this field should be set to the number of

units. I suspect that this is wrong since this is not a block device. As far

as I can see, this field should only ever be set to one since each unit will

actually have its own header with its own unique name.

A: CD-ROM device drivers are a hybrid of block and char device drivers and

are not technically legal as one or the other. Block drivers make some

assumptions about the media format that aren't meaningful for CD-ROM and

don't have a read call that can deal with CD-ROM's large data space. They

were made as char devices with some additional calls and rules. One of the

changes that was made for CD-ROM device drivers was to allow multiple sub-

units for the device so the treatment of this field is correct as specified

even though CD-ROM device drivers are character device drivers.

If one has more than one CD-ROM drive, one can approach supporting them

from several ways. One could have separate device drivers for each drive and

load one per drive, have a single driver with multiple device headers, or

have a single driver with one device header that supports sub-units. This

last method is borrowed from block drivers. For the case that the drives and

drive commands are all the same, using sub-units will allow you to

distinguish between which drive receives which command. The alternatives

clutter things up with drivers or device headers. Sub-units may not be legal

character device driver fields but conceptually, they're the right thing.

Since CD-ROM device drivers could not be block drivers and had to be char

device drivers, some liberties were taken with the specification to merge

the best of both specifications.

Q: Is there any support through MSCDEX for WRITE LONG? I have a need for

this to support a CD mastering system. I would like to be able to treat a

WORM drive as a CD-ROM and allow writing to the drive once to create a

master and then be able to test it out by using it as CD-ROM to verify that

our data has been correctly stored in "High Sierra" format.

A: Such a call exists. It only serves to define a standard interface for CD-

ROM device drivers that are running over re-writable mediaÄÄsuch as a CD

mastering system. It is in the latest copy of the driver spec released with

version 2.00 of the CD-ROM Extensions.

Q: How important is it that I should support RAW mode access in my driver?

What would this typically be used for?

A: Not important now. No drive presently support reading raw at the moment

that we know of. Since drives and their command capabilities are hardware

dependent, you would know based on the capabilities of your hardware if you

wanted to support it. This function was added for completeness. A standard

way was needed to define how to get at the 288 bytes of EDC/ECC if drives

allowed access and to have an avenue prepared if people found useful

applications that would not use EDC/ECC where they wanted the additional

space (such as gracefully degrading low-fidelity audio or graphics). It will

be useful for copying audio information or for audio systems that will want

to be able to manipulate audio tracks. Many people have expressed interest

in having this capability.

Q: Driver spec page 13 (in the structure for the UPC Code function): "The

UPC/EAN code (last 4 bits are zero)". Does this mean the low order or high

order 4 bits?

A: This is less ambiguous if you read Red Book under mode-2 of the Q-channel

info. This is now clarified in the UPC Code call. It should be the low

nibble of byte 7. Red Book specifies that MSB comes out first so the high

nibble will contain the 13th nibble of the UPC code and the 14th nibble will

be zero.

Unfortunately, scanning for the UPC code is time consuming especially if

it is done by the software. This is due to the design of the q-channel in

Red Book. It's a pity because this could be a useful number to verify the

correct disc has been inserted. Most CD-ROMs do not have a UPC code or have

it zeroed out. The same seems to be true for CD-audio as well. We believe

that CD-ROM drives should scan for the UPC code as they read the Table of

Contents when initializing from power up or a new disc. If the hardware does

not do this, the UPC code has to be scanned by polling the q-channel which

may occasionally miss the UPC code.

Q: It would be nice if the device driver spec included a list of what types

of disk access functions would and would not work so that users could get an

idea of what utilities and applications will and will not work with the

extensions.

A: The device driver specification describes just what is necessary for

writing a CD-ROM device driver. The information you would like concerning

things such as INT 25h/26h not supported as well as the behavior

CHKDSK/FORMAT/etc belongs and is mentioned in the MSCDEX overview.

Q: If I have a low priority request and if the system has time, can the

prefetch request read into and transfer the data into a transfer address?

A: We have looked at this for some time but the bottom line is that

asynchronous I/O under DOS is virtually impossible to support in all cases.

Because of this it is unlikely we will be implementing this or designing it

into the device driver interface. There is no way for MSCDEX or the CD-ROM

device driver to know that the transfer address is still valid because DOS

never notifies MSCDEX or the device driver if the requesting process was

been terminated. The request runs the risk of writing over another program.

The best approach now is if the driver wants to, it can reserve internal

buffer space for data from the disc and put prefetched data there. Then it

can copy the data to the read transfer address once the read request finally

arrives. Alternately, some of the caching or prefetching can reside in the

CD-ROM controller or in the drive itself. True asynchronous reads will have

to wait for OS/2.

Q: Is there any status indication that the transfer has occurred? or some

interaction with the read long command?

A: There is no way to tell if a prefetch request was successful or the state

of it. The prefetch simply provides a hint and the read request later is the

request that finally expects delivery of the data.

Q: Read (ioctl input) When audio is playing, can location of head move?

A: I'm not entirely sure what you mean by this question but I will attempt

to answer a couple different ways and hope I provide the information you

need.

While audio is playing, the head is moving on the CD. If the driver

receives a command asking where the head is, the driver should ask the drive

where the head is presently positioned and report that. So as audio is

playing, an application program that is controlling the audio can monitor

the progress of audio playback and can either synchronize actions with the

audio or report the present location to the user. For example, a program to

play audio discs would be able to report the present track number and time

elapsed on the computer monitor much like a CD-audio player reports things

on its LED display. If the driver is sent a command that requires the

movement of the head or a change in the state of the machine (SEEK, READ,

INITIALIZE, PLAY AUDIO etc.) then the audio will be interrupted so that the

drive can perform the new request. If the drive receives a command for

status or some other function that does not require the head to be moved or

the state of the machine to change, then audio play should continue.

Q: Are the parameters of Audio Disk Info and Audio Track Info BCD or

binary?

A: The parameters were vaguely specified in the device driver spec. The spec

has been clarified. They are as follows:

ţ  Audio Disk Info

binary    Lowest Track Number (1-99)

binary    Highest Track Number (1-99)

red book  Starting point of lead-out track

ţ  Audio Track Info

binary    Track Number (1-99)

red book  Starting point of track

as is     Track control information

The track control information is not a BCD or Binary number like the track

number. The byte contents as it appears on the disc should be carried

through unmodified by the driver and is interpreted according to the

Philips/Sony Red Book standard.

Q: Are the parameters for the Audio Q-channel info BCD or binary?

A: The parameters are as follows:

ţ  Audio Q-Channel Info

as is     Control and ADDR byte

as is     Track Number

as is     Point or Index (X)

red book  MIN/SEC/FRAME

zero      ZERO

red book  AMIN/ASEC/AFRAME or PMIN/PSEC/PFRAME

The notes on when to convert from BCD to red book addresses for

MIN/SEC/FRAME, AMIN/ASEC/AFRAME and PMIN/PSEC/PFRAME is already fairly clear

in the spec. The other fields are passed through as is from the disc. For

additional information, see the two code samples AUDIO.C and AUDIO.ASM.

Q: Must we support read sub-channel info and the read upc code?

A: No. It is not necessary that these be supported. At the present time and

in the forseeable future, MSCDEX will not be issuing these commands though

some applications may wish to.

Read Sub-Channel Information

At the present time, nobody is using channels P or R through W. The read

sub-channel command was added to provide a standard way to specify access to

these channels in the event that they are used and to specify in one way or

another access to all information on a CD-ROM. The error detection or

correction for information in these channels is not as good as it is for

data returned from READ commands.

Read UPC Code

This command is conceivably much more useful. It is advised that it be

supported so that application software can exactly identify the CD-ROM in

the drive. This may be a consideration for audio software that wishes to try

to identify inserted audio discs (if the UPC code is present) or for

software that is specifically tied to a version of a CD-ROM. Unfortunately,

the standard does not specify a specific location for this information so

that unless the hardware reads it on disc initialization as we recommend,

the device driver must poll the q-channel and hope that it locates it. See

also the sample code in AUDIO.ASM.

Q: My driver seems to work ok except that whenever I connect to a sub-

directory and do a directory, I am suddenly back in the root directory

again. What's going wrong?

A: What is most likely happening is the driver is returning an incorrect

value for MEDIA CHECK and MSCDEX thinks that the disc is changing all the

time. When this happens, MSCDEX rereads the volume descriptors and pathtable

and reinitializes what it knows about the disc and changes the current

working directory back to root as if the drawer had been opened, the disc

removed, and then reinserted. This will be accompanied with a larger amount

of disc activity than one would expect for a simple directory scan. Fixing

the driver to return the correct value when asked for a media check will

correct this behavior.

Q: What is the best way for my application to know if the disc has changed

since it was last accessed?

A: Use the DOS function find first and look for the volume id. When the disc

has been read and MSCDEX has already initialized the internal information it

keeps for each disc, this is a relatively inexpensive operation. The

information is in memory and the disc does not have to be touched, so

checking the volume id is very quick. Only if the disc has been changed does

the disc have to be touched. This operation takes considerably longer than

if the disc was not changed but even so, this has to be done anyway because

MSCDEX has to read and initialize what it knows about the new disc so it can

report the volume id correctly so the application can know if the disc in

the drive is the one that it is looking for.

Q: When I do a directory, the first couple filenames are either duplicated

or they are random characters. What might cause this?

A: The problem comes from having the incorrect bytes in the file identifier

field for the first two directory entries. The first directory entry in each

directory file is supposed begin with a copy of the directory record for

that directory file followed by a copy of the directory record for the

parent directory (also known as '.' and '..' on Unix or MS-DOS). The

filename or directory identifier is supposed to be 1 byte long

Q: When I do a directory, the first couple filenames are either duplicated

or they are random characters. What might cause this?

A: The problem comes from having the incorrect bytes in the file identifier

field for the first two directory entries. The first directory entry in each

directory file is supposed begin with a copy of the directory record for

that directory file followed by a copy of the directory record for the

parent directory (also known as '.' and '..' on Unix or MS-DOS). The

filename or directory identifier is supposed to be 1 byte long and the

contents are supposed to be 0 for the first directory entry and 1 for the

second directory entry. This is discussed in clause 6.8.2.2 of the ECMA

standard or the ISO-9660 proposal. Several places on the disc in question,

you have a 1 where there should be a 0 and in one directory, the file

identifier consists of 0x8A which is why DIR in that directory begins with

an "e". Incorrectly formatted discs will not be handled by the extensions

correctly. This is why it is a good idea to test your disc image using

MSCDEX before you press a disc to make sure your data is formatted correctly

and as MSCDEX expects it.

Q: I have a directory file that is 4Kb long but when I do a DIR in that

directory, it is slower than usual and random filenames are printed out. I

can tell by watching the device driver commands that MSCDEX is asking for

sectors far beyond the end of the directory. I can see how this might

account for the random filenames but why is it scanning so far?

A: Problems such as this result from having with multi-sector directory

files that include empty sectors in the directory file. The High Sierra

specification does not allow you to have empty directory sectors at the end

or to have gaps in the middle. The problem stems from the fact that your

directory length is too long. For example, for the disc in question, the

root directory begins at sector 28 and its length is 4096 bytes but the

second sector is completely blank (all 0's). This confuses MSCDEX because it

does not expect to see empty sectors.

Because LEN_DR of what would be the first directory entry in sector 29 is

0, MSCDEX thinks that there are no more entries in that sector. When it

reaches the end of the entries in each sector, MSCDEX rounds its offset up

to the start of the next sector:

offset += (SECTOR_SIZE - 1);

offset &= ~(SECTOR_SIZE - 1);

Since the offset did not changed when scanning sector 29 (there were no

entries in this sector to increment the offset) the above rounding algorithm

does not change the offset (2048 in, 2048 out). This is why MSCDEX continues

reading beyond the end of the directory file at sector 29 because the offset

did not grow past the directory length. MSCDEX continues reading blank

sectors (sectors 29 through 49 are all blank) until it reaches the first

non-blank sector.

It looks like what you are attempting to do is implement updatable High

Sierra and you want to allocate the directory space ahead of time and fill

it in later as needed. The format you are using though is not valid High

Sierra and also incurs the cost of reading the blank directory sectors at

the end of every directory. Instead, you should record the correct High

Sierra directory length in the directory length field for that directory (in

this case 2Kb). What remains is finding a place to store the value which

tells how many blocks have been reserved for each directory file. There are

a number of places this can be done, either in system/application use fields

in the directory record, in an XAR, or in a separate file either inside or

outside the High Sierra directory structure. The first is the easiest

approach to take.

Be aware thought that CDI may have plans to use the system use field in the

directory record so you may want to investigate Philips' plans to make sure

whatever scheme you use meshes well with what CDI has in mind. MSCDEX will

ignore the system use or application use information recorded so you can

store what you'd like in the form you like (ascii or binary). You may also

want a final pass over the directory hierarchy before mastering to remove

extraneous information from the directory record.

Q: I noticed that Function Request 0 Get Number of CD-ROM Drive Letters may

not always return unambiguous results. Suppose I start the network first and

use one of the drive letters for a network drive (F:). When I start the

Extensions, it will begin assigning drive letters after the last used drive

letter (C: on my machine). If I have 4 CD-ROM drives on my system, they

will be assigned drive letters D:, E:, G:, and H:. Function 0 returns 4 in

BX for the number of CD-ROM drives and 3 in CX for drive letter D:

correctly. But as you can see, the CD-ROM drives do not use contiguous drive

letters so I cannot deduce from what this function returns that drive F: is

not a CD-ROM drive.

A: That is correct. This is why function 0Dh Get CD-ROM drive letters was

added. To get an unambiguous list of CD-ROM drives, use this function or use

function 0Bh CD-ROM Drive Check to tell if a drive letter is for a CD-ROM

drive.

Q: Is it possible to do an absolute read using the Extensions. I am trying

to read mode 2 (uncooked) data using Function Request 8 Absolute Read. I use

a normal device I/O to turn off error correction and perform a read but all

I get back is 2048 bytes of data instead of the full 2356 bytes. Is there

another way in Int 2F to get the data uncooked?

A: Not at present. If you want to get at the data including error correction

code, you will have to communicate directly with the device driver. The

Extensions will provide the location of the device drivers if asked.

Q: Is it possible to access a non-High Sierra disc with the Extensions using

an absolute disc read?

A: One can use either the extensions to read a non-High Sierra disc using

INT 2Fh or one can communicate directly with the device driver to do this.

The device driver itself makes no distinction between High Sierra and non-

High Sierra discs so it can be used to read them although the burden of file

system translation and reading then falls on the application talking to the

driver. The INT 2Fh Absolute Read function simply packages the request to

read and sends it directly to the driver and returns the result.

Q: What we have done is, in AUTOEXEC.BAT, first loaded the MS-DOS CD-ROM

extensions and then the MS-NET software. The error message is "Redirector

already installed". The network software is then not loaded. We are using

MS-NET 1.1 in an HP product called ThinLAN. Any hints as to what they should

try next?

A: MSCDEX is a CD-ROM "redirector". It hooks into DOS the same way the

network redirector does to get requests for file access to files that are

not on local hard/floppy disks. As far as DOS is concerned, CD-ROM drives

look just like network drives. DOS passes all file accesses through the

redirector interface to the network redirector which in turn sends file

access requests out over the net. MSCDEX splices itself in front of the

network redirector and takes requests belonging to CD-ROM drives and passes

the rest to the network redirector.

The problem is that the network redirector code assumes that there will only

be one redirector installed (itself) whereas MSCDEX does not make this

assumption. If the network redirector is installed after MSCDEX (before it

in the interrupt chain), it will process all requests from DOS and never

pass any CD-ROM requests through to MSCDEX. For this reason, MSCDEX has to

be installed after the network redirector (before it in the interrupt chain)

and so MSCDEX prevents the network redirector from installing afterwards to

ensure this. Since you installed MSCDEX first, the network believes a

redirector is already installed so it does not install itself which is what

you are seeing. In order to install both, simply install your network

software first and MSCDEX second and you're set.

Q: CHKDSK, ASSIGN, and SUBST report that the CD-ROM is a network disc. Why

is this?

A: From the above explanation, you understand that to DOS, the CD-ROM drives

look like network drives. The programs CHKDSK, ASSIGN, and SUBST check the

same fields DOS does and think the same thing. There is no way to get around

this.

Q: RENAME gives error message "Duplicate file name or File not found"

instead of something that makes sense such as "Access denied" or "Can't

rename CD-ROM files".

A: The error message is coming from the code for RENAME and not MSCDEX. The

error condition is being returned correctly but the error code returned by

version 1.01 is correct according to DOS documentation. The problem seems to

be that there are two error codes for access denied - 5 and a special one 65

which is error_net_access_denied which is returned by the network redirector

when it has a problem. MSCDEX version 2.00 returns error code

error_net_access_denied and so RENAME now reports "Access denied".