Hi,

here comes the technical info about the new moviegrabber(tm)

PC-Hurricane(tm).

===========================================================================

This description was still written for our old DOS Software, that still

comes with the PC-Hurricane, for those who don't like to use Win3.1....

---------------------------------------------------------------------------

It is a hicolor color YUV (4:1:1) realtime video movie digitizer which

can store 25 frames/sec(PAL) or 30 frames/sec (NTSC)  into Expanded Memory.

It has support for ET4000 Hicolor and true color boards and all VESA1.2

compatible SVGA cards.

The software is a userfriendly DOS (or win3.1) software, which displays the

incoming video signal in almost realtime (about 5-10 frames/sec) on a

Hicolor board in 32768 colors/pixel or on a 256 color board.

This is okay for video purposes, cause the noise inside a video movie

almost does already the dithering to a true color quality.(simualar to

24 bit quality)

If you don't have a Hicolor card inside your PC, then there is also a

256 color version driver, which displays the video signal in almost

realtime in 3-3-2 color quantisation. (still the TGA pics are saved as

16 bit Hicolor/pixel, so quality from the saved output is the same as

in the Hicolor software driver)

If the sequence arrives, which you wanna digitize, just hit the space

key.

Then it is digitized into the Expanded Memory in realtime with 25 or

30 frames/sec maximum rate. This can be adjusted to 12.5 or 8 or 4

frames/sec !

During this storing process, you will not see any signal on your VGA

monitor, but if the RAM is filled with e.g.396 frames at 320x240, then

you can already watch the digitized movie play of the Expanded Memory!

It plays in slow motion the YUV coded fields to the SVGA card via the

ISA bus. This gives a slow motion Hicolor digital movie out of

the RAM !

So if you have 32 MBytes of Ram on your motherboard, you can be happy.

Then you can store about 16 seconds of realtime MPEG resolution

320x240 true color video with one shot ! Or doubble this, if you just

only take 12.5 frames/sec...

With the enclosed software you can save this with one command to

single TGA pics onto the harddisk or save it as a complete Hicolor

movie (uncompressed, big file size).

PC-Hurricane also digitizes the resolutions:

384x288, 360x240, 320x240 and 320x200. So max. resolution is 1 PAL

field (384x288).

Field digitisation is preferred, cause it gives no Interlace

artifacts, due to motion...

So if you want to make a FLI file out of the 16 seconds sequence, no

problem, just start DTA18g (Dave's Targa Animator) and it produces a

FLI animation file out of the single TGA pics..

DTA comes with it. This way you can also very easy produce AVI movies

for Video for Windows. Just load the FLI file into the Video Editor

of Video for Windows and compress it into a *.AVI file..

Also, if you have the Xing Technology software only MPEG encoder,

it will be possible to compress this about 400 single

TARGA(tm) pics with just one second command to a MPEG movie !

So you have in about 5 minutes of work a ready to display MPEG movie !

PC-Hurricane has NTSC/PAL switchable. So it can also digitize NTSC

video. It has Video(FBAS) and Y/C(SVHS/Hi8) inputs.

Best regards, Stefan Hartmann,c/o Gatz & Hartmann

email to: harti@mikro.ee.tu-berlin.de

Gatz & Hartmann

Ingenieurbuero fuer Multimedia-Anwendungen

Fehrbelliner Str. 32

13585 Berlin

Tel: 030-375 55 68

FAX: 030-375 55 68

International area code: ++49 30-

P.S.: We are accepting all other trademarks (tm).

Hi,

well, MPEG 2.1 for windows3.x is now available !

It is the digital Video player via a software only solution !

It displays in a 320x240 window (or 160x120)under win3.x a realtime decompressed

digital video !

Decompression is done only by software and it reaches

30 frames/sec on a 486 PC !

The new version has a very enhanced picture quality, because the

compression ratio with the encoder can now be adjusted !

The very new thing is the WAV-Sound and MPEG-Audio support !

So if you have a soundcard inside your PC, you will have a real

video-clip with accompagning sound !

We currently have 3 demo disks with the full featured Player, version

2.1 and lots of animations on the disks.

We ship it to you with airmail.

Then the price is 40 US$ inclusive shipping and handling and airmail

delivery. You have to send a cheque then first with your order to:

Gatz & Hartmann

Ingenieurbuero fuer Multimedia-Anwendungen

Fehrbelliner Str. 32

13585 Berlin , Germany

Subject: Re: Saving presentations to video tape

===============================================

Hi,

for recording SVGA animations or VGA (also 24 bit support)

presentations we currently have

the following products over here in Germany:

We currently have three solutions for you:

1. 24 bit to TV converter box which is a box connected outside of the PC to

the SVGA card via its VGA connector. There must be a TSR program run

in the background to change automatically the scanning frequencies of

the SVGA board. This works also very well with win3.1

It supports up to 800x600x 65K colors (only PAL) and 640x480 with 24 bits

color (true color) on the Genoa 7900 ET4000 SVGA board.

It has  Video (FBAS) and Y/C (SVHS/HI8)

outputs.

There is also a version of this box with additional RGB output, so you

get the best picture quality on a TV-set with SCART - RGB-inputs..

The VGA Monitor is also connected to the 24 Bit to TV converter box,

so if you don't start the driver software, your system is like normal:

You see the picture on your SVGA monitor.

The DC-power comes from the keyboard connector, so you don't have to

fiddle around with AC-DC-aptor power supplies !

Just don't start the driver software, if you don't need video out !

The box doesn't interfere with your daily work. Just forget it behind

you PC.

If you need video out, just start the 2 KBytes driver software and

then you can toggle between video out and VGA monitor display with a

hot key !

The driver software is adapted to SVGA FLIC-files from Autodesk

Animator PRO to give a 640x288 x256 colors Fullscreen-Overscan display

on PAL TV-sets. This works by using double scanning with 50 Hz

Non-Interlace !

Every scanline from the 288 is scanned twice, so it gives 640x576

Fullscreen display ! (with no flicker)

This way you can record your favourite SVGA animations to tape without

seeing any black borders !

There is also a NTSC version available, which works only up to 640x480

with 16, 256 ,32K , 65 K and 24 bits colors !

On NTSC model the 640x480 mode is already Fullscreen !

It also works with the new Hicolor FLI player, so you can play and

record hicolor animation directly from a RAMdisk to VCR tape..

You don't need NO single frame accurate expensive VCR anymore !

Cause it works also with win3.1, you can record all your daily windows

application work to video tape !

If you have a presentation program like Microsoft Powerpoint, no

problem ! Just record your presentation to video tape to show it via

video cassette to everyone, who has a VCR !

In windows the 24 bit to TV-box works in 50 or 60 Hz Interlace to

display the 640x480 or 800x600 (only PAL) scanlines.

This gives a little flicker on Menu-buttons, which have only very

small horizontal lines with big contrast in color. So use an

appropriate color palette under windows to reduce flicker..

With the Y/C output (SVHS/Hi8) you almost get SVGA monitor quality on your

video tape !

2. VGA2TV PRO adaptor. This is an internal board which is hooked inside

the PC to the feature connector of the VGA card. So the VGA or SVGA

board must have a feature connector !

The VGA2TV PRO samples the contens of the VGA RAM into its own dual ported

RAM and gives it out with the Video scanning frequencies to its Video

(FBAS) or Y/C output.

It also has a Genlock possibility. So it can be used to do titling in

front of a holiday movie video tape running in the background of the

computer graphics titles !

Input of the external video source could also be FBAS or Y/C.

VGA2TV PRO comes with integrated Flicker fixer.

It has the flicker filter for making the

Interlace flicker much less visible. It also has OverScan/Underscan

feature and the position of the screen on the TV set could be

controlled via software.

Max. resolutions to convert VGA to TV with this VGA2TV PRO board is

640x480 with 256 colors ! It has the advantage, that it does not need

any software ! Just plug and it plays !

Does not work with Cirrus Logic VESA Local Bus graphics cards !

3. GL 800 Genlock Interface PC-Card.

====================================

This is our brand new own development.

It works like the VGA2TV Pro card, but it has also 800x600x256 support

and also can Overlay, MIX and Fade-In and Fade-Out the VGA graphic over the

background video input.

It does not have a flicker fixer, but works very stable.

It has RGB, FBAS and Y/C outputs and Y/C and FBAS inputs.

We recommend this board for fullscreen PAL output from 800x600 SVGA screens.

This board is only 799.-DM incl. 15 % VAT.

This board also works with VESA Local Bus cards !

Please let me know, what you need.

Best regards Stefan Hartmann.

email to: harti@mikro.ee.tu-berlin.de

Gatz & Hartmann

Ingenierbuero fuer Multimedia-Anwendungen

Fehrbelliner Str. 32

13585 Berlin , Germany

Tel: ++ 49 30 - 375 55 68

FAX: ++ 49 30 - 375 55 68

===========================================================================

Ingenierbuero Gatz & Hartmann

Fehrbelliner Str. 32

13585 Berlin, Germany

Tel: 030- 375 55 68

FAX: 030- 375 55 68

International: ++ 49 30-

Produktuebersicht:

==================

Genoa 7900 SVGA 24 bit true color Multimedia Graphik-Karte.... 229.-DM

ET4000 Chip, 640x480x24 bit Windows3.x Treiber, etc..

Genoa 8500 SVGA Win3.1 Accelerator Karte, true color...........249.-DM

Genoa 8500VL SVGA Win3.1 Accelerator Karte,Vesa LB true color..269.-DM

Genoa VideoBlitz Karte, P9000 Weitek chipset, 1600x1200x256 NI!.. 999.-DM

24 bit to TV Konverter Box, passend zu den Genoa Karten.......349.-DM

24 bit to TV Konverter CARD, passend zu den Genoa Karten......299.-DM

24 bit to TV Box mit zusaetzlichem RGB-Video Ausgang auf Scart..449.-DM

24 bit to TV CARD mit zusaetzlichem RGB-Video Ausgang auf Scart.399.-DM

VGA2TV Pro Genoa Genlock-Karte,intern, Computergrafik ueber Video..999.-DM

GL 800 Genlock card, up to 800x600x256 support.....................799.-DM

PC-Titler Standard,Video Vertitelungs-Software fuer VGA-Karten.199.-DM

PC-Titler Deluxe, Profi-Version vom PC Titler, mehr Effekte....399.-DM

The Video Director, PC gesteuerter Video-Schnittplatz..........499.-DM

Xing MPEG Encoder, digitales Video per Software,fuer TGA+BMP-pics...199,-DM

MPEG Demo, 3 Disketten mit dem neuen WAV-support MPEG-player ....39.-DM

PC-Hurricane,25 Bilder/sec realtime movie-grabber(Film Digitizer).499.-DM

PC-Hurricane mit Xing MPEG Encoder und Player Software............699.-DM

FG-02 , Hicolor DTP Framegrabber, 512x512 pixel Videodigitizer.499.-DM

VT-Express,sehr schneller JPEG-Bild viewer, ca. 1.2 sec/Bild...199.-DM

Hicolor(32768Farben) Animations-Software f. TGA-Animationen ...149.-DM

Alle Preise inclusive 15 % Mehrwertsteuer. Verkauf nur an Endkunden.

Haendleranfragen bitte erst ab 5 Stueck/unit.

Sie koennen hier sofort per email bestellen !!

Mit freundlichen Gruessen, Stefan Hartmann, c/o Ingenierbuero Gatz & Hartmann.

Ingenierbuero Gatz & Hartmann

Fehrbelliner Str. 32

13585 Berlin

Tel: 030- 375 55 68

FAX: 030- 375 55 68

International: ++ 49 30-

Mit freundlichen Gruessen, Stefan Hartmann, c/o Ingenierbuero Gatz & Hartmann.

email to: harti@tron.gun.de

///////////////////////////////////////////////////////////////////////////

////  Public Access Unix System Berlin           SCO-XENIX 2.3.4 GT    ////

////          Firmen-Service                     zelator.in-berlin.de  ////

///////////////////////////////////////////////////////////////////////////

Gatz & Hartmann

Ingenierbuero fuer Multimedia-Anwendungen

Tel/FAX: 030- 375 55 68

Support Mailbox-Programm vom Zelator-Team

Modem-ports:     0 30 - 462 63 41

- 462 64 35

- 462 65 38

- 462 60 22

- 462 61 37

- 462 62 37

- 461 86 50

International: ++49 30 -

login: guh