Serial Fix 1993

by Medical Designs Software

Version 2.0

Written by: Bill Penner and Bruce D. Noonan, M.D.

This  program may be distributed and used, free of charge, provided

that  the program is distributed in complete form and UNmodified.  This

software is not to be sold, however, may be included on disks which are

sold  for  a  small  handling fee.  Medical Design Software retains all

copyright   exclusive   rights  except  exclusive  distribution.   This

software  consists  of  the following files: SERFX20.PRG, SERIALFX.CPX,

and SERIALFX.DOC (this file).

PURPOSE

Serial  Fix  is  now  a  universal serial port patch for use on the

ATARI  ST/STe/TT  computers.   Serial Fix now incorporates the TOS14FIX

and SERPATCH patches from ATARI, so no longer do you need several patch

programs.   A  problem  in the serial port software appears when a high

speed  or  high speed buffered modem is used in conjunction with ZMODEM

or  any  near-continuous flow transfer program.  The ST/TT has problems

properly  recognizing  when  the modem deasserts CTS (telling the ST/TT

that the buffer is full and don't send any more data), resulting in the

ST overflowing the modem's buffer.  This results in bad data being sent

which  results  in  CRC  errors,  packet  size problems, and other data

transmission errors.

TurboCTS  (an  old  program  of mine) was only a partial fix of the

problem.  TurboCTS did manage to speed things up to the point where CTS

response  was  generally  ok, however, that program did not work on the

TT,  did not support XBRA programming requirements, and only worked for

RTS/CTS handshaking.

Serial FIX works with all versions of TOS from 1.0 through 3.06 and

FALCON  TOS.   Serial  FIX  supports  both  RTS/CTS,  XON/XOFF,  and no

handshaking  modes.  The desired handshaking mode is set from the RS232

control  panel  or other program as you currently do now.  Simultaneous

XON/XOFF  and RTS/CTS can be selected by locking the RTS/CTS mode on in

the  SERIALFX  CPX  and selecting XON/XOFF from the control panel/RS232

CPX.   This is a REAL fix to the RTS/CTS problem.  Previous fixes (like

my TurboCTS) only fixed some of the symptoms of the problem.

INSTALLATION:

Serial  FIX is composed of two parts, a program for the AUTO folder

and  a  CPX  file to be used with the extensible CONTROL (XCONTROL.ACC)

panel.

SERFX20.PRG  is  the heart of the Serial FIX solution.  SERFX20.PRG

is intended to be installed in your AUTO folder, although it can be ran

from  the desktop.  Serial FIX replaces the current serial routines for

the ST compatible port.  The ST compatible port is the only port on the

ST  and  STe  machines,  or  the  Modem  1  port  on the MegaSTe and TT

machines.

SERIALFX.CPX  is  a  control  panel  to be used with the extensible

control  panel  (XCONTROL.ACC).   The  CPX  is  used to lock the serial

routines into using RTS/CTS handshaking, regardless of the desires of a

program.   The  file  SERIALFX.CPX is to be placed in in your folder of

CPX files (often called 'CPX').

OPERATION:

SERFX20.PRG:   Once  the program has been started (either booted in

the  'AUTO'  folder or run from the desktop), no special operations are

needed.   The  program  replaces the system routines and the port speed

and  handshaking  are  set  just  as  they  are now, either by terminal

software, CONTROL panel, or the extensible control panel.

SERIALFX.CPX:   This program will allow you to lock the handshaking

in  RTS/CTS  mode.   This  is often needed because a number of programs

have  been  written to ignore RTS/CTS mode (like FLASH).  If the CPX is

not  used  to  lock  the  mode  on, if the mode is set from the control

panel,  and  a  terminal  program  is started, very likely, the program

would  turn  off  RTS/CTS.   With  the mode locked, the program will be

unable  to  disable  RTS/CTS.  The CPX will give you the option to save

the  current  lock setting.  Saving the setting will cause the selected

mode  to  be entered every time the CPX boots up.  So, note, to be able

to  lock the RTS/CTS mode on, you must use both the SERFX20.PRG and the

SERIALFX.CPX.

MODEM PROBLEMS:

If you are having problems getting RTS/CTS to work with your modem,

you need to check a few things:

1.    Modem:    You will need to verify that the modem is set up to

use  RTS/CTS.   For  error correcting/compression modems the command us

usually  'AT&K3'  to  enable RTS/CTS mode.  Remember, when you get your

modem working, be sure to save the configuration into the modem with an

'AT&W'.   If  your  modem  is  a non-error correcting/compression type,

RTS/CTS  may not be desired as most of the modems do not really support

RTS/CTS,  and  besides,  it  is  not necessary since the modem does not

buffer  data.   For  most  modems, if you wish to check to see what the

settings are, use a terminal program and type 'AT&V' and the modem will

display the current settings.  Verify the &K option has a 3 shown (i.e.

&K3)  is  shown in the active configuration portion of the display.  If

you type the AT&V and the modem replies ERROR, then that command is not

supported  and  you will have to check the documentation with the modem

to determine how to verify the setting.

2.    Cable:     Most  manufactured  RS232  cables  should not be a

problem,  but some commercial cables and home built cables may not have

the RTS and CTS pins connected.  For RTS/CTS to work, the RTS pins (pin

4) and CTS pins (pin 5) need to be connected through the cable.  If you

want  to  check  the  cable and are familiar with the use of a meter in

doing  a  continuity test, verify that the pin 4s on the two connectors

are  connected  together,  the pin 5's are connected together, and that

the pin 4's are NOT connected to the pin 5's.

3.   Software incompatiblity: If you are still having problems, try

removing   all  other  AUTO  folder  programs  and  ACCESORIES  (except

SERFX20.PRG  and  XCONTROL.ACC).   If things still do not work, you may

have a hardware problem.

4.    Hardware Problem:   If you have checked everything above, and

RTS/CTS  is  still not working, you may have a hardware problem.  It is

possible  that the RTS or CTS circuitry in the computer or in the modem

may have failed.  Attempt to use the modem on another computer and/or a

different  modem  on  your computer to verify whether the problem is in

the modem or the computer.

5.    Last  (and hopefully LEAST) possiblity is that there is a bug

in  my  software (or an undocumented, possibly undesirable, feature, as

some call it).  Let me know if you cannot get SERIALFX to work and I'll

see what help I can provide.

PROGRAMMING INFORMATION:

This  version  of  the  software is fully XBRA compliant.  When the

software  starts and installs the replacement routines, the TOS version

is  checked  to  determine  the  best  method of installation (read for

maximum speed).  Under TOS 1.0, traps 1, 13, and 14 are intercepted and

processed.   Under  TOS  1.2,  1.4,  and  1.6 (non-Bconmap() compatible

machines after 1.2), the xcon vector table is used, and only trap 14 is

used  to  receive Rsconf() calls.  Under Bconmap() compatiable systems,

bconmap  (port  6)  and  xcon  vectors  are  modified  so  that no trap

interception is necessary.

Other  software  can be used to lock or unlock the serial routines.

To  perform  this function, look in the cookie jar for a 'MDSa' cookie.

The long data byte is a pointer to the following data structure:

struct {

int  lockfg;        /* RTS/CTS locked flag (bit 1 = locked) */

int  (*setlock)();  /* Routine to set lock status           */

char bcon[6];       /* Flag for following table = "BCON"    */

int  (*auxistat)(); /* AUX port input status routine        */

int  (*auxin)();    /* AUX port input character routine     */

int  (*auxostat)(); /* AUX port output status routine       */

int  (*auxout)();   /* AUX port output character routine    */

long (*rsconf)();   /* Rsconf() routine for AUX port        */

struct *iorec;      /* Pointer to Iorec buffer              */

} serfx;

The lock status is set by pushing an integer with the value of either 0

(to disable lock) or 1 (to enable) and then jsr'ing the setlock routine

(i.e.  (*serfx->setlock)(1)  to enable the lock).  The call can be made

in user or super mode.

Direct  access  to  the ST compatible serial port.  This is intended to

provide  faster  access  to serial routines on systems running an older

version  of  TOS.   Access  to  the  routines  is the same as accessing

current Bconmap compatible routines.

WRAP UP

As  normal,  PLEASE  let  me know if you have any problems with the

program.  I have attempted to make the program as compatible with other

software as possible.

Bill Penner

3235 Wright Avenue

Bremerton, WA  98310-4753

Phone: (206) 373-4840 (before 9pm Pacific)

GEnie: BPENNER

CHANGES SINCE VERSION 1.0

Two  bugs  that  were  discovered were fixed.  Both bugs would only

occur  if  no  cookie  jar  was  found.  One bug resulted in creating a

cookie  jar  that  was  full of garbage and the other would not build a

cookie  jar.  The second bug was the more common, which would result in

the  CPX  reporting  that  the  program  was  not  loaded.  Running the

SERIALFX.PRG program a second time often worked to fix the problem.

CHANGES SINCE VERSION 1.1

Two  more  bugs  were  found that prevented the software from being

loaded  on  some  systems and a couple of obscure problems were fixed.

SERPATCH  and TOS14FIX routines have been incorporated to make SERIALFX

a  universal patch.  A new vector table was added for people who wanted

to  have  faster access to the original ST serial port so that software

can  directly  interface to the serial routines (as with newer versions

of  TOS)  to  bypass  the  overhead  in  exception  vector  execution.

SERFX20.PRG replaces the older SERIALFX.PRG.