*************************************************************************

*                                                                       *

* LLMVAS.EXP                                                            *

*                                                                       *

* August 18, 1993                                                       *

*                                                                       *

* Lyon Lamb Video Animation Systems                                     *

* Video Tape Recorder                                                   *

* Autodesk Device Interface                                             *

*                                                                       *

*************************************************************************

This documentation will help you install the Lyon Lamb MiniVAS and PCVAS

ADI (LLMVAS.EXP) for Autodesk 3D Studio.

The following assumes that 3D Studio has been installed in the directory

C:\3DS.  If this is different on your system, please substitute the

appropriate directory when reading these instructions.

1) First preblack your video tape to automatically record control track,

frame code, and time code on it.

2) Copy LLMVAS.EXP into the 3D Studio drivers sub-directory.  This

will most likely be the sub-directory C:\3DS\DRIVERS.

3) Edit the file 3DS.SET in the C:\3DS directory to change the

VTR-DEVICE from the default [DIAQUEST] to VTPADI.

The proper syntax is:

VTR-DEVICE = VTPADI

4) In 3DS.SET, examine the "Rendering Display" section. Modify

the default rendering driver according to Autodesk's built-in

output drivers.

Change the rendering display parameters to match the video output

card in your system. A few compatible video graphic cards are VISTA,

TARGA or VISION16.

The proper syntax for a VISTA (756x486) is:

RENDER-DISPLAY = VISTAH

NOTE: You must have SET RDPADI or SET RCPADI statement in your

AUTOEXEC.BAT for these to function.  Consult your 3D Studio manual.

Alternatively, you can use the "3DS VIBCFG" command to change your

rendering display to match the video output card in your system.

Consult your 3D Studio Rel. 3 Installation Guide for details.

5) Add the following line to the end of your AUTOEXEC.BAT file:

SET VTPADI=C:\3DS\DRIVERS\LLMVAS.EXP

The line tells 3D Studio that the ADI video tape recorder driver

you want to use is LLMVAS.EXP.

6) Now you need to add another line to your AUTOEXEC.BAT file.  The line

tells the ADI driver how your system is set up.  The format of this line

depends on whether you have a MiniVAS or a PCVAS and how it is set up.

The format of the line is:

SET LLMVAS={F,T,L} {N,P} {C1,C2,C3,C4} I# Vaa R#

Immediately following the description of the parameters are several

examples of specific configurations and the proper SET LLMVAS= line

to place in your AUTOEXEC.BAT file.

Description of the parameters:

-----------------------------

{C1,C2,C3,C4}   C1 - Use COM port 1

C2 - Use COM port 2

C3 - Use COM port 3

C4 - Use COM port 4

I#              Specify the IRQ number to use for communication with the VAS

e.g. I10        means use IRQ 10

I5         means use IRQ 5

NOTE: MiniVAS users do not normally need to specify the I

parameter.

{F,T,L}         F - Use frame code

T - Use serial time code

L - Use longitudinal time code (available only for PCVAS)

{N,P}           N - Use NTSC timing (30 frames per second)

P - Use PAL timing (25 frames per second)

Vaa             Specify the VTR identification code (consult your MiniVAS

or PCVAS manual)

e.g. VSM        is the Sony VO-9850

VP7        is the Panasonic AG-7750

R#              Specify the preroll time (1 to 20 seconds) for recording

e.g. R6         set a preroll time of 6 seconds

R11        set a preroll time of 11 seconds

NOTE: Specifying R0 will use the default preroll time

stored in the MiniVAS's or PCVAS's memory.

Example SET LLMVAS= configurations:

----------------------------------

Example A:  A MiniVAS connected to COM1 (C1), using frame code (F),

NTSC timing (N), the Sony VO-9850 (VSM), and a recording

preroll time of 8 seconds (R8):

SET LLMVAS=C1 F N VSM R8

Example B:  A MiniVAS connected to COM2 (C2), using time code (T),

PAL timing (P), the Sony VO-9850 with time code option (VSN),

and a recording preroll time of 10 seconds (R10):

SET LLMVAS=C2 T P VSN R10

Example C:  A MiniVAS connected to COM3 (C3), using frame code (F),

NTSC timing (N), the Sony BVU-2500 (VSD), and the default

recording preroll time (R0):

SET LLMVAS=C3 F N VSD R0

Example D:  A PCVAS connected to COM3 (C3), IRQ number 10 (I10),

using frame code (F), NTSC timing (N), the Sony VO-9850 (VSM),

and a recording preroll time of 6 seconds (R6):

SET LLMVAS=C3 I10 F N VSM R6

Example E:  A PCVAS connected to COM1 (C1), IRQ number 4 (I4),

using frame code (F), PAL timing (P), the Hitachi HR-230 (VH3),

and the default recording preroll time (R0):

SET LLMVAS=C1 I4 F P VH3 R0

Example F:  A PCVAS connected to COM4 (C4), IRQ number 5 (I5),

using longitudinal time code (L), NTSC timing (N),

the Panasonic AG-7750 (P7), and a recording preroll time

of 7 seconds (R7):

SET LLMVAS=C4 I5 L N P7 R7

7) After you add the proper SET LLMVAS= line to your AUTOEXEC.BAT file

and save it, reboot your computer to put the changes into effect.

8)  Open 3D Studio, go to Keyframer (F4), and select the following from

the menu on the left:

Renderer ...

VTR Control ...

VTR Setup ...

Hit the Initialize button, wait, then a dialogue box appears reading:

VTR is On-line.

9)  Please refer back to the Studio 3D documentation for a

detailed explanation regarding the:

VTR Control...  and  VTR Setup...  menus.

================================================================

Detailed Description of the LLMVAS Environment Variable Settings

================================================================

The following is a full description of the LLMVAS environment

variable settings.

Syntax:

------

SET LLMVAS={C1|C2|C3|C4} {I#} {Axxx} {N|P} {F|T|L} {Vaa} {Rdd}

Default Setting:

---------------

If the LLMVAS environment variable is not defined, the driver assumes

the following default setting:

SET LLMVAS=C3 I10 A3E8 N F R0

This specifies COM port 3, IRQ 10, I/O base address 0x3e8, NTSC timing,

frame code operation, and the default preroll time of the VAS.

Notes on the Syntax:

-------------------

- The curly brackets "{" and "}" show groups of related options.

- Vertical bars "|" separate each available option in a group.

- Capital letters or lower case letters may be used.

- Unrecognized options will be ignored (e.g. the letter "Z").

- Spaces may optionally separate each option.

- Spaces may not separate the letters composing an option.

(e.g. "VSM" works, but "V SM" does not)

- Options can be given in any order but subsequent options from the same

group override previous options from the same group.

(e.g. "NP" means PAL operation since the P comes after the N)

Parameters:

----------

COMMUNICATIONS PORT:    {C1 | C2 | C3 | C4}

C1 - COM1, IRQ4, 0x3F8

C2 - COM2, IRQ3, 0x2F8

C3 - COM3, IRQ10, 0x3E8

C4 - COM4, IRQ11, 0x2E8

This option selects the serial port to which the MiniVAS or PCVAS

is connected.  The assumed IRQ and I/O base address for the

COM port are listed above.  However, specifying a different IRQ

using the I option (described below) overrides the assumed IRQ,

and specifying a different I/O base address using the A option

(also described below) overrides the assumed I/O base address.

Default: If this option is not specified, then the driver

assumes COM3 (C3).

IRQ NUMBER:             {I#}

This option selects the IRQ number to use for communication

with the PCVAS or MiniVAS.

The PCVAS has a set of selectable jumpers labelled (IRQ2, IRQ3,

IRQ4, IRQ5, IRQ7, IRQ10, IRQ11, and IRQ15) for selecting the

IRQ number.  You should use the I option to specify the IRQ

for which the PCVAS is set.

The MiniVAS does not have selectable jumpers for changing the

IRQ number, but the I/O card in your computer may have such jumpers.

Default: If this option is not specified, then the driver

assumes the default IRQ for the selected COM port.

Examples:       I2      specifies IRQ2

I11     specifies IRQ11

I/O BASE ADDRESS:       {Axxx}

This option selects the I/O base address to use for communication

with the PCVAS or MiniVAS.  The address is given as a three digit

hexadecimal number.

The PCVAS has a set of dip switches labelled (A3 to A8) for

selecting the I/O base address.  Normally you do not need to

specify this option because selecting a communications port

using the C option selects a default I/O base address.

Default: If this option is not specified, then the driver

assumes the default I/O base address for the selected

COM port.

Examples:       A200    specifies 0x200

A3D8    specifies 0x3D8

FRAME FORMAT:           {F | T | L}

F - Frame code

T - Serial time code

L - Longitudinal time code

This option selects the mode used for search operations and record

operations.

Default: If this option is not specified, the driver assumes

frame code (F) operation.

TIME STANDARD:          {N | P}

N - NTSC (30 frames per second)

P - PAL (25 frames per second)

Default: If this option is not specified, the driver assumes

NTSC (N) operation.

VTR ID CODE:            {Vaa}

This option identifies the VTR connected to the MiniVAS or PCVAS

using a two letter identification code.  This two letter code

can be found in your MiniVAS or PCVAS manual.

Specifying this option causes the driver to initialize the MiniVAS

or PCVAS with the given identification code.

Default: If this option is not specified, then the driver assumes

that the current VTR identification code stored in the

MiniVAS's or PCVAS's memory is correct.

Examples:       VP7     specifies the Panasonic AG-7750

VSN     specifies the Sony VO-9850

PREROLL TIME:           {R#}

This option specifies the preroll time for performing recordings.

The preroll time can be set to a number of seconds from 1 to 20.

To use the default preroll time of the MiniVAS or PCVAS,

specify (R0).

Default: If this option is not specified, the driver assumes

that the default preroll time of the MiniVAS or PCVAS

is correct.

Examples:       R5      specifies a 4 second preroll

R10     specifies a 10 second preroll

R0      specifies the default preroll time

===========================================================================

Notes on the Operation of the Driver

===========================================================================

If 3D Studio appears to have hung:

---------------------------------

If 3D Studio appears to have hung during a frame search using

Next Frame Forward, Next Frame Back, Goto, or a record operation

using either Record or Disk-to-VTR, wait at least 15 seconds.

For most operations, the driver has a time-out time of about 10

seconds.  If after 10 seconds, the driver detects that something

may be wrong, it returns to 3D Studio indicating that an error

occurred in the operation.

If after 15 seconds, you are not given control back, stop your

VTR and wait another minute.  The driver will wait a longer period

of time before timing out when it is waiting for the cut-in signal

and the ready to record signal.