BFX

Copyright (c) 1992 Richard D. Fothergill

Richard D. Fothergill

the Oak Hill BBS

141 Oak Hill Road

Pittsfield, MA  01201

Data/FAX - (413) 499-7245

WHAT IS BFX?

When  I purchased my ZyXEL  U-1496 Modem/Fax I wanted an easy way

to  be able to accept fax and modem calls on the same line.  This

would allow me to continue to operate my BBS software and be in a

position  to receive  faxes.  I  tried BinkleyTerm and FrontDoor.

Both  excellent programs, but  way more than  I needed or wanted,

not  having a  need for  network mailers  or a  built in terminal

program.   I wanted  something simple,  small, easy  to setup and

maintain.  Thus the beginning of BFX.

WHAT CAN I DO WITH BFX?

You  run BFX instead  of your BBS  software.  BFX will answer the

phone,  decide  what  the  incoming  call  is  and  exit  with an

errorlevel  which you  can  use in  a  batch file  for subsequent

operations.   A BBS  call will also  create a  batch file to call

your  BBS software.  In addition  you can program up to 10 events

with  a preset time  of execution and  exit errorlevel.  When BFX

encounters  one of  these it  will  exit with  your predetermined

errorlevel.   Again, you control further processing in your batch

file.

DISCLAIMER

IN  NO EVENT WILL  I BE LIABLE  TO YOU FOR ANY DAMAGES, INCLUDING

ANY   LOST  PROFITS,   LOST  SAVINGS   OR  OTHER   INCIDENTAL  OR

CONSEQUENTIAL DAMAGES ARISING OUT OF YOUR USE OR INABILITY TO USE

THE PROGRAM, OR FOR ANY CLAIM BY ANY OTHER PARTY.

BFX                                               -1-

Copr (c) 1992 Richard D. Fothergill

INSTALLATION

There  are three files that you must create to run BFX.  They are

BBS.BAT, BFX.CFG, BFX.EVT.  I will discuss these individually.

BBS.BAT

This batch file is the heart of your BBS/FAX system.  It controls

all  activity and  execution.  You  do not  have to  use the name

BBS.BAT,  it can be any  name of your choosing.  The following is

the BBS.BAT file I use.

@ECHO OFF

:loop

cls

BFX

if errorlevel 40 goto event

if errorlevel 30 goto fax

if errorlevel 20 goto spit

if errorlevel 10 goto spitl

if errorlevel  0 goto end

:event

sfmaint

goto loop

:fax

rcvfax 1 f:\sf\fax

goto loop

:spit

call bbsexe

goto loop

:spitl

spitfire 0

goto loop

:end

There are four hardcoded errorlevels.  They are:

0 - Terminate

10 - Local Logon

20 - BBS/Modem Connection

30 - FAX Connection

The  loop: label is where execution of the batch file will return

to  after any activity other  than a normal exit.  BFX is started

and  awaits a caller.  If a call is received or a scheduled event

occurs  BFX will exit with the corresponding errorlevel.  Control

then returns to this batch file.

BFX                                               -2-

Copr (c) 1992 Richard D. Fothergill

BFX.CFG

This is the configuration file you setup to inform BFX of certain

parameters  it  needs to  function.   The following  is  a sample

configuration file.

CM=2

BA=38400

PA=N

DA=8

SB=1

MI=AT&FM0V1E0H0S2=1X7#B1+FCLASS=6

AS=ATA

HY=ATH0

HN=ATH1

MR=ATZ

BB=SPITFIRE

BP=

CP=/C

EC=/E

ST=120

RC=2

LM=>> BFX >> logging on to...

There  are no spaces  allowed in this  file between the parameter

and  the = sign.  The items should appear in the sequence listed.

The parameters and the meanings are a follows:

CM  The communications port you have your modem on.

BA  The baud rate to initialize your modem to.

PA  Parity

DA  Data length

SB  Stop Bits

MI  Your initialization string for modem/fax reception.

AS  Modem answer string.

HY  Modem onhook string.

HN  Modem offhook string.

MR  Modem reset string.

BB  The name you would like placed in the BBSEXE.BAT file.

BP  The text you would like to preceed the baud rate parameter in

the BBSEXE.BAT file.

BFX                                               -3-

Copr (c) 1992 Richard D. Fothergill

CP  The text you would like to proceed the com port parameter in

the BBSEXE.BAT file.  Spitfire specifies that the parameter

read /C<comport>.  Check your BBS software.

EC  The text you would like in then BBSEXE.BAT file to notify

your BBS software of an error correcting connection.

ST  Sleep time out seconds.  This is the amount of time BFX

will wait before blanking the screen.  A display of the

program name and current time will appear at random locations

to remind you of the program running.  If ST=0 then screen

blanking is disabled.

RC  The number of rings on which you would like BFX to answer

the call.

LM  Log on message.  This is sent to the caller just before BFX

exits on a modem connection.

BFX.EVT

This file is used to define your events.  It is a comma delimited

file consisting of three parameters.  The first is a 40 character

description.  This will be written to the log file when the event

is executed.  The second is the time you want the event to occur.

The format is HH:MM and is in 24 hour time.  The last item is the

errorlevel.   This can  be any  integer value  except one  of the

hardcoded  errorlevels mentioned earlier.  Each event must appear

on  a separate line in the file and you are limited to 10 events.

A typical line in the event file is as follows:

DAILY MAINTENANCE,01:00,40

This  line would cause  BFX to  exit with an  errorlevel of 40 at

1:00 am.

FILES CREATED BY BFX

There  are three  files  created by  BFX.   A discussion  of each

follows.

BFX.LOG

If  this file  is not  found at  start-up BFX  creates a new one.

Each  time an activity occurs,  an entry is written to this file.

The entry will consist of the date, time, description of activity

and  either the errorlevel for an event or the CONNECT string for

a  modem connection.  Local logons  are not recorded in the file.

This  file has  no interaction with  your BBS  log file.  A modem

connection  may be made  and control passed  to your BBS software

and  the user might  hang up with  out logging on.  In this case,

there  would still be  a record of  the connection in the BFX.LOG

but in all probability, not in your BBS log.

BFX                                               -4-

Copr (c) 1992 Richard D. Fothergill

BFX.STS

This  is a  file used  strictly by  BFX.  Each  time BFX releases

control  back to DOS, BFX writes control data to this file.  When

it  restarts it  reads this  information  back in.   Although not

required,  it is suggested  that each time  you erase the BFX.LOG

file  to  clear  that,  you  erase  BFX.STS  to  reset  the  call

statistics.   You do not have  to recreate these files.  BFX will

create new ones if existing files are not found.

BBSEXE.BAT

When  a BBS call is  received, BFX will create a batch file which

contains  the name (BB),  followed by the  baud rate prefix (BP),

baud  rate, com  port prefix  (CP), com  port (CM)  and the error

correcting  string  (EC) .   The  parameters you  specify  in the

configuration  file should allow  you to  construct an entry that

will  allow you  to start your  BBS software.   The format of the

line in BBSEXE using the configuration parameters is:

[BB] [BP]<baud rate> [CP][CM] [EC]

SPITFIRE 14400 /C2 /E

GENERAL DISCUSSION

As  you can  see  from my  sample  BBS.BAT file  I  am set  up as

follows.

Errorlevel                Action

0         Normal program exit.  Modem reset.

10         Local logon to BBS.  Spitfire requires that you

call it with a baud rate of 0 to let it know that

it is a local log on and not try to detect a

carrier.

20         Remote call BBS logon.  Spitfire requires that you

call it with a baud rate and com port to notify it

that a connection has already been made and to

proceed to the user log on.

30         Remote call FAX transmission.  The batch file

passes control to ZyXEL's RCVFAX program which also

assumes a connection has been established.

40         User specified event.  I have this set to occur at

1:00 am.  This is routine BBS maintenance.  Please

remember, running in a system such a this, in all

probability events scheduled in your BBS software

will never occur.  Your BBS software is not the

program controlling the flow, BFX is, so events

should be schedule through BFX.

BFX                                               -5-

Copr (c) 1992 Richard D. Fothergill

MENU AND HOT KEYS

From  the Waiting  for Caller  screen you  have two  possible key

strokes.   Alt-X will close BFX and execute a normal exit.  Alt-M

will bring up a pop menu with the following selections available:

View Activity Log

Local Logon

Drop to DOS

Terminate BFX

Within  this  menu  your  mouse  will  be  active  to  locate the

highlight  bar or you  can use the  arrow keys.  Either press the

left mouse button or the enter key to make a selection.  Pressing

the  right mouse  button or  the Esc  key will  return you to the

Waiting for Caller screen.

View  Activity Log  - This  selection  will pop  up a  window and

display  the BFX.LOG file.  All  the direction keys are active as

well  as the mouse.  Press  the Esc key or the right mouse button

to return to the menu.

Local Logon - This will exit the system with an errorlevel of 10.

This  does not  have to  be a  local log  on.  You  can have your

BBS.BAT file execute any program you wish or none at all.

Drop  to DOS  - Self explanatory.   The DOS  prompt is amended to

remind you to EXIT back to the BFX menu.

Terminate  BFX - Same as  the Alt-X key press.  Executes a normal

exit back to dos.

BFX                                               -6-

Copr (c) 1992 Richard D. Fothergill