RELOCK V2.0 by Tim Pearson

DISCLAIMER

----------

This program is not in the public domain.  It is provided "as is" with

no warranty of any kind, including implied warranties of merchantibility

or fitness for a particular purpose.  By using this program, you argree to

accept sole responsibility for any damage that may result from said use.

What is RELOCK?

---------------

With the advent of the new 16.8Kbps HST's and new high performance UART's

like the 16550, serial communications speed has outstripped the ability of

many formerly well behaved BBS systems and BBS doors to handle the higher

speeds.  However, many FidoNet(tm) compatible front end mailers (D'Bridge is

one noteable example) are perfectly capable of taking advantage of higher

serial port to modem locked rates.

RELOCK is an attempt to give you the best possible throughput with your

mailer while allowing you to slow things down from "warp speed" when your

mailer passes a caller off to your BBS software.

RELOCK has been tested with X00, D'Bridge, QuickBBS, and a USR 16.8 dual

standard modem on an 80486 33mhz machine running under DesqView.

RELOCK Operational Overview

---------------------------

RELOCK, when provided with the proper command line parameters, will:

1.  Open the specified port, locked at the specified initial rate.

2.  Wait for all pending output to clear the output buffer.

3.  Send the modem escape sequence "+++" to place the modem in

command mode while still on line. (see HST DIP switch setting

below).

4.  Switch the fossil to the new bps rate.

5.  Send ATB1(cr) to lock the modem at the new rate.

6.  Send ATO(cr) to the modem to return it to "online" mode.

7.  Exit leaving the fossil "hot" & locked at the new rate.

Hardware Compatibility

----------------------

RELOCK is designed for use with U.S. Robotics(tm) HST modems.  If your

modem is not an HST it must conform to the following to be compatible with

RELOCK:

1.  Escape sequence +++ must return the modem to "command" mode

without dropping carrier.

2.  ATB1 must be the command to lock your modem's baud rate.

3.  ATO must be the command to return your modem to "online" mode.

4.  Your modem must be able to accept commands at other than its

currently locked bps rate.

Installing RELOCK

-----------------

1. Set DIP switch 9 on your HST for "No disconnect on +++".

2. Insure that HST register S2 equals 43 (ASCII "+").

3. Make certain that X00's companion program XU is located

in your DOS search path.

4. Place RELOCK in your mailer or BBS batch file at a point where it

will be invoked after your mailer exits but before the BBS software

is loaded. (run RELOCK from the directory where it resides).

BBS users will see "+++" on their screen as RELOCK executes.  You may

wish to place a statement like "Adjusting communications speed. Stand by."

in your mailer's "loading bbs" text message.

RELOCK tests for the presence of a carrier upon entry and immediately

after sending the +++ escape sequence.  The latter is done in case you

have not set DIP switch 9 on your HST correctly.  If carrier is lost,

RELOCK displays a message and exits.  It will not "hang" your batch file.

RELOCK also displays several progress, error, and warning messages during

execution as conditions warrant.

Command Line Parameters

-----------------------

Typing RELOCK with no parameters will display a help screen.

Syntax is: RELOCK fossil port bps1 bps2

fossil = X00 or XOO for the X00 fossil. Others NOT supported.

port = 0 for COM1, 1 for COM2, 2 for COM3 or 3 for COM4

bps1 = modem's current locked bps rate (300 to 57600 bps)

bps2 = bps rate modem should be changed to (300 to 57600 bps)

All four parameters are required.  An incorrect specification will

cause an error message and the RELOCK help screen to be displayed followed

immediately by program exit.  During installation debugging, it might be

wise to place a PAUSE statement in your .BAT file immediately following

the RELOCK statement.

If bps2 is greater than bps1, RELOCK will display a warning message and

proceed to re-lock the modem at the higher rate.  The warning is included

merely because the normal use for RELOCK is to slow down rather than speed

up the modem's locked rate.

EXAMPLE1:

RELOCK X00 0 57600 19200

Using  port 0 (COM1).  Modem is currently locked at

57,600 bps.  Relock it at 19,200 bps.

EXAMPLE2:

RELOCK XOO 1 38400 19200

Using port 1 (COM2).  Modem is currently locked at

38,400 bps.  Relock it at 19,200 bps.

Support

-------

If you have problems with RELOCK, you may contact the author by calling

the LANStar BBS at 417-673-BAUD (2283) or by e-mailing Tim Pearson at

1:286/703 @ fidonet.  The most recent version of RELOCK will be available

for download or file request at the same number/address.

The Shareware Concept

---------------------

Users of this program are free to evaluate it for a period of 30

days at no charge.  After said evaluation period, you must discontinue

its use or to send $10.00 US funds to:

Timothy Pearson

110 West Main Street

Carterville, MO  USA  64835

Upon registration, you will receive a key file good for this and all future

versions of RELOCK.  Registration will also eliminate the 10 second non-

registered delay.

See RELOCK.REG contained in the RELOCK archive for more information.