Use WATCHCD at your own risk.

**********************************************************

*NOTICE NOTICE NOTICE NOTICE NOTICE NOTICE NOTICE NOTICE *

*                                                        *

* WATCHCD V1.00 is not compatible with previous versions.*

* Also, V1.00 or greater of X00.SYS must be used.  There *

* may be no error messages generated if you mix the      *

* incompatible versions of WATCHCD and X00.SYS           *

**********************************************************

WATCHCD is  a carrier  detect watchdog.   That is, once activated

WATCHCD will cause a warm boot if carrier  detect is  lost on the

selected communications port.

To  correctly  perform  the  watchdog function, some code must be

resident in memory while  other  programs  are  executing.   This

resident code  is in  the device  driver X00.SYS.   Thus, WATCHCD

will not function without X00.SYS.

WATCHCD supports both the conventional method of identifying comm

ports, such  as COM1, COM2 etc, and by port number identification

when the reassignment features of X00.SYS are used.

To execute WATCHCD using conventional identification enter:

WATCHCD COM1 ON to activate the watchdog on COM1

WATCHCD COM1 OFF to de-activate the watchdog on COM1

You can specify COM1 through COM4.  When  the above  commands are

used the CD monitoring will always be on the COMn port as defined

by IBM, regardless of any reassignments done by X00.SYS.

To execute WATCHCD using port numbers  that you  may have defined

using X00.SYS, then enter:

WATCHCD 0 ON to activate the watchdog on port 0.

WATCHCD 0 OFF to de-activate the watchdog on port 0.

The port  number may  be as large as the number of ports that you

have specified to X00.SYS.   If  you used  the statement  in your

CONFIG.SYS file  DEVICE=X00.SYS 0=COM4,  and then execute WATCHCD

with WATCHCD 0 ON, you have activated a watchdog on COM4.  If you

have  not  made  any  port  reassignment  then  ports 0 through 3

correspond to COM1 through COM3.

It is the authors intent to later change the watchdog function of

the X00.SYS  to force  programs to exit (gracefully) when carrier

is  lost  (under  any  conditions).    The  "gracefully"  is  the

difficult part  of the  above.   This is preferable to booting on

multi-tasking systems.