--------------------------------------------------------------------

function 0  - initialize a port

fct entry : ah = 0,

al = port parameters

76543210

||||||||

||||||``-- Data bits : 00  = 5 bits

||||||                 01  = 6 bits

||||||                 10  = 7 bits

||||||                 11  = 8 bits

|||||`---- Stop bits :  0  = 1 bit

|||||                   1  = 2 bits

|||``----- Parity    : 00  = none

|||                    01  = odd

|||                    10  = none

|||                    11  = even

```------- Baud rate : 000 =   110 baud

= 19200 if EBIOS supported

001 =   150 baud

010 =   300 baud

011 =   600 baud

100 =  1200 baud

101 =  2400 baud

110 =  4800 baud

111 =  9600 baud

dx = any valid channel # (DX min - DX max)

returns : ah = 0ffh if error, otherwise :

= port status.

al = modem status.

--------------------------------------------------------------------

function 1h  - output a character

fct entry : ah = 01h

al = character

dx = any valid channel # (DX min - DX max)

returns   : ah = port status.

al = character sent.

--------------------------------------------------------------------

function 2h  - input a character

fct entry : ah = 02h

dx = any valid channel # (DX min - DX max)

returns   : ah = port status

al = received character

--------------------------------------------------------------------

function 3h  - return port status

fct entry : ah = 03h

dx = any valid channel # (DX min - DX max)

returns   : ah = port status

al = modem status register

--------------------------------------------------------------------

function 4h  - extended port initialization

( Differs from COM/XI, PC/X* and MC/Xi DOS driver )

fct entry : ah = 04h

al = break setting

0 = No break.

1 = Break (Break will be set until reset)

bh = Parity

0 = no parity

1 = odd parity

2 = even parity

3 = stick parity odd (not supported)

4 = stick parity even (not supported)

bl = Stop bits

0 = 1 stop bit

1 = 2 stop bits

ch = Character length

0 = 5 bits

1 = 6 bits

2 = 7 bits

3 = 8 bits

cl = Baud rate

00h -   110 baud

01h -   150 baud

02h -   300 baud

03h -   600 baud

04h -  1200 baud

05h -  2400 baud

06h -  4800 baud

07h -  9600 baud

08h - 19200 baud

09h - 38400 baud

0Ah - 57600 baud

0Bh - 76800 baud

0Ch -115200 baud

0Dh -    50 baud

0Eh -    75 baud

0Fh -   134 baud

10h -   200 baud

11h -  1800 baud

dx = any valid channel # (DX min - DX max)

returns   : ah = 0ffh on error, otherwise :

ah = port status

al = modem status

--------------------------------------------------------------------

function 5h  - extended communications port control

( Differs from COM/XI, PC/X* and MC/Xi DOS driver )

fct entry : ah = 05h,

al = 00h - read modem control register

al = 01h - write modem control register

bl  - modem control bits

bit 0 = DTR

bit 1 = RTS

dx = any valid channel # (DX min - DX max)

returns   : ah = port status

al = modem status

bl = modem control register

bit 0 = DTR

bit 1 = RTS

NOTE : Access to OUT1, OUT2, and Loop bits are not supported. Also,

RTS and DTR will not be affected while hardware handshaking is active.

--------------------------------------------------------------------

Function 6h  - Get Driver/Board Information

( New to PC/X* and MC/Xi DOS drivers, extends COM/Xi DOS driver function )

fct entry : ah = 6h

Sub-function 0

dx = any valid channel # (DX min - DX max)

al = 0h - get port name

return	:

ah    = 0FFh if error, else

al    = Highest INT 14h function supported.

es:bx = Pointer to 8 byte ASCII string

String = "NoDriver" if No DOS support

is provided (INT 14h only)

String = Channel's DOS name

(eg "COM5    ")

Sub-function 1

al = 1h - get driver information

dx = any valid channel # (DX min - DX max)

return :

ah = 0FFh if error, else

ax = Total number of channels supported.

bx = Driver Version # (e.g. V 2.0.1 : BX = 0201h)

cx = Total number of board's supported.

dx = DX min - Lowest DX value for INT 14h calls.

Maximum DX value = Minimum value + AX - 1

Sub-function 2h

al = 2h - get board information

bx = Board Number (0-3).

dx = any valid channel # (DX min - DX max).

return :

ah = 0FFh if error, else

ah = IRQ number.

al = Board Type .

1 = COM/Xi

2 = MC/Xi

3 = PC/Xe

4 = PC/Xi

5 = PC/Xm

bx = Board's Dual-Ported Memory Segment.

cx = Number of channels on the board.

dx = Board's I/O port Address.

si = INT 14h DX value for board's first channel

Sub-function 0ffh

al = 0ffh - get driver name

dx = any valid channel # (DX min - DX max).

return :

ax    = driver version #

cx    = number of channels supported.

es:bx = pointer to drivers name (8 bytes 'DIGIFEP5').

--------------------------------------------------------------------

function 7h  - Send Break

( New to PC/X* and MC/Xi DOS drivers. Old Com/Xi driver used

a 55 msec interval instead of this drivers 10 msec interval. )

fct entry : ah = 07h

al = 00h - Use default break time

= 01h   then bx = break length in # of 10 msec

intervals.

dx = any valid channel # (DX min - DX max)

returns   : ah = 0ffh on error

= 0    on success

--------------------------------------------------------------------

function 8h  - alternate status check

fct entry : ah = 08h

dx = any valid channel # (DX min - DX max)

returns   : zf = 0 and

ah = a non 0ffh number.

then  ah = line status

al = Character queued

zf = 1 and

ah = 0ffh

then no char queued

Note : If zf = 1 then and only then will ah = 0ffh. Thus, you only need to

check zf OR ah to determine the success or failure of the call.

--------------------------------------------------------------------

function 9h  - clear buffers

fct entry : ah = 09h,

dx = any valid channel # (DX min - DX max)

returns   : ah = 0    fct good

ah = 0ffh fct bad (ccb timeout error)

--------------------------------------------------------------------

function 0ah - input queue check

fct entry : ah = 0ah

dx = any valid channel # (DX min - DX max)

returns   : zf = 1 and dh = offh if error, otherwise

ax = number of queued characters

Note : This function differs from FCT 15h. Fct 0Ah returns the number

of data characters in the receive buffer, while function 15h

returns the number of bytes used in the buffer. Typically, each

recieved character uses only one byte of buffer space unless the

data byte is 0ffh or unless an error is associated with it. In

these cases, 2 and 3 bytes of buffer space are used, respectively.

Fct 0Ah disregards these extra bytes in it's returned count, while

function 15h does not.

Use fct 0Ah when you wish to know how many data bytes to read when

using functions 2h (input a character) or 0Fh (read string).

Use fct 15h when you want to know how many physical bytes of buffer

space are used.

If you were previously using the COM/Xi DOS driver "XIM232.SYS"

Functions 0Ah and 15h are swapped in this driver.

Also note that status bytes are not stored with every character in

the receive buffer as they where with the XIM232, XAPCM232 and

XIM232MC DOS drivers.

: If zf = 1 then and only then will dh = 0ffh. Thus, you only need to

check zf OR dh to determine the success or failure of the call.

--------------------------------------------------------------------

function 0bh  - Drop Handshake Lines

( New to PC/X* and MC/Xi DOS drivers )

fct entry : ah = 0bh

dx = any valid channel # (DX min - DX max)

returns   : ah = 0ffh if errors

ah = 0    on success

This Function drops DTR and RTS. If Hardware handshaking is

active, it is disabled, the lines are dropped, and then it is

re-enabled.

--------------------------------------------------------------------

function 0ch  - Get Channel Parameters

( New to PC/X* and MC/Xi DOS drivers  changes and extends

COM/Xi DOS driver function.				  )

fct entry : ah = 0ch

dx = any valid channel # (DX min - DX max)

returns   : ah = 0ffh on error

otherwise

ah = S/W flow control

bit 0 = tx s/w flow control (XON/XOFF)

bit 1 = rx  "    "     "        "

al = H/W flow control

bit 0 = DTR rx flow control

bit 1 = RTS rx flow control

bit 4 = CTS tx flow control

bit 5 = DSR tx flow control

bit 7 = DCD tx flow control

bh = Parity

0 = no parity

1 = odd parity

2 = even parity

bl = Stop bits

0 = 1 stop bit

1 = 2 stop bits

ch = Character length

0 = 5 bits

1 = 6 bits

2 = 7 bits

3 = 8 bits

cl = Baud rate

00h -   110 baud

01h -   150 baud

02h -   300 baud

03h -   600 baud

04h -  1200 baud

05h -  2400 baud

06h -  4800 baud

07h -  9600 baud

08h - 19200 baud

09h - 38400 baud

0Ah - 57400 baud

0Bh - 75600 baud

0Ch -115200 baud

0Dh -    50 baud

0Eh -    75 baud

0Fh -   134 baud

10h -   200 baud

11h -  1800 baud

--------------------------------------------------------------------

function 0dh - get pointer to character ready flag

fct entry : ah = 0ah

dx = any valid channel # (DX min - DX max)

returns   : es:bx = pointer to the flag

flag = 0    : rx buffer empty

flag = 0ffh : char available in rx buffer

note      : The driver uses interrupts from the board, therfore an

IRQ line must be selected for this function to work.

--------------------------------------------------------------------

function 0eh  -  write string

fct entry : ah    = 0eh

dx    = any valid channel # (DX min - DX max)

cx    = # of chars in string

es:bx = pointer to string

returns   : ax    = # chars transmitted

zf    = 1 and dh = 0ffh if errors

NOTE : If zf = 1 then and only then will dh = 0ffh. Thus, you only need to

check zf OR dh to determine the success or failure of the call.

--------------------------------------------------------------------

function 0fh  - read string

fct entry : ah    = 0fh

dx    = any valid channel # (DX min - DX max)

cx    = # of chars to read

es:bx = pointer to callers buffer

returns   : ax    = # chars read

zf    = 1 and dh = 0ffh if errors

(line sts or wrong # chars rtnd)

NOTE : If zf = 1 then and only then will dh = 0ffh. Thus, you only need to

check zf OR dh to determine the success or failure of the call.

--------------------------------------------------------------------

function 10h  - clear rx buffer

fct entry : ah = 10h

dx = any valid channel # (DX min - DX max)

returns   : ah = 0ffh error

ah = 0    success

--------------------------------------------------------------------

function 11h  - clear tx buffer

fct entry : ah = 11h

dx = any valid channel # (DX min - DX max)

returns   : ah = 0ffh fct error

ah = 0    fct good

--------------------------------------------------------------------

function 12h  - get tx buffer free space

fct entry : ah = 12h

dx = any valid channel # (DX min - DX max)

returns   : ax = # of free bytes

--------------------------------------------------------------------

function 13h  - Set Handshake Lines

( New to PC/X* and MC/Xi DOS drivers )

fct entry : ah = 13

dx = any valid channel # (DX min - DX max)

returns   : ah = 0ffh if errors

ah = 0    on success

This Function raises DTR and RTS. If Hardware handshaking is

active, it is disabled, the lines are raised, and then it is

re-enabled.

--------------------------------------------------------------------

function 14h  - Non destructive character read

( New to PC/X* and MC/Xi DOS drivers )

fct entry : ah = 014h

dx = any valid channel # (DX min - DX max)

returns   : ah = Line status

al = Character queued

--------------------------------------------------------------------

function 15h  - input byte count

fct entry : ah = 015h

dx = any valid channel # (DX min - DX max)

returns   : ah = # of used bytes in the receive buffer

( See note on FCT 0Ah )

------------------------------------------------------------------

function 16h  - Reserved

( Old PC/X* Send CCB Command )

------------------------------------------------------------------

function 17h  - Pause / Resume transmit

( New to PC/X* and MC/Xi, Old COM/Xi Get / Clear transmit pause )

fct entry:  ah = 17h

al = 0, pause transmit

= 1, resume transmit

dx = any valid channel # (DX min - DX max)

returns:  ah = 0ffh fct error

ah = 0    fct good

------------------------------------------------------------------

function 18h  - Pause / Resume receive

( Old PC/X* and MC/Xi Send Bios Command, New to COM/Xi)

fct entry:  ah = 18h

al = 0, pause receive

= 1, resume receive

dx = any valid channel # (DX min - DX max)

returns:  ah = 0ffh fct error

ah = 0    fct good

------------------------------------------------------------------

function 19h  - Reserved

( Old PC/X* set/reset special character Interrupt )

------------------------------------------------------------------

function 1ah  - Reserved

( Old PC/X* get pointer to special character flag / counter )

--------------------------------------------------------------------

function 1bh  -  Get water marks / buffer size

( New to PC/X*, MC/Xi and COM/Xi DOS drivers )

fct entry : ah = 1bh

al = 0 - Get Transmit Low water mark.

= 1 - Get Receive Low water mark.

= 2 - Get Receive High water mark.

dx = any valid channel # (DX min - DX max)

returns   : ax = requested water mark.

bx = tx buffer size if al was 0

= rx buffer size if al was 1 or 2

zf = 1 and dh = 0ffh if error

Note : Transmit low water mark - The host may request an event (IRQ) when the

number of characters in the transmit buffer falls below this number.

: Receive low water mark - When receive is paused and the number

of characters in the receive buffer falls below this number,

receive is resumed.

: Receive high water mark - When receive is not paused and the

the number of characters in the receive buffer meets or exceeds

this value, transmit is paused.

: If zf = 1 then and only then will dh = 0ffh. Thus, you only need to

check zf OR dh to determine the success or failure of the call.

--------------------------------------------------------------------

function 1ch  -  Set water marks

( New to PC/X*, MC/Xi and COM/Xi DOS drivers )

fct entry : ah = 1ch

al = 0 - Set Transmit Low water mark.

= 1 - Set Receive Low water mark.

= 2 - Set Receive High water mark.

bx = water mark value.

dx = any valid channel # (DX min - DX max)

returns   : ah = 0 on sucess.

= 0ffh on error.

--------------------------------------------------------------------

function 1dh  -  Reserved

--------------------------------------------------------------------

function 1eh  -  Set flow control

( New to PC/X*, MC/Xi and COM/Xi DOS drivers )

fct entry : ah = 1eh

bh = S/W flow control

bit 0 = tx s/w flow control (XON/XOFF)

bit 1 = rx  "    "     "        "

bit 2 = set new XON/XOFF characters in CX

bl = H/W flow control

bit 0 = DTR rx flow control

bit 1 = RTS rx flow control

bit 4 = CTS tx flow control

bit 5 = DSR tx flow control

bit 7 = DCD tx flow control

ch = new XOFF character if BX bit 2 = 1

cl = new XON  character if BX bit 2 = 1

dx = any valid channel # (DX min - DX max)

returns   : ah = 0 on sucess.

= 0ffh on error.

--------------------------------------------------------------------

function 1fh  -  Enable / Disable Events

( New to PC/X*, MC/Xi and COM/Xi DOS drivers )

fct entry : ah = 1fh

al = Event mask

76543210

|||||||`--- Event on BREAK received.

||||||`---- Event on TX low water mark.

|||||`----- Event on TX buffer empty.

||||`------ Event on RX data present.

|||`------- (Not Used)

||`-------- Event on Modem Change Signal.

``--------- (Not Used)

bl = Modem Signal if al bit 5 = 1

76543210

||||````-- (Not Used)

|||`------ CTS

||`------- DSR

|`-------- RI

`--------- DCD

dx = any valid channel # (DX min - DX max)

returns   : ah = 0 on success

ah = ff if error

--------------------------------------------------------------------

EBIOS function 0f4h  -  Presence Test

( New to PC/X*, MC/Xi and COM/Xi DOS drivers )

fct entry : ah = 0f4h

dx = any valid channel # (DX min - DX max)

return    : ax = 0

--------------------------------------------------------------------

EBIOS function 0fbh  -  Specify modem leads

( New to PC/X*, MC/Xi and COM/Xi DOS drivers )

fct entry : ah = 0fbh

al = modem leads

bit 0 = set/reset DTR output

bit 1 = set/reset RTS output

bit 4 = use CTS for input flow control.

bit 5 = use DSR for input flow control.

bit 6 = RI not supported

bit 7 = use DCD for input flow control.

dx = any valid channel # (DX min - DX max)

Note : If RTS or DTR flow control is active, these lines will not be

affected until that flow control is deselected.

--------------------------------------------------------------------

EBIOS function 0fch  -  receive with no wait

( New to PC/X*, MC/Xi and COM/Xi DOS drivers )

fct entry : ah = 0fch

dx = any valid channel # (DX min - DX max)

return    : ah = line status

al = character

--------------------------------------------------------------------

EBIOS function 0fdh  -  retrieve buffer count

( New to PC/X*, MC/Xi and COM/Xi DOS drivers )

fct entry : ah = 0fdh

al = 1 - Get transmit buffer count.

= 2 - Get receive buffer count

dx = any valid channel # (DX min - DX max)

return    : cx = character count

This function returns the number of characters in the specified

buffer.

--------------------------------------------------------------------

EBIOS function 0feh  -  Set pacing mode

( New to PC/X*, MC/Xi and COM/Xi DOS drivers )

fct entry : ah = 0feh

al = 0   - no pacing.

= 40h - XON/XOFF for Receive.

= 80h - XON/XOFF for Transmit, no RX pacing

= C0h - XON/XOFF for Receive and Transmit.

dx = any valid channel # (DX min - DX max)

Note : CTS and DSR transmit pacing is always activated by a call to this

function to emulate BIOS pacing.  All other flow control (if any)

is disabled.

--------------------------------------------------------------------

EBIOS function 0ffh  -  Set buffered mode of operation

( New to PC/X*, MC/Xi and COM/Xi DOS drivers )

fct entry : ah    = 0ffh

al    = 1 - Transmit buffer.

= 2 - Receive buffer.

cx    = buffer size, 0 = terminate buffer mode.

dx 	  = any valid channel # (DX min - DX max)

es:bx = pointer to buffer.

This function really has no effect, since the intellegent boards

use on-board buffering and thus have no need for host buffering.

--------------------------------------------------------------------