DMPEG V1.1

Public Domain MPEG decoder

by Stefan Eckart

June 1993

1. Features

===========

DMPEG is another MPEG decoder/player for the PC:

- decodes (nearly) the full MPEG video standard

(I,P,B frames, frame size up to at least 352x288)

- can save decoded sequence in 8 or 24bit raw file for

fast off-line display (two pass mode)

- optional on-screen display during decoding

- several dithering options for 8 bit displays:

ordered dither, Floyd-Steinberg, grayscale

- selectable color-space

- runs under DOS, 640KB RAM, no MS-Windows or '386 required

- compact (small code / small data models, 16 bit arithmetic)

- supports VGA, many Super-VGAs (including VESA) and

some TrueColor SVGAs

2. Introduction

===============

DMPEG is a Public Domain decoder for files containing MPEG video

sequences. MPEG is a standardized format for compressed storage of video

(and audio), similar in concept to JPEG for individual images. It,

however, provides higher compression by exploiting the similarity of

consecutive images of a sequence, so called interframe coding. As with

JPEG, the compression factor depends on the source material and the

desired quality of the coded sequence. A compression to less than 5% of

the original material is not uncommon.

DMPEG supports the full MPEG video standard, except D-frame sequences.

Decoding is not very fast, therefore a two-pass mode was implemented:

during the first pass the sequence is decoded and stored in an unencoded

file which then can be played at maximum speed during the second pass.

Of course DMPEG also has a one-pass mode where the sequence is displayed

immediately but at lower speed. The speed is about one seventh of that

of the Xing MPEG decoder (see appendix A). The Xing player, however, can

only decode I frame sequences of size 160x120 and needs a '386. I

suggest to use DMEG in addition to Xing's MPEG.EXE 320x200 DOS player.

The latter is appropriate for real-time display of '160x120, I-frame

only' Xing files, DMPEG is more suitable for larger size 'real' MPEGs

containing I, P and B type, 352x288 sized frames or for faster and

better quality 'off-line' display of the 160x120 MPEGs. Try it on one of

the larger MPEG files (see appendix, my recommendation: flowg.mpg) to

see that MPEG can also be used (and is intended) for storing really

sharp, virtually noise-free sequences.

To give an impression of the attainable speeds, here are some figures

measured on a 386DX/33 with 4MB RAM and a Conner 3104 100 MB hard disk.

The decoding time for flowg.mpg (352x240) was about 4 sec per frame, 9

minutes for 150 frames. waterski.mpg (336x208 and higher compression)

required only 2.4 sec per frame. A decoded 150 frame raw data file is

12 MB long and can be displayed at 5.2 frames/s. 160x120 MPEGs reach

22 frames/s while the decoding speed is 1.9 sec/frame. The display frame

rate is limited by the transfer speed from hard disk to memory (about

550 kByte/s on my PC). If you have enough RAM, you can obtain much

higher speed by playing from a large RAM-disk (e.g 23 frames/s for a

352x240 sequence).

This program is Public Domain and I don't take any responsibility

regarding its fitness, usefulness etc. (#include

<your_favourite_disclaimer>). Comments, bug reports, questions to:

Stefan Eckart

Kagerstr. 4

D-81669 Muenchen

Germany

email: stefan@lis.e-technik.tu-muenchen.de

Any feedback is welcome.

3. Usage

========

dmpeg [options] input.mpg [output.raw]

dmpeg [options] input.raw

input.mpg   any MPEG DIS 11172 compliant non multiplexed compression

layer video stream (D frame streams unsupported)

output.raw  the decoded and dithered 8 or 24bit raw output file;

if omitted: output to screen only

input.raw   previously decoded raw file to be displayed at maximum speed

Options:

-q     quiet mode; no text output (except error messages)

-v     verbosity level; can be increased by repeating this option;

enables display of decoded information (start codes, frame size,

quantization scales etc.), can't be combined with the -s option

-dx    dithering options:

-d0    ordered, saturation dominant 4x4 dither (default)

-d1    Floyd-Steinberg error-diffusion / blue-noise-shaping (2 weights)

-d2    Floyd-Steinberg error-diffusion / blue-noise-shaping (4 weights)

-d3    undithered grayscale output

-d4    24 bit true color output

-p0    use full color range palette

-p1    use restricted color range palette (default)

-sn    select graphics adapter:

-s0    either standard VGA or Super-VGA in 320x200x256 mode (default)

(options -d0..3) or a TrueColor VGA with VESA BIOS in

320x200x16M mode (option -d4)

-s1    VESA BIOS, either 640x480x256 (-d0..3) or 640x480x16M (-d4)

-s2    ET4000               \

-s3    ET3000               |

-s4    Video 7              |

-s5    Paradise             | 640x480x256

-s6    Trident              | (options -d0..3 only)

-s7    Chips & Technologies |

-s8    ATI                  |

-s9    Orchid               |

-s10   Oaktech              /

-lx   delay (x=0..65535, default 0), used only for offline display

-b    use 8 bit transfer (default: 16 bit) into (S)VGA memory

Options can be combined. Example:

dmpeg -d2s0 flowg.mpg flowg.raw

decodes MPEG file flowg.mpg to raw file flowg.raw using FS4 dithering

and displays it on the screen in 320x200x256 mode.

dmpeg -vvv flowg.mpg flowg.raw

as above but using ordered dither and printing much information about

startcodes, headers and parameters.

All SVGA drivers use the 640x480x256 display mode and require at least

512 kB of video memory. If your graphics card is not listed, the best

you can do (besides trying all drivers in the hope that one of them

might work) is to obtain a VESA BIOS extension for your card and use

mode -s1. A collection of such drivers had been posted to

comp.binaries.ibm.pc and is also available at

ftp.rahul.net:/pub/bryanw/pc/vesadrv2.zip (anonymous ftp).

All drivers except the standard VGA, ET4000, the VESA driver in

conjunction with the mentioned public domain VESA BIOS extension TSR for

the ET4000 and the TrueColor driver with a Cirrus Logic VGA are

untested. I'm very interested in email feedback which drivers work and

which don't.

The -l option controls playback speed. It is implemented as a simple

delay loop without synchronization to a timer or vertical retrace.

The -b option is probably superfluous. I don't have much experience in

PC graphics programming and since I saw some example driver routines

using bytewise transfer (rep movsb) instead of wordwise (rep movsw) I

preferred to include this as a fudge factor. You should try it only in

case of problems. Speed is reduced considerably if you activate this

switch (at least for cards with 16 bit bus interface).

Decoding can be terminated by pressing any key.

Offline display can be controlled with the following keys:

any key except

space, return

or escape       halt display at current frame and step one frame

forward each time a key is pressed

space           step one frame backwards

return          continue display

escape          quit program

4. Technical information

========================

The player is a rather straightforward implementation of the MPEG spec

[1]. The IDCT is based on the Chen-Wang 11 multiplication algorithm [2]

which was coded in assembler but does not use tables for multiplication.

Blocks with not more than two non-zero coefficients use a non-separated

direct multiply-accumulate 2D-IDCT, which turned out to be faster than a

'fast' algorithm in this (quite common) case. Dithering is pretty

standard. Main difference to the Berkeley decoder (except for the fewer

number of supported algorithms) is the use of 256 instead of 128 colors,

the (default) option to use a restricted color-space and the

implementation of a color saturation dominant ordered dither. This leads

to a somewhat superior quality of the dithered image.

Restricted color-space means that the U and V components are clipped to

+/-0.25 (instead of +/-0.5) and the display color-space points are

distributed over this restricted space. Since the distance between

color-space points is thus reduced by a factor of two, the color

resolution is doubled at the expense of not being able to represent

fully saturated colors.

Saturation dominant ordered dither is a method by which a color, lying

somewhere between the points of the display color-space, is approximated

by primarily alternating between two points of constant hue instead of

constant saturation. This yields subjectivly better quality due to the

lower sensitivity of the human viewing system to saturation changes than

to hue changes (the same reasoning as used by the PAL TV standard to

improve on NTSC). The improvement is particularly visible in dark brown

or redish areas.

5. File formats

===============

If you want to write your own player or to post-process the results,

here is the format of the 8 bit raw file:

Byte

0..7      compatibility bytes (to be ignored)

8..9      image width  (MSB first)

10..11    image height (MSB first)

12..31    compatibility / unused

32..799   color table, R[0],G[0],B[0],  ... R[255],G[255],B[255]

800..     image data in natural order (top left to bottom right,

all frames concatenated without any gaps or repeated

headers)

The format of 24bit true color files (-d4 option) is:

0..7      compatibility bytes (to be ignored)

8..9      image width  (MSB first)

10..11    image height (MSB first)

12..31    compatibility / unused

32..      image data (3 bytes per pixel: B,G,R, top left to bottom

right, all frames concatenated without any gaps or repeated

headers)

The 8 bit format happens to be compatible with the raw format used by a

shareware program called Image Alchemy. In fact you can use that program

to view the first frame of the sequence and to convert it into other

formats.

6. Changes

==========

This is a list of major changes relative to version 1.0.

- speed improved by a factor of two through recoding of the IDCT and

interframe prediction routines in assembler and optimization of the

dithering algorithms

- support of TrueColor VGAs with VESA BIOS (only tested with Cirrus

Logic)

- DMPEG and DMPLAY merged into one program

- SVGA / TrueColor support for immediate display during decoding

- increased robustness against errors in the file to be decoded:

program tries to resynchronize on the next slice/frame header

instead of terminating

- format of the 24 bit raw file changed (B,G,R instead of R,G,B)

7. References

=============

1. Coding of moving pictures and associated audio for digital storage

media up to about 1,5 Mbit/s, Draft International Standard ISO/IEC

DIS 11172, 1992.

2. Chen, Wang, IEEE ASSP-32, pp. 803-816, Aug. 1984.

Appendix A: Related Software

============================

This list is probably incomplete, but it's all I'm aware of. Of course

there are programs for other systems as well (Mac, Amiga etc.).

cmpeg          an MPEG encoder for the PC (DOS, 640K, no '386 req.)

for Targa, PBMPLUS and Alchemy RAW images

Author: myself

mpeg_play      MPEG Video Software Decoder (Version 2.0; Jan 27, 1993)

Authors: Lawrence A. Rowe, Ketan Patel, and Brian Smith

Computer Science Division-EECS, Univ. of Calif. at

Berkeley

toe.cs.berkeley.edu:/pub/multimedia/mpeg/mpeg-2.0.tar.Z

mpgplay        Online port of mpeg_play for DOS

by: Giampero Caprino, scompx@milano.oas.olivetti.com

(evaluation version, works only with Xing files)

mpeg386.exe    Offline port of mpeg_play for DOS

by: Greg Ennis, 93gke@cs.williams.edu

(based on mpgplay, works only with Xing files)

mpegwin        Online port of mpeg_play for MS-Windows

by: Michael Simmons, msimmons@ecel.uwa.edu.au

toe.cs.berkeley.edu:/pub/multimedia/mpeg/Ports/mpegw*

(HiColor & TrueColor support, Shareware)

mplay.exe,

mpeg.exe       DOS MPEG players from Xing Technologies

(very high speed, but decodes only a small subset of the

MPEG standard)

MPEGv1.1/1.2alpha

MPEG Software Encoder/Decoder

Authors: Portable Video Research Group (PVRG)

havefun.stanford.edu:/pub/mpeg/MPEGv*.tar.Z

mpgcodec       PVRG encoder/decoder for PC compiled with GNU gcc

('386 required)

posted in alt.binaries.pictures.utilities

APPENDIX B: MPEG files

======================

Two good sources for MPEG files:

toe.cs.berkeley.edu:/pub/multimedia/mpeg/movies

havefun.stanford.edu:/pub/mpeg

High quality MPEGs you simply can't afford to miss:

tennis.mpg

flowg.mpg

bike.mpg

--

Stefan Eckart, stefan@lis.e-technik.tu-muenchen.de, June 1993.