ALR206T

PK-232 TO ALINCO ALR206T MIKE CIRCUIT

/-<- .1UF CAPACITOR

(AUDIO)              WHITE WIRE ----------][--------

---- CONNECT TO PIN 1 ON

RED WIRE   -----/\/\/\/--------      ALR 206T

(P.T.T.)                                 \-<- 33K OHM RESISTOR

(GND)

BROWN AND SHIELD WIRE------CONNECT TO PIN 7 ON ALR 206T

WHITE WIRE = AFSK AUDIO OUT

RED WIRE =   PTT

BROWN WIRE = GROUND

SHEILD =     GROUND

CAPACITOR = .1 UF @ 25WVDC

RESISTOR =   33K OHM @ 1/8 WATT

A SIMILAR CIRCUIT MAY WORK FOR OTHERS THAT DO NOT HAVE A SEPARATE PTT LINE

RELAYED BY...N0KGX...GENE