BEARCAT 200XLT CELLULAR FREQUENCY RESTORATION

AKNOWLEDGEMENTS

---------------

The following was extracted, and partially edited, from literature

received from:

MONITORING TIMES

140 Dog Branch Road

Brasstown, North Carolina  28902

If the reader desires to obtain the original literature, which comes

complete with an illustration of the circuit board and indication of

the resistor to be cut, a check for $2.00 along with a stamped self-

addressed envelope may be mailed to the above address.  The turn-

around for my literature was very fast (Less than 1 week after

mailing my check).  Although the directions presented here should be

quite adequate, the quick turn-around on the mail order literature

might provide the extra measure of visual security that some desire.

The documentation also includes a review of the BC200/205XLT scanner.

NOTES

-----

It is unlawful to monitor cellular telephone conversations.  It is

possible to monitor signals from the deleted ranges even without

conversion.  Simply add 21.7 MHZ to the deleted frequency and enter

the higher (image) frequency.  Reception is virtually identical in

strength to that which would be heard on the deleted frequency.

The frequencies deleted at the factory may be restored, but the

procedure should not be attempted by anyone unfamiliar with

electronic circuitry.  No one anywhere, anytime, in anyway, etc...

assumes any reponsibility for damage caused by this procedure.

THIS MODIFICATION WILL VOID YOUR WARRANTY!

TOOLS

-----

Small Philips Screwdriver

Small Wire Cutters

DISASSEMBLY

-----------

1.  Slide off the battery pack and remove your antenna.

2.   Remove the two screws from the back of the scanner, the two

screws which hold the battery retaining spring at the base, and

the battery retaining spring itself.

3.  Carefully pry the bottom of the rear cover from the radio and

remove the cover.

4.   Locate the two small screws at the base of the circuit board and

remove them.  Gently pull the front panel from the mainframe at

the base and seperate them.

MODIFICATION

------------

5.   On the face of the circuit board that faces the front of the

scanner when installed, locate the microprocessor IC labeled

"UNIDEN UC-1147".  Locate the 10K ohm resistor (brown, black,

orange), which is positioned approximately along the longitud

inal centerline of the board, and next to the microprocessor.

The resistor is of the leadless type and should be positioned

directly next to the microprocessor and above the "DEN" on the

IC label.

6.  Using the small wire cutters, cut the resistor body in two with

out disturbing anything next to it. If the left solder pad comes

loose, it may be peeled from the board.  Brush or blow away any

debris.  This completes the restoration.

REASSEMBLY

----------

7.   Insert the top of the front panel into the slot under the

volume/squelch control panel and, noting carefully the align

ment of the dual-inline connector at the bottom of the board,

press the front panel firmly into place.  Be sure that the

holes at the bottom of the circuit board line up with the

holes in the plastic standoffs below them.  Insert the two

screws and gently tighten them.

8.  Replace the back cover by inserting the top of the cover into

the slot under the volume/squelch control panel; press the cover

into place, insert and tighten the screws.

9.  Reposition the battery retaining spring (slotted side toward

notched hole), insert the two remaining screws and gently but

securely tighten them.

10  Slide the battery pack into place; switch the scanner on to make

sure the display comes on.  If not, the battery is discharged or

the dual-inline connector was misaligned during assembly (see

step 7).

CHECK OUT

---------

11.  Assuming the display comes on, press: MANUAL, 845.0, E; within

two seconds the frequency 845.000 should appear on the display.