IC901

OUT OF BAND RECEIVE MODS

FOR THE IC-901

To modify your IC-901 for out of band receive on each of the bands remove

the following diodes in the control head.

28 MHz		D24

50 MHz            D28

220 MHz            D19

430 MHz            D25

TO ENABLE CROSS BAND REPEAT

Flip S1 on the logic board (located in main unit at the very front)

Dial up the frequency you need on the main and sub band

Turn off the IC-901

Push and hold "Check" and "Lock"

Turn the power to the IC-901 on

To disable the cross band repeat function depress the "Lock" button