Model:              IC-R7000

Division:           Amateur

Note:          This service bulletin is meant for technical per-

sonnel with experience working on solid state com-

munications equipment.  Damage caused by im-

properly installing this modification may cause

ICOM to charge for subsequent repairs to the

product.  ICOM does not warrant this modification.

Bulletin #:    3Nov86-001

Date:          November 3, 1986

Subject:       Reduce intermodulation from FM stations (70-108

MHz) and TV stations (165-245 MHz)

Procedure

Write to ICOM's customer service department for a copy of

this service bulletin.

Bulletin #:    3Nov86-004

Date:          November 3, 1986

Subject:       The frequency displayed and actually received are

not the same

Procedure

1.  Check the location of C24 and W80 on the logic board

(compare against the schematic) to insure that they are in-

stalled in the correct position.

Bulletin #:    24287-001

Subject:       The S-meter may not drop to 0 when using the

receiver between 500 MHz and 800 MHz with no sig-

nal.

Procedure

1.  Install a 10mf 16V electrolytic capacitor between the emit-

ter of Q5 and ground on the Dc to DC Converter board.

Bulletin #:    24587-002

Subject:       Increase the number of memory channels to 200

Procedure

1.  Cut foil trace to IC-8 pin 19 on the logic board to remove

it from ground.

2.  Add a 47K resistor and between pin 19 and pin 24 of IC-8.

3.  Add a SPST switch from pin IC-8 pin 19 to ground.

Operation

1.  With the new switch closed access to memory channel 0 to 99

is possible.

2.  With the new switch open access to memory channels 100 to

199 is possible.

Note:  It is not possible to scan between the two banks of

memory.

Bulletin #:    35187-001

Subject:       Audio becomes distorted (clipped) when the radio

is warm.  The period of time required for this

problem to become evident varies depending on the

ambient temperature.  This problem may not be evi-

dent at low audio levels.

Procedure

Write to ICOM's customer service department for a copy of

this service bulletin.

Bulletin #:    35887-003

Subject:       The remote relay sometimes stays energized after a

signal is no longer present.  The radio must be

turned off and back on to clear this situation

before the modification.

Procedure

1.  Remove the covers from the radio and remove the main circuit

board on the side of the radio.

2.  Cut the trace between the base of Q5 and R106 on the foil

side of the main board.

3.  Install a RD6A diode (this part is critical and must be ob-

tained from ICOM) with its cathode toward R106 across the

cut in the trace.

4.  Reinstall the board and insure mechanical clearance of the

new diode.

5.  Verify proper operation of the recorder function.

Bulletin #:    11688-001

Date:          April 25, 1988

Subject:       To improve audio quality in SSB operation

Procedure

1.  Remove the top and bottom cover from the radio.

2.  Remove the PLL board from the radio.

3.  Change IC-1 from a MA78M08A to a MA78M08 (ICOM P/N 900-

00191) on the PLL board.

4.  De-Solder and remove the VCO from the PLL board.

5.  Change C6 (33 mf 10V electrolytic) to a 220 mf 10V

electrolytic (ICOM P/N 918-01477).  Install shrink tubing on

the capacitor leads.

6.  Change C17 (33 mf 10V electrolytic) to a 220 mf 10V

electrolytic (ICOM P/N 918-01477).  Install shrink tubing on

the capacitor leads.

7.  Reinstall the VCO unit on the PLL board.

8.  Install the PLL board in the radio.

9.  Replace the top and bottom cover.

Bulletin #:    13889-015

Date:          April 1, 1988

Subject:       If display fails to operate at power on

Procedure

1.  Remove the top and bottom cover from the radio.

2.  Remove the front panel sub-assembly from the main chassis of

the radio.

3.  Remove the DC to DC converter from the radio.

4.  Replace the following parts on the indicated boards.

Current                      New        ICOM

Designator Part     Board               Part       P/N

Q1 and Q2  2CS1214  DC to DC Converter  2SC2655    906-00385

R1 and R2  3.3 K    DC to DC Converter  5.16K      915-01173

C2 and C4           DC to DC Converter  .001mf 50V 918-01500

Q11 & Q12           Display Board       2SC2655    906-00385

R31 & R32 3.3 K     Display             5.16 K     915-01173

C13 & C16           Display             .001mf 50V 918-01500

5.  Reassemble the boards and front panel on the radio.

6.  Reinstall the top and bottom cover on the radio.

7.  Test