R7000.5

ADD A COOLING FAN TO YOUR ICOM R-7000 RECEIVER

by Brian Kantor, WB6CYT1

In my R-7000, adding a  fan  makes  a  big  difference.

I've  had  it  on  continuously for three days now, and

it's still nice and cool.  Turns out most of  the  heat

comes  from the transformer core in the AC supply.  One

of those 3-1/2" 12V DC fans fits just nicely -  a  pair

of 1/4" standoffs screwed through two of the upper back

panel ventilation slots holds it in  place,  the  power

leads  snake  through  the lower ventilation slots, and

some duct tape to form a gasket, and it pulls  the  hot

air  right out.  If you pick the 12V off the power sup-

ply regulator board at W3 (orange lead), it  will  even

work properly when you run the radio off 12 volts.

[In order to blow hot air out, the  fan  must  suck  in

cool  air  from  somewhere.  Users are advised to be on

guard for dust being drawn into fan-equipped R-7000s. -

Editor]