MX7000.MOD

THERE IS A POPULAR MX7000 MOD WHICH IS ADDING A RESISTOR IN PARALLEL WITH ONE

ON THE CPU BOARD TO INCREASE SCAN/SEARCH SPEED.

BY CHANGING C3 FROM 33pF TO 8pF YOU DON'T NEED TO DECREASE THE RESISTANCE AS

MUCH WHICH YIELDS LONGER MEMORY BACKUP.